| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18-24 Oct. 2014 | Document: JCT3V-J0075 |
| --- | --- |

| Title: | Cross-check results on Depth intra skip coding mode (JCT3V-J0038) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520, JAPAN | Tel: Email: | +81-43-299-8526 tsukuba.takeshi@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution reported cross-check results for JCT3V-J0038 on depth intra skip coding mode by Samsung. Experimental results under CTC and all-intra configuration exactly matched the results provided by the proponent.

# Introduction

JCT3V-J0038 [1] proposes a depth intra skip coding mode, called DISC, in which samples on a CU are reconstructed by utilizing one of the horizontal and vertical prediction modes (INTRA_ANGULAR10 or INTRA_ANGULAR26) without residuals.

The source code provided by the proponents was verified. Simulations were performed under CTC and all-intra configuration described in JCT3V-I1101.

# Results

Table 1 and Table 2 summarizes the results under CTC and all-intra configuration, respectively. It is observed that coding gains for synthesis are 0.25% and 0.13% for CTC and all-intra configuration, respectively. The results exactly matched those provided by the proponents.

Table 1: Result under CTC

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.11% | 0.06% | 0.00% | -0.11% | -0.14% | 102.4% | 101.3% | 102.2% |
| Kendo | 0.00% | -0.15% | 0.20% | 0.04% | -0.17% | -0.36% | 100.6% | 100.1% | 100.0% |
| Newspaper_CC | 0.00% | 0.11% | 0.14% | 0.02% | -0.17% | -0.20% | 96.2% | 97.1% | 96.8% |
| GT_Fly | 0.00% | 0.03% | 0.08% | 0.01% | -0.10% | -0.04% | 102.8% | 101.1% | 101.6% |
| Poznan_Hall2 | 0.00% | 0.06% | -0.03% | 0.00% | -0.75% | -1.05% | 98.8% | 98.5% | 98.4% |
| Poznan_Street | 0.00% | 0.13% | 0.07% | 0.03% | 0.01% | -0.01% | 102.6% | 101.4% | 103.4% |
| Undo_Dancer | 0.00% | -0.30% | -0.31% | -0.08% | -0.24% | -0.36% | 94.5% | 97.4% | 94.4% |
| Shark | 0.00% | 0.02% | 0.11% | 0.02% | 0.07% | 0.14% | 98.4% | 98.5% | 98.1% |
| 1024x768 | 0.00% | -0.05% | 0.13% | 0.02% | -0.15% | -0.23% | 99.7% | 99.5% | 99.7% |
| 1920x1088 | 0.00% | -0.01% | -0.02% | 0.00% | -0.20% | -0.26% | 99.4% | 99.4% | 99.2% |
| average | 0.00% | -0.03% | 0.04% | 0.01% | -0.18% | -0.25% | 99.5% | 99.4% | 99.4% |

Table 2: Result under all-intra configuration

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | 0.00% | 0.00% | 0.00% | -0.18% | -0.12% | 97.7% | 100.5% | 100.1% |
| Kendo | 0.00% | 0.00% | 0.00% | 0.00% | -0.28% | -0.30% | 95.6% | 97.6% | 98.1% |
| Newspaper_CC | 0.00% | 0.00% | 0.00% | 0.00% | -0.17% | -0.11% | 102.7% | 104.2% | 103.9% |
| GT_Fly | 0.00% | 0.00% | 0.00% | 0.00% | -0.15% | 0.06% | 100.7% | 101.2% | 101.9% |
| Poznan_Hall2 | 0.00% | 0.00% | 0.00% | 0.00% | -0.80% | -0.64% | 94.4% | 99.0% | 101.8% |
| Poznan_Street | 0.00% | 0.00% | 0.00% | 0.00% | -0.12% | 0.00% | 99.2% | 100.9% | 100.4% |
| Undo_Dancer | 0.00% | 0.00% | 0.00% | 0.00% | -0.26% | -0.19% | 89.2% | 96.0% | 96.1% |
| Shark | 0.00% | 0.00% | 0.00% | 0.00% | 0.06% | 0.27% | 98.9% | 100.4% | 98.5% |
| 1024x768 | 0.00% | 0.00% | 0.00% | 0.00% | -0.21% | -0.17% | 98.7% | 100.8% | 100.7% |
| 1920x1088 | 0.00% | 0.00% | 0.00% | 0.00% | -0.25% | -0.10% | 96.4% | 99.5% | 99.7% |
| average | 0.00% | 0.00% | 0.00% | 0.00% | -0.24% | -0.13% | 97.3% | 100.0% | 100.1% |

# Conclusion

The source code and WD provided by Samsung were confirmed. Cross-check results exactly matched those provided by the proponent.

References

[1] J. Y. Lee, M. W. Park, C. Kim “Depth intra skip coding (DISC) mode “, JCT3V-J0038, Oct. 2014