| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18-24 Oct. 2014 | Document: JCT3V-J0077 |
| --- | --- |

| Title: | Cross-check on ARP, IC and DBBP Flags Signaling for 3D-HEVC (JCT3V-J0037) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520, JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution reports cross-check the result for JCT3V-J0037 item 1 and item 2 on ARP conditions. Experimental results exactly match with the results provided by the proponent.

# Introduction

[item1]

JCT3V-J0037 item 1 adds one condition to check at least one inter-view picture should be available in reference picture list when signaling iv_res_pred_weight_idx.

- The variable RpRefPicAvailFlag is set equal to ( RpRefPicAvailFlagL0  | |  RpRefPicAvailFlagL1 ) && DefaultRefViewIdxAvailableFlag.

[item2]

JCT3V-J0037 item 2 removes the restriction of ARP in the case of DISP_DEFAULT where Neighbour DV is not available but Default DV is available.  JCT3V-J0041 also addresses the issue with almost same way.

The cross-checker agrees that there seems no reason ARP should be disabled when a disparity vector is available and it is not clear why DISP_DEFAULT should be introduced.

| Availability of Neighbour DV | Availability of Default DV | HTM120 | JCT3V-J0037 item 2 [1] | JCT3V-J0041 [2 |
| --- | --- | --- | --- | --- |
| YES | (YES) | DISP_AVAILABLE (ARP on) | DISP_AVAILABLE (ARP on) | DISP_AVAILABLE (ARP on) |
| NO | YES | DISP_DEFAULT (ARP off) |  |  |
| NO | NO | DISP_NONE (ARP off) | DISP_NONE (ARP off) | DISP_NONE (ARP off) |

# Results

Test1 [item1] ARP condition including DefaultRefViewIdxAvailableFlag)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.0% | 101.4% | 99.1% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.6% | 103.1% | 100.1% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.4% | 101.2% | 99.8% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.1% | 99.0% | 99.6% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.2% | 98.7% | 101.6% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 99.9% | 101.0% | 100.0% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.1% | 100.5% | 100.2% |
| Shark | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.1% | 99.3% | 100.0% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.3% | 101.9% | 99.6% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.1% | 99.7% | 100.3% |
| average | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.2% | 100.5% | 100.0% |

Test 2 [item2] remove ARP condition on DISP_DEFAULT

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.1% | -0.1% | -0.01% | -0.01% | 0.05% | 100.2% | 99.0% | 99.6% |
| Kendo | 0.0% | -0.1% | -0.1% | 0.00% | -0.01% | -0.05% | 100.4% | 100.1% | 100.1% |
| Newspaper_CC | 0.0% | 0.0% | -0.1% | -0.02% | -0.02% | -0.06% | 100.3% | 102.9% | 99.5% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.3% | 98.7% | 99.2% |
| Poznan_Hall2 | 0.0% | -0.1% | 0.1% | 0.00% | -0.01% | 0.06% | 100.3% | 98.4% | 100.6% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | -0.01% | -0.01% | -0.03% | 100.1% | 101.5% | 100.0% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.2% | 100.7% | 99.7% |
| Shark | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | -0.01% | 100.0% | 100.1% | 99.5% |
| 1024x768 | 0.0% | 0.0% | -0.1% | -0.01% | -0.01% | -0.02% | 100.3% | 100.7% | 99.8% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.01% | 100.2% | 99.9% | 99.8% |
| average | 0.0% | 0.0% | 0.0% | -0.01% | -0.01% | 0.00% | 100.2% | 100.2% | 99.8% |

# References

[1] M. W. Park, J. Y. Lee, B. Choi, C. Kim, “ARP, IC and DBBP Flags Signaling for 3D-HEVC (JCT3V-J0037) “, JCT3V-J0037, JCT-3V 10th Meeting: Strasbourg, FR, 18-24 Oct. 2014

[2] S. Yoo, S. Yea, “Simplification of an NBDV availability check “, JCT3V-J0041, JCT-3V 10th Meeting: Strasbourg, FR, 18-24 Oct. 2014

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).