| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18-24 Oct. 2014 | Document: JCT3V-J0083 |
| --- | --- |

| Title: | Cross-check on Motion buffer reduction for depth (JCT3V-J0055) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520, JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution reports cross-check results in JCT3V-J0055 on motion buffer reduction for depth. Experimental results exactly match with the results provided by the proponent.

# Introduction

JCT3V-J0055 reduces temporal (inter-view) motion buffer for depth view. It is noted that motion buffer for texture view is NOT modified.

This contribution reports the results of case 3, where motion buffer for depth view is decimated by 64x64 unit.

|  | Texture | Depth |
| --- | --- | --- |
| HEVC | 16x16 | - |
| HTM12 | 8x8 => 16x16 | 8x8 => 16x16 |
| JCT3V-J0055 case 3 |  | 64x64 |

# Results

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.1% | 0.0% | 0.00% | 0.02% | 0.15% | 100.8% | 102.1% | 99.9% |
| Kendo | 0.0% | -0.1% | 0.0% | 0.00% | 0.04% | 0.11% | 101.3% | 100.1% | 101.1% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | -0.01% | 0.01% | -0.03% | 101.6% | 99.6% | 101.0% |
| GT_Fly | 0.0% | 0.0% | -0.1% | -0.01% | 0.01% | 0.02% | 100.3% | 98.0% | 99.6% |
| Poznan_Hall2 | 0.0% | 0.1% | 0.0% | 0.01% | 0.02% | 0.11% | 99.9% | 97.9% | 100.9% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.00% | 0.03% | 0.05% | 100.1% | 100.2% | 99.9% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.00% | 0.04% | 0.12% | 100.4% | 99.4% | 100.2% |
| Shark | 0.0% | -0.1% | 0.0% | 0.00% | 0.04% | 0.11% | 100.2% | 99.3% | 99.4% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.00% | 0.02% | 0.08% | 101.2% | 100.6% | 100.7% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.00% | 0.03% | 0.08% | 100.2% | 99.0% | 100.0% |
| average | 0.0% | 0.0% | 0.0% | 0.00% | 0.03% | 0.08% | 100.6% | 99.6% | 100.3% |

# References

[1] G. Bang, Y. S. Heo, W. W. Gwun, G. H. Park, G. S. Lee, N. H. Hur, “Motion buffer reduction for depth “, JCT3V-J0055, JCT-3V 10th Meeting: Strasbourg, FR, 18-24 Oct. 2014

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).