| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18-24 Oct. 2014 | Document: JCT3V-J0092 |
| --- | --- |

| Title: | Cross-check on Removal of redundant VSP candidates in Merge mode (JCT3V-J0039) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520, JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution reports cross-check results for JCT3V-J0039 on VSP inheritance and VSP candidate position. Experimental results exactly match with the results provided by the proponent.

# Introduction

JCT3V-J0039 addresses the VSP inheritance’s redundancy with refined VSP candidate position. There are two options called test 1 and test 2.

- test 1 removes VSP inheritance with modified VSP candidate position

- test 2 utilises A1 only VSP inheritance with modified VSP candidate position

|  | VSP inheritance | VSP position |
| --- | --- | --- |
| HTM12 | A1 B1 B0 A0 B2 | T D IvMC A1 B1 B0 IvDC VSP A0 B2 |
| JCT3V-J0039 test 1 | - (No inheritance) | T D IvMC A1 B1 VSP B0 IvDC A0 B2 |
| JCT3V-J0039 test 2 | A1 (only) | T D IvMC A1 B1 VSP B0 IvDC A0 B2 |

# Results

Test1 Remove VSP inheritance with modified VSP candidate position

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | 0.1% | 0.01% | 0.02% | 0.04% | 100.6% | 103.4% | 99.4% |
| Kendo | 0.0% | -0.1% | -0.1% | -0.02% | -0.03% | -0.03% | 100.7% | 100.6% | 99.6% |
| Newspaper_CC | 0.0% | 0.1% | 0.0% | 0.02% | 0.00% | -0.11% | 100.9% | 103.9% | 99.5% |
| GT_Fly | 0.0% | -0.3% | 0.0% | -0.04% | -0.05% | -0.04% | 100.2% | 99.7% | 100.0% |
| Poznan_Hall2 | 0.0% | 0.2% | 0.4% | 0.08% | 0.10% | 0.16% | 100.3% | 99.2% | 101.1% |
| Poznan_Street | 0.0% | 0.1% | 0.0% | 0.02% | 0.02% | 0.00% | 100.2% | 100.1% | 100.1% |
| Undo_Dancer | 0.0% | 0.1% | 0.1% | 0.00% | -0.01% | 0.05% | 100.4% | 99.9% | 100.0% |
| Shark | 0.0% | 0.0% | -0.3% | -0.04% | -0.04% | -0.03% | 100.2% | 99.2% | 98.6% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | -0.03% | 100.7% | 102.6% | 99.5% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.03% | 100.2% | 99.6% | 100.0% |
| average | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.01% | 100.4% | 100.7% | 99.8% |

Test1 A1 only VSP inheritance with modified VSP candidate position

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | 0.0% | -0.02% | -0.02% | -0.03% | 100.6% | 100.8% | 99.8% |
| Kendo | 0.0% | -0.1% | -0.1% | -0.02% | -0.02% | -0.01% | 100.9% | 97.8% | 99.9% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | -0.01% | -0.04% | -0.08% | 100.8% | 99.8% | 102.3% |
| GT_Fly | 0.0% | -0.6% | -0.5% | -0.13% | -0.14% | -0.14% | 100.5% | 99.2% | 99.6% |
| Poznan_Hall2 | 0.0% | 0.1% | 0.2% | 0.04% | 0.04% | 0.03% | 100.2% | 98.5% | 100.2% |
| Poznan_Street | 0.0% | -0.1% | -0.2% | -0.03% | -0.02% | -0.04% | 100.2% | 102.4% | 100.0% |
| Undo_Dancer | 0.0% | -0.5% | -0.5% | -0.14% | -0.13% | -0.11% | 100.1% | 100.0% | 99.8% |
| Shark | 0.0% | -0.2% | -0.4% | -0.07% | -0.06% | -0.07% | 100.4% | 100.3% | 99.4% |
| 1024x768 | 0.0% | -0.1% | 0.0% | -0.02% | -0.02% | -0.04% | 100.7% | 99.4% | 100.7% |
| 1920x1088 | 0.0% | -0.3% | -0.3% | -0.07% | -0.06% | -0.06% | 100.3% | 100.1% | 99.8% |
| average | 0.0% | -0.2% | -0.2% | -0.05% | -0.05% | -0.06% | 100.4% | 99.8% | 100.1% |

# References

[1] M. W. Park, J. Y. Lee, C. Kim, “Removal of redundant VSP candidates in Merge mode“, JCT3V-J0039, JCT-3V 10th Meeting: Strasbourg, FR, 18-24 Oct. 2014

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).