| Joint Collaborative Team on 3D Video Coding Extension of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18–24 Oct. 2014 | Document: JCT3V-J0108 |
| --- | --- |

| Title: | Centralized Texture-Depth Packing (CTDP) SEI Message Syntax (JCTVC-S0031) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jar-Ferr Yang Ke-Ying Liao Ya-Han Hu Hung-Ming Wang | Tel: Email: | +886-6-2757575#62362 jefyang@mail.ncku.edu.tw |
| Source: | National Cheng Kung University (NCKU), 1 University Road, Tainan, Taiwan 701 |  |  |

_____________________________

# Abstract

This contribution proposes a new Centralized Texture Depth Packing (CTDP) SEI Message to cover a series of the CTDP formats to represent 3D videos with texture and depth information efficiently. With the arrange the texture in the center of the frame and the proposed colored depth concept, the most important feature of the CTDP formats with better coding performances is that they could be directly viewed in 2DTV displays without any extra computation. Besides, associated with the HEVC coding system (HM13.0), the CTDP formats show better performances in both texture and depth coding and virtual view rendering results compared to the texture-and-depth SbS packing method. Before 3D-HEVC chips have been delivered to all the receivers, the newly-proposed Centralized Texture Depth Packing (CTDP) SEI Message including a series of the CTDP formats could simply help to delivery of 3D video services embedded with 2D display viewable capability through the existing HEVC and AVC video coding standards.

# Introduction

Currently, several well-known 2D frame compatible side-by-side, top-and-bottom, and checkerboard stereo packing formats, which could convey left and right views in one frame, allow the broadcasters to deliver 3D television programs in one channel. However, the existing frame compatible stereo packing formats have two problems: 1) The stereo packing formats produce seriously annoying effects while showing in the 2D displays; 2) They cannot directly support naked-eyes multiview 3D displays with a reasonable cost unless the the receivers implement real-time stereo matching and multiple depth image-based rendering algorithms; It is noted the stereo 3D TV sets only occupy a small fraction of TV sets. From the broadcaster point of view, the stereo packing 3D videos only can serve this small number of 3D TV users. Not viewable for the 2D TVs, the 3D video services with stereo packing formats will be very difficult to attract commercial advertisements. Besides, the naked-eyes 3D displays, which will be a preferable 3D solution, need additional depth information to generate multiple views.

To generate multi-view content, a possible transmission format of 3DTV broadcasting systems [1] could be the 2D color texture with additional transmission of gray depth information. Hereafter, the color texture and the gray depth are just simply called the texture and the depth, respectively. With the texture-plus-depth packing format [2], [3], the 3D displays can properly show autostereoscopic multi-view contents if the depth-image-based rendering (DIBR) technique is built in. However, texture-plus-depth packing format with the arrangement of texture and depth in the SbS fashion will produce even worse visualization for 2D displays than the stereo packing ones. The issues of the bandwidth limitation, frame compatibility and comfortable visualization in 2D displays still need to be solved for the 3D broadcasters. To solve the viewable problem of the packing formats, we have suggested several centralized texture-depth packing (CTDP) formats associated with objective and subjective quality measurements in MPEG and AVS standard meetings [4]-[8]. In HEVC and AVC standards, however, there is still no SEI Message, which can properly support the CTDP packing formats. Therefore, in this contribution, we propose the syntax of the CTDP SEI Message such that we properly introduce the CTDP formats into MPEG HEVC and AVC video standards. In Section 2, we first review the packing and depacking procedures of the CTDP format in details. The coding performances of the CTDP formats comparing to that of texture-and-depth SbS packing format are exhibited in Section 3. In Section 4, we then propose the syntax of CTDP SEI Message to effectively include all the CTDP formats and their future variations.

# Proposed frame compatible CTDP formats

Fig. 1 shows the flowchart of for the computation of the texture-3/4 CTDP format [4], we could downscale the vertical resolution of texture and depth frames into three fourth of the original resolution. Here, we use the common bilinear method to resize the texture and depth frames. Since only Y channel of the depth pixel is utilized, three depth pixels can be rearranged into one colored-depth pixel, which is with R, G, and B subpixels. The colored-depth frame in subpixels color representation can reduce the original depth into one third of the depth resolution and the colored depth frame can be treated as a portion of packed texture frame. As shown in Fig. 2, nine lines of the depth frame are arranged into three color lines of the colored-depth frame in the vertical direction. Thus, the vertical resolution of the colored-depth frame becomes one third of the depth frame. In other words, the depth frame after 3/4 downscaling and colored subpixels arrangement in the vertical direction, the resized-and-colored depth frame becomes one fourth of the original resolution in the vertical direction. In stead of YCrCb color space, arranging three depth pixels into RGB color space will keep the colored depth frame close to gray color, which its Cr and Cb component are close to zero, since nearby depth values are almost the same.

Fig. 1. Flowchart for computating the proposed texture-3/4 CTDP format

Fig. 2. Procedure of rearranging the depth frame into RGB colored depth frame vertically

After RGB subpixels arrangement, we can convert (4, 4, 4) RGB color space into (4, 4, 4) YCbCr color space by,

YCbCr=0.25680.50410.0979−0.1482−0.29100.43920.4392−0.3678−0.0714RGB+16128128.

Thus, the RGB colored depth frame becomes the YCbCr colored depth frame with only one fourth of the original vertical resolution. It is noted that the YCbCr colored depth frame in (4, 4, 4) format could be further converted to (4, 2, 2) or (4, 2, 0) format according to the setups of HEVC and AVC coders.

Then, we split the resizeed-and-colored depth frame evenly into two separated parts with one eighth resolution and flip them vertically. The splitting and flipping parts are designed for better line-up of resized-and-colored depth frame to the texture frame to achieve better coding performance and better visualization in 2D displays. Finally, we obtain the packing frame by combining the first half of reduced-and-colored-and-flipped depth frame, resized texture frame, and the other half of reduced-and-colored-and-flipped depth frame from top to bottom sequentially.

The CTDP depacking procedure consists of simple reverse processes of CTDP packing procedure. The flow diagram for depacking the texture-3/4 CTDP format is shown in Fig. 3 [4]. After 4/3 upscaling depth frame in the vertical direction, we could optionally perform the depth enhancement by a bilateral or trilateral filter to enhance the depacked depth frame.

Fig. 3. Flow chart of the proposed Texture-3/4 CTDP depacking procedure

For the extension of the texture-3/4 CTDP format, we can set different resizing factors for texture and depth frame. For examples, to obtain the texture-7/8 and texture-15/16 CTDP formats [6],  the texture frame sizes are downscaled to 7/8 and 15/16 while the depth frame sizes should be downscaled to 3/8 and 3/16 in the vertical direction, respectively. After the color subpixels rearrangements, the horizontal sizes of the reduced-and-colored depth frames will become 1/8 and 1/16 sizes, respectively. After the combination of the first half of reduced depth frame, resized texture frame, and the other half of reduced depth frame from top to bottom, Fig. 4 exhibits the outlooks of the packed frames for texture-3/4, texture-7/8, and texture-15/16 CTDP formats.

It is noted that the reduced depth could be arranged in left and right side, while the downsizing of texture and depth frames and reduction of depth frame are all down in horizontally.

Fig. 4. Packed frames for texture-3/4, texture-7/8, and texture-15/16 CTDP formats

Fig. 5 shows the usage of the proposed frame-compatible texture-3/4 CTDP format, which could be adopted in any TV broadcasting networks associated with paired HEVC or AVC video encoder and decoder. Without any supporting hardware, the traditional 2DTV can directly exhibit their 2D programs. For 2DTV users, the texture-15/16 will show the best visualization performance while 3/4-texture CTDP format will have a slightly worse visualization. Even in texture-3/4 CTDP format, the subjective test shows that the CTDP format outperforms the other well-known SbS and TaB stereopacking formats in 2D visualization [7].

Fig. 5. The scope of the proposed CTDP formats used in TV broadcasting systems

# Performance Measurements

To verify the coding performances in HEVC coders, the proposed CTDP formats are compared to texture and depth SbS packing format [2]. The texture-depth SbS and texture-3/4, 7/8, and 15/16 CTDP formats are shown in Fig. 6. We adopted five 3D video sequences, which are Poznan Hall (1920x1088), Poznan Street (1920x1088), Kendo (1024x768), Balloons (1024x768), and Newspaper (1024x768) to conduct the objective measurements. Besides, the packed formats are executed on HEVC HM13.0, whose compression quality parameters are 22、27、32、37.

Texture-3/4 CCDP

Texture-7/8 CCDP

Texture-15/16 CCDP

Texture-depth SbS

Fig. 6. Schematics of texture and depth packaging formats used in simulations

Tables 1 and 2 respectively show the performances of the averaged BDPSNR in dB and the averaged BDBR in % under three different testing conditions as All Intra (ai), Low Delay (ldp), and Random Access (ra). For comparisons, the coding performances of texture and depth information are separately presented. We calculate the PSNR of reconstructed texture and depth frames to the original view and original depth ones, respectively. To calculate BDPSNR and BDBR, with respect to texture-depth SbS format, are exhibited where the bitstreams are the consuming bits to restore reconstructed the texture and depth frames, respectively.

In order to compare the view synthesized performances, the depacked texture and depth with corresponding camera parameters are used to generate a new viewpoint image through the depth image based rendering (DIBR), where we fill the holes by directly extending nearby background.

Table 1. Average BDPSNR (dB) results for different sequences

Table 2. Average BDBR (%) results for different sequences

To verify our CTDP coding capability, we also compare the coding performances of texture-3/4, 7/8, and 15/16 CTDP formats coded by HEVC HM13.0 to the coding performances of 1-view and 1-depth coded by 3D-HEVC HTM8.0. The experiments executed on HM13.0 are with compression quality parameters are 22, 27, 32, and 37 while those executed on HTM8.0 are with compression quality parameters are 25, 30, 35, and 40.

Tables 3 and 4 respectively show the performances of the averaged BDPSNR in dB and the averaged BDBR in % under Random Access (ra) condition. For the view synthesis, 3D-HEVC HTM8.0 uses VSRS to generate the virtual view, while HEVC HM 13.0 with CTDP formats utilizes the depth image based rendering (DIBR), which fills the hole areas by directly extending nearby background. We found that the coding performances of generated virtual view from the CTDP formats coded by HEVC HM13.0 are slightly worse than those of generated virtual from the view and depth coded by 3D-HEVC HTM8.0. It is noted that we did not perform any depth enhancement method in the enlisted simulation results for all simulations.

Table 3. Average BDPSNR (dB) results for different sequences

Table 4. Average BDBR (%) results for different sequences

# Centralized_texture-depth_packing SEI message Syntax

In order to properly signal the CTDP formats and their setups associated with HEVC and AVC coders, we propose a new centralized_texture-depth_packing (CTDP) SEI message as:

centralized_texture-depth_packing (CTDP) SEI Message:

| centralized_texture-depth_packing( payloadSize ) { | Descriptor |
| --- | --- |
| centralized_texture-depth_packing_id | ue(v) |
| centralized_texture-depth_packing_cancel_flag | u(1) |
| if( ! centralized_texture-depth_packing_cancel_flag ) { |  |
| centralized_texture-depth_packing_type | u(7) |
| centralized_texture-depth_packing_content_interpretation_type | u(6) |
| depth_spatial_flipping_flag | u(1) |
| depth_subpixel_arrangement_type | u(4) |
| texture_samping_type | u(8) |
| depth_samping_type | u(8) |
| centralized_texture-depth_packing_reserved_byte | u(8) |
| centralized_texture-depth_packing_persistence_flag | u(1) |
| } |  |
| } |  |

This SEI message informs the decoder that the output cropped decoded picture contains samples of multiple distinct spatially packed constituent frames that are packed into one frame using a centralized texture-depth packing arrangement. This information can be used by the decoder to appropriately rearrange the samples and process the samples of the constituent frames appropriately for display or other purposes (which are outside the scope of this Specification).

This SEI message may be associated with pictures that are either frames (when field_seq_flag is equal to 0) or fields (when field_seq_flag is equal to 1). The centralized texture-depth packing of the samples is specified in terms of the sampling structure of a frame in order to define a texture-depth packing arrangement structure that is invariant with respect to whether a picture is a single field of such a packed frame or is a complete packed frame.

When general_non_packed_constraint_flag is equal to 1 for a CVS, there shall be no centralized texture-depth packing arrangement SEI messages in the CVS.

centralized_texture-depth_packing_id contains an identifying number that may be used to identify the usage of the centralized texture-depth packing SEI message. The value of centralized_texture-depth_packing_id shall be in the range of 0 to 232 − 2, inclusive.

Values of centralized_texture-depth_packing_id from 0 to 255 and from 512 to 231 − 1 may be used as determined by the application. Values of centralized_texture-depth_packing_id from 256 to 511 and from 231 to 232 − 2 are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all centralized texture-depth packing arrangement SEI messages containing a value of centralized texture-depth_packing_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, and bitstreams shall not contain such values.

centralized_texture-depth_packing_cancel_flag equal to 1 indicates that the centralized texture-depth packing arrangement SEI message cancels the persistence of any previous the centralized texture-depth packing in output order. centralized_texture-depth_packing_cancel_flag equal to 0 indicates that indicates that centralized texture-depth packing follows.

centralized_texture-depth_packing_type indicates the type of packing arrangement of the frames as specified in Table X-.

Table X-1 – Definition of centralized_texture_depth_packing _type

| Value | Interpretation |
| --- | --- |
| 4 | Texture and depth plane of the decoded frames contains a texture-3/4 CTDP arrangement of corresponding planes of texture and depth frames as illustrated in Figure X-1. |
| 6 | Texture and depth plane of the decoded frames contains a texture-5/6 CTDP arrangement of corresponding planes of texture and depth frames as illustrated in Figure X-2. |
| 8 | Texture and depth plane of the decoded frames contains a texture-7/8 CTDP arrangement of corresponding planes of texture and depth frames as illustrated in Figure X-3. |
| 12 | Texture and depth plane of the decoded frames contains a texture-11/12 CTDP arrangement of corresponding planes of texture and depth frames as illustrated in Figure X-4. |
| 16 | Texture and depth plane of the decoded frames contains a texture-15/16 CTDP arrangement of corresponding planes of texture and depth frames as illustrated in Figure X-5. |

Figure X-1 Frame arrangement of texture-3/4 CTDP format

Figure X-2 Frame arrangement of texture-5/6 CTDP format

Figure X-3 Frame arrangement of texture-7/8 CTDP format

Figure X-4 Frame arrangement of texture-11/12 CTDP format

Figure X-5 Frame arrangement of texture-15/16 CTDP format

centralized_texture-depth_packing_content_interpretation_type indicates the intended interpretation of the constituent frames as specified in Table X-2. Values of centralized_texture-depth_packing_content_interpretation_type  that do not appear in Table X-2 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore centralized texture-depth packing SEI messages that contain reserved values of centralized_texture-depth_packing_content_interpretation_type.

For centralized_texture_depth_packing_type equal to 4, 6, 8, 12 or 16 and centralized_texture-depth_packing_content_interpretation_type equal to 1 or 2, texture and depth frames that are arranged by the specific order.

Table X-2 – Definition of centralized_texture-depth_packing_content_interpretation_type

| Value | Interpretation |
| --- | --- |
| 0 | Unspecified relationship between the centralized texture-depth packed constituent frames |
| 1 | Indicates that the packed frame is arranged in order of reduced_depth_1, resized_texture, and reduced_depth_2 from top to bottom |
| 2 | Indicates that the packed frame is arranged in order of reduced_depth_1, resized_texture, and reduced_depth_2 from left to right |

NOTE 1 – The value 2 for centralized_texture-depth_packing_content_interpretation_type is not expected to be prevalent in industry use of this SEI message. However, the value was specified herein for purposes of completeness

When centralized_texture-depth_packing_type is not equal to 4, 6, 8, 12, or 16, it is a requirement of bitstream conformance that centralized_texture-depth_packing_content_interpretation_type shall be equal to 0.

When centralized_texture-depth_packing_type is not equal to 4, 6, 8, 12, or 16, the value 1 or 2 for centralized_texture-depth_packing_content_interpretation_type is reserved for future use by ITU-T | ISO/IEC. When centralized_texture-depth_packing_type is not equal to 4, 6, 8, 12, or 16, decoders shall ignore the value 1 or 2 for centralized_texture-depth_packing_content_interpretation_type.

depth_spatial_flipping_flag equal to 1, when centralized_texture-depth_packing_type is not equal to 4, 6, 8, 12, or 16 and centralized_texture-depth_packing_content_interpretation_type equal 1 or 2, indicates that halves of the reduced and colored depth frame is spatially flipped relative to its intended orientation for display or other such purposes.

When centralized_texture-depth_packing_type is equal to 4, 6, 8, 12, or 16, centralized_texture-depth_packing_content_interpretation_type equal 1 or 2, and spatial_flipping_flag is equal to 1, the type of depth spatial flipping that is indicated is as follows:

–	If centralized_texture-depth_packing_content_interpretation_type is equal to 1, the indicated depth spatial flipping is vertical flipping.

–	If centralized_texture-depth_packing_content_interpretation_type is equal to 2, the indicated depth spatial flipping is horizontal flipping.

–	Otherwise (depth_spatial_flipping_flag is equal to 0), the indicated spatial flipping is not necessary.

When centralized_texture-depth_packing_type is not equal to 4, 6, 8, 12, or 16, it is a requirement of bitstream conformance that spatial_flipping_flag shall be equal to 0. When centralized_texture-depth_packing_type is not equal to 4, 6, 8, 12, or 16, the value 1 or 2 for depth_spatial_flipping_flag is reserved for future use by ITU-T | ISO/IEC. When centralized_texture-depth_packing_type is not equal to 4, 6, 8, 12, or 16, decoders shall ignore the value 1 or 2 for depth_spatial_flipping_flag.

depth_subpixel_arrangement_type equal to 1 indicates that embedding three Y values of the depth frame into R, G, and B channels of the colored depth frame is used a line-offset rotation method as specified in Figure X-6. depth_subpixel_arrangement_flag equal to 0 indicates that embedding three Y values of the depth frame into R, G, and B channels in the colored depth frame is used by R, G, and, B in the sequence order as specified in Figure X-6.

texture_sampling_type indicates that the method of the texture sampling is used by an encoder to indicate to a decoder as specified in Table X-3.

Figure X-6, Colored depth subpixel arrangement for depth_subpixel_arrangement_type equal to 1 or 0

Table X-3 – Definition of texture_sampling_type

| Value | Interpretation |
| --- | --- |
| 0 | Encoder using bilinear method to downscale the vertical or horizontal resolution of texture frame |

NOTE 2–When centralized_texture-depth_packing_content_interpretation_type equal to 1, the value 0 for texture_sampling_type, the texture frame is down scale the vertical resolution by using bilinear method. When centralized_texture-depth_packing_content_interpretation_type equal to 2, the value 0 for texture_sampling_type, the texture frame is down scaled in the horizontal resolution by using bilinear method.

Values of texture_sampling_type that do not appear in Table X.3 are reserved for future specification by ITU-T |ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore centralized texture-depth packing SEI messages that contain reserved values of texture_sampling_type.

depth_sampling_type indicates that the method of the depth sampling is used by an encoder to indicate to a decoder as specified in Table X-4.

Table X-4 – Definition of depth_sampling_type

| Value | Interpretation |
| --- | --- |
| 0 | Encoder using bilinear method to downscale the vertical or horizontal resolution of depth frame |

NOTE 3–When centralized_texture-depth_packing_content_interpretation_type equal to 1, the value 0 for depth_sampling_type, the depth frame is down scale the vertical resolution by using bilinear method. When centralized_texture-depth_packing_content_interpretation_type equal to 2, the value 0 for depth_sampling_type, the depth frame is down scaled in the horizontal resolution by using bilinear method.

Values of depth_sampling_type that do not appear in Table X.4 are reserved for future specification by ITU-T |ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore centralized texture-depth packing SEI messages that contain reserved values of depth_sampling_type.

centralized_texture-depth_packing_reserved_byte is reserved for future use by ITU-T | ISO/IEC. It is a requirement of bitstream conformance that the value of centralized_texture-depth_packing_reserved_byte shall be equal to 0. All other values of centralized_texture-depth_packing_reserved_byte are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of centralized_texture-depth_packing_reserved_byte.

centralized_texture_depth_packing_persistence_flag specifies the persistence of the centralized texture-depth packing arrangement SEI message.

centralized_texture-depth_packing_persistence_flag equal to 0 specifies that the centralized texture-depth packing arrangement SEI message applies to the current decoded frame only.

centralized_texture-depth_packing_persistence_flag equal to 1 specifies that the centralized texture-depth packing arrangement SEI message persists in output order until any of the following conditions are true:

–	A new CVS begins.

–	The bitstream ends.

–	A frame in an access unit containing a centralized texture-depth packing SEI message with the same value of centralized_texture-depth_packing_id is output having PicOrderCntVal greater than PicOrderCnt( CurrPic ).

# Conclusions

In this contribution, the objective measurements of the proposed texture-3/4, texture-7/8 and texture-15/16 centralized texture-depth packing (CTDP) formats are comparing to those of the frame compatible texture-and-depth (SbS) packing format. To include the texture-3/4, texture-5/6, texture-7/8, texture-11/12, and texture-15/16 CTDP formats and their settings in the AVC and HEVC standards, we propose a new Centralized Texture Depth Packing SEI message syntax. The proposed CTDP SEI Message could properly include all CTDP formats and their future variaations, which could help to attain better image quality in the receivers compared to the texture-and-depth SbS packing format for both 2D-TV and 3D-TV displays. Before 3D-HEVC decoders could be delivered to all the receivers, the CTDP format combining with HEVC or AVC coding system would be one of the best selections for providing 3D video services in the current broadcasting systems.

# References

[1]	Digital Video Broadcasting (DVB); Plano-stereoscopic 3DTV; ETSI TS 101 547 V1.1.1

[2]	Philips 3D Solutions: 3D Interface Specifications White Paper

[3]	A. Redert, R.-P. Berretty, C. Varekamp, O. Willemsen1, J. Swillens, H. Driessen, "Philips 3D Solutions: From Content Creation to Visualization", Third International Symposium on 3D Data Processing, Visualization, and Transmission (3DPVT'06).

[4] J.-F. Yang, H.-M. Wang and, A.-T. Chiang, “2D Backwards Compatible Centralized Color-Depth Packing,”, Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, the 6th Meeting: Document: JCT3V-F0087, Geneva, November 2013

[5]

J.-F. Yang, H.-M. Wang, Y.-A. Chiang and K. Y. Liao, “2D Frame Compatible Centralized Color Depth Packing Format,” AVS-M3225, AVS 47th Meeting, Dec. 2013, Beijing, China. (Translation from Chinese)

[6] J.-F. Yang, H.-M. Wang, Y.-A. Chiang and K.-Y. Liao, “Performance Evaluation of Different 2D Frame Compatible Centralized Color Depth Packing Formats,” AVS-M3302, AVS 48th Meeting, Apr. 2014, Shenzhen, China. (Translation from Chinese)

[7] J.-F. Yang, H.-M. Wang, Y.-A. Chiang and K.-Y. Liao, “Objective and Subjective Tests of Different 2D Frame Compatible Centralized Color Depth Packing Formats,” AVS-M3375, AVS 49th Meeting, June 2014, Dalian, China. (Translation from Chinese)

[8] J.-F. Yang, H.-M. Wang, K.-Y. Liao, and Y.-A. Chiang, “AVS2 Syntax Message for 2D Frame Compatible Centralized Color Depth Packing Formats,” AVS-M3472, AVS 50th Meeting, Oct. 2014, Nanjing, China (Translation from Chinese)

[9]	B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, Y.-K. Wang, T. Wiegand, High Efficiency Video Coding (HEVC) text specification draft 10 (for FDIS & Consent), Document no. JCTVC-L1003, Jan. 2013.

# Patent rights declaration(s)

National Cheng Kung University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).