| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18–24 Oct. 2014 | Document: JCT3V-J0111 |
| --- | --- |

| Title: | BoG on HEVC Conformance for 3D extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Ying Chen 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | cheny@qti.qualcomm.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

This document provides a report of the BoG on conformance tests for HEVC 3D extensions, including MV-HEVC and 3D-HEVC, from 2:30 pm to 4:15 pm. Detail procedure on conformance bitstream generation and validation, as well as several open issues on the current specifications for MV-HEVC conformance and 3D-HEVC conformance has been discussed.

# Introduction

The BoG thanks the volunteers for providing the conformance bitstreams for MV-HEVC and 3D-HEVC. Especially those who have provided the initial version of the assigned bitstreams. The companies that have provided the 3D-HEVC bitstreams are Sharp, Qualcomm, and MediaTek. NTT has have provided the assigned MV-HEVC bitstreams.

The BoG further confirmed that most of the missing conformance bitstreams will be provided by the assigned companies by the time of next JCT-3V meeting.

# Bitstream verification procedure

## Configuration files

For now, we don’t use common configuration files to generation bitstreams. It is encouraged that some configuration files can be cross checked and even shared among the volunteers in a later stage of the conformance test.

## Files to be uploaded

### Naming

A bitstream is contained in one zip achieve, as a package. The zip file should be named with the company name, such as MVHEVCS_H_LGE_5.

There should be no revision number or company name for files within each package.

The name of the YUV file for each layer and the bitstream should contain bitstream name as defined in the specification with minimal suffix, wherein the number (as in the suffix) indicates the layer id. An example is provided as follows. A zip file of 3DHC_DT_B_NTT_X.zip ( wherein the “X” is the revision number of the conformance bitstream set) contains the following files:

3DHC_DT_B_0.yuv

3DHC_DT_B_1.yuv

3DHC_DT_B_2.yuv

3DHC_DT_B_3.yuv

3DHC_DT_B_4.yuv

3DHC_DT_B_5.yuv

3DHC_DT_B.bit

3DHC_DT_B.txt

3DHC_DT_B.md5

### Md5 and other information associated with the bitstreams

Currently, it was suggested to upload the bitstream, short description, and md5 checksum in a zip archive. All files inside the zip archive should have the same base name. Only the extension is changed in the following way:

.bit - bitstream

.txt - description

.md5 - MD5 sum of the complete decoded file

As shown in one example bitstream provided by Sharp, one md5 file is provided. It contains the following

Md5 sum for each of the views (layers)

Md5 sum for the whole bitstream

It is discussed that whether md5 for each picture is needed. It is concluded that per picture md5 is not additionally needed. However, the decoded picture as hash SEI message containing the md5 information for each picture may be used.

The decoded picture as hash SEI message is a picture level SEI message.

A question has been raised whether the final conformance bitstreams need to include this SEI message.

It is suggested that we can follow the convention of HEVC version 1 conformance. Some HEVC version 1 conformance bitstreams contain this SEI message.

It was  discussed wether such SEI message should be present in the bitstream, it seems that it is helpful and we could encourage containing such SEI in our bitstreams, knowing that the implementation of the SEI is not mandatary for the conforming decoders.

The BoG also discussed whether we want to have additional metadata as per picture or access unit information. The JCT-3V experts in the BoG don’t think we need such metadata.

It has been discussed whether the spreadsheet to summarize the information of each bitstream may be provided. Such information includes the values of each syntax elements in the sequence level. The BoG suggested not to follow the same practice, which seems to have been discontinued for HEVC version 1 conformance.

### Cross checking of the bitstreams

For each category, a responsible person is assigned. The responsible person coordinates the cross checking (with 2 or 3 persons) of each bitstream of that category.

3D-HEVC

Texture tool: T. Ikai (Sharp)

Depth tool: J. -L. Lin (MediaTek), X. Zheng (HiSilicon)

Depth dependent texture tool: S. Shimizu (NTT)

Texture dependent depth tool: H. Liu (Qualcomm)

Others: J. Nam. (LG)

MV-HEVC

Prediction Structure (2-view): T. Suzuki (Sony)

Prediction Structure (3-view): Y. Chen (Qualcomm)

Hybrid scalability: Nokia may be interested

# Bitstream preparation status

## Work plan

Start a MPEG request document for the MV-HEVC conformance draft.

We further discussed when to have such bitstreams. If we plan to have the DAM (of MV-HEVC) by Feb. 2015, all the bitstreams need to be available before the next meeting.

It is also suggested that the MV-HEVC and 3D-HEVC may be covered within one specification.

Therefore a work plan, as listed below, has been discussed and agreed.

Reference software HTM 13 available date:				2014/11/12/

Bitstream available date based on HTM 12.1:						2014/11/12

Cross check of the bitstreams:					2014/11/26

Generation of the bitstreams based on HTM 13.0:		2014/11/27

Uploading of the final bitstream packages:			2014/12/03

People are encouraged to proceed faster for each step as mentioned above. The dates given are deallines. Discussions should occur in the JCT-3V mailing list if there is any delay compared with the deadlines.

For 3D-HEVC, the HTM version 12.1 will be used to generate the conformance bitstreams.

For MV-HEVC, the HTM version 12.1 is to be used.

The above work plan applies to both MV-HEVC conformance test and 3D-HEVC conformance test.

### 3D-HEVC

10 bitstreams out of 25 have been provided. Among the remaining bitstreams, we conformed the volunteers would like to provide them before the next JCT-3V meeting.

In the provided column, “X” indicates the first version of the bitstream has been provided; “Y” indicates that it was conformed that the bitstream will be provided; “?” indicates that either a volunteer is not assigned or the volunteer didn’t conform that the bitstream can be generated.

| Categories | Sub category | Bitstream | File name | Profile and Level | Provided |
| --- | --- | --- | --- | --- | --- |
| Texture tool | ARP | 3DHC_T_A | 3DHC_T_A_Qualcomm_0 | 5.1 and higher | X |
|  | Sub-PU inter-view motion prediction | 3DHC_T_B | 3DHC_T_B_MediaTek_0 | 5.1 and higher | X |
|  | Illumination compensation | 3DHC_T_C | 3DHC_T_C_Sharp_0 | 5.1 and higher | X |
|  | Combined | 3DHC_T_D | 3DHC_T_D_Sharp_0 | 5.1 and higher | X |
|  | Combined texture only bitstream | 3DHC_T_E | 3DHC_T_E_HHI_0 | 5.1 and higher | Y |
| Depth tool | Depth Intra (DMM1) | 3DHC_D1_A | 3DHC_D1_A_HHI_0 | 5.1 and higher | Y |
|  | Depth Intra (DMM1) | 3DHC_D1_B | 3DHC_D1_B_HHI_0 | 5.1 and higher | Y |
|  | Depth Intra (SDC) | 3DHC_D1_C | 3DHC_D1_C_RWTH_0 | 5.1 and higher | Y |
|  | Depth Intra (SDC) | 3DHC_D1_D | 3DHC_D1_D_RWTH_0 | 5.1 and higher | Y |
|  | Depth Intra (single depth) | 3DHC_D1_E | 3DHC_D1_E_MediaTek_0 | 5.1 and higher | X |
|  | Depth Intra (single depth) | 3DHC_D1_F | 3DHC_D1_F_MediaTek_0 | 5.1 and higher | X |
|  | Depth Intra (combined) | 3DHC_D1_G | 3DHC_D1_G_RWTH_0 | 5.1 and higher | Y |
|  | Depth Intra (combined) | 3DHC_D1_H | 3DHC_D1_H_RWTH_0 | 5.1 and higher | Y |
| Depth tool | Depth Inter (inter-view motion) | 3DHC_D2_A | 3DHC_D2_A_Samsung_0 | 5.1 and higher | Y |
|  | Depth Inter (inter-SDC) | 3DHC_D2_B | 3DHC_D2_B_LGE_0 | 5.1 and higher | Y |
| Depth dependent texture tool | DoNBDV | 3DHC_DT_A | 3DHC_DT_A_MediaTek_0 | 5.1 and higher | X |
|  | VSP | 3DHC_DT_B | 3DHC_DT_B_NTT_0 | 5.1 and higher | Y |
|  | DBBP | 3DHC_DT_C | 3DHC_DT_C_Hisilicon_0 | 5.1 and higher | X |
|  | Combined | 3DHC_DT_D | 3DHC_DT_D_NTT_0 | 5.1 and higher | Y |
| Texture dependent depth tool | Sub-PU MPI | 3DHC_TD_A | 3DHC_TD_A_Qualcomm_0 | 5.1 and higher | X |
|  | MPI | 3DHC_TD_B | 3DHC_TD_B_MediaTek_0 | 5.1 and higher | X |
|  | QTL | 3DHC_TD_D | 3DHC_TD_D_Orange_0 | 5.1 and higher | Y |
| Others | 3-view random access | 3DHC_C_A | 3DHC_C_A_HHI_0 | 5.1 and higher | Y |
|  | 3-view all Intra | 3DHC_C_B | 3DHC_C_B_HHI_0 | 5.1 and higher | Y |
|  | 2-view random access | 3DHC_C_C | 3DHC_C_C_Sharp_0 | 5.1 and higher | X |

### MV-HEVC

| Categories | Sub category | Bitstream | File name | Provided |
| --- | --- | --- | --- | --- |
| Prediction Structure (2-view) | Inter-view prediction | MVHEVCS-A | MVHEVCS_A_Qualcomm_0 | Y |
|  | All Intra | MVHEVCS-B | MVHEVCS_B_Sharp_0 | Y |
|  | Simulcast | MVHEVCS-C | MVHEVCS_C_Sony_0 | Y |
|  | Simulcast with asymmetric resolutions | MVHEVCS-D | MVHEVCS_D_NTT_0 | X |
|  | Inter-view prediction and hierarchical B | MVHEVCS-E | MVHEVCS_E_Qualcomm_0 | Y |
|  | Inter-view prediction for IRAP AUs only | MVHEVCS-F | MVHEVCS_F_Qualcomm_0 | Y |
| Prediction Structure (3-view) | Inter-view prediction | MVHEVCS-G | MVHEVCS_G_NTT_0 | Y |
|  | Inter-view prediction with PIP view structure | MVHEVCS-H | MVHEVCS_H_LGE_0 | Y |
|  | Inter-view prediction with IBP view structure | MVHEVCS-I | MVHEVCS_I_Nokia_0 | Y |
| Hybrid scalability |  | MVHEVCS-J | MVHEVCS_J_XXX_5 | ?? |

The layer and output mechanisms might be further tested in MV-HEVC conformance bitstream.

Test the case the number of views is up to 63 (with/or maybe without) gap of layer id.

# Open issues of the current MV-HEVC draft and 3D-HEVC draft

We would like to highlight the major issues for the current specifications.

Missing one volunteer for the hybrid scalability bitstream.

Compatibility of the 3D-HEVC needs to be discussed, especially for the Main 10 and Multiview Main Profiles.

It is agreed that capability to the main 10 for the base view (of 3D-HEVC) is not needed.

In addition, 3D-HEVC should be compatiable to Multiview Main.