| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18–24 Oct. 2014 | Document: JCT3V-J0112 |
| --- | --- |

| Title: | Combination of JCT3V-J0025 and JCT3V-J0035 for DMM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai3, Xianguo Zhang1, Kai Zhang1, Jicheng An1, Han Huang1, Jian-Liang Lin2, Takeshi Tsukuba3, Shawmin Lei2 1MediaTek Beijing: Building 1-B, No. 6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 31-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520, JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp Shawmin Lei +886-3-5670766 ext. 25555 {xianguo.zhang, kai.zhang, jicheng.an, h.huang, jl.lin, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. and SHARP Corporation |  |  |

_____________________________

# Abstract

For the DMM1 intra prediction, it requires 48.3 k bytes to store the lookup table in the current design. In this contribution, a combination of JCT3V-J0025 and JCT3V-J0035 is proposed. It is firstly proposed to downsample the wedgelet patterns for DMM1 to reduce the lookup table size as JCT3V-J0035. Secondly, it is proposed to simplify the DMM table derivation process by rotating as JCT3V-J0025. The combination claims to achieve 22.6 k bytes (uncompressed) and 3.6 k bytes (compressed), while HTM12 table is reportedly compressed to 16.3 k bytes with run-length compression.

Experimental results reportedly show that the proposed method’s BD-rate change is 0.00 %, -0.02 %, 0.04 % in texture, video and synthesis respectively in CTC. And the BD-rate change is 0.00%, -0.01%, 0.00% in AI.

# Introduction

In order to reduce the size for the DMM1 lookup table in current 3D-HEVC [1], while accessing the wedgelet patterns, intra 32x32 PU re-uses the lookup table of 16x16 PU to obtain the starting and ending point of the wedgelet segmentation [4]. However, even with this simplification, the lookup table information for the current DMM1’s wedgelet patterns still requires about 400k bits storage as listed in Table 1.

Table 1. Number of bits for wedgelet pattern at different block size in the current HTM12

| PU size | Number of wedgelet pattern | Bins for decoding the index | Number of bits for one wedgelet pattern | Total Table Size(bits) | Accumulated Table Size (bits) | bits ratio [%] |
| --- | --- | --- | --- | --- | --- | --- |
| 4×4 | 86 | 7 | 16 | 1,376 | 1,376 | 0.35%=1376/396000 |
| 8×8 | 766 | 10 | 64 | 46,024 | 50,400 | 11.62%=46024/396000 |
| >=16×16 | 1,350 | 11 | 256 | 345,600 | 396,000 | 87.27%=345600/396000 |

Besides, the DMM table consists of six different orientations, in the range of wedgeOri =0...5. Currently each table has to be generated with a line segment process and orientation specific filling and sub-sampling process. However, since the table with the different orientation has the similar structure, simple rotation is sufficient to produce. With this, we can remove orientation specific filling process, sub-sampling process and pruning process in non rotating tables.

# Proposed method

Component 1:

We propose to downsample the lookup table size for PU>=16x16. The proposed simplification is illustrated as Fig. 1 and described as follows:

- For the wedgelet patterns with starting point and ending point located in adjacent edges: only the ones with even-position (relative to the corresponding corner point) starting and ending points are kept.

- For the wedgelet patterns with starting point and ending point located in opposite edges: with even-x-or-y-coordinate starting points are kept.

Figure 1. Two categories of wedgelet segmentations

Component 2:

Considering the above table-derivation problems, we further propose to modify the table generation by

- reusing generated tables by rotation

- one filling

- one sub-sampling

- reduced pruning

# Table Size Analysis

Table 2, 3 and 4 present the table size analysis of the proposed method.

Table 2. Size of proposed DMM1 table (uncompressed)

| PU size | Number of wedgelet pattern | Bins for decoding the index | Number of bits for one wedgelet pattern | Total Table Size[bits] | Accumulated Table Size [bits] | Total [Kbytes] |
| --- | --- | --- | --- | --- | --- | --- |
| 4×4 | 108 | 7 | 16 | 1728 | 1728 | 22.6 |
| 8×8 | 824 | 10 | 64 | 52736 | 17,504 |  |
| >=16×16 | 510 | 9 | 256 | 130560 | 185024 |  |

Table 3. Size of proposed DMM1 table (compressed)

| PU size | Number of wedgelet pattern | Bins for decoding the index | Number of bits for one wedgelet pattern | Total Table Size[bits] | Accumulated Table Size [bits] | Total [Kbytes] |
| --- | --- | --- | --- | --- | --- | --- |
| 4×4 | 30 | 7 | 12 | 360 | 360 | 3.6 |
| 8×8 | 266 | 10 | 32 | 8512 | 8872 |  |
| >=16×16 | 255 | 9 | 80 | 20400 | 29272 |  |

Table 4. Table size comparison

|  | short description | uncompressed [kbytes] | Compressed [kbytes] |
| --- | --- | --- | --- |
| HTM12.0 | Anchor | 48.3 | 16.3*1 |
| J0025+J0035 | rotate+reduce16x16 | 22.6 | 3.6*2 |

*1: run-length compression for each line is used. The table size decreases MxM bit to (1+log2(M)) x M bits (i.e. 8x8 table can be represented by 4*8 = 32 bits)

*2: Only not-rotating table is stored with run-length compression.

# Complexity

Table 5 shows the additional operations produced by run-length compression and our proposed method.

Table 5. Additional operations comparison

|  |  | ADD/CMP | TBL |
| --- | --- | --- | --- |
| HTM12 | Uncompress | 0 | 0 |
| HTM12 | Compress | MxN+2 | 0 |
| J0025+J0035 | Uncompress | 0 | 0 |
| J0025+J0035 | Compress | MxN+M+2 | 0 |

As shown bellow, Run-length(HTM12,Compress) and Run-length+rotate(J0025+J0035, Compress) can be carried out with the same way, so the complexity of Run-length + rotate is almost the same as that of Run-length.

- Run-length MxN Add/Substitution+ 2 comparison

for (y = 0; y < N; y++)

{

Pel = compressedPattern[y][0]

Run = compressedPattern[y][1]

for (x = 0; x < Run == 0 ? M - 1: Run; x++)

Pattern[x][y] = Pel

Pel = !Pel

for (;x < M - 1; x++)

Pattern[x][y] = Pel

}

- Run-length+Rotate MxN+M Add/Substitution+ 2 comparison

for (x = M - 1; y >= 0; y--)

{

xx = M – 1 - x

Pel = compressedPattern[xx][0]

Run = compressedPattern[xx][1]

for (y = 0; y < Run == 0 ? N - 1 : Run; y++)

Pattern[x][y] = Pel

Pel = !Pel

for (;y < N - 1; y++)

Pattern[x][y] = Pel

}

# Experimental results

The proposed simplification is conducted based on HTM-12.0 [2]. The simulation results under the common test conditions (CTC) [3] and all intra cases are shown in Table 6 and Table 7, respectively. As shown in the results, the simplification only introduces 0.04% BD-rate loss for CTC and no loss for all intra cases.

Table 6.  The results under all intra test conditions

Table 7.  The results under CTC

# Text specification changes

Table Error: Reference source not found-1 –Values of wedgeFullTabIdxBits[ log2PUSize ]

| Initialization variable | wedgeFullTabIdxBits |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| log2PbSize | 2 | 3 | 4 | 5 | 6 |
| Value | 7 | 10 | 119 | 119 | 13 |

- I.8.4.4.2.11 Specification of tables WedgePatternTable

NOTE 1 − Tables and values resulting from the processes specified in the following are independent of any information contained in the bitstream. Therefore the derivation process described in this subclause can be carried out once as part of the initialization of the decoding process. Alternatively, the tables and values can be stored within the decoder (read-only) memory as fixed lookup tables, such that the derivation process described in this subclause does not need to be implemented in the decoder at all.

The list WedgePatternTable[ log2BlkSize ] of binary partition patterns of size (1  <<  log2BlkSize )x(1  <<  log2BlkSize), the variable NumWedgePattern[ log2BlkSize ] specifying the number of binary partition patterns in list WedgePatternTable[ log2BlkSize ] are derived as specified in the following:

- For log2BlkSize ranging from 2 to 4, inclusive, the following applies:

  - The variable resShift is set equal to ( log2BlkSize  = =  4 ) ? 0 : 1.

  - The variable wBlkSize is set equal to ( 1  <<  ( log2BlkSize + resShift ) )

  - The patternSize is set equal to ( 1  <<  log2BlkSize )

  - The variables posWedgePattern is set equal to 0,

  - For wedgeOri in the range of 0 to 5, inclusive, the following ordered steps apply.

    - Depending on wedgeOri the variables xPosS, yPosS, xPosE, yPosE, xIncS, yIncS, xIncE, yIncE are derived as specified in Table Error: Reference source not found−2 − Specification of xPosS, yPosS, xPosE, yPosE, xIncS, yIncS, xIncE, yIncE.

- Table Error: Reference source not found−2 − Specification of xPosS, yPosS, xPosE, yPosE, xIncS, yIncS, xIncE, yIncE

| wedgeOri | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| xPosS | 0 | wBlkSize − 1 | wBlkSize − 1 | 0 | 0 | wBlkSize − 1 |
| yPosS | 0 | 0 | wBlkSize − 1 | wBlkSize − 1 | 0 | 0 |
| xPosE | 0 | wBlkSize − 1 | wBlkSize− 1 | 0 | 0 | 0 |
| yPosE | 0 | 0 | wBlkSize − 1 | wBlkSize − 1 | wBlkSize − 1 | 0 |
| xIncS | 1 | 0 | −1 | 0 | 1 | 0 |
| yIncS | 0 | 1 | 0 | −1 | 0 | 1 |
| xIncE | 0 | −1 | 0 | 1 | 1 | 0 |
| yIncE | 1 | 0 | −1 | 0 | 0 | 1 |

    - The variables startPos is set equal to posWedgePattern

    - If wedgeOri is equal to 0 or 4, the following applies:

      - sizeScaleS = 1+ (log2BlkSize > 3 ) ;

      - sizeScaleE = wedgeOri<4 ?1+ (log2BlkSize > 3 ) : 1;

      - For m in the range of 0 to wBlkSize/ sizeScaleS − 1, inclusive, the following applies:

        - For n in the range of 0 to wBlkSize/ sizeScaleE − 1, inclusive,  the following applies:

          - xS = m* sizeScaleS

          - yS = 0

          - xE = (wedgeOri==0) ? 0 : n* sizeScaleE

          - yE = (wedgeOri==0) ? n* sizeScaleE : wBlkSize − sizeScaleE

          - The Wedgelet pattern generation process as specified in subclause  is invoked with patternSize being equal to ( 1  <<  log2BlkSize ), the variable resShift, variable wedgeOri, xS, yS, xE, yExS being equal to ( xPosS + m * xIncS ), yS being equal to 0 ( yPosS + m * yIncS ), xE being equal to ( xPosE + n * xIncE ) and yE being equal to ( yPosE + n * yIncE ) as inputs, and the output is the binary array curWedgePattern.

          - The wedgelet pattern list insertion process as specified in subclause  is invoked with log2BlkSize, and the binary partition pattern curWedgePattern, wedgeOri and startPos as inputs, output is the variable posWedgePattern

      - The variables endPos is set equal to posWedgePattern - 1

    - Otherwise (wedgeOri is equal to 1..3 or wedgeOri is equal to 5), the following applies:

      - For pos in the range of startPos to endPos, inclusive, the following applies:

        - The Wedgelet pattern rotation process as specified in subclause  is invoked with the variable patternSize, wedgeOri, and the binary partition pattern WedgePatternTable[ log2BlkSize ][ pos ] as inputs, and the output is the binary array curWedgePattern.

        - The wedgelet pattern list insertion process as specified in subclause  is invoked with log2BlkSize, and the binary partition pattern curWedgePattern and wedgeOri as inputs, output is the variable posWedgePattern

- The wedgelet pattern list insertion process as specified in subclause  is invoked with log2BlkSize, and the binary partition pattern curWedgePattern as inputs.NumWedgePattern[ 5 ] is set equal to NumWedgePattern[ 4 ].

- For k = 0..NumWedgePattern[ 5 ] − 1 the following applies:

  - For x, y =0..( 1  <<  5 ) − 1 the following applies:

    - WedgePatternTable[ 5 ][ k ][ x ][ y ] = WedgePatternTable[ 4 ][ k ][ x  >>  1 ][ y  >> 1 ]

NOTE 1 − WedgePatternTable[ 5 ] might be derived on-the-fly in the decoding process from WedgePatternTable[ 4 ].

- I.8.4.4.2.11.1 Wedgelet pattern generation process

Inputs to this process are:

- a variable patternSize specifying the binary partition pattern size,

- a resolution shift value resShift specifying the precision of the wedgelet partition start and end positions relative to patternSize,

- a variable wedgeOri specifying the orientation identifier of the wedgelet pattern,

- a variable xS specifying the partition line start horizontal position,

- a variable yS specifying the partition line start vertical position,

- a variable xE specifying the partition line end horizontal position,

- a variable yE specifying the partition line end vertical position.

Output of this process is:

- binary array wedgePattern[ x ][ y ] of size ( patternSize )x( patternSize ).

- The variable curSize specifying the size of the current partition pattern is derived as follows:.

  - curSize = ( resShift  = =  1 ) ? ( patternSize  <<  1 ) : patternSize	(Error: Reference source not found-1)

- The values of variable wedgePatterncurPattern[ x ][ y ], are derived as specified by the following ordered steps.

  - For x, y = 0.. patternSizecurSize − 1, wedgePatterncurPattern[ x ][ y ] is set equal to 0.

  - The samples of the array wedgePatterncurPattern that form a line between ( xS, yS ) and ( xE, yE ) are set equal to 1 as specified in the following:

x0 = xS
		y0 = yS
		x1 = xE
		y1 = yE
		if( abs( yE − yS ) > abs( xE − xS ) ) {
			( x0, y0 ) = Swap( x0, y0 )
			( x1, y1 ) = Swap( x1, y1 )
		}
		if( x0 > x1 ) {
			( x0, x1 ) = Swap( x0, x1 )
			( y0, y1 ) = Swap( y0, y1 )
		}
		sumErr = 0
		posY = y0
		for( posX = x0; posX  <=  x1; posX ++ ) {
			if( abs( yE − yS ) > abs( xE − xS ) )  
				wedgePatterncurPattern[ posY  >> resShift ][ posX  >> resShift ] = 1 
			else 
				wedgePatterncurPattern[ posX >> resShift ][ posY   >> resShift ] = 1 
			sumErr  +=  ( abs( y1 − y0 )  <<  1 )
			if( sumErr  >=  ( x1 − x0 ) ) {
				posY  +=  ( y0 < y1 ) ? 1 : −1
				sumErr  −=  ( x1 − x0 )  <<  1
			}
		}

  - The samples of wedgePatterncurPattern belonging to the a smaller partition are set equal to 1 as specified in the following:

if( wedgeOri  = =  0 ) 
			for( iX = 0; iX < xS; iX ++ )  
				for( iY = 0; curPattern[ iX ][ iY ]  = =  0; iY++ )  
					curPattern[ iX ][ iY ] = 1
		else if( wedgeOri  = =  1 ) 
			for( iY = 0; iY < yS; iY++ )  
				for( iX = curSize − 1; curPattern[ iX ][ iY ]  = =  0 ; iX− − )
					curPattern[ iX ][ iY ] = 1
		else if( wedgeOri  = =  2 ) 
			for( iX = curSize − 1; iX > xS; iX− − ) 
				for( iY = curSize − 1; curPattern[ iX ][ iY ]  = =  0 ; iY− −)
					curPattern[ iX ][ iY ] = 1
		else if( wedgeOri  = =  3 ) 
			for( iY = curSize − 1; iY > yS; iY− − )  
				for( iX = 0; curPattern[ iX ][ iY ]  = =  0 ; iX++) 
					curPattern[ iX ][ iY ] = 1
		else if( wedgeOri  = =  4 ) && ( ( xS + xE ) < curSize ) ) 
			for( iY = 0; iY < curSize; iY++ ) 
				for( iX = 0; curPattern[ iX ][ iY ]  = = 0  ; iX++) 
					curPattern[ iX ][ iY ] = 1
		else if( wedgeOri  = =  4 )
		 	for( iY = 0; iY < curSize; iY++ )  
				for( iX = curSize − 1; curPattern[ iX ][ iY ]  = =  0 ; iX− −) 
					curPattern[ iX ][ iY ] = 1
		else if( wedgeOri  = =  5 )  &&  ( ( yS + yE ) < curSize ) )
			for( iX = 0; iX < curSize; iX ++ )  
				for( iY = 0; curPattern[ iX ][ iY ] = = 0 ; iY++ )
					curPattern[ iX ][ iY ] = 1
		else if( wedgeOri  = =  5 ) 
			for( iX = 0; iX < curSize; iX++ )  
				for( iY = curSize − 1; curPattern[ iX ][ iY ]  = =  0 ; iY− − )  
					curPattern[ iX ][ iY ] = 1

for( iY = 0; iY <= yE>> resShift; iY++ )
			for( iX = 0; iX <= patternSize − 1 || wedgePattern [ iX ][ iY ]  = =  0 ; iX++ )
				wedgePattern [ iX ][ iY ] = 1

  - The binary partition pattern wedgePattern[ x ][ y ], with x, y = 0..patternSize − 1, is derived as specified in the following:

    - If resShift is equal to 1, the following applies:

      - Depending on wedgeOri, the variables xOff and yOff are set as specified in Table Error: Reference source not found−3.

Table Error: Reference source not found−3 − Specification of xOff, yOff

| wedgeOri | ( xS + xE ) < curSize | xOff | yOff |
| --- | --- | --- | --- |
| 0 |  | 0 | 0 |
| 1 |  | 1 | 0 |
| 2 |  | 1 | 1 |
| 3 |  | 0 | 1 |
| 4 | 0 | 1 | 0 |
| 4 | 1 | 0 | 0 |
| 5 | 0 | 0 | 1 |
| 5 | 1 | 0 | 0 |

      - For x, y = 0 ..patternSize − 1 the following applies:

        - wedgePattern[ x ][ y ] = curPattern[ ( x  <<  1 ) + xOff ][ ( y  <<  1 ) + yOff ]	(Error: Reference source not found-2)

Otherwise (resShift is not equal to 1), wedgePattern is set equal to curPattern.

- I.8.4.4.2.11.2 Wedgelet pattern rotation process

Inputs to this process are:

- a variable patternSize specifying the binary partition pattern size,

- a variable wedgeOri specifying the orientation identifier of the wedgelet pattern,

- a binary array refPattern[ x ][ y ] of size ( patternSize )x( patternSize ).

Output of this process is:

- binary array wedgePattern[ x ][ y ] of size ( patternSize )x( patternSize ).

- The values of variable wedgePattern[ x ][ y ] are derived as specified in the following:

- cInc = wedgeOri == 2 ? − 1 : 0;

- sInc = (wedgeOri == 1 ? 1 : − 1) * (1 + xInc);

- iOffset = cInc + sInc < 0 ? patternSize − 1 ? 0

- jOffset = −sInc + cInc < 0 ? patternSize − 1 ? 0

  - for( y = 0; y < patternSize; y++ )  
				for( x = 0; x < patternSize; x++ )  {

  - i = iOffset + cInc * x + sInc  * y

  - j = jOffset − sInc * x + cInc  * y

- wedgePattern[ x ][ y ] = refPattern [ iOffset ][ jOffset ] ? refPattern [ i ][ j ] : 1 -refPattern [ i ][ j ]

- }

- I.8.4.4.2.11.3 Wedgelet pattern list insertion process

Inputs to this process are:

- a variable log2BlkSize specifying the binary partition pattern size as ( 1  <<  log2BlkSize ),

- binary partition pattern wedgePattern[ x ][ y ], with x, y =0..( 1  <<  log2BlkSize ) − 1.

- a variable wedgeOri specifying the orientation identifier of the wedgelet pattern,

Output of this process is:

- a variable posWedgePattern.

- The variable isValidFlag specifying whether the binary partition pattern wedgePattern is added to the list WedgePatternTable[ log2BlkSize ] not is set 	equal to 0.

- If wedgeOri is equal to 0 or 4, the value of isValidFlag is derived as specified by the following ordered steps.

  - For x, y = 0..( 1  <<  log2BlkSize ) − 1 the following applies:

    - When wedgePattern[ x ][ y ] is not equal to wedgePattern[ 0 ][ 0 ] the flag isValidFlag is set to 1.

  - For k = 0..NumWedgePattern[ log2BlkSize ] − 1 the following applies:

    - The flag patIdenticalFlag is set equal to 1.

    - For x, y =0..( 1  <<  log2BlkSize ) − 1 the following applies:

      - When wedgePattern[ x ][ y ] is not equal to WedgePatternTable[ log2BlkSize ][ k ][ x ][ y ], patIdenticalFlag is set to 0.

    - When patIdenticalFlag is equal to 1, isValidFlag is set to 0.

  - For k = 0..NumWedgePattern[ log2BlkSize ] − 1 the following applies:

    - The flag patInvIdenticalFlag is set to 1.

    - For x, y =0..( 1  <<  log2BlkSize ) − 1 the following applies:

      - When wedgePattern[ x ][ y ] is equal to WedgePatternTable[ log2BlkSize ][ k ][ x ][ y ], patInvIdenticalFlag is set to 0.

    - When patInvIdenticalFlag is equal to 1, isValidFlag is set to 0.

- Otherwise (wedgeOri is equal to 1..3 or wedgeOri is equal to 5), the value of isValidFlag is set equal to 1.

- When isValidFlag is equal to 1, the following applies:

  - The pattern WedgePatternTable[ log2BlkSize ][ NumWedgePattern[ log2BlkSize ] ] is set equal to wedgePattern.

  - The value of NumWedgePattern[ log2BlkSize ] is increased by one.

The variable posWedgePattern is set equal to NumWedgePattern[ log2BlkSize ]

# Conclusion

It is proposed to further reduce the look up table size of DMM1 by down sampling in 16x16 and symmetric pattern. The proposed combination claims to achieve 22.6 k bytes (uncompressed) and 3.6 k bytes (compressed) compared to the current design 48.3 k bytes (uncompressed) and 16.3 k bytes (compressed) in terms of the table size.

# Patent rights declaration(s)

MediaTek Inc. and Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, "3D-HEVC Draft Text 5," Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG JCT3V-I1001, July. 2014

- HTM-12.0, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-12.0

- D. Rusanovksyy, A. Vetro, and K. Müller, "Common Test Conditions for 3D Video Extensions Development," Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG JCT3V-E1100, July, 2013.

- Tomohiro Ikai, "Lookup table size reduction in DMM1," Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG JCT3V-I0110, July, 2014.