| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18–24 Oct. 2014 | Document: JCT3V-J0115 |
| --- | --- |

| Title: | Single Depth Intra Mode Simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xu Chen, Xiaozhen Zheng, Yongbing Lin, Jianhua Zheng, Sunmi Yoo*, Sehoon Yea*, Gun Bang**, Young Su Heo***, Woo Woen Gwun***, Gwang Hoon Park***, Gwang Soon Lee**, Nam Ho Hur** HiSilicon technologies Shenzhen, P. R. China, 518129 LG Electronics* 19, Yangjae-daero 11gil, Seocho-gu, Seoul 137-130 South Korea ETRI** 218 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, Korea Media Lab., Kyung Hee University*** 1, Seocheon, Giheung, Yongin, Gyeonggi, 446-701, Korea | Tel: Email: | +86-755-28425333 +82-2-6912-6477 +82-42-860-6791 +82-31-201-3680 anderson.chen@huawei.com sunmi.yoo*@lge.com {gbang**, gslee**, namho**} @etri.re.kr {medist***, woo_gwun***, ghpark***} @khu.ac.kr |
| Source: | HiSilicon Technologies LG Electronics* ETRI** and Kyung Hee University*** |  |  |

_____________________________

# Abstract

At the 9th JCT-3V meeting, a method of single depth intra coding was adopted in JCT3V-I0095 [1] that is reconstructed by filling this CU with one single depth value when a CU is coded as single depth mode. The candidate list derivation process in JCT3V-I0095 is relatively high complex, as it needs to insert into the candidate list by analyzing up to five reconstructed spatial neighboring samples around the current CU in the order with pruning process. In this contribution, a unified solution for a simplified candidate list construction for single depth mode, presented in JCT3V-J0040[2], JCT3V-J0054[3], and JCT3V-J0058[4], is proposed. The impact on the coding efficiency of single depth intra coding is negligible.

# Original Candidate List Derivation Algorithm

In the original scheme, the size of the sample candidate list is fixed to 2. Five reconstructed spatial neighboring samples around the current CU in the order (An/2, Bn/2, B0, A0 and B-1) with pruning process. If empty entry still exists in the candidate list after the pruning and the derivation of candidates from the spatial neighboring samples, offset candidates which are derived by adding offsets to first available sample candidate will be used to fill in those empty entries. After the sample candidate list is constructed, a candidate index is signaled to indicate which sample candidate is selected to fill the current CU.

Figure 1. The spatial sample candidates used to construct the sample candidate list.

# Proposed Partitioning Derivation Algorithm

In this unification, it is proposed to reduce the number of candidate of SDM from 5 to 2. Also, the pruning process for SDM candidates is removed.

## Up to two reconstructed spatial neighboring samples

As shown on Figure 2, two reconstructed spatial neighboring samples around the current CU in the order (An/2, Bn/2). J0040 and J0058 part 1 proposes to use only two samples as the candidate samples of SDM.

Figure 2. The spatial sample candidates used to construct the sample candidate list.

## Remove pruning process

J0054 and J0058 part 2 proposes to remove pruning process. With this proposed method, the comparisons to construct candidate samples can be further removed.

- Proposed Text Modification

The newly added parts compared to working draft are highlighted in yellow and the removed parts are marked with strikethrough. The text bug-fix parts are highlighted in green.

- I.8.4.4.2.9 Specification of intra prediction mode INTRA_SINGLE_SAMPLE

Inputs to this process are:

- a sample location ( xTb, yTb ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

- the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1,

- a variable nTbS specifying the transform block size.

Output of this process is:

- the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

The lists predSample[ i ] and availableFlag[ i ] are derived as specified in the following:

- For i in the range of 0 to 41, inclusive, the following applies:

  - Depending on i, the luma location ( xN, yN ) is specified in Error: Reference source not foundTable I-6.

  - The variable availableFlag[ i ] is derived as specified in the following:

    - If p[ xN, yN ] is marked as "available for intra prediction", availableFlag[ i ] is set equal to 1.

    - Otherwise (p[ xN, yN ] is marked as "not available for intra prediction"), availableFlag[ i ] is set equal to 0.

  - The variable predSample[ i ] is set equal to p[ xN, yN ].

Table I-6 – Specification of xN and yN depending on i

| i | 0 | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- |
| xN | nTbS >> 1-1 | 0nTbS >> 1 | 0 | −1 | −1 |
| yN | 0nTbS >> 1 | nTbS >> 1-1 | −1 | 0 | −1 |

- The list availableFlag[ i ] is modified as specified in the following:

- For i in the range of 1 to 4, inclusive, the following applies:

  - For j in the range of 0 to ( i − 1 ), inclusive, the following applies:

      - availableFlag[ i ] = availableFlag[ i ]  &&
	!( availableFlag[ j ]  &&  ( predSample[ j ]  = =  predSample[ i ] ) )	(Error: Reference source not found-1)

The list sampleCandList is derived as specified in the following:

numCand = 0 	
	for( i = 0; i < 52; i++ )
		if ( availableFlag[ i ]  && numCand < 2 ) 
			sampleCandList[ numCand++ ] = predSample[ i ]
	if( numCand  = =  0 )
		sampleCandList[ numCand++ ] = ( 1  <<  ( BitDepthY − 1 ) )
	if( numCand  = =  1 )
		sampleCandList[ numCand++ ] = sampleCandList[ 0 ] + 1

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1 are derived as follows:

- predSamples[ x ][ y ] = sampleCandList[ single_sample_idx ]	(Error: Reference source not found-2)

- Experimental Results

The proposed methods were implemented on top of HTM-12.0[2] and were carried out on the common test condition. The results are summarized in Table 1 - 2.

Table 1 shows the proposed method introduces a negligible coding loss (0.02% loss for synthesized views).

Table 1. Comparison results between CTC anchor and Proposed Method

Table 2 shows the results of the proposed method provides negligible coding loss (0.01% loss for AI).

Table 2. Comparison results between AI anchor and Proposed Method

- Conclusions

In this contribution, a unified solution of J0040, J0054, and J0058 is proposed to simplify the process of candidate list construction for single depth intra coding. The experiments results show negligible coding efficiency impact.

- Patent rights declaration(s)

HiSilicon Technologies, LG Electronics, ETRI and Kyung Hee University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

- References

- Y.-W. Chen, J.-L. Lin, Y.-W. Huang, S. Lei, “CE2: Single depth intra mode for 3D-HEVC,” Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-I0095, Sapporo, JP, July 2014.

- S. Yoo, S. Yea, “ Simplification for single depth mode pruning process,” Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-J0040, Strasbourg, FR, October 2014.

- G. Bang(ETRI),Y.S. Heo, W.W. Gwun, G.H.Park(KHU),G.S. Lee, N.H.Hur(ETRI), “Simplification of single depth intra mode,” Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-J0054, Strasbourg, FR, October 2014.

- X. Chen, X. Zheng, Y. Lin, J. Zheng , “Simplification on candidate list construction for single depth mode,” Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-J0058, Strasbourg, FR, October 2014.

- HTM-12.0, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-12.0