| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18–24 Oct. 2014 | Document: JCT3V-J00116 |
| --- | --- |

| Title: | Unfication report on further cleanup of Single Depth Intra Mode simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianhua Zheng, Zhouye Gu* Xu Chen, Xiaozhen Zheng, Yongbing Lin, Nam Ling* Philipp Zhang | Tel: Email: | +1-206-816-2367 zhengjianhua@hisilicon.com zgu@scu.edu anderson.chen@huawei.com |
| Source: | HiSilicon Technologies Santa Clara University* |  |  |

_____________________________

# Abstract

In this contribution, a unified solution for a simplified single depth mode, presented in JCT3V-J0065[1], JCT3V-J0040Error: Reference source not found, JCT3V-J0054Error: Reference source not found, and JCT3V-J0058Error: Reference source not found, is proposed. The impact on the coding efficiency of single depth intra coding is negligible.

# Original Candidate List Derivation Algorithm

In the original scheme, the size of the sample candidate list is fixed to 2. Five reconstructed spatial neighboring samples around the current CU in the order (An/2, Bn/2, B0, A0 and B-1) with pruning process. If empty entry still exists in the candidate list after the pruning and the derivation of candidates from the spatial neighboring samples, offset candidates which are derived by adding offsets to first available sample candidate will be used to fill in those empty entries. After the sample candidate list is constructed, a candidate index is signaled to indicate which sample candidate is selected to fill the current CU.

Figure 1. The spatial sample candidates used to construct the sample candidate list.

# Proposed Partitioning Derivation Algorithm

In this unification, it is proposed to reduce the number of candidate of SDM from 5 to 2. Also, the pruning process as well as the candidate list derivation process for SDM is removed. The unified solution is identical to and also presented in the J0065 method3.

In this meeting, JCT3V-J0040, JCT3V-J0054, JCT3V-J0058p1&2 have been unified to get a simplified SDM solution, which can be found in JCT3V-J0115. The unified solution in JCT3V-J0115 is identical to the JCT3V-J0058p1&2.

The features of the SDM simplification proposals can be summarized as,

Table 1  Configuration for tests

|  | # of cand samples | Pruning | Remove default case | With Candidate list Derivation |
| --- | --- | --- | --- | --- |
| J0040(J0058p1) | 2 | Yes | No | Yes |
| J0054 | 5 | No | Yes | Yes |
| J0058p2 | 5 | No | No | Yes |
| J0058p1&p2 | 2 | No | No | Yes |
| JCT3V-J0115 (indentical to J0058p1&p2) | 2 | No | No | Yes |
| J0065 | 2 | No | No | No |

From above summary, it is clearly that JCT3V-J0065 offer several features. Beside the features it has been proposed in JCT3V-J0115(reduce candidate number from 5 to 2, and remove pruning process), JCT3V-J0065 also propose to further cleanup the candidate list derivation process.

We proposed here a further clean-up unified solution, which is harmolized to the JCT3V-J0115.

The table for each cases is summarized as in Table2.

Table2.1

| i | 0 (A) | 1 (B) |
| --- | --- | --- |
| xN | 0 | nTbS >> 1 |
| yN | nTbS >> 1 | 0 |

Table2.2  Available candidate checking

|  | single_sample_idx=0 | single_sample_idx=1 |  |
| --- | --- | --- | --- |
| (A available, B available) | A | B |  |
| (A NOT available, B available) | 1 << ( BitDepthY − 1 ) | B |  |
| (A available, B NOT available) | A | 1 << ( BitDepthY − 1 ) |  |
| (A NOT available, B NOT available) | 1 << ( BitDepthY − 1 ) |  | 1 << ( BitDepthY − 1 ) |

## Reduce to two reconstructed spatial neighboring samples

As shown on Figure 2, two reconstructed spatial neighboring samples around the current CU in the order (An/2, Bn/2). J0040 and J0058 part 1 proposes to use only two samples as the candidate samples of SDM.

Figure 2. The spatial sample candidates used to construct the sample candidate list.

## Remove pruning process

J0065, J0054 and J0058 part 2 proposes to remove pruning process. With this proposed method, the comparisons to construct candidate samples can be further removed.

## Remove candidate list derivation process

J0065 proposes to remove candidate list derivation process. With this proposed method, candidate list derivation process is fuether cleanup and the worse case for the number of checking availablilty of the candidates is further reduced.

- Proposed Text Modification

The newly added parts compared to working draft are highlighted in yellow and the removed parts are marked with strikethrough. The text bug-fix parts are highlighted in green. The text further cleanup by JCT3V-J0065 based on unification solution1(JCT3V-J0115 )are marked as following.

- I.8.4.4.2.9 Specification of intra prediction mode INTRA_SINGLE_SAMPLE

Inputs to this process are:

- a sample location ( xTb, yTb ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

- the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1,

- a variable nTbS specifying the transform block size.

- a index single_sample_idx specifies index of the single sample candidates.

Output of this process is:

- the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

The lists predSample[ i ] and availableFlag[ i ] are derived as specified in the following:

- Set depthVal equal to 1  <<  ( BitDepthY − 1 ).

- If single_sample_idx is equal to 0, set i as 0, otherwise, set i as 1.

- For i in the range of 0 to 41, inclusive, the following applies:

  - Depending on i, the luma location ( xN, yN ) is specified in Error: Reference source not foundTable I-6.

  - The variable availableFlag[ i ] is derived as specified in the following:

    - If p[ xN, yN ] is marked as "available for intra prediction", availableFlag[ i ] is set equal to 1. depthVal is set equal to p[ xN, yN ].

    - Otherwise (p[ xN, yN ] is marked as "not available for intra prediction"), availableFlag[ i ] is set equal to 0.

  - The variable predSample[ i ] is set equal to p[ xN, yN ].

- The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1 are derived as follows:

- predSamples[ x ][ y ] = depthVal                    (Error: Reference source not found-XX)

Table Error: Reference source not found-1 – Specification of xN and yN depending on i

| i | 0 | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- |
| xN | nTbS >> 1-1 | 0nTbS >> 1 | 0 | −1 | −1 |
| yN | 0nTbS >> 1 | nTbS >> 1-1 | −1 | 0 | −1 |

- The list availableFlag[ i ] is modified as specified in the following:

- For i in the range of 1 to 4, inclusive, the following applies:

  - For j in the range of 0 to ( i − 1 ), inclusive, the following applies:

      - availableFlag[ i ] = availableFlag[ i ]  &&
	!( availableFlag[ j ]  &&  ( predSample[ j ]  = =  predSample[ i ] ) )	(Error: Reference source not found-1)

The list sampleCandList is derived as specified in the following:

numCand = 0 	
	for( i = 0; i < 52; i++ )
		if ( availableFlag[ i ]  && numCand < 2 ) 
			sampleCandList[ numCand++ ] = predSample[ i ]
	if( numCand  = =  0 )
		sampleCandList[ numCand++ ] = ( 1  <<  ( BitDepthY − 1 ) )
	if( numCand  = =  1 )
		sampleCandList[ numCand++ ] = sampleCandList[ 0 ] + 1

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1 are derived as follows:

predSamples[ x ][ y ] = sampleCandList[ single_sample_idx ]

- Experimental Results

The proposed methods were implemented on top of HTM-12.0[2] and were carried out on the common test condition. The results are summarized in Table 1 - 2.

4.1  Result of proposed method(2.1&2.2)

Table 1 shows the proposed method(2.1&2.2) introduces a negligible coding loss (0.02% loss for synthesized views).

Table 1. Comparison results between CTC anchor and Proposed Method

Table 2 shows the results of the proposed method provides negligible coding loss (0.01% loss for AI).

Table 2. Comparison results between AI anchor and Proposed Method

4.2  Result of proposed method(2.1&2.2&2.3)

Table 3 and Table4 shows the proposed method(2.1&2.2&2.3) introduces a negligible coding loss.

Table 3: BD rate results for 3-view case under CTC

Table 4: BD rate results for 3-view case under All Intra

- Conclusions

In this contribution, a unified solution of J0065, J0040, J0054, and J0058 is proposed to simplify the single depth intra coding. The experiments results show negligible coding efficiency impact.

- References

- Z. Gu(SCU), J. Zheng(HiSilicon), N. Ling(SCU), P. Zhang(HiSilicon), “ On Single Depth Mode Simplification,”Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-J0054, Strasbourg, FR, October 2014.

- S. Yoo, S. Yea, “ Simplification for single depth mode pruning process,” Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-J0040, Strasbourg, FR, October 2014.

- G. Bang(ETRI),Y.S. Heo, W.W. Gwun, G.H.Park(KHU),G.S. Lee, N.H.Hur(ETRI), “Simplification of single depth intra mode,” Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-J0054, Strasbourg, FR, October 2014.

- X. Chen, X. Zheng, Y. Lin, J. Zheng , “Simplification on candidate list construction for single depth mode,” Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-J0058, Strasbourg, FR, October 2014.

- Patent rights declaration(s)

HiSilicon Technologies, Santa ClaraUniversity may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).