| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18–24 October 2014 | Document: JCT3V-J1006r3 |
| --- | --- |

| Title: | Draft of Texture and Depth View Packing SEI Message |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-3V |  |  |
| Purpose: | Draft Text |  |  |
| Author(s) or Contact(s): | Takanori Senoh NICT Ying Chen Qualcomm Miska M. Hannuksela Nokia Jens-Rainer Ohm RWTH Aachen University Gary J. Sullivan Microsoft | Email: Email: Email: Email: Email: | senoh@nict.go.jp cheny@qti.qualcomm.com miska.hannuksela@nokia.com ohm@ient.rwth-aachen.de garysull@microsoft.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This amendment defines a texture and depth view packing SEI message.

In Table D.1, add the following after “depth_view_info ( payloadSize )”:

| else if( payloadType = = 56 ) |  |  |
| --- | --- | --- |
| texture_and_depth_view_packing( payloadSize ) /* specified in Annex J */ | 5 |  |

Add a new subclause J.13.1.2 “Texture and depth view packing SEI message syntax” as follows:

      - Texture and depth view packing SEI message syntax

| texture_and_depth_view_packing( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| depth_type | 5 | ue(v) |
| if( depth_type = = 0 ) { |  |  |
| num_residual_texture_views_minus1 | 5 | ue(v) |
| residual_depth_flag | 5 | u(1) |
| z_flag | 5 | u(1) |
| intrinsic_param_flag | 5 | u(1) |
| rotation_flag | 5 | u(1) |
| translation_flag | 5 | u(1) |
| if( z_flag ) |  |  |
| for( i = 0; i <= num_residual_texture_views_minus1 + 1; i++ ) { |  |  |
| z_near_sign_flag[ i ] | 5 | u(1) |
| z_near_exponent[ i ] | 5 | u(7) |
| z_near_mantissa_len_minus1[ i ] | 5 | u(5) |
| z_near mantissa[ i ] | 5 | u(v) |
| z_far_sign_flag[ i ] | 5 | u(1) |
| z_far_exponent[ i ] | 5 | u(7) |
| z_far_mantissa_len_minus1[ i ] | 5 | u(5) |
| z_far_mantissa[ i ] | 5 | u(v) |
| } |  |  |
| if( intrinsic_param_flag ) { |  |  |
| prec_focal_length | 5 | ue(v) |
| prec_principal_point | 5 | ue(v) |
| } |  |  |
| if( rotation_flag ) |  |  |
| prec_rotation_param | 5 | ue(v) |
| if ( translation_flag ) |  |  |
| prec_translation_param | 5 | ue(v) |
| for( i = 0; i <= num_residual_texture_views_minus1 + 1; i++ ) { |  |  |
| if( intrinsic_param_flag ) { |  |  |
| sign_focal_length_x[ i ] | 5 | u(1) |
| exponent_focal_length_x[ i ] | 5 | u(6) |
| mantissa_focal_length_x[ i ] | 5 | u(v) |
| sign_focal_length_y[ i ] | 5 | u(1) |
| exponent_focal_length_y[ i ] | 5 | u(6) |
| mantissa_focal_length_y[ i ] | 5 | u(v) |
| sign_principal_point_x[ i ] | 5 | u(1) |
| exponent_principal_point_x[ i ] | 5 | u(6) |
| mantissa_principal_point_x[ i ] | 5 | u(v) |
| sign_principal_point_y[ i ] | 5 | u(1) |
| exponent_principal_point_y[ i ] | 5 | u(6) |
| mantissa_principal_point_y[ i ] | 5 | u(v) |
| } |  |  |
| if( rotation_flag ) |  |  |
| for ( j = 1;j <= 3;j++ ) /* row */ |  |  |
| for( k = 1;k <= 3;k++ ) { /* column */ |  |  |
| sign_r[ i ][ j ][ k ] | 5 | u(1) |
| exponent_r[ i ][ j ][ k ] | 5 | u(6) |
| mantissa_r[ i ][ j ][ k ] | 5 | u(v) |
| } |  |  |
| if ( translation_flag ) { |  |  |
| sign_t_x[ i ] | 5 | u(1) |
| exponent_t_x[ i ] | 5 | u(6) |
| mantissa_t_x[ i ] | 5 | u(v) |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

Add a new subclause J.13.2.2 “Texture and depth view packing SEI message semantics” as follows, with “J-xx” replaced with the appropriate table number in the sequence of tables present in the text.

      - Texture and depth view packing SEI message semantics

The texture and depth view packing SEI message informs the decoder that view components of one output view contain samples of multiple distinct spatially packed constituent pictures. The related output view is not suitable to be displayed directly. This SEI message can be used after the decoder output to appropriately rearrange the samples to produce additional views that are appropriate for display or to server for other purposes (which are outside the scope of this Recommendation | International Standard).

Texture and depth view accompanied by this SEI message conform to the specifications in Annex I and J. They are 8 bit, 4:2:0 sampled. The depth_representation_type is defined in the depth_representation_info_SEI message which shall be always present. Depth view is only carried in luma component.

The texture and depth view packing SEI message, when present, shall be associated with an IDR access unit. The information signalled in this SEI message applies to all the access units in the coded video sequence. Also, num_anchor_refs_l0[ i ], num_anchor_refs_l1[ i ], num_non_anchor_refs_l0[ i ] and num_non_anchor_refs_l1[ i ] shall be equal to 0 in the active SPS RBSP syntax structure for the coded video sequence and avc_3d_extension_flag shall be 0 in all VCL NAL units of the coded video sequence.

NOTE 1 – These constraints disable inter-view and inter-component prediction.

In such a coded video sequence, there shall be two output views, wherein the base view is a complete view and a non-base view, which contains a packing arrangement of 1-4 sub-sampled pictures of additional views. Such sub-sampled pictures are denoted as constituent pictures here and after.

NOTE 2 – The view_id of the non-base view is not used for the view output in the context of this SEI message.

Each constituent picture represents a picture generated by 2:1 decimation from the original picture horizontally and vertically, starting from the sample at the co-ordinates (0, 0).

The parameter i, with a value from 0 to num_residual_texture_views_minus1 + 1 inclusive, indicates the location of the constituent pictures in the non-base texture view as shown in Table J-xx. i equal to 0 indicates the base texture view. The other i indicates the constituent texture view.

Let the output non-base texture view has a width equal to W and a height equal to H for the luma component. This SEI message shall not be present unless W is a multiple of 2 * SubWidthC and H is a multiple of 2 * SubHeightC.

- Table J-xx – The origin coordinates of constituent pictures packed in a non-base texture view of width=W and height=H.

| i | The origin coordinates in a non-base texture view |
| --- | --- |
| 1 | (0, 0) |
| 2 | (H/2, 0) |
| 3 | (0, W/2) |
| 4 | (H/2, W/2) |

The decoded depth views have the same structure as the decoded texture views, namely they are base depth view (i = 0) and non-base depth view. The non-base depth view contains up to four constituent depth pictures (i = 1,…4) for the constituent views. They are packed in the non-base depth view in the same arrangement as the constituent texture pictures shown in Table J-xx.

depth_type shall be equal to 0. Other values are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this Specification. Decoders shall ignore texture and depth view packing SEI messages in which such other values are present.

num_residual_texture_views_minus1 plus 1 specifies the number of constituent texture pictures packed into each texture view component of a non-base view. num_residual_texture_views_minus1 shall be in the range of 0 to 3, inclusive.

residual_depth_flag equal to 1 specifies that constituent depth pictures are packed into the depth components of the non-base view, with a packing scheme that is identical to the one used for the constituent texture pictures.

Each constituent depth picture of the non-base depth view component is associated with a constituent picture of the non-base texture view component in the same relative location. The number of the coded depth views in the coded video sequence shall be equal to 1 + residual_depth_flag. The depth components of the base view shall always be present, independent of the value of residual_depth_flag.

NOTE 3 – The following SEI message parameters can be used along with the depth maps to project samples from the base view into the co-ordinates of constituent views such that reconstructed views can be generated by combining projected samples and samples from the constituent views.

z_flag equal to 1 indicates the presence of the syntax elements z_near_sign_flag[ i ], z_near_exponent[ i ],  z_near_mantissa_len_minus1[ i ], z_near_mantissa[ i ], z_far_sign_flag[ i ], z_far_exponent[ i ], z_far_mantissa_len_minus1[ i ], and z_far_mantissa[ i ], for i ranging from 0 to num_residual_texture_views_minus1 + 1. z_flag equal to 0 indicates that these syntax elements are not present.

intrinsic_param_flag equal to 1 indicates the presence of intrinsic camera parameter syntax elements. intrinsic_param_flag equal to 0 indicates that these syntax elements are not present.

rotation_flag equal to 1 indicates the presence of rotation camera parameter syntax elements. rotation_flag equal to 0 indicates that these syntax elements are not present. When rotation_flag is 0, a default rotation camera parameter of a unit matrix value is inferred..

translation_flag equal to 1 indicates the presence of horizontal translation camera parameter syntax elements. translation_flag equal to 0 indicates that these syntax elements are not present.

z_near_sign_flag[ i ] equal to 0 indicates that the sign of the nearest depth value of the i-th camera is positive. z_near _sign_flag[ i ] equal to 1 indicates that the sign of the nearest depth value of the i-th camera is negative.

z_near_exponent[ i ] specifies the exponent part of the nearest depth value of the i-th camera. The value of z_near_exponent[ i ] shall be in the range of 0 to 27-2, inclusive. The value 27-1 is reserved for future use by ITU T | ISO/IEC. When z_near_exponent[ i ] is equal to 27-1, the value of z near[ i ] is unspecified.

z_near_mantissa_len_minus1[ i ] + 1 specifies the number of bits in the mantissa of the nearest depth value of the i-th camera. The value of z_near_mantissa_len_minus1 [ i ] shall be in the range of 0 to 31, inclusive.

z_near_mantissa[ i ] specifies the mantissa part of the nearest depth value of the i-th camera.

z_far_sign_flag[ i ] equal to 0 indicates that the sign of the farthest depth value of the i-th camera is positive. z_far_sign_flag[ i ] equal to 1 indicates that the sign of the farthest depth value of the i-th camera is negative.

z_far_exponent[ i ] specifies the exponent part of the farthest depth value of the i-th camera. The value of z_far_exponent[ i ] shall be in the range of 0 to 27-2, inclusive. The value 27-1 is reserved for future use by ITU T | ISO/IEC. When z_far_exponent[ i ] is equal to 27-1, the value of z far[ i ] is unspecified.

z_far_mantissa_len_minus1[ i ] + 1 specifies the number of bits in the mantissa of the farthest depth value of the i-th camera. The value of z_far_ mantissa_len_minus1[ i ] shall be in the range of 0 to 31, inclusive.

z_far_mantissa[ i ] specifies the mantissa part of the farthest depth value of the i-th camera.

prec_focal_length specifies the exponent of the maximum allowable truncation error for focal_length_x[ i ] and focal_length_y[ i ] as given by 2−prec_focal_length. The value of prec_focal_length shall be in the range of 0 to 31, inclusive.

prec_principal_point specifies the exponent of the maximum allowable truncation error for principal_point_x[ i ] and principal_point_y[ i ] as given by 2−prec_principal_point. The value of prec_principal_point shall be in the range of 0 to 31, inclusive.

prec_rotation_param specifies the exponent of the maximum allowable truncation error for r[ i ][ j ][ k ] as given by 2−prec_rotation_param. The value of prec_rotation_param shall be in the range of 0 to 31, inclusive.

prec_translation_param specifies the exponent of the maximum allowable truncation error for t_x[ i ] as given by 2−prec_translation_param. The value of prec_ translation_param shall be in the range of 0 to 31, inclusive.

sign_focal_length_x[ i ] equal to 0 indicates that the sign of the focal length of the i-th camera in the horizontal direction is positive. sign_focal_length_x[ i ] equal to 1 indicates that the sign of the focal length of the i-th camera in the horizontal direction is negative.

exponent_focal_length_x[ i ] specifies the exponent part of the focal length of the i-th camera in the horizontal direction. The value of exponent_focal_length_x[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU T | ISO/IEC. When exponent_focal_length_x[ i ] is equal to 63, the value of focal length of the horizontal direction for the i-th camera is unspecified.

mantissa_focal_length_x[ i ] specifies the mantissa part of the focal length of the i-th camera in the horizontal direction. The length of the mantissa_focal_length_x[ i ] syntax element is determined as follows:

–	If exponent_focal_length_x[ i ] = = 0, the length is Max( 0, prec_focal_length − 30 ).

–	Otherwise (0 < exponent_focal_length_x[ i ] < 63), the length is Max( 0, exponent_focal_length_x[ i ] + prec_focal_length − 31 ).

sign_focal_length_y[ i ] equal to 0 indicates that the sign of the focal length of the i-th camera in the vertical direction is positive. sign_focal_length_y[ i ] equal to 1 indicates that the sign of the focal length of the i-th camera in the vertical direction is negative.

exponent_focal_length_y[ i ] specifies the exponent part of the focal length of the i-th camera in the vertical direction. The value of exponent_focal_length_y[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU T | ISO/IEC. When exponent_focal_length_y[ i ] is equal to 63, the value of focal length of the vertical direction is unspecified.

mantissa_focal_length_y[ i ] specifies the mantissa part of the focal length of the i-th camera in the vertical direction.

The length of the mantissa_focal_length_y[ i ] syntax element is determined as follows:

–	If exponent_focal_length_y[ i ] = = 0, the length is Max( 0, prec_focal_length − 30 ).

–	Otherwise (0 < exponent_focal_length_y[ i ] < 63), the length is Max( 0, exponent_focal_length_y[ i ] + prec_focal_length − 31 ).

sign_principal_point_x[ i ] equal to 0 indicates that the sign of the principal point of the i-th camera in the horizontal direction is positive. sign_principal_point_x[ i ] equal to 1 indicates that the sign of the principal point of the i-th camera in the horizontal direction is negative.

exponent_principal_point_x[ i ] specifies the exponent part of the principal point of the i-th camera in the horizontal direction. The value of exponent_principal_point_x[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU T | ISO/IEC. When exponent_principal_point_x[ i ] is equal to 63, the value of principal point in the horizontal direction for the i-th camera is unspecified.

mantissa_principal_point_x[ i ] specifies the mantissa part of the principal point of the i-th camera in the horizontal direction The length of the mantissa_principal_point_x[ i ] syntax element in units of bits is determined as follows:

–	If exponent_principal_point_x[ i ] = = 0, the length is Max( 0, prec_principal_point − 30 ).

–	Otherwise (0 < exponent_principal_point_x[ i ] < 63), the length is Max( 0, exponent_principal_point_x[ i ] + prec_principal_point − 31 ).

sign_principal_point_y[ i ] equal to 0 indicates that the sign of the principal point of the i-th camera in the vertical direction is positive. sign_principal_point_y[ i ] equal to 1 indicates that the sign of the principal point of the i-th camera in the vertical direction is negative.

exponent_principal_point_y[ i ] specifies the exponent part of the principal point of the i-th camera in the vertical direction. The value of exponent_principal_point_y[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU T | ISO/IEC. When exponent_principal_point_y[ i ] is equal to 63, the value of principal point in the vertical direction for the i-th camera is unspecified.

mantissa_principal_point_y[ i ] specifies the mantissa part of the principal point of the i-th camera in the vertical direction The length of the mantissa_principal_point_y[ i ] syntax element in units of bits is determined as follows:

–		If exponent_principal_point_y[ i ] = = 0, the length is Max( 0, prec_principal_point − 30 ).

–		Otherwise (0 < exponent_principal_point_y[ i ] < 63), the length is Max( 0, exponent_principal_point_y[ i ] + prec_principal_point − 31 ).

sign_r[ i ][ j ][ k ] equal to 0 indicates that the sign of (j, k) component of the rotation matrix for the i-th camera is positive. sign_r[ i ][ j ][ k ] equal to 1 indicates that the sign of (j, k) component of the rotation matrix for the i-th camera is negative.

exponent_r[ i ][ j ][ k ] specifies the exponent part of (j, k) component of the rotation matrix for the i-th camera. The value of exponent_r[ i ][ j ][ k ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU T | ISO/IEC. When exponent_r[ i ][ j ][ k ] is equal to 63, the value of rotation matrix is unspecified.

mantissa_r[ i ][ j ][ k ] specifies the mantissa part of (j, k) component of the rotation matrix for the i-th camera.

The length of the mantissa_r[ i ][ j ][ k ] syntax element in units of bits is determined as follows:

–	If exponent_r[ i ] = = 0, the length is Max( 0, prec_rotation_param − 30 ).

–	Otherwise (0 < exponent_r[ i ] < 63), the length is Max( 0, exponent_r[ i ] + prec_rotation_param − 31 ).

The rotation matrix R[ i ] for i-th camera is represented as follows:

r[i][0][0]r[i][0][1]r[i][0][2]r[ i ][ 1 ][ 0 ]r[i][1][1]r[i][1][2]r[ i ][ 2 ][ 0 ]r[i][2][1]r[i][2][2] 	(J-XX)

sign_t_x[ i ] equal to 0 indicates that the sign of the horizontal component of the translation vector for the i-th camera is positive. sign_trans_x[ i ] equal to 1 indicates that the sign of the horizontal component of the translation vector for the i-th camera is negative.

exponent_t_x[ i ] specifies the exponent part of the horizontal component of the translation vector for the i-th camera. The value of exponent_t_x[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU T | ISO/IEC. When exponent_t_x[ i ] is equal to 63, the value of the translation vector is unspecified.

mantissa_t_x[ i ] specifies the mantissa part of the horizontal component of the translation vector for the i-th camera.

The length v of the mantissa_t_x[ i ] syntax element in units of bits is determined as follows:

–	If exponent_t_x[ i ] = = 0, the length v = Max( 0, prec_translation_param − 30 ).

–	Otherwise (0 < exponent_t_x[ i ] < 63), the length v = Max( 0, exponent_t_x[ i ] + prec_translation_param − 31 ).

The variable x, where x refers to a focal length, principal point, rotation, or translation is computed as follows, where e = exponent, s = sign, n = mantissa and v = mantissa_len_minus1 + 1:

–	If 0 < e < 63, x = (−1)s * 2e−31 * (1 + n  2v).

–	Otherwise (e is equal to 0), x = (−1)s * 2−(30+v) * n.

NOTE 4 – The above specification is similar to what found in IEC 60559:1989, Binary floating-point arithmetic for microprocessor systems.