| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18–24 October 2014 | Document: JCT3V-J1006r1 |
| --- | --- |

| Title: | Draft 1 of Texture and Depth View Packing SEI Message |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-3V |  |  |
| Purpose: | Draft Text |  |  |
| Author(s) or Contact(s): | Takanori Senoh NICT Ying Chen Qualcomm Miska M. Hannuksela Nokia Jens-Rainer Ohm RWTH Aachen University Gary J. Sullivan Microsoft | Email: Email: Email: Email: Email: | senoh@nict.go.jp cheny@qti.qualcomm.com miska.hannuksela@nokia.com ohm@ient.rwth-aachen.de garysull@microsoft.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This amendment defines a texture and depth view packing SEI message.

In Table D.1, add the following after “depth_view_info ( payloadSize )”:

| else if( payloadType = = 56 ) |  |  |
| --- | --- | --- |
| texture_and_depth_view_packing( payloadSize ) /* specified in Annex J */ | 5 |  |

Add a new subclause J.13.1.2 “Texture and depth view packing SEI message syntax” as follows:

      - Texture and depth view packing SEI message syntax

| texture_and_depth_view_packing( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| depth_type | 5 | ue(v) |
| if( depth_type = = 0 ) { |  |  |
| num_residual_texture_views_minus1 | 5 | ue(v) |
| residual_depth_flag | 5 | u(1) |
| z_flag | 5 | u(1) |
| intrinsic_param_flag | 5 | u(1) |
| rotation_flag | 5 | u(1) |
| translation_flag | 5 | u(1) |
| if( z_flag ) |  |  |
| for( i = 0; i <= num_residual_texture_views_minus1 + 1; i++ ) { |  |  |
| z_near_sign_flag[ i ] | 5 | u(1) |
| z_near_exponent[ i ] | 5 | u(7) |
| z_near_mantissa_len_minus1[ i ] | 5 | u(5) |
| z_near mantissa[ i ] | 5 | u(v) |
| z_far_sign_flag[ i ] | 5 | u(1) |
| z_far_exponent[ i ] | 5 | u(7) |
| z_far_mantissa_len_minus1[ i ] | 5 | u(5) |
| z_far_mantissa[ i ] | 5 | u(v) |
| } |  |  |
| if( intrinsic_param_flag ) { |  |  |
| prec_focal_length | 5 | ue(v) |
| prec_principal_point | 5 | ue(v) |
| } |  |  |
| if( rotation_flag ) |  |  |
| prec_rotation_param | 5 | ue(v) |
| if ( translation_flag ) |  |  |
| prec_translation_param | 5 | ue(v) |
| for( i = 0; i <= num_residual_texture_views_minus1 + 1; i++ ) { |  |  |
| if( intrinsic_param_flag ) { |  |  |
| sign_focal_length_x[ i ] | 5 | u(1) |
| exponent_focal_length_x[ i ] | 5 | u(6) |
| mantissa_focal_length_x[ i ] | 5 | u(v) |
| sign_focal_length_y[ i ] | 5 | u(1) |
| exponent_focal_length_y[ i ] | 5 | u(6) |
| mantissa_focal_length_y[ i ] | 5 | u(v) |
| sign_principal_point_x[ i ] | 5 | u(1) |
| exponent_principal_point_x[ i ] | 5 | u(6) |
| mantissa_principal_point_x[ i ] | 5 | u(v) |
| sign_principal_point_y[ i ] | 5 | u(1) |
| exponent_principal_point_y[ i ] | 5 | u(6) |
| mantissa_principal_point_y[ i ] | 5 | u(v) |
| } |  |  |
| if( rotation_flag ) |  |  |
| for ( j = 1;j <= 3;j++ ) /* row */ |  |  |
| for( k = 1;k <= 3;k++ ) { /* column */ |  |  |
| sign_r[ i ][ j ][ k ] | 5 | u(1) |
| exponent_r[ i ][ j ][ k ] | 5 | u(6) |
| mantissa_r[ i ][ j ][ k ] | 5 | u(v) |
| } |  |  |
| if ( translation_flag ) { |  |  |
| sign_t_x[ i ] | 5 | u(1) |
| exponent_t_x[ i ] | 5 | u(6) |
| mantissa_t_x[ i ] | 5 | u(v) |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

Add a new subclause J.13.2.2 “Texture and depth view packing SEI message semantics” as follows, with “J-xx” replaced with the appropriate table number in the sequence of tables present in the text.

      - Texture and depth view packing SEI message semantics

The texture and depth view packing SEI message informs the decoder that view components of one output view contain samples of multiple distinct spatially packed constituent residual pictures. The related output view is not suitable to be displayed directly and may be converted to the displayable views by using this SEI message. This information can be used after the decoder output to appropriately rearrange the samples to produce the displayable views that are appropriate for display or to server for other purposes (which are outside the scope of this Recommendation | International Standard).

The texture and depth view packing SEI message, when present, shall be associated with an IDR access unit. The information signalled in this SEI message applies to all the access units in the coded video sequence. Also, num_anchor_refs_l0[ i ], num_anchor_refs_l1[ i ], num_non_anchor_refs_l0[ i ] and num_non_anchor_refs_l1[ i ] shall be equal to 0 in the active SPS RBSP syntax structure for the coded video sequence and avc_3d_extension_flag shall be 0 in all VCL NAL units of the coded video sequence.

NOTE – These constraints disable inter-view and inter-component prediction.

In such a coded video sequence, there are at least two output views, wherein the base view is coded as a displayable view and a non-base view is a residual view. A residual view is not a displayable view. Instead, it contains a sequence of difference pictures which are decoded view components of the residual view. The difference picture consists of the samples which are not predictable by the base view, namely occluded samples and out-of-picture samples in the base picture. Each difference picture is packed in a frame as up to four constituent residual pictures.

Each constituent residual picture represents a picture generated by 2:1 decimation from the residual picture horizontally and vertically, starting from the top-left most sample.

The parameter i, with a value from 0 to num_residual_texture_views_minus1 + 1 inclusive, indicates the location of the constituent residual pictures in the non-base texture view as shown in Table J-xx. i equal to 0 indicates the base texture view.

Let the output non-base texture view has a width equal to W and a height equal to H for the luma component. This SEI message shall not be present unless W is a multiple of 2 * SubWidthC and H is a multiple of 2 * SubHeightC.

- Table J-xx – Top-left corner coordinates of constituent residual pictures packed in a non-base texture view of width=W and height=H.

| i | Top-left corner coordinate |
| --- | --- |
| 1 | (0, 0) |
| 2 | (H/2, 0) |
| 3 | (0, W/2) |
| 4 | (H/2, W/2) |

The decoded depth views have the same structure as the decoded texture views, namely they are base depth view and non-base depth view. The size of these depth views are half of the decoded texture views. Base depth view (i = 0) is a sample-wise median of the same coordinates of all depth views and is associated with the base texture view (i = 0). The non-base depth view contains up to four constituent depth difference pictures (i = 1,…4) for the constituent residual views. They are packed in the non-base depth view in the same positions as the constituent residual texture pictures shown in Table J-xx.

The following SEI message parameters are used for the reconstruction of the displayable view.

depth_type specifies the arrangement how distinct spatially packed constituent residual pictures are packed into one view component of the non-base view. The values of depth_type shall be equal to 0. Other values are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this Specification. Decoders shall ignore texture and depth view packing SEI messages in which such other values are present.

num_residual_texture_views_minus1 plus 1 specifies the number of constituent residual texture pictures packed into each texture view component of a non-base view. num_residual_texture_views_minus1 shall be in the range of 0 to 3, inclusive.

residual_depth_flag equal to 1 specifies that residual depth pictures are packed into the depth components of the non-base view, with a packing scheme that is identical to the one used for the residual texture pictures.

Each constituent residual depth picture of the non-base depth view component is associated with a constituent residual picture of the non-base texture view component in the same relative location. The number of the coded depth views in the coded video sequence shall be equal to 1 + residual_depth_flag. The depth components of the base view shall always be present, independent of the value of residual_depth_flag.

z_flag equal to 1 indicates the presence of the syntax elements z_near_sign_flag[ i ], z_near_exponent[ i ],  z_near_mantissa_len_minus1[ i ], z_near_mantissa[ i ], z_far_sign_flag[ i ], z_far_exponent[ i ], z_far_mantissa_len_minus1[ i ], and z_far_mantissa[ i ], for i ranging from 0 to num_residual_texture_views_minus1 + 1. z_flag equal to 0 indicates that these syntax elements are not present.

intrinsic_param_flag equal to 1 indicates the presence of intrinsic camera parameter syntax elements. intrinsic_param_flag equal to 0 indicates that these syntax elements are not present.

rotation_flag equal to 1 indicates the presence of rotation camera parameter syntax elements. rotation_flag equal to 0 indicates that these syntax elements are not present. When rotation_flag is 0, a default rotation camera parameter of a unit matrix value is inferred..

translation_flag equal to 1 indicates the presence of horizontal translation camera parameter syntax elements. translation_flag equal to 0 indicates that these syntax elements are not present.

z_near_sign_flag[ i ] equal to 0 indicates that the sign of the nearest depth value of the i-th camera is positive. z_near _sign_flag[ i ] equal to 1 indicates that the sign of the nearest depth value of the i-th camera is negative.

z_near_exponent[ i ] specifies the exponent part of the nearest depth value of the i-th camera. The value of z_near_exponent[ i ] shall be in the range of 0 to 27-2, inclusive. The value 27-1 is reserved for future use by ITU T | ISO/IEC. When z_near_exponent[ i ] is equal to 27-1, the value of z near[ i ] is unspecified.

z_near_mantissa_len_minus1[ i ] + 1 specifies the number of bits in the mantissa of the nearest depth value of the i-th camera. The value of z_near_mantissa_len_minus1 [ i ] shall be in the range of 0 to 31, inclusive.

z_near_mantissa[ i ] specifies the mantissa part of the nearest depth value of the i-th camera.

z_far_sign_flag[ i ] equal to 0 indicates that the sign of the farthest depth value of the i-th camera is positive. z_far_sign_flag[ i ] equal to 1 indicates that the sign of the farthest depth value of the i-th camera is negative.

z_far_exponent[ i ] specifies the exponent part of the farthest depth value of the i-th camera. The value of z_far_exponent[ i ] shall be in the range of 0 to 27-2, inclusive. The value 27-1 is reserved for future use by ITU T | ISO/IEC. When z_far_exponent[ i ] is equal to 27-1, the value of z far[ i ] is unspecified.

z_far_mantissa_len_minus1[ i ] + 1 specifies the number of bits in the mantissa of the farthest depth value of the i-th camera. The value of z_far_ mantissa_len_minus1[ i ] shall be in the range of 0 to 31, inclusive.

z_far_mantissa[ i ] specifies the mantissa part of the farthest depth value of the i-th camera.

prec_focal_length specifies the exponent of the maximum allowable truncation error for focal_length_x[ i ] and focal_length_y[ i ] as given by 2−prec_focal_length. The value of prec_focal_length shall be in the range of 0 to 31, inclusive.

prec_principal_point specifies the exponent of the maximum allowable truncation error for principal_point_x[ i ] and principal_point_y[ i ] as given by 2−prec_principal_point. The value of prec_principal_point shall be in the range of 0 to 31, inclusive.

prec_rotation_param specifies the exponent of the maximum allowable truncation error for r[ i ][ j ][ k ] as given by 2−prec_rotation_param. The value of prec_rotation_param shall be in the range of 0 to 31, inclusive.

prec_translation_param specifies the exponent of the maximum allowable truncation error for t_x[ i ] as given by 2−prec_translation_param. The value of prec_ translation_param shall be in the range of 0 to 31, inclusive.

sign_focal_length_x[ i ] equal to 0 indicates that the sign of the focal length of the i-th camera in the horizontal direction is positive. sign_focal_length_x[ i ] equal to 1 indicates that the sign of the focal length of the i-th camera in the horizontal direction is negative.

exponent_focal_length_x[ i ] specifies the exponent part of the focal length of the i-th camera in the horizontal direction. The value of exponent_focal_length_x[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU T | ISO/IEC. When exponent_focal_length_x[ i ] is equal to 63, the value of focal length of the horizontal direction for the i-th camera is unspecified.

mantissa_focal_length_x[ i ] specifies the mantissa part of the focal length of the i-th camera in the horizontal direction. The length of the mantissa_focal_length_x[ i ] syntax element is determined as follows:

–	If exponent_focal_length_x[ i ] = = 0, the length is Max( 0, prec_focal_length − 30 ).

–	Otherwise (0 < exponent_focal_length_x[ i ] < 63), the length is Max( 0, exponent_focal_length_x[ i ] + prec_focal_length − 31 ).

sign_focal_length_y[ i ] equal to 0 indicates that the sign of the focal length of the i-th camera in the vertical direction is positive. sign_focal_length_y[ i ] equal to 1 indicates that the sign of the focal length of the i-th camera in the vertical direction is negative.

exponent_focal_length_y[ i ] specifies the exponent part of the focal length of the i-th camera in the vertical direction. The value of exponent_focal_length_y[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU T | ISO/IEC. When exponent_focal_length_y[ i ] is equal to 63, the value of focal length of the vertical direction is unspecified.

mantissa_focal_length_y[ i ] specifies the mantissa part of the focal length of the i-th camera in the vertical direction.

The length of the mantissa_focal_length_y[ i ] syntax element is determined as follows:

–	If exponent_focal_length_y[ i ] = = 0, the length is Max( 0, prec_focal_length − 30 ).

–	Otherwise (0 < exponent_focal_length_y[ i ] < 63), the length is Max( 0, exponent_focal_length_y[ i ] + prec_focal_length − 31 ).

sign_principal_point_x[ i ] equal to 0 indicates that the sign of the principal point of the i-th camera in the horizontal direction is positive. sign_principal_point_x[ i ] equal to 1 indicates that the sign of the principal point of the i-th camera in the horizontal direction is negative.

exponent_principal_point_x[ i ] specifies the exponent part of the principal point of the i-th camera in the horizontal direction. The value of exponent_principal_point_x[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU T | ISO/IEC. When exponent_principal_point_x[ i ] is equal to 63, the value of principal point in the horizontal direction for the i-th camera is unspecified.

mantissa_principal_point_x[ i ] specifies the mantissa part of the principal point of the i-th camera in the horizontal direction The length of the mantissa_principal_point_x[ i ] syntax element in units of bits is determined as follows:

–	If exponent_principal_point_x[ i ] = = 0, the length is Max( 0, prec_principal_point − 30 ).

–	Otherwise (0 < exponent_principal_point_x[ i ] < 63), the length is Max( 0, exponent_principal_point_x[ i ] + prec_principal_point − 31 ).

sign_principal_point_y[ i ] equal to 0 indicates that the sign of the principal point of the i-th camera in the vertical direction is positive. sign_principal_point_y[ i ] equal to 1 indicates that the sign of the principal point of the i-th camera in the vertical direction is negative.

exponent_principal_point_y[ i ] specifies the exponent part of the principal point of the i-th camera in the vertical direction. The value of exponent_principal_point_y[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU T | ISO/IEC. When exponent_principal_point_y[ i ] is equal to 63, the value of principal point in the vertical direction for the i-th camera is unspecified.

mantissa_principal_point_y[ i ] specifies the mantissa part of the principal point of the i-th camera in the vertical direction The length of the mantissa_principal_point_y[ i ] syntax element in units of bits is determined as follows:

–		If exponent_principal_point_y[ i ] = = 0, the length is Max( 0, prec_principal_point − 30 ).

–		Otherwise (0 < exponent_principal_point_y[ i ] < 63), the length is Max( 0, exponent_principal_point_y[ i ] + prec_principal_point − 31 ).

sign_r[ i ][ j ][ k ] equal to 0 indicates that the sign of (j, k) component of the rotation matrix for the i-th camera is positive. sign_r[ i ][ j ][ k ] equal to 1 indicates that the sign of (j, k) component of the rotation matrix for the i-th camera is negative.

exponent_r[ i ][ j ][ k ] specifies the exponent part of (j, k) component of the rotation matrix for the i-th camera. The value of exponent_r[ i ][ j ][ k ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU T | ISO/IEC. When exponent_r[ i ][ j ][ k ] is equal to 63, the value of rotation matrix is unspecified.

mantissa_r[ i ][ j ][ k ] specifies the mantissa part of (j, k) component of the rotation matrix for the i-th camera.

The length of the mantissa_r[ i ][ j ][ k ] syntax element in units of bits is determined as follows:

–	If exponent_r[ i ] = = 0, the length is Max( 0, prec_rotation_param − 30 ).

–	Otherwise (0 < exponent_r[ i ] < 63), the length is Max( 0, exponent_r[ i ] + prec_rotation_param − 31 ).

The rotation matrix R[ i ] for i-th camera is represented as follows:

r[i][0][0]r[i][0][1]r[i][0][2]r[ i ][ 1 ][ 0 ]r[i][1][1]r[i][1][2]r[ i ][ 2 ][ 0 ]r[i][2][1]r[i][2][2] 	(J-XX)

sign_t_x[ i ] equal to 0 indicates that the sign of the horizontal component of the translation vector for the i-th camera is positive. sign_trans_x[ i ] equal to 1 indicates that the sign of the horizontal component of the translation vector for the i-th camera is negative.

exponent_t_x[ i ] specifies the exponent part of the horizontal component of the translation vector for the i-th camera. The value of exponent_t_x[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU T | ISO/IEC. When exponent_t_x[ i ] is equal to 63, the value of the translation vector is unspecified.

mantissa_t_x[ i ] specifies the mantissa part of the horizontal component of the translation vector for the i-th camera.

The length v of the mantissa_t_x[ i ] syntax element in units of bits is determined as follows:

–	If exponent_t_x[ i ] = = 0, the length v = Max( 0, prec_translation_param − 30 ).

–	Otherwise (0 < exponent_t_x[ i ] < 63), the length v = Max( 0, exponent_t_x[ i ] + prec_translation_param − 31 ).

The variable x, where x refers to a focal length, principal point, rotation, or translation is computed as follows, where e = exponent, s = sign, n = mantissa and v = mantissa_len_minus1 + 1:

–	If 0 < e < 63, x = (−1)s * 2e−31 * (1 + n  2v).

–	Otherwise (e is equal to 0), x = (−1)s * 2−(30+v) * n.

NOTE – The above specification is similar to what found in IEC 60559:1989, Binary floating-point arithmetic for microprocessor systems.

Add a new subclause J.13.2.2.1 “Reconstruction process for displayable texture view (Informative)” as follows, with “J-XX+i” replaced with the appropriate equation number in the sequence of equations present in the text:

      - Reconstruction process for displayable texture view (Informative)

This subclause describes the reconstruction process for a displayable view A by using this SEI message.

Inputs to this process are:

- decoded base view of texture and depth.

- decoded non-base view of texture and depth, depending on the value of residual_depth_flag.

- decoded SEI message including sequence information.

The decoded SEI message is in the forms of:

  - GVDViewSynth.cfg

  - GVDCameraParam.txt

  - GVDCameraParam.CSV.

GVDViewSynth.cfg contains:

    - depth_type

    - num_residual_texture_views_minus1

    - residual_depth_flag

    - An indicater for GVDCameraParam.txt.

    - An indicater for GVDCameraParam.CSV

    - An indicater for base texture view

    - An indicator for base depth view

    - An indicator for non-base texture view

    - An indicator for non-base depth view

    - Indicaters for target views, including view A

GVDCameraParam.txt contains:

for(n = 0; n <= 4 * (num_residual_texture_views_minus1 + 1); n++)

      - intrinsic camera parameter

        - focal_length_x[ n ], principal_point_x[ n ]

        - focal_lenght_y[ n ], principal_point_y[ n ]

      - rotation matrix r[ n ][ j ][ k ]

      - translation t_x[ n ]

GVDCameraParam.CSV contains:

for(n = 0; n <= 4 * (num_residual_texture_views_minus1 + 1); n++)

      - Znear[ n ]

      - Zfar[ n ]

      - t_x[ n ]

n denotes the view number which is synthesized form the decoded views, including the decoded view itself. The relationship between the number n and the number i in the SEI message is as follows.

If num_residual_texture_views_minus1 <= 1, The SEI message parameters for i = 0 are assigned to the parameters for n = 4, The SEI message parameters for i = 1are assigned to the parameters for n = 0. The SEI message parameters for i = 2 are assigned to the parameters for n = 8.

Otherwise (2 <= num_residual_texture_views_minus1 <= 3), the SEI message parameters for i = 0 are assigned to the parameters for n = 8,The SEI message parameters for i = 1are assigned to the parameters for n = 4. The SEI message parameters for i = 2 are assigned to the parameters for n = 12. The SEI message parameters for i = 3 are assigned to the parameters for n = 0. The SEI message parameters for i = 4 are assigned to the parameters for n = 16.

The parameter values for other value of n are interpolated from the assigned parameters.

Output of this process is a displayable view, which is a target view A, for n = 0 to 4 * (num_residual_texture_views_minus1 + 1), excluding the base texture view.

The target view A is reconstructed by the following ordered steps:

–	When num_residual_texture_views_minus1 < 2, base view is called as base view B, value 4 is assigned to the variable b and value 0 is assigned to variable r1 in the following steps.

–		If the target view A is in-between the residual view of i = 1 and the base view of i = 0, the residual view of i = 1 is called as the residual view R and value 0 is assigned to the variable r in the following steps.

–		Otherwise, the target view A is in-between the base view of i = 0 and the residual view of i = 2, the residual view of i = 2 is called as the residual view R and value 8 is assigned to the variable r in the following steps.

–	Otherwise, base view is called as base view B, value 8 is assigned to the variable b and value 4 is assigned to variable r1 in the following steps.

–		If the target view A is in-between the residual view of i = 1 and the base view of i = 0, the residual view of i = 1 is called as the residual view R and value 4 is assigned to the variable r in the following steps.

–		If the target view A is in-between the base view of i = 0 and the residual view of i = 2, the residual view of i = 2 is called as the residual view R and value 12 is assigned to the variable r in the following steps.

–		If the target view A is in-between the residual view of i = 3 and the residual view of i = 1, the residual view of i = 3 is called as the residual view R and value 0 is assigned to the variable r in the following equations.

–		If the target view A is in-between the residual view of i = 2 and the residual view of i = 4, the residual view of i = 4 is called as the residual view R and value 16 is assigned to the variable r in the following steps.

- Depth reconstruction process

- –	When residual_depth_flag is equal to 1, every constituent residual depth picture associated with the residual view R is extracted out of the non-base depth view. The constituent residual depth picture is a down-sampled depth picture, which contains samples only in the 8-sample-dilated area where samples in the base depth view B are not projected. The constituent residual depth picture is 1:2 upsampled horizontally and then vertically two times to the residual depth view R, which is same size as the base texture view. The upsampling filer coefficients are (0.5, 0.5).

The upsampled residual depth view R is projected to the target view A by using the input parameters to this process. The projected depth sample address (xA, yA) in the target view A is given by the following equation.

xA = xR + f * LRA * (1 / Znear – 1 / Zfar) * D(xR, yR) / 255 + f * LRA / Zfar	(J-XX+1)

yA = yR

Where, (xR, yR) = sample address in the residual view R, f = focal_length of view R, LRA = baseline length between view R and view A = (translation_x[ r ] – translation_x[ n ]), Znear is the Znear of view R, Zfar is Zfar of view R, D(xR, yR) = depth value of sample (xR, yR) in view R.

- Samples in the target depth view A which have smaller depth values are overwritten by the larger values of the projected depth samples.

- –	In the base depth view, an intermediate depth sample in a slope of depth gap larger than 5 is changed its value to the closer depth value of the gap edges. This process eliminates undesirable ringing in the reconstructed displayable view.

- –	The base depth view is 1:2 upsampled horizontally and then vertically to the base depth view B. The upsanple filter is as follows.

- If (|Di - 1 – Di + 1| < g),  Di = (Di - 1 + Di + 1) / 2

- otherwise, if (Di - 1 > Di + 1),  Di = Di - 1 ,

- otherwise, Di = Di + 1

- Where, g is empirically given such as g = 5. Di - 1, Di and Di + 1 are depth values of horizontally or vertically contiguous three samples in the upsampled base depth view B. This filter eliminates undesirable ringing in the reconstructed view.

- –	The upsampled base depth view B is filtered by a median filter of 5 * 5 samples to smoothen the jaggy diagonal edges in the upsampled depth view B.

–	The filtered depth view B is projected to the target view A by using the input parameters to this process. The projected sample address (xA, yA) in the target view A is given by the following equation.

xA = xB + f * LBA * (1/Znear – 1/Zfar) * D(xB, yB) / 255 + f * LBA / Zfar	(J-XX+2)

yA = yB

Where, (xB, yB) = pixel address in the base view B, f = focal_length of view B, LBA = baseline length between view B and view A = translation_x[ b ] – translation_x[ n ], Znear is the Znear of view B, Zfar is the Zfar of view B and D(xB, yB) = depth value of sample (xB, yB) in view B.

–	When residual_depth_flag is equal to 0 and num_residual_texture_views_minus1 is an even number, the base depth view B is projected to the target depth view A from its center location between the base view B and the residual view R (i=1). Such pixel address (xA, yA) in the view A is given by the above equation J-XX+2, assuming LBA = (translation_x[ b ] + translation_x[ r1 ] ) / 2 – translation_x[ n ]) .

–	Samples in the target depth view A which have smaller depth values are overwritten by the larger values of the projected depth samples.

–	The resultant depth view A is filtered by a median filter of 3*3 samples to smoothen the diagonal edges.

- –	Remaining holes in the target depth view A are inpainted by the smaller depth value of existing left or right neighbour sample.

- Hole mask HB(x, y) generation and base view B projection process

- –	If the target view A is in-between the residual view R of i = 1 or 3 and the base view B (i = 0), project the base texture view B to the target view A as follows.

- –	for (δ = f * LBA / Znear ; δ >= 0; δ –= f * LBA / Znear / 16) , check the depth value D(x, y) in view A and generate a hole mask HB(x, y) of view A as follows.

- DΔ1 = D(x + δ, y) – D(x, y)	(J-XX+3)

- disp = f * LBA * DΔ1 * (1 / Znear  – 1 / Zfar) / 255

- if (DΔ1 >= ga)

- DΔ2 = D(x + disp, y) – D(x, y)

- if (DΔ2 > c * DΔ1)

- HB(x, y) = hole

Where, ga is empirically given such as 8, f = focal_length of view B, LBA = baseline length between view B and view A = (translation_x[ b ] – translation_x[ n ]), Znear is the Znear of view B, Zfar is the Zfar of view B and c is empirically given such as 0.5..

–		A texture sample B(u, v) in the base view B is projected to the sample A(x, y) in the target view A, where HB(x, y) ≠ hole. The sample address (u, v) in the base view B is given by the following equation.

u = x – f * LBA * (1/Znear – 1/Zfar) * D(x, y) / 255 – f * LBA / Zfar	(J-XX+4)

v = y

–	Otherwise, the target view A is in-between the base view B (i = 0) and the residual view R of i = 2 or 4. Project the base texture view B to the target view A as follows..

–	for (δ = f * LAB / Znear ; δ >= 0; δ –= f * LAB / Znear / 16) , check the depth value D(x, y) in view A and generate a hole mask HB(x, y) of view A as follows.

DΔ1 = D(x – δ, y) – D(x, y)	(J-XX+5)

disp = f * LAB * DΔ1 * (1 / Znear – 1 / Zfar) / 255

if (DΔ1 >= ga)

DΔ2 = D(x – disp, y) – D(x, y)

if (DΔ2 > c * DΔ1)

HB(x, y) = hole

Where, ga is empirically given such as 8, f = focal_length of view B, LAB = baseline length between view A and view B = (translation_x[ n ] – translation_x[ b ]), Znear is the Znear of view B, Zfar is the Zfar of view B and c is empirically given such as 0.5..

–		A texture sample B(u, v) in the base view B is projected to the sample A(x, y) in the target view A, where HB(x, y) ≠ hole. The sample address (u, v) in the base view B is given by the following equation.

u = x + f * LAB * (1 / Znear – 1 / Zfar) * D(x, y) / 255 + f * LAB / Zfar	(J-XX+6)

v = y

–	If u is not an integer, the sample value A(x, y) is given by the weighted average of two contiguous samples by the following equation.

A(x, y) = (int(u) + 1 – u) * B(int(u), v) + (u – int(u) * B(int(u) + 1, v)	(J-XX+7)

- Hole mask HR(x, y) generation and residual view R projection process

–	Extract every constituent residual picture out of the non-base view. The constituent residual picture is a down-sampled picture, which contains samples only in the 8-sample-dilated area where samples in the base view B are not projected. This area might be determined by logical OR operation with the corresponding such area in the other intermediate target views. The constituent residual picture is 1:2 upsampled horizontally and then vertically to the residual view R, which is same size as the base texture view B. The upsampling filer coefficients are (0.5, 0.5).

- –	Dilate hole mask HB(x, y) by 1 sample to allow the coded degradation of the depth edge.

- –	for (HB(x, y) == hole), check the depth value D(x, y) in view A and project the residual texture view R to the target view A as follows.

–	if the target view A is in-between the residual view R of i = 1or 3 and the base view B (i = 0), check the depth value D(x, y) in view A and generate hole mask HR(x, y) of view A as follows.

–	for (δ = f * LAR / Znear ; δ >= 0; δ –= f * LAR / Znear / 16)	(J-XX+8)

DΔ1 = D(x – δ, y) – D(x, y)

disp = f * LAR * DΔ1 *(1 / Znear  – 1 / Zfar) / 255

if (DΔ1 >= ga)

DΔ2 = D(x – disp, y) – D(x, y)

if (DΔ2 > c * DΔ1)

HR(x, y) = hole

Where, ga is empirically given such as 8, f = focal_length of view R, LAR = baseline length between view A and view R = (translation_x[ n ] – translation_x[ r ]), Znear is the Znear of view R, Zfar is the Zfar of view R and c is empirically given such as 0.5..

–	A texture sample R(u, v) in the residual view R is projected to the sample A(x, y) in the target view A, where HR(x, y) ≠ hole. The sample address (u, v) in the residual view R is given by the following equation.

u = x + f * LAR * (1 / Znear – 1 / Zfar) * D(x, y) / 255 + f * LAR / Zfar	(J-XX+9)

v = y

–	Otherwise, target view A is in-between the base view B (i = 0) and the residual view R of i = 2 or 4. Check the depth value D(x, y) in view A and generate hole mask HR(x, y) of view A as follows.

–	for (δ = f * LRA / Znear ; δ >= 0; δ –= f * LRA / Znear / 16) , check the depth view A to find holes as follows.

DΔ1 = D(x + δ, y) – D(x, y)	(J-XX+10)

disp = f * LRA * DΔ1 * (1 / Znear – 1 / Zfar) / 255

if (DΔ1 >= ga)

DΔ2 = D(x + disp, y) – D(x, y)

if (DΔ2 > c * DΔ1)

HR(x, y) = hole

Where, ga is empirically given such as 8, f = focal_length of view R, LRA = baseline length between view R and view A = (translation_x[ r ] – translation_x[ n ]), Znear is the Znear of view R, Zfar is the Zfar of view R and c is empirically given such as 0.5..

–		A texture sample R(u, v) in the residual view R is projected to the sample A(x, y) in the target view A, where HR(x, y) ≠ hole. The sample address (u, v) in the base view R is given by the following equation.

u = x – f * LRA * (1 / Znear– 1 / Zfar) * D(x, y) / 255 – f * LRA / Zfar	(J-XX+11)

v = y

–	If u is not an integer, the sample value A(x, y) is given by the weighted average of two contiguous samples by the following equation.

A(x, y) = (int(u) + 1 – u) * R(int(u), v) + (u – int(u) * R(int(u) + 1, v)	(J-XX+12)

- Remaining hole inpainting and a displayable view A outputting process

–	Iinpaint samples A(x, y), where HB(x, y) = hole and HR(x, y) = hole, by the existing surrounding samples.

–	Smoothen the two samples A(x, y) at the both side of the edge of hole mask HB(x, y) and HR(x, y) by a 3-tap filter of (0.25, 0.5, 0.25) vertically and horizontally.

–	Output the resultant texture view A as a displayable view.