| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18–24 Oct. 2014 | Document: JCT3V-J1007 |
| --- | --- |

| Title: | Draft 2 of MFC plus Depth |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-3V |  |  |
| Purpose: | Draft Text |  |  |
| Author(s) or Contact(s): | Peng Yin Dolby Laboratories, Inc. Miska M. Hannuksela Nokia Ying Chen Qualcomm Jens-Rainer Ohm RWTH Aachen University Gary J. Sullivan Microsoft | Email: Email: Email: Email: Email: | pyin@dolby.com miska.hannuksela@nokia.com cheny@qti.qualcomm.com ohm@ient.rwth-aachen.de garysull@microsoft.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This amendment defines a new profile appropriate for inclusion of MFC plus depth. The new profile is specified as MFC Depth High profile. This amendment also includes the addition of ST. 2084, ST. 2085 and ST. 2086.

Subclause Error: Reference source not found specifies two profiles for MVCD (Multiview Depth High, and MFC Depth High).

In 7.3.2.1.1 (Sequence parameter set data syntax), replace the syntax table with the following:

| seq_parameter_set_data( ) { | C | Descriptor |
| --- | --- | --- |
| profile_idc | 0 | u(8) |
| constraint_set0_flag | 0 | u(1) |
| constraint_set1_flag | 0 | u(1) |
| constraint_set2_flag | 0 | u(1) |
| constraint_set3_flag | 0 | u(1) |
| constraint_set4_flag | 0 | u(1) |
| constraint_set5_flag | 0 | u(1) |
| reserved_zero_2bits /* equal to 0 */ | 0 | u(2) |
| level_idc | 0 | u(8) |
| seq_parameter_set_id | 0 | ue(v) |
| if( profile_idc = = 100 ¦ ¦ profile_idc = = 110 ¦ ¦ profile_idc = = 122 ¦ ¦ profile_idc = = 244 ¦ ¦ profile_idc = = 44 ¦ ¦ profile_idc = = 83 ¦ ¦ profile_idc = = 86 ¦ ¦ profile_idc = = 118 ¦ ¦ profile_idc = = 128 ¦ ¦ profile_idc = = 138 ¦ ¦ profile_idc = = 139 ¦ ¦ profile_idc = = 134 ¦ ¦ profile_idc = = 135 ) { |  |  |
| chroma_format_idc | 0 | ue(v) |
| if( chroma_format_idc = = 3 ) |  |  |
| separate_colour_plane_flag | 0 | u(1) |
| bit_depth_luma_minus8 | 0 | ue(v) |
| bit_depth_chroma_minus8 | 0 | ue(v) |
| qpprime_y_zero_transform_bypass_flag | 0 | u(1) |
| seq_scaling_matrix_present_flag | 0 | u(1) |
| if( seq_scaling_matrix_present_flag ) |  |  |
| for( i = 0; i < ( ( chroma_format_idc != 3 ) ? 8 : 12 ); i++ ) { |  |  |
| seq_scaling_list_present_flag[ i ] | 0 | u(1) |
| if( seq_scaling_list_present_flag[ i ] ) |  |  |
| if( i < 6 ) |  |  |
| scaling_list( ScalingList4x4[ i ], 16, UseDefaultScalingMatrix4x4Flag[ i ]) | 0 |  |
| else |  |  |
| scaling_list( ScalingList8x8[ i − 6 ], 64, UseDefaultScalingMatrix8x8Flag[ i − 6 ] ) | 0 |  |
| } |  |  |
| } |  |  |
| log2_max_frame_num_minus4 | 0 | ue(v) |
| pic_order_cnt_type | 0 | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |  |
| log2_max_pic_order_cnt_lsb_minus4 | 0 | ue(v) |
| else if( pic_order_cnt_type = = 1 ) { |  |  |
| delta_pic_order_always_zero_flag | 0 | u(1) |
| offset_for_non_ref_pic | 0 | se(v) |
| offset_for_top_to_bottom_field | 0 | se(v) |
| num_ref_frames_in_pic_order_cnt_cycle | 0 | ue(v) |
| for( i = 0; i < num_ref_frames_in_pic_order_cnt_cycle; i++ ) |  |  |
| offset_for_ref_frame[ i ] | 0 | se(v) |
| } |  |  |
| max_num_ref_frames | 0 | ue(v) |
| gaps_in_frame_num_value_allowed_flag | 0 | u(1) |
| pic_width_in_mbs_minus1 | 0 | ue(v) |
| pic_height_in_map_units_minus1 | 0 | ue(v) |
| frame_mbs_only_flag | 0 | u(1) |
| if( !frame_mbs_only_flag ) |  |  |
| mb_adaptive_frame_field_flag | 0 | u(1) |
| direct_8x8_inference_flag | 0 | u(1) |
| frame_cropping_flag | 0 | u(1) |
| if( frame_cropping_flag ) { |  |  |
| frame_crop_left_offset | 0 | ue(v) |
| frame_crop_right_offset | 0 | ue(v) |
| frame_crop_top_offset | 0 | ue(v) |
| frame_crop_bottom_offset | 0 | ue(v) |
| } |  |  |
| vui_parameters_present_flag | 0 | u(1) |
| if( vui_parameters_present_flag ) |  |  |
| vui_parameters( ) | 0 |  |
| } |  |  |

In 7.3.2.1.3 (Subset sequence parameter set RBSP syntax), replace the syntax table with the following:

| subset_seq_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| seq_parameter_set_data( ) | 0 |  |
| if( profile_idc = = 83 ¦ ¦ profile_idc = = 86 ) { |  |  |
| seq_parameter_set_svc_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| svc_vui_parameters_present_flag | 0 | u(1) |
| if( svc_vui_parameters_present_flag = = 1 ) |  |  |
| svc_vui_parameters_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| } else if( profile_idc = = 118 ¦ ¦ profile_idc = = 128 ¦ ¦ profile_idc = = 134 ) { |  |  |
| bit_equal_to_one /* equal to 1 */ | 0 | f(1) |
| seq_parameter_set_mvc_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| mvc_vui_parameters_present_flag | 0 | u(1) |
| if( mvc_vui_parameters_present_flag = = 1 ) |  |  |
| mvc_vui_parameters_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| } else if( profile_idc = = 138 ¦ ¦ profile_idc = = 135 ) { |  |  |
| bit_equal_to_one /* equal to 1 */ | 0 | f(1) |
| seq_parameter_set_mvcd_extension( ) /* specified in Annex Error: Reference source not found */ |  |  |
| } else if( profile_idc = = 139 ) { |  |  |
| bit_equal_to_one /* equal to 1 */ | 0 | f(1) |
| seq_parameter_set_mvcd_extension( ) /* specified in Annex Error: Reference source not found */ |  |  |
| seq_parameter_set_3davc_extension( ) /* specified in Annex J */ | 0 |  |
| } |  |  |
| additional_extension2_flag | 0 | u(1) |
| if( additional_extension2_flag = = 1 ) |  |  |
| while( more_rbsp_data( ) ) |  |  |
| additional_extension2_data_flag | 0 | u(1) |
| rbsp_trailing_bits( ) | 0 |  |
| } |  |  |

In D.1 (SEI payload syntax), insert the following before the row containing only "else":

| else if( payloadType = = 137 ) |  |  |
| --- | --- | --- |
| mastering_display_colour_volume( payloadSize ) | 5 |  |

Renumber D.1.27 (Reserved SEI message syntax) as D.1.28.

Add the following new subclause as D.1.27:

D.1.27	Mastering display colour volume SEI message syntax

| mastering_display_colour_volume( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| for( c = 0; c < 3; c++ ) { |  |  |
| display_primaries_x[ c ] | 5 | u(16) |
| display_primaries_y[ c ] | 5 | u(16) |
| } |  |  |
| white_point_x | 5 | u(16) |
| white_point_y | 5 | u(16) |
| max_display_mastering_luminance | 5 | u(32) |
| min_display_mastering_luminance | 5 | u(32) |
| } |  |  |

Renumber D.2.27 (Reserved SEI message semantics) as D.2.28.

Add the following new subclause as D.2.27:

D.2.27	Mastering display colour volume SEI message semantics

This SEI message identifies the colour volume (the colour primaries, white point, and luminance range) of a display considered to be the mastering display for the associated video content – e.g., the colour volume of a display that was used for viewing while authoring the video content. The described mastering display is a three-colour additive display system that has been configured to use the indicated mastering colour volume.

This SEI message does not specify the measurement methodologies and procedures used for determining the indicated values or any description of the mastering environment. It also does not provide information on colour transformations that would be appropriate to preserve creative intent on displays with colour volumes different from that of the described mastering display.

The information conveyed in this SEI message is intended to be adequate for purposes corresponding to the use of Society of Motion Picture and Television Engineers ST 2086.

The following constraints apply for the presence of mastering display colour volume SEI messages in IDR access units:

–	When a mastering display colour volume SEI message is present in any access unit of a coded video sequence and the mastering display colour volume SEI message is not contained within any other SEI message, a mastering display colour volume SEI message that is not contained within any other SEI message shall be present in the IDR access unit that is the first access unit of the coded video sequence.

–	When a mastering display colour volume SEI message is present in any access unit of a coded video sequence and the mastering display colour volume SEI message is contained in a scalable nesting SEI message applying to dependency_id dId, quality_id qId, and temporal_id tId, a mastering display colour volume SEI message that is contained in a scalable nesting SEI message applying to dependency_id equal to dId, quality_id equal to qId, and temporal_id equal to tId shall be present in the IDR access unit that is the first access unit of the coded video sequence.

–	When a mastering display colour volume SEI message is present in any access unit of a coded video sequence and the mastering display colour volume SEI message is contained in an MVC scalable nesting SEI message applying to view_id vId and temporal_id tId, a mastering display colour volume SEI message that is contained in an MVC scalable nesting SEI message applying to view_id equal to vId and temporal_id equal to tId shall be present in the IDR access unit that is the first access unit of the coded video sequence.

–	When a mastering display colour volume SEI message is present in any access unit of a coded video sequence and the mastering display colour volume SEI message is contained in an MVCD scalable nesting SEI message applying to texture views with view_id vId and temporal_id tId, a mastering display colour volume SEI message that is contained in an MVCD scalable nesting SEI message applying to texture views with view_id equal to vId and temporal_id equal to tId shall be present in the IDR access unit that is the first access unit of the coded video sequence.

The mastering display colour volume SEI message persists in decoding order from the current access until the end of the coded video sequence.

When a mastering display colour volume SEI message is not contained within any other SEI message, it pertains only to VCL NAL units with nal_unit_type in the range of 1 to 5, inclusive.

NOTE – When the bitstream is a scalable video bitstream according to Annex G, a mastering display colour volume SEI message that is not contained within any other SEI message applies only to the base layer bitstream. When the bitstream is a multiview video bitstream according to Annex H, a mastering display colour volume SEI message that is not contained within any other SEI message applies only to the base layer bitstream. When the bitstream is a multiview video bitstream with depth according to Annex I or Annex J, a mastering display colour volume SEI message that is not contained within any other SEI message applies only to the base texture view.

The following constraints apply for the content of mastering display colour volume SEI messages:

–	All mastering display colour volume SEI messages that apply to the same coded video sequence and are not contained within any other SEI message shall have the same content.

–	All mastering display colour volume SEI messages that apply to the same coded video sequence and are contained in a scalable nesting SEI message applying to particular values of dependency_id, quality_id, and temporal_id shall have the same content.

–	All mastering display colour volume SEI messages that apply to the same coded video sequence and are contained in an MVC scalable nesting SEI message applying to particular values of view_id and temporal_id shall have the same content.

–	All mastering display colour volume SEI messages that apply to the same coded video sequence and are contained in an MVCD scalable nesting SEI message applying to texture views with particular values of view_id and temporal_id shall have the same content.

display_primaries_x[ c ] and display_primaries_y[ c ] specify the normalized x and y chromaticity coordinates, respectively, of the colour primary component c of the mastering display in increments of 0.00002, according to the CIE 1931 definition of x and y as specified in ISO 11664-1 (see also ISO 11664-3 and CIE 15). For describing mastering displays that use red, green, and blue colour primaries, it is suggested that index value c equal to 0 should correspond to the green primary, c equal to 1 should correspond to the blue primary, and c equal to 2 should correspond to the red colour primary (see also Annex Error: Reference source not found and Table E-1). The values of display_primaries_x[ c ] and display_primaries_y[ c ] shall be in the range of 0 to 50 000, inclusive.

white_point_x and white_point_y specify the normalized x and y chromaticity coordinates, respectively, of the white point of the mastering display in normalized increments of 0.00002, according to the CIE 1931 definition of x and y as specified in ISO 11664-1 (see also ISO 11664-3 and CIE 15). The values of white_point_x and white_point_y shall be in the range of 0 to 50 000.

max_display_mastering_luminance and min_display_mastering_luminance specify the nominal maximum and minimum display luminance, respectively, of the mastering display in units of 0.0001 candelas per square metre. min_display_mastering_luminance shall be less than max_display_mastering_luminance.

At minimum luminance, the mastering display is considered to have the same nominal chromaticity as the white point.

Replace Table E-3 with the following:

Table E-1 – Colour primaries

| - Value | - Primaries | - Informative Remark |
| --- | --- | --- |
| - 0 | - Reserved | - For future use by ITU-T ¦ ISO/IEC |
| - 1 | - primary x y - green 0.300 0.600 - blue 0.150 0.060 - red 0.640 0.330 - white D65 0.3127 0.3290 | Rec. ITU-R BT.709-5 Rec. ITU-R BT.1361 conventional colour gamut system and extended colour gamut system IEC 61966-2-1 (sRGB or sYCC) IEC 61966-2-4 Society of Motion Picture and Television Engineers RP 177 (1993) Annex B |
| - 2 | - Unspecified | - Image characteristics are unknown or are determined by the application. |
| - 3 | - Reserved | - For future use by ITU-T ¦ ISO/IEC |
| - 4 | - primary x y - green 0.21 0.71 - blue 0.14 0.08 - red 0.67 0.33 - white C 0.310 0.316 | - Rec. ITU-R BT.470-6 System M (historical) - United States National Television System Committee 1953 Recommendation for transmission standards for colour television - United States Federal Communications Commission Title 47 Code of Federal Regulations (2003) 73.682 (a) (20) |
| - 5 | - primary x y - green 0.29 0.60 - blue 0.15 0.06 - red 0.64 0.33 - white D65 0.3127 0.3290 | - Rec. ITU-R BT.470-6 System B, G (historical) - Rec. ITU-R BT.601-6 625 - Rec. ITU-R BT.1358 625 - Rec. ITU-R BT.1700 625 PAL and 625 SECAM |
| - 6 | - primary x y - green 0.310 0.595 - blue 0.155 0.070 - red 0.630 0.340 - white D65 0.3127 0.3290 | - Rec. ITU-R BT.601-6 525 - Rec. ITU-R BT.1358 525 - Rec. ITU-R BT.1700 NTSC - Society of Motion Picture and Television Engineers 170M (2004) - (functionally the same as the value 7) |
| - 7 | - primary x y - green 0.310 0.595 - blue 0.155 0.070 - red 0.630 0.340 - white D65 0.3127 0.3290 | - Society of Motion Picture and Television Engineers 240M (1999) - (functionally the same as the value 6) |
| - 8 | - primary x y - green 0.243 0.692 (Wratten 58) - blue 0.145 0.049 (Wratten 47) - red 0.681 0.319 (Wratten 25) - white C 0.310 0.316 | - Generic film (colour filters using Illuminant C) |
| - 9 | - primary x y - green 0.170 0.797 - blue 0.131 0.046 - red 0.708 0.292 - white D65 0.3127 0.3290 | - Rec. ITU-R BT.2020 |
| - 10 | - primary x y - Y 0.0 1.0 - Z 0.0 0.0 - X 1.0 0.0 - centre white 1 ÷ 3 1 ÷ 3 | - Society of Motion Picture and Television Engineers ST 428-1 - (CIE 1931 XYZ) |
| - 11 | - primary x y - green 0.264 0.690 - blue 0.150 0.060 - red 0.680 0.320 - white DCI 0.314 0.351 | - Society of Motion Picture and Television Engineers ST 431-2 - (DCI P3) |
| - 12 | - primary x y - green 0.264 0.690 - blue 0.150 0.060 - red 0.680 0.320 - white D65 0.3127 0.3290 | - Society of Motion Picture and Television Engineers ST 432-1 - (P3 D65) |
| - 13..255 | - Reserved | - For future use by ITU-T ¦ ISO/IEC |

In E.2 (VUI semantics), replace the semantics of transfer_characteristics with the following:

transfer_characteristics indicates the opto-electronic transfer characteristic of the source picture as specified in Table E-2 as a function of a linear optical intensity input Lc with a nominal real-valued range of 0 to 1. For interpretation of entries in Table E-2 that are expressed in terms of multiple curve segments parameterized by the variable  over a region bounded by the variable  or by the variables  and , the values of  and  are defined to be the positive constants necessary for the curve segments that meet at the value  to have continuity of value and continuity of slope at the value , and the value of , when applicable, is defined to be the positive constant necessary for the associated curve segments to meet at the value . For example, for transfer_characteristics equal to 1, 6, 14, or 15,  has the value 1 + 5.5 *  = 1.099 296 826 809 442... and  has the value 0.018 053 968 510 807....

When the transfer_characteristics syntax element is not present, the value of transfer_characteristics is inferred to be equal to 2 (the transfer characteristics are unspecified or are determined by the application). Values of transfer_characteristics that are identified as reserved in Table E-2 are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall interpret reserved values of transfer_characteristics as equivalent to the value 2.

Replace Table E-4 with the following:

Table E-2 – Transfer characteristics

| Value | Transfer Characteristic | Informative Remark |
| --- | --- | --- |
| 0 | Reserved | For future use by ITU-T ¦ ISO/IEC |
| 1 | V =  * Lc0.45 − (  − 1 ) for 1 >= Lc >=  V = 4.500 * Lc for  > Lc >= 0 | Rec. ITU-R BT.709-5 Rec. ITU-R BT.1361 conventional colour gamut system (functionally the same as the values 6, 14, and 15) |
| 2 | Unspecified | Image characteristics are unknown or are determined by the application. |
| 3 | Reserved | For future use by ITU-T ¦ ISO/IEC |
| 4 | Assumed display gamma 2.2 | Rec. ITU-R BT.470-6 System M (historical) United States National Television System Committee 1953 Recommendation for transmission standards for colour television United States Federal Communications Commission Title 47 Code of Federal Regulations (2003) 73.682 (a) (20) Rec. ITU-R BT.1700 (2007 revision) 625 PAL and 625 SECAM |
| 5 | Assumed display gamma 2.8 | Rec. ITU-R BT.470-6 System B, G (historical) |
| 6 | V =  * Lc0.45 − (  − 1 ) for 1 >= Lc >=  V = 4.500 * Lc for  > Lc >= 0 | Rec. ITU-R BT.601-6 525 or 625 Rec. ITU-R BT.1358 525 or 625 Rec. ITU-R BT.1700 NTSC Society of Motion Picture and Television Engineers 170M (2004) (functionally the same as the values 1, 14, and 15) |
| 7 | V =  * Lc0.45 − (  − 1 ) for 1 >= Lc >=  V = 4.0 * Lc for  > Lc >= 0 | Society of Motion Picture and Television Engineers 240M (1999) |
| 8 | V = Lc for all values of Lc | Linear transfer characteristics |
| 9 | V = 1.0 + Log10( Lc )  2 for 1 >= Lc >= 0.01 V = 0.0 for 0.01 > Lc >= 0 | Logarithmic transfer characteristic (100:1 range) |
| 10 | V = 1.0 + Log10( Lc )  2.5 for 1 >= Lc >= Sqrt( 10 )  1000 V = 0.0 for Sqrt( 10 )  1000 > Lc >= 0 | Logarithmic transfer characteristic (100 * Sqrt( 10 ) : 1 range) |
| 11 | V =  * Lc0.45 − (  − 1 ) for Lc >=  V = 4.500 * Lc for  > Lc > − V = − * ( −Lc )0.45 + (  − 1 ) for − >= Lc | IEC 61966-2-4 |
| 12 | V =  * Lc0.45 − (  − 1 ) for 1.33 > Lc >=  V = 4.500 * Lc for  > Lc >= − V = −(  * ( −4 * Lc )0.45 − (  − 1 ) )  4 for − > Lc >= −0.25 | Rec. ITU-R BT.1361 extended colour gamut system |
| 13 | V =  * Lc( 1  2.4 ) − (  − 1 ) for 1 >= Lc >=  V = 12.92 * Lc for  > Lc >= 0 | IEC 61966-2-1 (sRGB or sYCC) |
| 14 | V = * Lc0.45 − (  − 1 ) for 1 >= Lc >=  V = 4.500 * Lc for  > Lc >= 0 | Rec. ITU-R BT.2020 (functionally the same as the values 1, 6, and 15) |
| 15 | V = * Lc0.45 − (  − 1 ) for 1 >= Lc >=  V = 4.500 * Lc for  > Lc >= 0 | Rec. ITU-R BT.2020 (functionally the same as the values 1, 6, and 14) |
| 16 | V = ( ( c1 + c2 * Lcn ) ÷ ( 1 + c3 * Lcn ) )m for all values of Lc c1 = c3 − c2 + 1 = 3424 ÷ 4096 = 0.8359375 c2 = 32 * 2413 ÷ 4096 = 18.8515625 c3 = 32 * 2392 ÷ 4096 = 18.6875 m = 128 * 2523 ÷ 4096 = 78.84375 n = 0.25 * 2610 ÷ 4096 = 0.1593017578125 for which Lc equal to 1 for peak white is ordinarily intended to correspond to a display luminance level of 10 000 candelas per square metre | Society of Motion Picture and Television Engineers ST 2084 for 10, 12, 14, and 16-bit systems. |
| 17 | V = ( 48 * Lc ÷ 52.37 )( 1  2.6 ) for all values of Lc for which Lc equal to 1 for peak white is ordinarily intended to correspond to a display luminance level of 48 candelas per square metre | Society of Motion Picture and Television Engineers ST 428-1 |
| 18..255 | Reserved | For future use by ITU-T ¦ ISO/IEC |

In E.2.1 (VUI parameters semantics), replace the semantics of matrix_coefficients with the following:

- matrix_coefficients describes the matrix coefficients used in deriving luma and chroma signals from the green, blue, and red, or Y, Z, and X primaries, as specified in Replace Table E-5 with the following:.

- matrix_coefficients shall not be equal to 0 unless one or more of the following conditions are true:

- –	BitDepthC is equal to BitDepthY.

- –	chroma_format_idc is equal to 3 (4:4:4).

- The specification of the use of matrix_coefficients equal to 0 under all other conditions is reserved for future use by ITU-T | ISO/IEC.

- matrix_coefficients shall not be equal to 8 unless one of the following conditions is true:

- –	BitDepthC is equal to BitDepthY,

- –	BitDepthC is equal to BitDepthY + 1 and chroma_format_idc is equal to 3 (4:4:4).

- The specification of the use of matrix_coefficients equal to 8 under all other conditions is reserved for future use by ITU-T | ISO/IEC.

- When the matrix_coefficients syntax element is not present, the value of matrix_coefficients is inferred to be equal to 2 (unspecified).

- The interpretation of matrix_coefficients, together with colour_primaries and transfer_characteristics, is specified by the equations below.

NOTE 3 – For purposes of YZX representation when matrix_coefficients is equal to 0, the symbols R, G, and B are substituted for X, Y, and Z, respectively, in the following descriptions of Equations E-19 to E-3, E-7 to E-9, E-13 to E-15, and E-19 to E-21.

- ER, EG, and EB are defined as "linear-domain" real-valued signals based on the indicated colour primaries before application of the transfer characteristics function. The application of the transfer characteristics function is denoted by ( x )′ for an argument x. The signals E′R, E′G, and E′B are determined by application of the transfer characteristics function as follows:

E′R = ( ER )′		(E-1)

E′G = ( EG )′		(E-2)

E′B = ( EB )′		(E-3)

- The range of E′R, E′G, and E′B is specified as follows:

–	If transfer_characteristics is not equal to 11 or 12, E′R, E′G, and E′B are real numbers with values in the range of 0 to 1, inclusive.

–	Otherwise, (transfer_characteristics is equal to 11 (IEC 61966-2-4) or 12 (Rec. ITU-R BT.1361 extended colour gamut system) ), E′R, E′G and E′B are real numbers with a larger range not specified in this Specification.

Nominal white is specified as having E′R equal to 1, E′G equal to 1, and E′B equal to 1.

Nominal black is specified as having E′R equal to 0, E′G equal to 0, and E′B equal to 0.

- The interpretation of matrix_coefficients is specified as follows:

–	If video_full_range_flag is equal to 0, the following applies:

–	If matrix_coefficients is equal to 1, 4, 5, 6, 7, 9, 10 or 11, the following equations apply:

Y = Clip1Y( Round( ( 1  <<  ( BitDepthY − 8 ) ) * ( 219 * E′Y + 16 ) ) )	(E-4)

Cb = Clip1C( Round( ( 1  <<  ( BitDepthC − 8 ) ) * ( 224 * E′PB + 128 ) ) )	(E-5)

Cr = Clip1C( Round( ( 1  <<  ( BitDepthC − 8 ) ) * ( 224 * E′PR + 128 ) ) )	(E-6)

–	Otherwise, if matrix_coefficients is equal to 0 or 8, the following equations apply:

R = Clip1Y( ( 1  <<  ( BitDepthY − 8 ) ) * ( 219 * E′R + 16 ) )	(E-7)

G = Clip1Y( ( 1  <<  ( BitDepthY − 8 ) ) * ( 219 * E′G + 16 ) )	(E-8)

B = Clip1Y( ( 1  <<  ( BitDepthY − 8 ) ) * ( 219 * E′B + 16 ) )	(E-9)

–	Otherwise, if matrix_coefficients is equal to 2, the interpretation of the matrix_coefficients syntax element is unknown or is determined by the application.

–	Otherwise (matrix_coefficients is not equal to 0, 1, 2, 4, 5, 6, 7, 8, 9, 10 or 11), the interpretation of the matrix_coefficients syntax element is reserved for future definition by ITU-T | ISO/IEC.

–	Otherwise (video_full_range_flag is equal to 1), the following applies:

–	If matrix_coefficients is equal to 1, 4, 5, 6, 7, 9, 10 or 11 the following equations apply:

Y = Clip1Y( Round( ( ( 1  <<  BitDepthY ) − 1 ) * E′Y ) )	(E-10)

Cb = Clip1C( Round( ( ( 1  <<  BitDepthC ) − 1 )  * E′PB + ( 1  <<  ( BitDepthC − 1 ) ) ) )	(E-11)

Cr = Clip1C( Round( ( ( 1  <<  BitDepthC ) − 1 )  * E′PR + ( 1  <<  ( BitDepthC − 1 ) ) ) )	(E-12)

–	Otherwise, if matrix_coefficients is equal to 0 or 8, the following equations apply:

R = Clip1Y( ( ( 1  <<  BitDepthY ) − 1 ) * E′R )		(E-13)

G = Clip1Y( ( ( 1  <<  BitDepthY ) − 1 ) * E′G )		(E-14)

B = Clip1Y( ( ( 1  <<  BitDepthY ) − 1 ) * E′B )		(E-15)

–	Otherwise, if matrix_coefficients is equal to 2, the interpretation of the matrix_coefficients syntax element is unknown or is determined by the application.

–	Otherwise (matrix_coefficients is not equal to 0, 1, 2, 4, 5, 6, 7, 8, 9, 10 or 11), the interpretation of the matrix_coefficients syntax element is reserved for future definition by ITU-T | ISO/IEC. Reserved values for matrix_coefficients shall not be present in bitstreams conforming to this version of this Specification. Decoders shall interpret reserved values of matrix_coefficients as equivalent to the value 2.

- The variables E′Y, E′PB, and E′PR (for matrix_coefficients not equal to 0 or 8) or Y, Cb, and Cr (for matrix_coefficients equal to 0 or 8) are specified as follows:

–	If matrix_coefficients is not equal to 0, 8, 10, or 11, the following equations apply:

E′Y = KR * E′R + ( 1 − KR − KB ) * E′G + KB * E′B		(E-16)

E′PB = 0.5 * ( E′B − E′Y ) ÷ ( 1 − KB )		(E-17)

E′PR = 0.5 * ( E′R − E′Y ) ÷ ( 1 − KR )		(E-18)

NOTE 4 – E′Y is a real number with the value 0 associated with nominal black and the value 1 associated with nominal white. E′PB and E′PR are real numbers with the value 0 associated with both nominal black and nominal white. When transfer_characteristics is not equal to 11 or 12, E′Y is a real number with values in the range of 0 to 1, inclusive. When transfer_characteristics is not equal to 11 or 12, E′PB and E′PR are real numbers with values in the range of −0.5 to 0.5, inclusive. When transfer_characteristics is equal to 11 (IEC 61966-2-4), or 12 (ITU-R BT.1361 extended colour gamut system), E′Y, E′PB and E′PR are real numbers with a larger range not specified in this Specification.

–	Otherwise, if matrix_coefficients is equal to 0, the following equations apply:

Y   = Round( G )			(E-19)

Cb = Round( B )			(E-20)

Cr = Round( R )			(E-21)

–	Otherwise, if matrix_coefficients is equal to 8, the following applies:

–	If BitDepthC is equal to BitDepthY, the following equations apply:

Y   = Round( 0.5 * G + 0.25 * ( R + B ) )		(E-22)

Cb = Round( 0.5 * G − 0.25 * ( R + B ) ) + ( 1  <<  ( BitDepthC − 1 ) )	(E-23)

Cr = Round( 0.5 * (R − B ) ) + ( 1  <<  ( BitDepthC − 1 ) )	(E-24)

NOTE 5 – For purposes of the YCgCo nomenclature used in Replace Table E-5 with the following:, Cb and Cr of Equations E-23 and E-24 may be referred to as Cg and Co, respectively. The inverse conversion for the above three equations should be computed as:

t   = Y − ( Cb − ( 1  <<  ( BitDepthC − 1 ) ) )		(E-25)

G = Clip1Y( Y + ( Cb − ( 1  <<  ( BitDepthC − 1 ) ) ) )	(E-26)

B = Clip1Y( t − ( Cr − ( 1  <<  ( BitDepthC − 1 ) ) ) )	(E-27)

R = Clip1Y( t + ( Cr − ( 1  <<  ( BitDepthC − 1 ) ) ) )	(E-28)

–	Otherwise (BitDepthC is not equal to BitDepthY), the following equations apply:

Cr = Round( R ) − Round( B ) + ( 1  <<  ( BitDepthC − 1 ) )	(E-29)

t = Round( B ) + ( ( Cr − ( 1  <<  ( BitDepthC − 1 ) ) )  >>  1 )	(E-30)

Cb = Round( G ) − t + ( 1  <<  ( BitDepthC − 1 ) )	(E-31)

Y = t + ( ( Cb − ( 1  <<  ( BitDepthC − 1 ) ) )  >>  1 )	(E-32)

NOTE 6 – For purposes of the YCgCo nomenclature used in Replace Table E-5 with the following:, Cb and Cr of Equations E-31 and E-29 may be referred to as Cg and Co, respectively. The inverse conversion for the above four equations should be computed as.

t   = Y − ( ( Cb − ( 1  <<  ( BitDepthC − 1 ) ) )  >>  1 )	(E-33)

G = Clip1Y( t + ( Cb − ( 1  <<  ( BitDepthC − 1 ) ) ) )	(E-34)

B = Clip1Y( t − ( ( Cr − ( 1  <<  ( BitDepthC − 1 ) ) )  >>  1 ) )	(E-35)

R = Clip1Y( B + ( Cr − ( 1  <<  ( BitDepthC − 1 ) ) ) )	(E-36)

–	Otherwise, if matrix_coefficients is equal to 10, the following applies:

EY = KR * ER + ( 1 − KR − KB ) * EG + KB * EB		(E-37)

E′Y = ( EY )′				(E-38)

NOTE 7 – In this case, EY is defined from the "linear-domain" signals for ER, EG, and EB, prior to application of the transfer characteristics function, which is then applied to produce the signal E′Y. EY and E′Y are analogue with the value 0 associated with nominal black and the value 1 associated with nominal white.

E′PB = ( E′B − E′Y ) ÷ ( 2 * NB )	for − NB  <=  E′B − E′Y  <=  0	(E-39)

E′PB = ( E′B − E′Y ) ÷ ( 2 * PB )	for 0 < E′B − E′Y  <=  PB	(E-40)

E′PR = ( E′R − E′Y ) ÷ ( 2 * NR )	for − NR  <=  E′R − E′Y   <=  0	(E-41)

E′PR = ( E′R − E′Y ) ÷ ( 2 * PR )	for 0 < E′R − E′Y   <=  PR	(E-42)

where the constants NB, PB, NR, and PR are determined by application of the transfer characteristics function to expressions involving the constants KB and KR as follows:

NB = ( 1 − KB )′		(E-43)

PB = 1 – ( KB )′ 		(E-44)

NR = ( 1 − KR )′		(E-45)

PR = 1 – ( KR )′			(E-46)

–	Otherwise (matrix_coefficients is equal to 11), the signal E′Y is determined by application of the transfer characteristics function as in Equation E-47, and Equations E-48 and E-49 apply for specification of the signals E′PB and E′PR:

E′Y = ( EY )′		(E-47)

E′PB = ( 0.986566 *E′B − E′Y ) ÷ 2	 	(E-48)

E′PR = ( E′R – 0.991902 *E′Y ) ÷ 2		(E-49)

NOTE 8 – For purposes of the Y′D′zD′x nomenclature used in Table E-3, E′PB and E′PR of Equations E-48 and E-49 may be referred to as D′z and D′x, respectively.

Replace Table E-5 with the following:

Table E-3 – Matrix coefficients

| - Value | - Matrix | - Informative remark |
| --- | --- | --- |
| - 0 | - GBR | - The identity matrix. - Typically used for GBR (often referred to as RGB); however, may also be used for YZX (often referred to as XYZ) - IEC 61966-2-1 (sRGB) - Society of Motion Picture and Television Engineers ST 428-1 (XYZ) - See Equations E-19 to E-21 |
| - 1 | - KR = 0.2126; KB = 0.0722 | ITU-R Rec. BT.709-5 ITU-R Rec. BT.1361 conventional colour gamut system and extended colour gamut system IEC 61966-2-1 (sRGB) IEC 61966-2-4 xvYCC709 Society of Motion Picture and Television Engineers RP 177 (1993) Annex B See Equations E-16 to E-18 |
| - 2 | - Unspecified | - Image characteristics are unknown or are determined by the application. |
| - 3 | - Reserved | - For future use by ITU-T ¦ ISO/IEC |
| - 4 | - KR = 0.30; KB = 0.11 | - United States Federal Communications Commission Title 47 Code of Federal Regulations (2003) 73.682 (a) (20) - See Equations E-16 to E-18 |
| - 5 | - KR = 0.299; KB = 0.114 | - ITU-R Rec. BT.470-6 System B, G (historical) - ITU-R Rec. BT.601-6 625 - ITU-R Rec. BT.1358 625 - ITU-R Rec. BT.1700 625 PAL and 625 SECAM - IEC 61966-2-4 xvYCC601 - (functionally the same as the value 6) - See Equations E-16 to E-18 |
| - 6 | - KR = 0.299; KB = 0.114 | - ITU-R Rec. BT.601-6 525 - ITU-R Rec. BT.1358 525 - ITU-R Rec. BT.1700 NTSC - Society of Motion Picture and Television Engineers 170M (2004) - (functionally the same as the value 5) - See Equations E-16 to E-18 |
| - 7 | - KR = 0.212; KB = 0.087 | - Society of Motion Picture and Television Engineers 240M (1999) - See Equations E-16 to E-18 |
| - 8 | - YCgCo | - See Equations E-22 to E-36 |
| - 9 | - KR = 0.2627; KB = 0.0593 | - Rec. ITU-R BT.2020 non-constant luminance system - See Equations E-16 to E-18 |
| - 10 | - KR = 0.2627; KB = 0.0593 | - Rec. ITU-R BT.2020 constant luminance system - See Equations E-37 to E-46 |
| - 12 | - Y′D′z D′x | - Society of Motion Picture and Television Engineers FCD 2085 (2014) - See Equations E-47 to E-49 |
| - 13..255 | - Reserved | - For future use by ITU-T ¦ ISO/IEC |

In Annex E, renumber the prior Equations E-43 through E-47 as Equations E-50 through E-54, and update the cross-references to these equations in A.3.1 and A.3.3 accordingly.

Add a new subclause I.10.1.2 as follows:

- I.10.1.2	 MFC Depth High profile

Bitstreams conforming to the MFC Depth High profile shall obey the following constraints:

–	The base view bitstream as specified in clause Error: Reference source not found shall obey all constraints of the High profile specified in clause Error: Reference source not found, and all active sequence parameter sets shall fulfill one of the following conditions:

–	profile_idc is equal to 77 or constraint_set1_flag is equal to 1,

–	profile_idc is equal to 100.

–	The sub-bitstream of stereoscopic texture bitstream, as specified in clause Error: Reference source not found, shall obey all constraints of the MFC High profile specified in clause Error: Reference source not found and all active MVC sequence parameter sets shall fulfill one of the following conditions:

–	profile_idc is equal to 134,

–	profile_idc is equal to 100,

–	profile_idc is equal to 77 or constraint_set1_flag is equal to 1.

–	Only I, P, and B slice types may be present.

–	NAL unit streams shall not contain nal_unit_type values in the range of 2 to 4, inclusive.

–	Arbitrary slice order is not allowed.

–	Picture parameter sets shall have num_slice_groups_minus1 equal to 0 only.

–	Picture parameter sets shall have redundant_pic_cnt_present_flag equal to 0 only.

–	When frame_mbs_only_flag is equal to 1 in an active sequence parameter set for a texture view, frame_mbs_only_flag shall be equal to 1 in the active sequence parameter set for the depth view having the same view_id.

–	When frame_mbs_only_flag is equal to 0 in an active sequence parameter set for a depth view, mb_adaptive_frame_field_flag shall be equal to 0.

–	MVCD sequence parameter sets for the depth views shall have chroma_format_idc equal to 0 only.

–	MVCD sequence parameter sets shall have bit_depth_luma_minus8 equal to 0 only.

–	MVCD sequence parameter sets shall have bit_depth_chroma_minus8 equal to 0 only.

–	MVCD sequence parameter sets shall have qpprime_y_zero_transform_bypass_flag equal to 0 only.

–	For each access unit, the value of level_idc for all active view MVCD sequence parameter set RBSPs shall be the same as the value of level_idc for the active MVCD sequence parameter set RBSP.

–	The level constraints specified for the MFC Depth High profile in clause Error: Reference source not found shall be fulfilled.

Conformance of a bitstream to the MFC Depth High profile is indicated by profile_idc being equal to 135.

Decoders conforming to the MFC Depth High profile at a specific level shall be capable of decoding all bitstreams in which both of the following conditions are true:

- All active MVCD sequence parameter sets have one or more of the following conditions fulfilled:

–	profile_idc is equal to 135,

–	profile_idc is equal to 138,

–	profile_idc is equal to 134,

–	profile_idc is equal to 128,

–	profile_idc is equal to 118 and constraint_set5_flag is equal to 1,

–	profile_idc is equal to 100,

–	profile_idc is equal to 77 or constraint_set1_flag is equal to 1.

- All active MVCD sequence parameter sets have one or more of the following conditions fulfilled:

–	level_idc or (level_idc and constraint_set3_flag) represent a level less than or equal to the specific level,

–	level_idc[ i ] or (level_idc[ i ] and constraint_set3_flag) represent a level less than or equal to the specific level.

In J.7.3.2.1.5 (Sequence parameter set MVCD extension syntax), replace the syntax table with the following:

| seq_parameter_set_3davc_extension( ) { | C | Descriptor |
| --- | --- | --- |
| if( NumDepthViews > 0 ) { |  |  |
| 3dv_acquisition_idc | 0 | ue(v) |
| for( i = 0; i < NumDepthViews; i++ ) |  |  |
| view_id_3dv[ i ] | 0 | ue(v) |
| if( 3dv_acquisition_idc ) { |  |  |
| depth_ranges( NumDepthViews, 2, 0 ) |  |  |
| vsp_param( NumDepthViews, 2, 0 ) |  |  |
| } |  |  |
| reduced_resolution_flag | 0 | u(1) |
| if( reduced_resolution_flag ) { |  |  |
| depth_pic_width_in_mbs_minus1 | 0 | ue(v) |
| depth_pic_height_in_map_units_minus1 | 0 | ue(v) |
| depth_hor_mult_minus1 | 0 | ue(v) |
| depth_ver_mult_minus1 | 0 | ue(v) |
| depth_hor_rsh | 0 | ue(v) |
| depth_ver_rsh | 0 | ue(v) |
| } |  |  |
| depth_frame_cropping_flag | 0 | u(1) |
| if( depth_frame_cropping_flag ) { |  |  |
| depth_frame_crop_left_offset | 0 | ue(v) |
| depth_frame_crop_right_offset | 0 | ue(v) |
| depth_frame_crop_top_offset | 0 | ue(v) |
| depth_frame_crop_bottom_offset | 0 | ue(v) |
| } |  |  |
| grid_pos_num_views | 0 | ue(v) |
| for( i = 0; i < grid_pos_num_views; i++ ) { |  |  |
| grid_pos_view_id[ i ] | 0 | ue(v) |
| grid_pos_x[ grid_pos_view_id[ i ] ] | 0 | se(v) |
| grid_pos_y[ grid_pos_view_id[ i ] ] | 0 | se(v) |
| } |  |  |
| slice_header_prediction_flag | 0 | u(1) |
| seq_view_synthesis_flag | 0 | u(1) |
| } |  |  |
| alc_sps_enable_flag | 0 | u(1) |
| enable_rle_skip_flag | 0 | u(1) |
| if( !AllViewsPairedFlag ) { |  |  |
| for( i = 1; i <= num_views_minus1; i++ ) |  |  |
| if( texture_view_present_flag[ i ] ) { |  |  |
| num_anchor_refs_l0[ i ] | 0 | ue(v) |
| for( j = 0; j < num_anchor_refs_l0[ i ]; j++ ) |  |  |
| anchor_ref_l0[ i ][ j ] | 0 | ue(v) |
| num_anchor_refs_l1[ i ] | 0 | ue(v) |
| for( j = 0; j < num_anchor_refs_l1[ i ]; j++ ) |  |  |
| anchor_ref_l1[ i ][ j ] | 0 | ue(v) |
| } |  |  |
| for( i = 1; i <= num_views_minus1; i++ ) |  |  |
| if( texture_view_present_flag[ i ] ) { |  |  |
| num_non_anchor_refs_l0[ i ] | 0 | ue(v) |
| for( j = 0; j < num_non_anchor_refs_l0[ i ]; j++ ) |  |  |
| non_anchor_ref_l0[ i ][ j ] | 0 | ue(v) |
| num_non_anchor_refs_l1[ i ] | 0 | ue(v) |
| for( j = 0; j < num_non_anchor_refs_l1[ i ]; j++ ) |  |  |
| non_anchor_ref_l1[ i ][ j ] | 0 | ue(v) |
| } |  |  |
| } |  |  |
| } |  |  |

Add the following at the end of J.7.4.2.1.5 (Sequence parameter set 3D-AVC extension semantics):

The variable AllViewsPairedFlag is derived as follows:

AllViewsPairedFlag = 1
for( i = 1; i <= num_views_minus1; i++ )
	AllViewsPairedFlag = ( AllViewsPairedFlag  &  depth_view_present_flag[ i ]  &  	(J-3)
		texture_view_present_flag[ i ] )

For num_anchor_refs_l0[ i ], anchor_ref_l0[ i ][ j ], num_anchor_refs_l1[ i ], anchor_ref_l1[ i ][ j ], num_non_anchor_refs_l0[ i ], non_anchor_ref_l0[ i ][ j ], num_non_anchor_refs_l1[ i ], and non_anchor_ref_l1[ i ][ j ], the semantics specified in subclause H.7.4.2.1.4 is applied with the substitution of texture view component for view component. When num_anchor_refs_l0[ i ], anchor_ref_l0[ i ][ j ], num_anchor_refs_l1[ i ], anchor_ref_l1[ i ][ j ], num_non_anchor_refs_l0[ i ], non_anchor_ref_l0[ i ][ j ], num_non_anchor_refs_l1[ i ], and non_anchor_ref_l1[ i ][ j ] are not present, they are inferred to have the same values as the respective syntax elements in the seq_parameter_set_mvcd_extension( ) syntax structure in the same subset_seq_parameter_set_rbsp( ) syntax structure that also contains this seq_parameter_set_3davc_extension( ) syntax structure.

In Annex J, renumber the prior Equations J-3 through J-81 as Equations J-4 through J-82.