| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18–24 Oct. 2014 | Document: JCT3V-J1008 |
| --- | --- |

| Title: | MV-HEVC and 3D-HEVC Conformance Draft 2 |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Draft |  |  |
| Author(s) or Contact(s): | Ying Chen Qualcomm Incorporated. Tomohiro Ikai Sharp Kei Kawamura KDDI Shinya Shimizu NTT Teruhiko Suzuki Sony | Tel: Email: | +1-858-845-6589 cheny@qti.qualcomm.com ikai.tomohiro@sharp.co.jp ki-kawamura@kddi.com shimizu.shinya@lab.ntt.co.jp teruhikos@jp.sony.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document is a proposed draft 2 of conformance test of MV-HEVC and 3D-HEVC.

The following text specifies the normative tests for verifying conformance of Rec. ITU-T H.265 | ISO/IEC 23008-2 video bitstreams and decoders conforming to the Multiview Main profile and the 3D Main Profile.

[Ed. Note: The conformance bitstreams are available at the following two links for MV-HEVC (Multiview Main Profile) and 3D-HEVC (3D Main Profile) respectively:

http://wftp3.itu.int/av-arch/jct3v-site/bitstream_exchange/under_test/MV-HEVC/

http://wftp3.itu.int/av-arch/jct3v-site/bitstream_exchange/under_test/3D-HEVC/]

- MV-HEVC and 3D-HEVC Conformance Draft 2

In subclause 6.5.7, add the following text at the end of the subclause:

A decoder that conforms to the Multiview Main profile at specific level shall be capable of decoding the specified bitstreams in Table 3. In addition to the bitstreams defined in Table 3, a decoder that conforms to the Multiview Main profile shall be capable of decoding the Main profile bitstreams specified in Table 1.

A decoder that conforms to the 3D Main profile (as specified in subclause I.11 of Rec. ITU-T H.265 | ISO/IEC 23008-2) at specific level shall be capable of decoding the specified bitstreams in Table 4. In addition to the bitstreams defined in Table 4, a decoder that conforms to the 3D Main profile shall be capable of decoding the Multiview Main profile bitstreams specified in Table 3.

In subclause 6.6, add the following text at the end of the subclause:

### 6.6.13	Test bitstreams – MV-HEVC

#### 6.6.13.1	Test bitstream #MVHEVCS-A

Specification: All slices are coded as I, P or B slices. Only the first picture of each view is coded as IDR picture. Each picture contains only one slice. NumViews is equal to 2. NumDirectRefLayers of the non-base view is equal to 1. For each picture in the non-base view, inter-view prediction is enabled. The two views are with the same spatial resolution. All NAL units are encapsulated into the byte stream format specified in Rec. ITU-T H.265 | ISO/IEC 23008-2.

Functional stage: Decoding of two views with inter-view prediction and inter-prediction.

Purpose: To conform the most normal case.

Contributor: Qualcomm

#### 6.6.13.2	Test bitstream #MVHEVCS-B

Specification: All slices of the base view are coded as I slices and all slices of the non-base view are coded with only inter-view prediction, thus P slices. Only the first picture of each view is coded as IDR picture. Each picture contains only one slice. NumViews is equal to 2. NumDirectRefLayers of the non-base view is equal to 1. The two views are with the same spatial resolution. All NAL units are encapsulated into the byte stream format specified in Rec. ITU-T H.265 | ISO/IEC 23008-2.

Functional stage: Decoding of two views with only inter-view prediction and intra-prediction.

Purpose: To conform the all-Intra case.

Contributor: Sharp.

#### 6.6.13.3	Test bitstream #MVHEVCS-C

Specification: All slices are coded as I, P or B slices. Only the first picture of each view is coded as IDR picture. Each picture contains only one slice. NumViews is equal to 2. NumDirectRefLayers is always equal to 0, meaning inter-view prediction is disabled. The two views are with the same spatial resolution. All NAL units are encapsulated into the byte stream format specified in Rec. ITU-T H.265 | ISO/IEC 23008-2.

Functional stage: Decoding of two views with only inter-prediction and intra-prediction.

Purpose: To conform the case of simulcast coding.

Contributor: Sony.

#### 6.6.13.4	Test bitstream #MVHEVCS-D

Specification: All slices are coded as I, P or B slices. Only the first picture of each view is coded as IDR picture. Each picture contains only one slice. NumViews is equal to 2. NumDirectRefLayers is always equal to 0, meaning inter-view prediction is disabled. The two views are with different spatial resolutions. All NAL units are encapsulated into the byte stream format specified in Rec. ITU-T H.265 | ISO/IEC 23008-2.

Functional stage: Decoding of two views with only inter-prediction and intra-prediction.

Purpose: To conform the case when different spatial resolutions for two views are used.

Contributor: NTT.

#### 6.6.13.5	Test bitstream #MVHEVCS-E

Specification: All slices are coded as I, P or B slices. Only the first picture of each view is coded as IDR picture and thus an IRAP access unit. In addition, every two GOPs start with an access unit which contains pictures that are all CRA pictures. Each picture contains only one slice. NumViews is equal to 2. NumDirectRefLayers of the non-base view is equal to 1. For each picture in the non-base view, inter-view prediction is enabled. The two views are with the same spatial resolution. All NAL units are encapsulated into the byte stream format specified in Rec. ITU-T H.265 | ISO/IEC 23008-2.

Functional stage: Decoding of two views with inter-view prediction and inter-prediction.

Purpose: To conform the random access hierarchical B case.

Contributor: Qualcomm.

#### 6.6.13.6	Test bitstream #MVHEVCS-F

Specification: All slices are coded as I, P or B slices. Only the first picture of each view is coded as IDR picture and thus an IRAP access unit. In addition, for each of every two following GOPs, an access unit which contains pictures that are all CRA pictures are requested. Other pictures are non-IRAP pictures. Each picture contains only one slice. NumViews is equal to 2. NumActiveRefLayerPics is equal to 1 for each picture in an IRAP access unit of the non-base view, and 0 otherwise, i.e., only for each picture in the non-base view in the IRAP access unit, inter-view prediction is enabled. The two views are with the same spatial resolution. All NAL units are encapsulated into the byte stream format specified in Rec. ITU-T H.265 | ISO/IEC 23008-2.

Functional stage: Decoding of two views with inter-view prediction only for random access points.

Purpose: To conform the flexibility of inter-view prediction applicability scope (inter-view pred. only for IRAP pictures).

Contributor: Qualcomm.

#### 6.6.13.7	Test bitstream #MVHEVCS-G

Specification: All slices are coded as I, P or B slices. Only the first picture of each view is coded as IDR picture and thus an IRAP access unit. In addition, every two GOPs start with an access unit which contains pictures that are all CRA pictures. Each picture contains only one slice. NumViews is equal to 3. NumDirectRefLayers of the non-base view is equal to 1. For each picture in the non-base view, inter-view prediction is enabled. The three views are with the same spatial resolution. All NAL units are encapsulated into the byte stream format specified in Rec. ITU-T H.265 | ISO/IEC 23008-2.

Functional stage: Decoding of three views with inter-view prediction and inter-prediction.

Purpose: To conform the random access hierarchical B case (random access 3-view case, PIP cfg.).

Contributor: NTT.

#### 6.6.13.8	Test bitstream #MVHEVCS-H

Specification: All slices are coded as I, P or B slices. Only the first picture of each view is coded as IDR picture and thus an IRAP access unit. In addition, every two GOPs start with an access unit which contains pictures that are all CRA pictures. Each picture contains only one slice. NumViews is equal to 3. NumDirectRefLayers of the first non-base view is equal to 1 and NumDirectRefLayers of the second non-base view is equal to 2. For each picture in the non-base view, inter-view prediction is enabled. The three views are with the same spatial resolution. All NAL units are encapsulated into the byte stream format specified in Rec. ITU-T H.265 | ISO/IEC 23008-2.

Functional stage: Decoding of three views with inter-view prediction and inter-prediction.

Purpose: To conform the random access hierarchical B case (random access 3-view case, IBP cfg.).

Tentative contributor: LGE.

#### 6.6.13.9	Test bitstream #MVHEVCS-I

Specification: All slices are coded as I, P or B slices. Only the first picture of each view is coded as IDR picture and thus an IRAP access unit. In addition, every two GOPs start with an access unit which contains pictures that are all CRA pictures. Each picture contains only one slice. NumViews is equal to 3. NumDirectRefLayers of the first non-base view is equal to 1 and NumDirectRefLayers of the second non-base view is equal to 2. The P view is coded with viewOrderIdx equal to 1. For each picture in the non-base view, inter-view prediction is enabled. Each view contains both texture and depth and the views are with the same spatial resolution. All NAL units are encapsulated into the byte stream format specified in Rec. ITU-T H.265 | ISO/IEC 23008-2. The vertical constraint of the inter-view motion vector applies only to the P view (for both texture layer and depth layer of this view).

Functional stage: Decoding of three views with inter-view prediction and inter-prediction.

Purpose: To conform the random access hierarchical B case (random access 3-view case, IBP cfg. MV-HEVC plus depth).

Tentative contributor: Nokia.

#### 6.6.13.10	Test bitstream #MVHEVCS-J

Specification: All slices are coded as I, P or B slices. For the non-base view, only the first picture of is coded as IDR picture and thus an IRAP access unit. In addition, every two GOPs start with an access unit which contains pictures that are all CRA pictures. Each picture contains only one slice. NumViews is equal to 2. NumDirectRefLayers of the non-base view is equal to 1. For each picture in the non-base view, inter-view prediction is enabled. The two views are with the same spatial resolution. The base view contains pictures derived by external means (may be set as dummy pictures). All NAL units are encapsulated into the byte stream format specified in Rec. ITU-T H.265 | ISO/IEC 23008-2.

Functional stage: Decoding of two views with the base view being external.

Purpose: To conform the functionality of hybrid scalability.

Tentative contributor: tbd.

### 6.6.14	Test bitstreams – 3D-HEVC

AI configuration: A bitstream in this configuration contains two or three views, each containing one texture view and one depth view. All slices of the base view are coded as I slices and all slices of the non-base view are coded with only inter-view predictions, thus P slices. Only the first picture of each view is coded as IDR picture. Each picture contains only one slice. NumDirectRefLayers of the non-base views is equal to 1. All texture and depth views are with the same spatial resolution. All NAL units are encapsulated into the byte stream format specified in Rec. ITU-T H.265 | ISO/IEC 23008-2.

Random access configuration: A bitstream in this configuration contains two or three views, each containing one texture view and one depth view. All slices are coded as I, P or B slices. Only the first picture of each view is coded as IDR picture. In addition, every two GOPs start with an access unit which contains pictures that are all CRA pictures. Each picture contains only one slice. NumDirectRefLayers of the non-base view is equal to 1. For each picture in a non-base view, inter-view predictions are enabled. All texture and depth views are with the same spatial resolution. All NAL units are encapsulated into the byte stream format specified in Rec. ITU-T H.265 | ISO/IEC 23008-2.

3-view configuration: A bitstream in this configuration contains three views, each containing one texture view and one depth view. NumViews is equal to 3. It can be either AI configuration or random access configuration.

2-view configuration: A bitstream in this configuration contains three views, each containing one texture view and one depth view. NumViews is equal to 2. It can be either AI configuration or random access configuration.

#### 6.6.14.1	Test bitstreams – texture tools

In this category, test bitstreams are tested with the following default tools enabled. The default tools include disparity motion compensation, Neighboring Block based Disparity Vector (NBDV), the merge candidate list construction (with DoNBDV and VSP disabled). In general, Depth-oriented NBDV (DoNBDV), VSP and Depth Block Based Partitioning (DBBP) are turned off.

In the test bistreams of this category, unless specified explicitly, the depth views are coded with MV-HEVC mechanisms (i.e., only inter-view sample prediction, a.k.a. disparity motion compensation).

### 6.6.14.1.1	Test bitstream #3DHC_T_A

Specification: The bitstream is coded with random access configuration as well as 3-view configuration. In addition to inter-view motion prediction, advanced residual prediction is enabled in the non-base texture views.

Functional stage: Decoding of three views with inter-view motion prediction and advanced residual prediction (ARP).

Purpose: To conform the ARP.

Contributor: Qualcomm.

### 6.6.14.1.2	Test bitstream ##3DHC_T_B

Specification: The bitstream is coded with random access configuration as well as 3-view configuration. In addition to inter-view motion prediction, sub-PU inter-view motion prediction is enabled in the non-base texture views.

Functional stage: Decoding of three views with sub-PU inter-view motion prediction.

Purpose: To conform the sub-PU inter-view motion prediction.

Contributor: MediaTek.

### 6.6.14.1.3	Test bitstream ##3DHC_T_C

Specification: The bitstream is coded with random access configuration as well as 3-view configuration. In addition to inter-view motion prediction, illumination compensation is enabled in the non-base texture views.

Functional stage: Decoding of three views with illumination compensation.

Purpose: To conform the illumination compensation.

Contributor: Sharp.

### 6.6.14.1.4	Test bitstream ##3DHC_T_D

Specification: The bitstream is coded with random access configuration as well as 3-view configuration. In addition to inter-view motion prediction, ARP, sub-PU inter-view motion prediction and illumination compensation are enabled in the non-base texture views.

Functional stage: Decoding of three views with all texture coding tools enabled.

Purpose: To conform the combined texture coding tools.

Contributor: Sharp.

### 6.6.14.1.5	Test bitstream #3DHC_T_E

Specification: The bitstream in this configuration contains two or three views, each containing only one texture view but no depth view. All slices are coded as I, P or B slices. Only the first picture of each view is coded as IDR picture. In addition, every two GOPs start with an access unit which contains pictures that are all CRA pictures. Each picture contains only one slice. NumDirectRefLayers of the non-base view is equal to 1. For each picture in a non-base view, inter-view predictions are enabled, with inter-view motion prediction, ARP, sub-PU inter-view motion prediction and illumination compensation. All NAL units are encapsulated into the byte stream format specified in Rec. ITU-T H.265 | ISO/IEC 23008-2.

Functional stage: Decoding of three texture views only with all texture coding tools enabled.

Purpose: To conform the texture coding tools for texture only bitstream.

Contributor: HHI.

#### 6.6.14.2	Test bitstreams – depth tools

In this category, test bitstreams are tested with the default tools enabled for texture coding. The default tools include disparity motion compensation, Neighboring Block based Disparity Vector (NBDV), the merge candidate list construction. DoNBDV, VSP and DBBP are disabled. Depth views are coded by default with MV-HEVC mechanisms (i.e., only inter-view sample prediction, a.k.a. disparity motion compensation).

#### 6.6.14.2.1	Test bitstreams – depth Intra tools

Depth Intra coding tools should be tested in both random access configuration and AI configuration.

### 6.6.14.2.1.1	Test bitstream #3DHC_D1_A

Specification: The bitstream is coded with random access configuration as well as 3-view configuration. In addition to inter-view sample prediction for depth, DMM 1 is enabled for depth coding.

Functional stage: Decoding of three views with DMM 1 enabled for depth.

Purpose: To conform the DMM 1 in random access configurations.

Contributor: HHI.

#### 6.6.14.2.1.2	Test bitstream #3DHC_D1_B

Specification: The bitstream is coded with AI configuration as well as 3-view configuration. DMM 1 is enabled for depth coding.

Functional stage: Decoding of three views with DMM 1 enabled for depth.

Purpose: To conform the DMM 1 in AI configuration.

Contributor: HHI.

#### 6.6.14.2.1.3	Test bitstream #3DHC_D1_C

Specification: The bitstream is coded with random access configuration as well as 3-view configuration. In addition to inter-view sample prediction for depth, SDC is enabled for depth coding.

Functional stage: Decoding of three views with SDC enabled for depth.

Purpose: To conform the DMM 1 in random access configurations.

Contributor: RWTH.

#### 6.6.14.2.1.4	Test bitstream #3DHC_D1_D

Specification: The bitstream is coded with AI configuration as well as 3-view configuration. In addition, SDC is enabled for depth coding.

Functional stage: Decoding of three views with SDC enabled for depth.

Purpose: To conform the SDC in AI configuration.

Contributor: RWTH.

#### 6.6.14.2.1.5	Test bitstream #3DHC_D1_E

Specification: The bitstream is coded with random access configuration as well as 3-view configuration. In addition to inter-view sample prediction for depth, single depth mode is enabled for depth coding.

Functional stage: Decoding of three views with single depth mode enabled for depth.

Purpose: To conform the single depth mode in random access configurations.

Contributor: MediaTek.

#### 6.6.14.2.1.6	Test bitstream #3DHC_D1_F

Specification: The bitstream is coded with AI configuration as well as 3-view configuration. In addition, single depth mode is enabled for depth coding.

Functional stage: Decoding of three views with single depth mode enabled for depth.

Purpose: To conform the single depth mode in AI configuration.

Contributor: MediaTek.

#### 6.6.14.2.1.7	Test bitstream #3DHC_D1_G

Specification: The bitstream is coded with random access configuration as well as 3-view configuration. In addition to inter-view sample prediction for depth, the all depth Intra tools are enabled together with DLT.

Functional stage: Decoding of three views with depth Intra coding tools enabled.

Purpose: To conform the depth Intra coding tools in random access configuration.

Contributor: RWTH/Hisilicon.

#### 6.6.14.2.1.8	Test bitstream #3DHC_D1_H

Specification: The bitstream is coded with AI configuration as well as 3-view configuration. In addition, the all depth Intra tools are enabled together with DLT

Functional stage: Decoding of three views with depth Intra coding tools enabled.

Purpose: To conform the depth Intra coding tools in AI configuration.

Contributor: RWTH/Hisilicon.

### 6.6.14.2.2	Test bitstreams – depth Inter tools

Depth Intra coding tools are tested in random access configuration.

### 6.6.14.2.2.1	Test bitstream #3DHC_D2_A

Specification: The bitstream is coded with random access configuration as well as 3-view configuration. In addition to inter-view sample prediction for depth, the inter-view motion prediction is enabled for depth.

Functional stage: Decoding of three views with inter-view motion prediction for depth views.

Purpose: To conform the inter-view motion prediction for depth.

Contributor: Samsung (not confirmed).

### 6.6.14.2.2.2	Test bitstream #3DHC_D2_B

Specification: The bitstream is coded with random access configuration as well as 3-view configuration. In addition to inter-view sample prediction for depth, the inter SDC is enabled for depth coding.

Functional stage: Decoding of three views with inter SDC enabled for depth views.

Purpose: To conform the inter SDC for depth.

Contributor: LGE.

#### 6.6.14.3	Test bitstreams – depth dependent texture tools

In this category, test bitstreams are tested with the all texture coding tools enabled. The default tools include disparity motion compensation, Neighboring Block based Disparity Vector (NBDV), the merge candidate list construction. DoNBDV, VSP and DBBP are disabled. Depth views are coded by default with MV-HEVC mechanisms (i.e., only inter-view sample prediction, a.k.a. disparity motion compensation).

### 6.6.14.3.1	Test bitstream #3DHC_DT_A

Specification: The bitstream is coded with random access configuration as well as 3-view configuration. In addition to ARP, sub-PU inter-view motion prediction and illumination compensation, DoNBDV is enabled for non-base texture views.

Functional stage: Decoding of three views with DoNBDV enabled for texture views.

Purpose: To conform the DoNBDV for texture.

Contributor: MediaTek.

### 6.6.14.3.2	Test bitstream #3DHC_DT_B

Specification: The bitstream is coded with random access configuration as well as 3-view configuration. In addition to ARP, sub-PU inter-view motion prediction and illumination compensation, VSP is enabled for non-base texture views.

Functional stage: Decoding of three views with VSP enabled for texture views.

Purpose: To conform the inter VSP for texture.

Contributor: NTT.

### 6.6.14.3.3	Test bitstream #3DHC_DT_C

Specification: The bitstream is coded with random access configuration as well as 3-view configuration. In addition to ARP, sub-PU inter-view motion prediction and illumination compensation, DBBP is enabled for non-base texture views.

Functional stage: Decoding of three views with DBBP enabled for texture views.

Purpose: To conform the inter DBBP for texture.

Contributor: Hisilicon.

### 6.6.14.3.4	Test bitstream #3DHC_DT_D

Specification: The bitstream is coded with random access configuration as well as 3-view configuration. In addition to ARP, sub-PU inter-view motion prediction and illumination compensation, DoNBDV, VSP and DBBP are enabled for non-base texture views.

Functional stage: Decoding of three views with depth dependent texture tools.

Purpose: To conform the combined depth dependent texture tools.

Contributor: NTT.

## Test bitstreams – texture dependent depth tools

In this category, test bitstreams are tested with the all texture coding tools and depth dependent texture tools enabled for non-base texture views. Depth views are coded with depth coding tools enabled and by default, texture dependent depth tools disabled unless otherwise specified.

### 6.6.14.4.1	Test bitstream #3DHC_TD_A

Specification: The bitstream is coded with random access configuration as well as 3-view configuration. In addition to all the texture coding tools, all the depth coding tools and depth dependent texture coding tools, sub-PU MPI (Motion Parameter Inheritance) are turned on.

Functional stage: Decoding of three views with sub-PU MPI enabled for depth views.

Purpose: To conform the sub-PU MPI for depth.

Contributor: Qualcomm.

### 6.6.14.4.2	Test bitstream #3DHC_TD_B

Specification: The bitstream is coded with random access configuration as well as 3-view configuration. In addition to all the texture coding tools, all the depth coding tools and depth dependent texture coding tools, MPI (with sub-PU block size set equal to PU size) is turned on.

Functional stage: Decoding of three views with MPI enabled for depth views.

Purpose: To conform the MPI for depth.

Contributor: MediaTek.

### 6.6.14.4.3	Test bitstream #3DHC_TD_C

Void.

### 6.6.14.4.4	Test bitstream #3DHC_TD_D

Specification: The bitstream is coded with random access configuration as well as 3-view configuration. In addition to all the texture coding tools, all the depth coding tools and depth dependent texture coding tools, QTL (Quad-Tree Limitation) is turned on.

Functional stage: Decoding of three views with QTL enabled for depth views.

Purpose: To conform the QTL for depth.

Contributor: Orange/MediaTek.

#### 6.6.14.5	Test bitstreams – other combined cases

### 6.6.14.5.1	Test bitstream #3DHC_C_A

Specification: The bitstream is coded with random access configuration as well as 3-view configuration. All the texture coding tools, all the depth coding tools, all the depth dependent texture coding tools and all the texture dependent depth coding tools are turned on.

Functional stage: Decoding of three views with coding tools enabled.

Purpose: To conform the common test condition case.

Contributor: HHI.

### 6.6.14.5.2	Test bitstream #3DHC_C_B

Specification: The bitstream is coded with AI configuration as well as 3-view configuration. All the texture coding tools, all the depth coding tools, all the depth dependent texture coding tools and all the texture dependent depth coding tools are turned on.

Functional stage: Decoding of three views with coding tools enabled for AI configuration.

Purpose: To conform the AI configuration.

Contributor: HHI.

### 6.6.14.5.3	Test bitstream #3DHC_C_C

Specification: The bitstream is coded with random access configuration as well as 2-view configuration. All the texture coding tools, all the depth coding tools, all the depth dependent texture coding tools and all the texture dependent depth coding tools are turned on.

Functional stage: Decoding of two views with coding tools enabled.

Purpose: To conform the 2-view case.

Contributor: Sharp.

| Table 3 – Bitstreams for Multiview Main profile |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Categories | Sub category | Bitstream | File name | Main | Main 10 | Main still picture |  | Main tier | Level | Frame rate (Frames/sec) |
| Prediction Structure (2-view) | Inter-view prediction | MVHEVCS-A | MVHEVCS_A_Qualcomm_1 | X | X |  |  | X | 5.1 and higher | 30 |
|  | All Intra | MVHEVCS-B | MVHEVCS_B_Sharp_1 | X | X |  |  | X | 5.1 and higher | 30 |
|  | Simulcast | MVHEVCS-C | MVHEVCS_C_Sony_1 | X | X |  |  | X | 5.1 and higher | 30 |
|  | Simulcast with asymmetric resolutions | MVHEVCS-D | MVHEVCS_D_NTT_1 | X | X |  |  | X | 5.1 and higher | 30 |
|  | Inter-view prediction and hierarchical B | MVHEVCS-E | MVHEVCS_E_Qualcomm_1 | X | X |  |  | X | 5.1 and higher | 30 |
|  | Inter-view prediction for IRAP AUs only | MVHEVCS-F | MVHEVCS_F_Qualcomm_1 | X | X |  |  | X | 5.1 and higher | 30 |
| Prediction Structure (3-view) | Inter-view prediction PIP view structure | MVHEVCS-G | MVHEVCS_G_NTT_1 | X | X |  |  | X | 5.1 and higher | 30 |
|  | Inter-view prediction with IBP view structure | MVHEVCS-H | MVHEVCS_H_LGE_1 | X | X |  |  | X | 5.1 and higher | 30 |
|  | Inter-view prediction with IBP view structure and auxiliary depth | MVHEVCS-I | MVHEVCS_I_Nokia_1 | X | X |  |  | X | 5.1 and higher | 30 |
| Hybrid scalability |  | MVHEVCS-J | MVHEVCS_J_XXX_1 | X | X |  |  | X | 5.1 and higher | 30 |

[Ed. (CY): the level values for the MV-HEVC bitstreams may need to be revisited.]

| Table 4 – Bitstreams for 3D Main profile |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Categories | Sub category | Bitstream | File name | Main | Main 10 | Multiview Main |  | Main tier | Level | Frame rate (Frames/sec) |
| Texture tool | ARP | 3DHC_T_A | 3DHC_T_A_Qualcomm_1 | X | X | X |  | X | 5.1 and higher | 30 |
|  | Sub-PU inter-view motion prediction | 3DHC_T_B | 3DHC_T_B_MediaTek_1 | X | X | X |  | X | 5.1 and higher | 30 |
|  | Illumination compensation | 3DHC_T_C | 3DHC_T_C_Sharp_1 | X | X | X |  | X | 5.1 and higher | 30 |
|  | Combined | 3DHC_T_D | 3DHC_T_D_Sharp_1 | X | X | X |  | X | 5.1 and higher | 30 |
|  | Combined texture only bitstream | 3DHC_T_E | 3DHC_T_E_HHI_1 | X | X | X |  | X | 5.1 and higher | 30 |
| Depth tool | Depth Intra (DMM1) | 3DHC_D1_A | 3DHC_D1_A_HHI_1 | X | X | X |  | X | 5.1 and higher | 30 |
|  | Depth Intra (DMM1) | 3DHC_D1_B | 3DHC_D1_B_HHI_1 | X | X | X |  | X | 5.1 and higher | 30 |
|  | Depth Intra (SDC) | 3DHC_D1_C | 3DHC_D1_C_RWTH_1 | X | X | X |  | X | 5.1 and higher | 30 |
|  | Depth Intra (SDC) | 3DHC_D1_D | 3DHC_D1_D_RWTH_1 | X | X | X |  | X | 5.1 and higher | 30 |
|  | Depth Intra (single depth) | 3DHC_D1_E | 3DHC_D1_E_MediaTek_1 | X | X | X |  | X | 5.1 and higher | 30 |
|  | Depth Intra (single depth) | 3DHC_D1_F | 3DHC_D1_F_MediaTek_1 | X | X | X |  | X | 5.1 and higher | 30 |
|  | Depth Intra (combined) | 3DHC_D1_G | 3DHC_D1_G_Hisilicon_1 | X | X | X |  | X | 5.1 and higher | 30 |
|  | Depth Intra (combined) | 3DHC_D1_H | 3DHC_D1_H_Hisilicon_1 | X | X | X |  | X | 5.1 and higher | 30 |
| Depth tool | Depth Inter (inter-view motion) | 3DHC_D2_A | 3DHC_D2_A_Samsung_1 | X | X | X |  | X | 5.1 and higher | 30 |
|  | Depth Inter (inter-SDC) | 3DHC_D2_B | 3DHC_D2_B_LGE_1 | X | X | X |  | X | 5.1 and higher | 30 |
| Depth dependent texture tool | DoNBDV | 3DHC_DT_A | 3DHC_DT_A_MediaTek_1 | X | X | X |  | X | 5.1 and higher | 30 |
|  | VSP | 3DHC_DT_B | 3DHC_DT_B_NTT_1 | X | X | X |  | X | 5.1 and higher | 30 |
|  | DBBP | 3DHC_DT_C | 3DHC_DT_C_Hisilicon_1 | X | X | X |  | X | 5.1 and higher | 30 |
|  | Combined | 3DHC_DT_D | 3DHC_DT_D_NTT_1 | X | X | X |  | X | 5.1 and higher | 30 |
| Texture dependent depth tool | Sub-PU MPI | 3DHC_TD_A | 3DHC_TD_A_Qualcomm_1 | X | X | X |  | X | 5.1 and higher | 30 |
|  | MPI | 3DHC_TD_B | 3DHC_TD_B_MediaTek_1 | X | X | X |  | X | 5.1 and higher | 30 |
|  | QTL | 3DHC_TD_D | 3DHC_TD_D_MediaTek_1 | X | X | X |  | X | 5.1 and higher | 30 |
| Others | 3-view random access | 3DHC_C_A | 3DHC_C_A_HHI_1 | X | X | X |  | X | 5.1 and higher | 30 |
|  | 3-view all Intra | 3DHC_C_B | 3DHC_C_B_HHI_1 | X | X | X |  | X | 5.1 and higher | 30 |
|  | 2-view random access | 3DHC_C_C | 3DHC_C_C_Sharp_1 | X | X | X |  | X | 5.1 and higher | 30 |