| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3 – 9 July 2014 | Document: JCT3V-J1101 |
| --- | --- |

| Title: | Description of Core Experiment 1 (CE1) on Improved Depth Coding |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Jian-Liang Lin Jin Young Lee | Tel: Email: Email: | +886-3-5670766 ext. 25250 jl.lin@mediatek.com jinyoung79.lee@samsung.com |
| Source: | CE coordinators |  |  |

_____________________________

- Abstract

This document defines Core Experiment 1 on improved depth coding to be performed for the 11th JCT-3V meeting.

- Introduction

The goal of this Core Experiment (CE) is to investigate the methods on improved depth coding proposed at the 10th JCT-3V meeting. Tools under test will be evaluated according to their impact on compression efficiency and implementation complexity

- Participants

  - CE1

| Participant | Contact | Email | Type |
| --- | --- | --- | --- |
| MediaTek | Kai Zhang Jian-Liang Lin Yu-Wen Huang | kai.zhang@mediatek.com jl.lin@mediatek.com yuwen.huang@mediatek.com | P/C |
| Samsung | Jin Young Lee Min Woo Park | jinyoung79.lee@samsung.com m.w.park@samsung.com | P/C |
| Sharp | Tomohiro Ikai | ikai.tomohiro@sharp.co.jp | C |
| NTT | S. Shimizu | shimizu.shinya@lab.ntt.co.jp | C |
| Qualcomm | Ying Chen | cheny@qti.qualcomm.com |  |

- Tools under Test

  - Segmental prediction (J0032)

In this contribution, a segmental prediction method is proposed in depth coding. When the segmental prediction is enabled, two steps are applied to obtain the reconstructed block from the prediction block.

- In the first step, samples in the current block are classified into one, two or three segments.

- In the second step, a single value is derived for each segment in the reconstructed block

The following aspects should be further studied in this context:

-	Use only the four corner samples to derive the threshold.

-	Restriction of two segments only.

-	Analysis the encoder complexity especially on the RD optimization.

  - Depth intra skip coding mode (J0038)

This contribution proposes a depth intra skip coding mode for the efficient depth coding. The proposed depth intra skip coding mode uses the vertical and horizontal prediction modes. The residual information is not compressed and the best prediction mode is determined through RDO. A flag is signaled to distinguish the two different prediction modes.

The following aspects should be further studied in this context:

-	Further study to identify whether there would be alternative ways to achieve the gain by enabling CBF at the RQT root in intra.

-	Further study to identify whether there would be alternative ways to achieve the gain by reducing the number of intra modes for depth maps.

-	Test the coding performance of the depth intra skip mode on top of single depth mode.

- Mandates

The goals of this core experiment is to investigate the methods on segmental prediction and intra skip coding mode proposed at the 10th JCT-3V meeting.

- Software, Configuration and Evaluation

  - Software

Experiments in CE1 will use the 3D-HTM 13.0 software that is recommended in JCT3V-G1100. Proponents are requested to provide software that can be compiled under Windows and Linux platforms.

  - Test Sequences, Bit Rates and Coding Conditions

The CE will use the test sequences, configuration and conditions that are recommended in JCT3V-G1100. Moreover, proponents and cross checkers are required to provide simulation results for the Random access configuration as specified in JCT3V-G1100.

  - Evaluation of CE Results

- Measure impact on bitrate/PSNR. PSNR shall be calculated for the decoded texture views, relative to original texture views and for the synthesized views relative to uncompressed synthesized views. Use 4-point BD-PSNR and BD-Rate according to common conditions. The anchors will be generated according to common test conditions.

- Measure impact on encoding and decoding runtimes for all test cases relative to the unmodified test model software.

- Timelines

  - CE1

2014/11/20		Release HTM 13.0

2015/01/22		Make source code, simulation results, and draft text available for ALL CE1 participants, not only for the cross-checks.

2015/02/05		Register documents for the 11th JCT-3V meeting

2015/02/05		Upload contributions to the 11th JCT-3V meeting

2015/02/12-02/18	Start the 11th JCT-3V meeting

- References

- D. Rusanovksyy, K. Müller, and A. Vetro, " Common Test Conditions of 3DV Core Experiments," Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG, JCT3V-G1100, San José, Jan., 2014.