| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Geneva, CH, 12 – 18 February 2015 | Document: JCT3V-K0022 |
| --- | --- |

| Title: | AHG8: Specification text fixes for the VSP modes of 3D-AVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Technologies Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 10 4488000 firstname.lastname@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

It is asserted that the 3D-AVC specification text contains a few issues related to macroblock partition and sub-macroblock size constraints and derivation as well as their use in inverse scanning processes when view synthesis prediction is used. The following changes are proposed:

- sub_mb_type is semantically constrained in clause J.7.4.6.2 to indicate a sub-macroblock type having an 8x8 sub-macroblock size, when bvsp_flag_lX is equal to 1.

- Table 7-13 (macroblock type values for P and SP slices) is overridden in Annex J so that when mb_type is equal to P_skip and MbVSSkipFlag is equal to 1, NumMbPart( P_Skip ) is specified to be equal to 4, MbPartWidth( P_Skip ) is specified to be equal to 8, and MbPartHeight( P_Skip ) is specified to be equal to 8.

- Clause 6.4.2.2 (inverse sub-macroblock partition scanning process) is overridden in Annex J so that it operates otherwise as earlier but when a VSP mode is in use and mb_type is equal to B_Skip or B_Direct_16x16, or mb_type is equal to B_8x8 and sub_mb_type[ mbPartIdx ] is equal to B_Direct_8x8, its operation uses sub-macroblocks of size 8x8.

It is suggested that no changes in the 3DV-ATM software are needed, as the software presumably already works correctly. This assumption is suggested to be confirmed.

# Problem Statement

The following issues were discovered when reviewing the view synthesis prediction related specification text of 3D-AVC:

- sub_mb_type[ mbPartIdx ] is not semantically constrained when bvsp_flagLX[ mbPartIdx ] is equal to 1 and can indicate a sub-macroblock size that is less than 8x8. However, it is asserted that the design intent has been to disallow sub-macroblocks smaller than 8x8, which is also enforced by equations (J-6) and (J-7), as shown in the following excerpt:

–	If mb_type is not equal to P_8x8, P_8x8ref0, B_Skip, B_Direct_16x16, or B_8x8, subMbPartIdx is set equal to 0 and the following applies:

–	If vspPartFlag is equal to 1 or both mb_type is equal to P_Skip and MbVSSkipFlag is equal to 1, partWidth and partHeight are derived as:

partWidth = 8		(J-6)

partHeight = 8		(J-7)

This issue was first noticed by Li Zhang (Qualcomm) in T-REC-H.264-201402-P!!MSW-E_annotated_g2_l1.docx shared among the specification editors.

- Table 7-13 specifies that NumMbPart( P_Skip ) is equal to 1, MbPartWidth( P_Skip ) is equal to 16, and MbPartHeight( P_Skip ) is equal to 16. However, when MbVSSkipFlag is equal to 1, it is asserted that the intent is to specify NumMbPart( P_Skip ) to be equal to 4, MbPartWidth( P_Skip ) to be equal to 8, and MbPartHeight( P_Skip ) to be equal to 8. In other words, the MB VSP skip mode is supposed to work on 8x8 block basis in disparity vector derivation and sample prediction block generation.

The specification text in clause J.8.2 correctly derives partWidth and partHeight to be equal to 8 and mbPartIdx to proceed over values 0 to 3, inclusive, for macroblocks with MBVSSkipFlag equal to 1. However, clause J.8.2 (and its subclauses) also invoke the inverse macroblock partition scanning process of clause 6.4.2.1, which uses functions MbPartWidth( mb_type ) and MbPartHeight( mb_type ) specified in Table 7-13.

When mb_type is equal to P_skip, MbVSSkipFlag is equal to 1, and mbPartIdx is greater than 0, it is asserted that the operation of clause 6.4.2.1 is not specified.

- Subclause J.8.2 correctly derives partWidth and partHeight to be equal to 8 and subMbPartIdx to be equal to 0 for B skip/direct modes using B-VSP, as shown in the following excerpt:

–	Otherwise (mb_type is equal to B_Skip or B_Direct_16x16, or mb_type is equal to B_8x8 and sub_mb_type[ mbPartIdx ] is equal to B_Direct_8x8), the following applies:

–	If either MbVSSkipFlag or mb_direct_type_flag is equal to 1, subMbPartIdx is set to 0 and partWidth and partHeight are derived as:

partWidth = 8 		(J-12)

partHeight = 8 		(J-13)

However, clause J.8.2 (and its subclauses) also invoke the inverse sub-macroblock partition scanning process of clause 6.4.2.2, which is uses hard-coded mapping between mb_type values and the sub-macroblock sizes, as shown in the following excerpt of clause 6.4.2.2:

The inverse sub-macroblock partition scanning process is specified as follows:

–	If mb_type is equal to P_8x8, P_8x8ref0, or B_8x8,

x = InverseRasterScan( subMbPartIdx, SubMbPartWidth( sub_mb_type[ mbPartIdx ] ), 
		SubMbPartHeight( sub_mb_type[ mbPartIdx ] ), 8, 0 ) 	(6-13)

y = InverseRasterScan( subMbPartIdx, SubMbPartWidth( sub_mb_type[ mbPartIdx ] ), 
		SubMbPartHeight( sub_mb_type[ mbPartIdx ] ), 8, 1 ) 	(6-14)

–	Otherwise (mb_type is not equal to P_8x8, P_8x8ref0, or B_8x8),

x = InverseRasterScan( subMbPartIdx, 4, 4, 8, 0 )	(6-15)

y = InverseRasterScan( subMbPartIdx, 4, 4, 8, 1 )	(6-16)

When both ( mb_type is equal to B_Skip or B_Direct_16x16, or mb_type is equal to B_8x8 and sub_mb_type[ mbPartIdx ] is equal to B_Direct_8x8 ) and a VSP mode is in use, it is asserted that the operation of clause 6.4.2.2 is incorrect, i.e. assumes sub-macroblocks of size 4x4, whereas it should assume sub-macroblocks of size 8x8.

# Proposal

The detailed proposal is indicated with change marks of user identity "JCT3V-K0022v1" in the accompanying specification text document. The proposal is provided on top of the specification text provided for ITU-T's editorial process in producing the published H.264 version 9 standard, having the file name T-REC-H 264-201402-P!!MSW-E_annotated_g4_mh0.docx.

The proposal can be summarized as follows, with enumeration matching the problem statement above:

- sub_mb_type is semantically constrained in clause J.7.4.6.2 to indicate a sub-macroblock type having an 8x8 sub-macroblock size, when bvsp_flag_lX is equal to 1.

- Table 7-13 (macroblock type values for P and SP slices) is overridden in Annex J so that when mb_type is equal to P_skip and MbVSSkipFlag is equal to 1, NumMbPart( P_Skip ) is specified to be equal to 4, MbPartWidth( P_Skip ) is specified to be equal to 8, and MbPartHeight( P_Skip ) is specified to be equal to 8.

- Clause 6.4.2.2 (inverse sub-macroblock partition scanning process) is overridden in Annex J so that it operates otherwise as earlier but when a VSP mode is in use and mb_type is equal to B_Skip or B_Direct_16x16, or mb_type is equal to B_8x8 and sub_mb_type[ mbPartIdx ] is equal to B_Direct_8x8, its operation uses sub-macroblocks of size 8x8.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).