| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Geneva, CH, 12–18 Feb. 2015 | Document: JCT3V-K0024 |
| --- | --- |

| Title: | [A Virtual View Quality Estimation Method] |  |  |
| --- | --- | --- | --- |
| Status: | [Input Document to JCT-3V] |  |  |
| Purpose: | [Proposal] |  |  |
| Author(s) or Contact(s): | Hui Yuan School of Information Science of Engineering, Shandong University, Ji’nan, China, 250100. | Tel: Email: | [+86-15169092917] [yuanhui0325@gmail.com] |
| Source: | [Shandong University] |  |  |

_____________________________

# Abstract

In order to reduce the complexity of 3DV system, it is better to estimate virtual view distortions or PSNRs directly. In this proposal, the virtual view synthesis procedure and the distortion propagation from existing views to virtual views are analyzed, and then a virtual view quality estimation model is derived line by line based on a typical view synthesis procedure with forward warping which has been adopted by the 3DVC platform, i.e. 3D-HTM. Experimental results demonstrate that the proposed method could estimate PSNRs of virtual views accurately. The correlation coefficients between the estimated distortion and the actual distortion of virtual views are all larger than 0.99 for all the test pictures. Besides, the proposed method is also friendly for parallel design.

# Proposed Method

In a 3DV system, depth maps are used for rendering virtual views. Depth maps compression will affect the quality of virtual views indirectly. For a set of well calibrated 3D videos captured by cameras with parallel arrangement, DIBR could be represented as a disparity compensation procedure, in which the relationship between a depth value and the corresponding disparity value could be written as (1),

,                        (1)

where I denotes a pixel value of depth maps, disp denotes the corresponding disparity value, f is the focal length, l is the baseline width (horizontal interval between two view points), Znear and Zfar are the nearest and farthest object distance of the scene. Equation (1) could be rewritten as a simple form,

,                                   (2)

where α and β depends on camera parameters, i.e. f, l, Znear, and Zfar. When depth maps are compressed, the reconstructed pixel value Iꞌ could be represented as,

,                                   (3)

where dispꞌ is the corresponding disparity value of Iꞌ.

Fig. 1. Flow diagram for VSRS 1D mode [5].

In a typical virtual view synthesis procedure, as shown in Fig. 1, the existing texture videos (left view and right view) and the associate depth maps (left depth map and right depth map) are first warped to the virtual view position to generate 2 virtual views (generated from the left view and right view respectively) and 2 virtual depth maps (generated from the left depth map and the right depth map). At the same time, two boundary masks are also generated from the left and right depth maps. Then, the two virtual views and virtual depth maps are merged to generate a virtual view and virtual depth map, while the two boundary masks are merged to generate a hole mask. Finally, the merged virtual view and depth map are modified by the hole mask to generate the final virtual view and virtual depth map.

Therefore, the coding error of depth maps will first affect the quality of the 2 virtual views (denoted as Vlm and Vrm respectively) generated from the left and right views, and then affect the quality of the final merged virtual view (denoted as Vm). Let ed,l{x} and ed,r{x} represent the coding error of pixel x in the left and the right depth map respectively, Il(x) and Ir(x) represent the original depth pixel value of x, Ilꞌ(x) and Irꞌ(x) represent the reconstructed depth pixel value of x. Accordingly, the corresponding disparity of Il(x), Ir(x), Ilꞌ(x), and Irꞌ(x) could be represented as disp,l(x), disp,r(x), disp,lꞌ (x), and disp,rꞌ (x). When depth maps are not compressed, the pixel xl in the left view will be warped to pixel position xvl of the Vl, i.e.,

,                                 (4)

where Tl(xl) represents the xlth pixel value of the left view. When depth maps is compressed, the pixel value in the xvlth position of Vlm will be filled by another pixel value Tl(yl), i.e.

.                                  (5)

Subsequently, the squared error (SEline,l) of the xvlth pixel in the current line of the virtual view could be written as,

,                            (6)

in which the relationship between xl and yl could be written as,

.                               (7)

By taking (2) and (3) into consideration, (12) could be rewritten as,

(8)

where Δl(xl)=ed,l(xl)/α, τl(xl)=[Ilꞌ(xl)-Ilꞌ(yl)]/α. Accordingly, SEline,l could be represented as,

(9)

It could be observed from (9) that the SEline,l could be calculated when Δl(xl) and τl(xl) are confirmed. Because Δl(xl) could be obtained from ed,l(xl) and α directly, the main problem is how to obtain τl(xl). From (8), we can observe that τl(xl) depends on Ilꞌ(xl)-Ilꞌ(yl) in which Ilꞌ(xl) and Ilꞌ(yl) represent the pixel values of position xl and yl in the reconstructed left view depth maps. Therefore, in order to calculate τl(xl), yl must be confirmed. For a certain pixel position xl, yl and is hard to be known; however, since yl depends on the compression of depth maps, i.e. the probability distribution of yl is the same with that of ed,l(xl). Therefore, yl could be thought as a stochastic variables whose probability distribution is the same with that of ed(xl). Subsequently, for a set of pixel positions, the expectation of τl(xl) could be calculated by (10),

,                  (10)

where bl is the maximum distance between xl and yl, and could be calculated by (11),

(11)

where Δl,0, Δl,1, … and Δl,W-1 are the disparity errors of each pixel position in the current line. As a result, the mean squared error of the current line in Vlm could be rewritten as,

,                   (12)

where W is the width of the picture. Subsequently, by using the same way, MSEline, r could be calculated as,

.                  (13)

Then, for each line of the merged virtual view, the mean squared error (denoted as MSEline) could be written as,

(14)

where ωl and ωr are weighted coefficients (defined as 0.5 for both ωl and ωr) for the left and the right view respectively. Finally, for the whole virtual view image, the mean squared error (MSE) could be written as,

(15)

where H is the height of the virtual view image.

Based on the above analysis, the virtual view distortion could be estimated by using the following procedure.

Step 1: Calculate the coding error, i.e. ed,l and ed,r, as well as the corresponding Δl and Δr for each pixel of the left and the right view compressed depth maps.

Step 2: Calculate bl and b based on the maximum value of Δl and Δr.

Step 3: Calculate E{τl(xl)}and E{τr(xr)} by using (10).

Step 4: Calculate MSEline,l and MSEline,r for the each line based on (12) and (13).

Step 5. Calculate MSEline based on (14) for each line, and get the final MSE of virtual view based on (15).

# Experimental Results

In order to verify the accuracy of the proposed distortion estimation method, 8 3DV sequences [1] which are adopted by Joint Collaborative Team on 3D Video Coding (JCT-3V), i.e. BookArrival, Newspapercc, Kendo, Balloons , GhostTwonFly, UndoDancer, PoznanStreet, and PoznanHall2[2], with different content and camera parameters were used. Those sequences are encoded with texture/depth QP pairs of (15, 24), (20, 29), (25, 34), (30, 39), (35, 42), and (40, 45) by 3D video coding test platform version 9.2 (3D-HTM9.2) [3] with configurations defined by JCT-3V common test condition [4]. The View Synthesis Reference Software [3] adopted by JCT-3V are used for rendering virtual views.

Fig. 2 compares the estimated PSNRs by the proposed method with the actual PSNRs of the synthesized virtual view. It can be observed that the trends of the estimated PSNRs by the proposed method and the actual PSNRs are the same for all the tested pictures for the proposed method. Because that there are a lot of quality enhancement filtering algorithms in the view synthesis procedure, the estimated PSNRs are a little lower than the actual PSNRs. From TABLE I, it can be observed that the squared correlation coefficients (SCC) and the root of mean squared error (RMSE) between the estimated PSNRs by the proposed method and the actual PSNRs are 0.998388 and 2.011843 on average respectively, which indicates that the proposed distortion estimation method is accurate.

(a)                                   (b)

(c)                                   (d)

(e)                                   (f)

(g)                                   (h)

Fig. 2. Actual PSNRs and estimated PSNRs comparison.

TABLE I. SCC and RMSE between the actual PSNRs and the estimated PSNRs.

| Sequence | Proposed Method |  |
| --- | --- | --- |
|  | SCC | RMSE |
| Balloons | 0.999978 | 1.799801 |
| BookArrival | 0.999154 | 2.683928 |
| Kendo | 0.999978 | 1.862134 |
| Newspapercc | 0.997245 | 2.835631 |
| GhostTownFly | 0.998464 | 1.419062 |
| UndoDancer | 0.992881 | 1.396316 |
| PoznanHall2 | 0.999911 | 2.554155 |
| PoznanStreet | 0.999494 | 1.543717 |
| Average | 0.998388 | 2.011843 |

- Joint Collaborative Team for 3DV, 3D Video Test Sequences [Online]. Available: ftp.hhi.fraunhofer.de, Sep. 2013.

- Marek Domański, Tomasz Grajek, Krzysztof Klimaszewski, Maciej Kurc, Olgierd Stankiewicz, Jakub Stankowski, Krzysztof Wegner "Poznań Multiview Video Test Sequences and Camera Parameters", ISO/IEC JTC1/SC29/WG11 MPEG 2009/M17050, Xian, China, October 2009

- Joint Collaborative Team for 3DV, 3D-HTM software platform [Online]. Available: https://hevc.hhi.fraunhofer.de/svn/ svn_3DVCSoftware/tags/, Mar. 2013.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” in Proc.3rd Meeting of ITU-T/ISO/IEC Joint Collaborative Team on 3D Video Coding (JCT-3V), document JCT3V-C1100, Jan. 2013.

# Acknowledgement

This work was supported in part by the National Natural Science Foundation of China under Grants 61201211; in part by the Ph.D. Programs Foundation, Ministry of Education of China under Grant 20120131120032; in part by the Excellent Youth Scientist Award Foundation of Shandong Province under Grants BS2012DX021.

# Patent rights declaration(s)

Shandong University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).