| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Geneva, CH, 12 – 18 February 2015 | Document: JCT3V-K0025 |
| --- | --- |

| Title: | Alignment of TMVP derivation process between 3D-HEVC and HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jae-Yung Lee Jong-Ki Han 910A Choongmu-Gwan Gunja-Dong, Gwangjin-Gu, 143-747, Seoul, Korea | Tel: Email: | +82-2-3408-3739 ljy321456@gmail.com hjk@sejong.edu |
| Source: | Sejong University |  |  |

_____________________________

# Abstract

In the current version of 3D-HEVC, because the interview reference picture is marked as long-term reference picture, the codec is unable to discriminate between interview prediction and temporal long-term prediction. It results in crash in the scaling process to derive the TMVP for merge mode when IvMvScalingFlag is set to ‘1’ and the collocated block is predicted from the temporal long-term reference pictures. Such problem is also occurred when IvMvScalingFlag is set to ‘0’ and the current block and the collocated block are predicted from a temporal long-term reference picture and an inter-view reference picture, respectively. In those cases, the scaling factor used to derive TMVP becomes ‘infinity’, because the denominator of the equation to calculate the scaling factor is ‘0’. This contribution solves the problem by disabling the TMVP for those cases.

# Introduction

As shown in Fig. 1, when both the current block and the collocated block are predicted from their interview reference pictures, respectively, those reference pictures are marked as ‘long-term prediction’ in the current version of 3D-HEVC. In this case, the scaling factor used to derive the TMVP depends on the differences between view IDs of the related pictures.

Figure 1. Both the current block and the collocated block are predicted from their interview reference pictures, respectively. The reference pictures of the current block and the collocated block are marked as ‘long-term reference picture’.

Marking the interview reference pictures as ‘long-term reference picture’ may result in crash in some cases, because it doesn’t discriminate between the inter-view prediction and the temporal long-term prediction.

# Problem statement

## Problem case 1

Figure 2 shows a case where the current block is predicted from a block in the reference picture of the other view, while the collocated block is predicted from the temporal long-term reference picture. In this case, those two reference pictures are marked as ‘long-term reference picture’. Because IvMvScalingFlag is set to ‘1’, the scaling factor used to derive the TMVP is calculated by using the differences between view IDs of the related pictures. In this case, the scaling factor used to derive the TMVP becomes ‘infinity’, because the difference between view IDs of the collocated block and its reference picture is ‘0’. It means that the scaling process results in software crash.

Figure 2. An example of case 1

## Problem case 2

Fig. 3 shows another example case which results in software crash, where the current block is predicted from the temporal long-term reference picture while the collocated block is referencing to a block of picture in other view. In this case, IvMvScalingFlag is set to 0, and the scaling factor used to derive TMVP is calculated by using the differences between POCs of the related pictures. Because the difference between POCs of the collocated block and the reference picture of the collocated block is ‘0’, the denominator of the scale factor used to derive the TMVP is equal to ‘0’. Thus, the scaling factor becomes ‘infinity’.

Figure 3. An example of case 2

# Proposed method

## Solution

This contribution proposes a method to avoid cases which results in crash in the scaling process to derive TMVP. The proposed method checks the difference between View IDs to specify the referencing type (among the inter-view prediction and temporal prediction) for the current and collocated blocks. The proposed process to derive the TMVP is summarized in Table 1.

Table 1. Summary of proposed TMVP derivation process

| Current block | Collocated block | Derived TMVP |
| --- | --- | --- |
| Inter-view prediction | Inter-view prediction | Scaled TMVP based on ViewID |
| Non Inter-view prediction | Inter-view prediction | Disabling TMVP |
| Inter-view prediction | Non Inter-view prediction | Disabling TMVP |
| Non Inter-view prediction | Non Inter-view prediction | Derive TMVP as HEVC v.1 does |

The related part in the draft text [1] should be modified as follows: (added part is highlighted, and the deleted part is strikethrough)

I.8.5.3.2.9 Derivation process for collocated motion vectors in merge mode

….

and mvLXCol and availableFlagLXCol are derived as follows:

  - The variables curIvFlag and colIvFlag, specifying whether inter-view prediction is utilized for the current and collocated PU are derived as:

      - curIvFlag = LongTermRefPic( currPic, currPb, refIdxLX, LX )	(I-129)

      - colIvFlag = LongTermRefPic( ColPic, colPb, refIdxCol, listCol )	(I-130)

- curIvFlag = (DiffViewId( currPic, RefPicListX[ refIdxLX ] ) != 0)	(I-129)

      - colIvFlag = (DiffViewId( ColPic, refPicListCol[ refIdxCol ] ) != 0)	(I-130)

  - When curIvFlag is not equal to colIvFlag LongTermRefPic( currPic, currPb, refIdxLX, LX ) is not equal to LongTermRefPic( ColPic, colPb, refIdxCol, listCol ), and AltRefIdxLX is not equal to −1, the variables, AltRefFlagLX, refIdxLX and curIvFlag are modified as follows:

      - AltRefFlagLX = 1		(I-131)

      - refIdxLX = AltRefIdxLX		(I-132)

      - curIvFlag = LongTermRefPic( currPic, currPb, refIdxLX, LX )	(I-133)

      - curIvFlag = (DiffViewId( currPic, RefPicListX[ refIdxLX ] ) != 0)	(I-133)

  - The motion vector mvLXCol is modified as follows:

    - If curIvFlag is not equal to colIvFlag or LongTermRefPic( currPic, currPb, refIdxLX, LX ) is not equal to LongTermRefPic( ColPic, colPb, refIdxCol, listCol ), both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

    - Otherwise, the variable availableFlagLXCol is set equal to 1, refPicListCol[ refIdxCol ] is set to be the picture with reference index refIdxCol in the reference picture list listCol of the slice containing prediction block currPb in the collocated picture specified by ColPic, and the following applies:

        - The variables colDiff and currDiff specifying a POC or ViewId difference are derived as follows:

          - If curIvFlag is equal to 0 or ( ( ViewIdx  !=  0 )  &&  IvMvScalingFlag ) is equal to 0, the following applies:

              - colDiff = DiffPicOrderCnt( ColPic, refPicListCol[ refIdxCol ] )	(I-134)

              - currDiff = DiffPicOrderCnt( currPic, RefPicListX[ refIdxLX ] )	(I-135)

          - Otherwise (curIvFlag is equal to 1 and ( ( ViewIdx  !=  0  )  &&  IvMvScalingFlag ) is equal to 1), the following applies:

              - colDiff = DiffViewId( ColPic, refPicListCol[ refIdxCol ] )	(I-136)

              - currDiff = DiffViewId( currPic, RefPicListX[ refIdxLX ] )	(I-137)

        - If colDiff is equal to currDiff or (LongTermRefPic( currPic, currPb, refIdxLX, LX) == 1 && curIvFlag != 1) is equal to 1, mvLXCol is derived as follows:

            - mvLXCol = mvCol		(I-138)

….

With this modifications, the software run normally without crash under a test condition where both the temporal long-term prediction and the interview prediction are used.

## Bug-fix: Alternative reference index in merge mode

When the TMVP is derived in merge mode, MV of the right-bottom block of the collocated block is considered as a candidate for TMVP. If the MV can’t be used because the prediction types of the current block and the right-bottom block are different from each other, for example, one is referencing to the short-term reference picture and other is encoded with the long-term reference picture, then the reference index of the current block is set to AltRefIdx. Note that the reference index of the current block had been initialized with 0 before considering the right-bottom block. If the MV of the right-bottom block is not able to be used as a candidate for TMVP in spite of using AltRefIdx, then MV of the center block of the collocated block is considered. Note that the reference index of the current block should be initialized with 0, before considering MV of the center block. However, in the current reference software [2], the 2nd initialization is omitted. While the reference index of the current block is setting to AltRefIdx, considering the MV of the center block may result in crash. We propose to fix the bug by adding the 2nd initialization just before considering the MV of the center block.

The related part in the draft text [1] should be modified as follows: (added part is highlighted, and the deleted part is strikethrough)

I.8.5.3.2.2 Derivation process for luma motion vectors for merge mode

…

.The variables AltRefFlagL0 and AltRefFlagL1 are set equal to 0.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived by the following ordered steps:

  - The derivation process for the base merge candidate list as specified in clause Error: Reference source not found is invoked with the luma location ( xCb, yCb ), the luma location ( xPb, yPb ), the variables nCbS, nPbW, nPbH, and the partition index partIdx as inputs, and the output being a modified luma location ( xPb, yPb ), the modified variables nPbW and nPbH, the modified variable partIdx, the luma location ( xOrigP, yOrigP ), the variables nOrigPbW and nOrigPbH, the merge candidate list baseMergeCandList, the luma motion vectors mvL0N and mvL1N, the reference indices refIdxL0N and refIdxL1N, and the prediction list utilization flags predFlagL0N and predFlagL1N, with N being replaced by all elements of baseMergeCandList.

  - The reference indices for the temporal merging candidate, refIdxLXCol, with X being 0 or 1, are set equal to 0.

  - For N being replaced by A1, B1, B0, A0 and B2, the following applies:

    - If N is an element in baseMergeCandList, availableFlagN is set equal to 1.

    - Otherwise (N is not an element in baseMergeCandList), availableFlagN is set equal to 0.

…

I.8.5.3.2.8 Derivation process for temporal luma motion vector prediction in merge mode

…

The variables mvLXCol and availableFlagLXCol are derived as follows:

- If slice_temporal_mvp_enabled_flag is equal to 0, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- Otherwise, the following ordered steps apply:

- The bottom right collocated motion vector is derived as follows:

xColBr = xPb + nPbW	(8-1)

yColBr = yPb + nPbH	(8-2)

- If yPb  >>  CtbLog2SizeY is equal to yColBr  >>  CtbLog2SizeY, yColBr is less than pic_height_in_luma_samples, and xColBr is less than pic_width_in_luma_samples, the following applies:

- The variable colPb specifies the luma prediction block covering the modified location given by ( ( xColBr  >>  4 )  <<  4, ( yColBr  >>  4 )  <<  4 ) inside the collocated picture specified by ColPic.

- The luma location ( xColPb, yColPb ) is set equal to the top-left sample of the collocated luma prediction block specified by colPb relative to the top-left luma sample of the collocated picture specified by ColPic.

- The derivation process for collocated motion vectors as specified in subclause Error: Reference source not found is invoked with currPb, colPb, ( xColPb, yColPb ), and refIdxLX as inputs, and the output is assigned to mvLXCol and availableFlagLXCol.

- Otherwise, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- When availableFlagLXCol is equal to 0, the central collocated motion vector is derived as follows:

xColCtr = xPb + ( nPbW  >>  1 )	(8-3)

yColCtr = yPb + ( nPbH  >>  1 )	(8-4)

- The reference indices for the temporal merging candidate, refIdxLXCol, with X being 0 or 1, are set equal to 0.

- The variable colPb specifies the luma prediction block covering the modified location given by ( ( xColCtr  >>  4 )  <<  4, ( yColCtr  >>  4 )  <<  4 ) inside the collocated picture specified by ColPic.

- The luma location ( xColPb, yColPb ) is set equal to the top-left sample of the collocated luma prediction block specified by colPb relative to the top-left luma sample of the collocated picture specified by ColPic.

…

- All invocations of the process specified in clause 8.5.3.2.9 are replaced with invocations of the process specified in clause Error: Reference source not found.

With this modifications, the software run normally without crash under a test condition where both the temporal long-term prediction and the interview prediction are used.

# Simulations result

The simulations for the proposed method are conducted based on HTM-13.0 [2]. Because the temporal long-term reference pictures are not considered in the common test conditions (CTC) [3], we make a Modified Test Condition (MTC) by modifying the CTC. The MTC configuration sets the intra period to 48, and marks the picture having POC 8 as ‘temporal long-term reference picture’.

The simulation results of the proposed method under the CTC are shown in Table 2. As shown in Table 2, no loss is generated from the proposed method under CTC.

Table 2. The results of the proposed method under CTC (3 View Case)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 100.1% | 116.8% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 100.0% | 100.1% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 99.8% | 102.1% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 92.9% | 99.4% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 103.6% | 102.8% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 102.4% | 99.1% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 101.9% | 104.7% |
| Shark | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 101.4% | 91.3% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 99.9% | 106.4% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 100.4% | 99.5% |
| average | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 100.3% | 102.1% |

The simulation of the current version of 3D-HEVC under the MTC results in software crash, whereas the proposed version of 3D-HEVC encodes and decodes the video sequences without any problem under the MTC. Because comparison between the current version and the proposed version of 3D-HEVC is meaningless, we don’t show the BD rate and BD PSNR of the codecs under MTC. The proposed method has been crosschecked by multiple companies.

# Patent rights declaration(s)

Sejong University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 6,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-J1001, Oct. 2014.

- HTM-13.0, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-13.0

- A. Vetro, and K. Müller, "Common Test Conditions for 3D Core Experiments," Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG JCT3V-G1100, Jan, 2014.