| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Geneva, CH, 12–18 Feb. 2015 | Document: JCT3V-K0027 |
| --- | --- |

| Title: | Centralized Texture-Depth Packing (CTDP) SEI Message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jar-Ferr Yang Ke-Ying Liao Hung-Ming Wang Chiu-Yu Chen | Tel: Email: | +886-6-2757575#62362 jefyang@mail.ncku.edu.tw |
| Source: | National Cheng Kung University (NCKU), 1 University Road, Tainan, Taiwan 701 |  |  |

_____________________________

# Abstract

This contribution defines a modified SEI Messages for inclusion of Centralized Texture-Depth Packing (CTDP) format, which is a prior contribution, JCT3V-J0108 [1]. This contribution adds a new CTDP content interpretation type to include the packing modes with two texture-plus-depth views, limits the frame size of half-reduced depth maps to be even, and provides additional experiment results. Besides, to promote the adoption of the CTDP formats, we have signed two agreements with ChungHwa Telecom (CHT) Company and Hsin Yeong An (HYA) Cable TV Company. The CHT MOD broadcasting system with about 1.4 million users provides digital video services through their high speed data networks in Taiwan. The HYA Cable TV Company with about 145 thousand users, which is a Tainan cable company and very near the National Cheng Kung University, will be a good test site for verifying the best CTDP parameters for 3D video services.

# Centralized_texture-depth_packing SEI message Syntax

- Replace the following specifications at clause 7.4.4:

general_non_packed_constraint_flag equal to 1 specifies that there are neither frame packing arrangement SEI messages, segmented rectangular frame packing arrangement SEI messages nor centralized texture-depth packing SEI messages present in the CVS. general_non_packed_constraint_flag equal to 0 indicates that there may or may not be one or more frame packing arrangement SEI messages, segmented rectangular frame packing arrangement SEI messages or centralized texture-depth packing SEI messages present in the CVS.

NOTE 1– Decoders may ignore the value of general_non_packed_constraint_flag, as there are no decoding process requirements associated with the presence or interpretation of frame packing arrangement SEI messages, segmented rectangular frame packing arrangement SEI messages or centralized texture-depth packing SEI messages.

In Subclause D.2.1 - General SEI message syntax, adds “centralized_texture-depth_packing (payloadSize)” :

| else if( payloadType == X ) | Descriptor |
| --- | --- |
| centralized_texture-depth_packing ( payloadSize ) /* specified in Annex D */ |  |

where “X” is an appropriate payloadType number in the sequence of defined payloadType numbers.

Add a new Subclause D.2.X “centralized_texture-depth_packing SEI message syntax”, where. X will be replaced with an appropriate subclause number in the sequence of subclauses present in the text.

- D.2.X	Centralized texture-depth packing (CTDP) SEI message syntax

| centralized_texture-depth_packing( payloadSize ) { | Descriptor |
| --- | --- |
| centralized_texture-depth_packing_cancel_flag | u(1) |
| if( ! centralized_texture-depth_packing_cancel_flag ) { |  |
| depth_sampling_factor | u(3) |
| centralized_texture-depth_packing_content_interpretation_type | u(3) |
| depth_spatial_flipping_flag | u(1) |
| depth_subpixel_arrangement_type | u(2) |
| texture_sampling_type | u(3) |
| depth_sampling_type | u(3) |
| depth_chroma_sampling_type | u(3) |
| centralized_texture-depth_packing_persistence_flag | u(1) |
| } |  |
| } |  |

In Table D-1, add a new persistence scope for centralized_texture-depth_packing SEI message as follows:

| - SEI message | - Persistence scope |
| --- | --- |
| centralized_texture-depth_packing | - Specified by the syntax of the SEI message |

Add a new Subclause D.3.X “Centralized texture-depth packing SEI message semantics” where X will be replaced with an appropriate subclause number in the sequence of subclauses present in the text. In the following Subclause D.3.X, the numbers of tables, figures, and equations started from Table D-X1, Figure D-X1, D-X1 will be also replaced with an appropriate numbers in the sequence of tables, figures, and equations present in the text, respectively.

- D.3.X Centralized texture-depth packing SEI message semantics

This SEI message informs the decoder that the output cropped decoded picture contains samples of multiple distinct spatially packed constituent frames that are packed into one frame using a centralized texture-depth packing arrangement. This information can be used by the decoder to appropriately rearrange the samples and process the samples of the constituent frames appropriately for display or other purposes (which are outside the scope of this Specification).

When general_non_packed_constraint_flag is equal to 1 in the active SPS for the current layer, there shall be no centralized texture-depth packing SEI message applicable for any picture of the CLVS of the current layer.

When a frame packing arrangement SEI message or a segmented rectangular frame packing arrangement SEI message is applicable for any picture of the CLVS of the current layer, there shall be no centralized texture-depth packing SEI messages applicable for any picture of the CLVS of the current layer.

centralized_texture-depth_packing_cancel_flag equal to 1 indicates that the centralized texture-depth packing arrangement SEI message cancels the persistence of any previous the centralized texture-depth packing in output order. centralized_texture-depth_packing_cancel_flag equal to 0 indicates that indicates that centralized texture-depth packing follows.

depth_sampling_factor indicates the sampling ratio of depth frames and their corresponding texture frames as specified in Table D-X1.

Table D-X1 – Definition of depth_sampling_factor

| Value | Interpretation |
| --- | --- |
| 0 | Unspecified the downscale method between the centralized texture-depth packed constituent frames. |
| 1 | Each depth frame in centralized texture-depth packing arrangement represents about 4:1 decimation from the original depth frame horizontally or vertically and about 12:11 decimation from the original texture frame horizontally or vertically, starting from the top or left-most sample. |
| 2 | Each depth frame in centralized texture-depth packing arrangement represents about 4:2 decimation from the original depth frame horizontally or vertically and about 12:10 decimation from the original texture frame horizontally or vertically, starting from the top or left-most sample. |
| 3 | Each depth frame in centralized texture-depth packing arrangement represents about 4:3 decimation from the original depth frame horizontally or vertically and about 12:9 decimation from the original texture frame horizontally or vertically, starting from the top or left-most sample. |

Values of depth_sampling_factor that do not appear in Table D-X1 are reserved for future specification by ITU-T |ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore centralized texture-depth packing SEI messages that contain reserved values of depth_sampling_factor.

centralized_texture-depth_packing_content_interpretation_type indicates the intended interpretation of the constituent frames as specified in Table D-X2.

Table D-X2 – Definition of centralized_texture-depth_packing_content_interpretation_type

| Value | Interpretation |
| --- | --- |
| 0 | Indicates that the packed frame is arranged in order of reduced_depth_top (RDT), resized_texture (RT), and reduced_depth_bottom (RDB) from top to bottom called as CTDP TB type as illustrated in Figure D-X1. |
| 1 | Indicates that the packed frame is arranged in order of reduced_depth_left (RDL), resized_texture (RT), and reduced_depth_right (RDR) from left to right called as CTDP LR type as illustrated in Figure D-X2. |
| 2 | Indicates that the packed frame, which provides two texture-plus-depth views, is arranged in order of reduced_depth_leftview_top (RDLT), reduced_depth_rightview_top (RDRT), resized_texture_leftview (RTL), resized_texture_rightview (RTR), reduced_depth_leftview_bottom (RDLB), and reduced_depth_rightview_bottom (RDRB), from upper left, upper right to lower right called as CTDP 2TB type as illustrated in Figure D-X3. |
| 3 | Indicates that the packed frame, which provides two texture-plus-depth views, is arranged in order of reduced_depth_topview_left (RDTL), reduced_depth_bottomview_left (RDBL), resized_texture_topview (RTT), resized_texture_bottomview (RTB), reduced_depth_topview_right (RDTR) and reduced_depth_bottomview_right (RDBR) from upper left, upper center to lower right called as CTDP 2LR type as illustrated in Figure D-X4. |

Values of centralized_texture-depth_packing_content_interpretation_type that do not appear in Table D-X2 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore centralized texture-depth packing SEI messages that contain reserved values of centralized_texture-depth_packing_content_interpretation_type.

Let W and H be the width and height as pic_width_in_luma_samples and pic_height_in_luma_samples, respectively, of the decoded frame output from the decoder in units of luma samples. For CTDP-TB and CTDP-2TB packing types, HRD denotes the height of the reduced depth regions, which are RDT, RDB, RDLT, RDRT, RDLB, and RDRB. For CTDP-LR and CTDP-2LR packing types, WRD denotes the width of the reduced depth regions, which are RDL, RDR, RDTL, RDTR, RDBL, or RDBR.

- if depth_sampling_factor equal to 1, the width or the height of the reduced depth region becomes

WRD = Floor(W/48)*2		(D-X1)

or

HRD = Floor(H/48)*2.		(D-X2)

Otherwise, if depth_sampling_factor equal to 2, the width or the height of the reduced depth region becomes

WRD = Floor(W/24)*2		(D-X3)

or

HRD = Floor(H/24)*2.		(D-X4)

Otherwise, if depth_sampling_factor equal to 3, the width or the height of the reduced depth region becomes

WRD = Floor(W/16)*2		(D-X5)

or

HRD = Floor(H/16)*2.		(D-X6)

Figure D-X1 Rearrangement and upconversion of a CTDP TB arrangement with centralized_texture-depth_packing_content_interpretation_type equal to 0

- When centralized_texture-depth_packing_content_interpretation_type equal to 0. The centralized texture-depth packing arrangement is composed of three-resized rectangular regions identified as RDT, RT, and RDB as illustrated in Figure D-X1.

The height of RDT and RDB regions is specified as HRD in units of frame luma samples and the height of RT region is specified in units of frame luma samples as:

HRT = H  HRD *2.		(D-X7)

After upsampling and rearrangement, the upconverted depth pictures are composed of two-rectangular regions identified as DT and DB as illustrated in Figure D-X1. After CTDP depacking, the width and height of the upconverted texture picture, T region, are W and H, respectively. The width and the height of DT and DB regions in units of frame luma samples are specified as W and H/2 in units of frame luma samples, respectively.

Figure D-X2 Rearrangement and upconversion of a CTDP LR arrangement with centralized_texture-depth_packing_content_interpretation_type equal to 1

- When centralized_texture-depth_packing_content_interpretation_type equal to 1. The centralized texture-depth packing arrangement is composed of three-resized rectangular regions identified as RDL, RT, and RDR as illustrated in Figure D-X2.

The width of regions RDL and RDR are specified as WRD in units of frame luma samples and the width of the region RT is specified in units of frame luma samples as:

WRT = W WRD *2.		(D-X8)

After upsampling and rearrangement, the upconverted depth pictures are composed of two-rectangular regions identified as DL and DR as illustrated in Figure D-X2. After CTDP depacking, the width and height of the upconverted texture picture, T region, are W and H, respectively. The width and the height of regions DL and DB are specified as W/2 and H in units of frame luma samples, respectively.

- When centralized_texture-depth_packing_content_interpretation_type equal to 2. The centralized texture-depth packing arrangement is composed of six-resized rectangular regions identified as RDLT, RDRT, RTL, RTR, RDLB, and RDRB as illustrated in Figure D-X3.

The height and the width of RDLT, RDRT, RDLB, and RDRB regions are specified as HRD and W/2 in units of frame luma samples, respectively.

The height and the width of RTL and RTR regions in units of frame luma sample are specified s as:

HRT = H  HRD *2,		(D-X9)

and W/2, respectively.

After upsampling and rearrangement, the upconverted depth pictures are composed of four-rectangular regions identified as DLT, DLB, DRT and DRB as illustrated in Figure D-X3. The width and the height of DLT, DLB, DRT and DRB regions are specified as H/2 and W, in units of frame luma samples, respectively. The width and the height of the upconverted texture pictures, TL and TR regions, are W and H, respectively.

Figure D-X3 Rearrangement and upconversion of a CTDP 2TB arrangement with centralized_texture-depth_packing_content_interpretation_type equal to 2

- When centralized_texture-depth_packing_content_interpretation_type equal to 3. The centralized texture-depth packing arrangement is composed of six-resized rectangular regions identified as RDTL, RDTR, RTT, RTB, RDBL, and RDBR as illustrated in Figure D-X4.

The height and the width of RDTL, RDTR, RDBL, and RDBR regions are specified H/2 and WRD as in units of frame luma samples, respectively.

The width and the height of RTT and RTB regions are specified in units of frame luma samples as follows:

WRT = W WRD *2		(D-X10)

and H/2, respectively.

After upsampling and rearrangement, the upconverted depth pictures are composed of four-rectangular regions identified as DTL, DTR, DBL, and DBR as illustrated in Figure D-X4. The height and the width of DTL, DTR, DBL, and DBR regions are specified as H and W/2 in units of frame luma samples, respectively. The width and the height of upsampled texture pictures, TT and TB regions, are W and H, respectively.

For centralized_texture-depth_packing_content_interpretation_type equal to 0, 1, 2, or 3 and depth_sampling_factor equal to 1, 2 or 3, texture and depth frames that are arranged by the specific order.

When centralized_texture-depth_packing_content_interpretation_type is not equal to 0, 1, 2, or 3, the value 0, 1 or 2 for depth_sampling_factor is reserved for future use by ITU-T | ISO/IEC. When centralized_texture-depth_packing_content_interpretation_type is not equal to 0, 1, 2, or 3, decoders shall ignore the value of depth_sampling_factor.

Figure D-X4 Rearrangement and upconversion of a CTDP 2LR arrangement with centralized_texture-depth_packing_content_interpretation_type equal to 3

depth_spatial_flipping_flag indicates that the reduced depth region are spatially flipped relative to its intended orientation for display or other such purposes.

When depth_spatial_flipping_flag is equal to 1, the type of depth spatial flipping that is indicated is as follows:

–	If centralized_texture-depth_packing_content_interpretation_type is equal to 0, the indicated depth spatial flipping is vertical flipping. The depth spatial vertical flipping process for each color component with Y, Cb, and Cr is given as:

for( x = 0; x < W; x++ )

for( y = 0; y < HRD; y++ )

{

tRDT[x][y] = RDT[x][HRD −y−1]

tRDB[x][y] = RDB[x][HRD −y−1]

}

where W and HRD are the width and the height of RDT and RDB regions of the decoded picture in samples in Figure D-X1, respectively. RDT[x][y], RDB[x][y] and tRDT[x][y], tRDB[x][y] are the sample values at the location (x, y) before and after the vertical flipping of the decoded picture, respectively. It is noted that the tRDT and tRDB are the temporary memories to store the sample values after the flipping. Finally, the temporary memories are restored as RDT = tRDT, RDB = tRDB.

–	If centralized_texture-depth_packing_content_interpretation_type is equal to 1, the indicated depth spatial flipping is horizontal flipping. The depth spatial horizontal flipping process for each component with Y, Cb, and Cr is given as:

for( x = 0; x < WRD; x++ )

for( y = 0; y < H; y++ )

{

tRDL[x][y] =RDL[WRD −x−1][y]

tRDR[x][y] = RDR[WRD −x−1][y]

}

where WRD and H are the width and the height of RDL and RDR regions of the decoded picture in samples in Figure D-X2, respectively. RDL[x][y], RDR[x][y] and tRDL[x][y], tRDR[x][y] denote the sample values at the location (x, y) before and after the horizontal flipping, respectively. It is noted that the tRDL and tRDR are the temporary memories to store the sample values after the flipping. Finally, the temporary memories are restored as RDL = tRDL and RDR = tRDR.

–	If centralized_texture-depth_packing_content_interpretation_type is equal to 2, the indicated depth spatial flipping is vertical flipping. The depth spatial vertical flipping process for each component with Y, Cb, and Cr is given as:

for( x = 0; x < W/2; x++ )

for( y = 0; y < HRD; y++ )

{

tRDLT[x][y] = RDLT[x][HRD −y−1]

tRDRT[x][y] = RDRT[x][HRD −y−1]

tRDLB[x][y] = RDLB[x][HRD −y−1]

tRDRB[x][y] = RDRB[x][HRD −y−1]

}

where HRD and W/2 are the height and the width of RDLT, RDRT, RDLB and RDRB regions of the decoded picture in samples in Figure D-X3, respectively. RDLT[x][y], RDRT[x][y], RDLB[x][y], RDRB[x][y] and tRDLT[x][y], tRDRT[x][y], tRDLB[x][y], tRDRB[x][y] are the sample values at the location (x, y) before and after the vertical flipping, respectively. It is noted that the tRDLT, tRDRT, tRDLB, and tRDRB are the temporary memories to store the sample values after the flipping. Finally, the temporary memories are restored as RDLT = tRDLT, RDRT = tRDRT, RDLB = tRDLB and RDRB = tRDRB.

–	If centralized_texture-depth_packing_content_interpretation_type is equal to 3, the indicated depth spatial flipping is horizontal flipping. The depth spatial horizontal flipping process for each component with Y, Cb, and Cr is given as:

for( x = 0; x < WRD; x++ )

for( y = 0; y < H/2; y++ )

{

tRDTL[x][y] =RDTL[WRD −x−1][y]

tRDTR[x][y] =RDTR[WRD −x−1][y]

tRDBL[x][y] = RDBL[WRD −x−1][y]

tRDBR[x][y] = RDBR[WRD −x−1][y]

}

where WRD and H/2 are the width and the height of RDTL, RDTR, RDBL and RDBR regions of the decoded picture in samples in Figure D-X4, respectively. RDLT[x][y], RDRT[x][y], RDLB[x][y], RDRB[x][y] and tRDLT[x][y], tRDRT[x][y], tRDLB[x][y], tRDRB[x][y] are the sample values at the location (x, y) before and after the vertical flipping, respectively. Finally, the temporary memories are restored as RDTL = tRDTL, RDTR = tRDTR, RDBL = tRDBL and RDBR = tRDBR.

–	Otherwise (depth_spatial_flipping_flag is equal to 0), the indicated spatial flipping is not necessary.

When centralized_texture-depth_packing_content_interpretation_type is not equal to 0, 1, 2, or 3, it is a requirement of bitstream conformance that depth_spatial_flipping_flag shall be equal to 0. When centralized_texture-depth_packing_content_interpretation_type is not equal to 0, 1, 2, or 3, the value 0 or 1 for depth_sampling_factor is reserved for future use by ITU-T | ISO/IEC. When centralized_texture-depth_packing_content_interpretation_type is not equal to 0, 1, 2, or 3, decoders shall ignore the value 1 for depth_spatial_flipping_flag.

depth_subpixel_arrangement_type indicates that the arrangement order of R, G, and B channels of a reduced depth region are rearranged into three depth pixels of resized depth region.

Figure D-X5. RGB-channels of a W’× H’ reduced depth region are arranged to three single-channels of resized depth in sequential order if depth_subpixel_arrangement_type equal to 0

–	When depth_subpixel_arrangement_type is equal to 0 indicates that R, G, and B channels of the reduced depth region are rearranged into three depth pixels of the resized depth region by using the sequence order specified in Figure D-X5.

if centralized_texture-depth_packing_content_interpretation_type is equal to 0 or 2, one horizontal RGB-channel line of reduced depth frame is expanded to three horizontal single-channel lines of resized depth frame such that the height, H’ will be increased to 3 times, 3H’, where the reduced depth region could be RDT or RDB and RDLT, RDLB, RDRT, or RDRB region specified in Figure D-X1 and Figure D-X3, respectively.

if centralized_texture-depth_packing_content_interpretation_type is equal to 1 or 3, one vertical line of reduced depth region is expanded to three vertical lines of resized depth region such that the width W’ will be increased to 3 times, 3W’, where the reduced depth region could be RDL or RDR and RDTL, RDBL, RDTR, or RDBR region specified in Figure D-X2 and Figure D-X4, respectively.

Figure D-X6 RGB-channels of a W’× H’ reduced depth region are arranged to three single-channels of resized depth in rotation order if depth_subpixel_arrangement_type equal to 1

–	When depth_subpixel_arrangement_type is equal to 1 indicates that R, G, and B channels of the reduced depth region are rearranged into three depth pixels of the resized depth region by using the rotation order specified in Figure D-X6.

if centralized_texture-depth_packing_content_interpretation_type is equal to 0 or 2, one horizontal RGB-channel line of reduced depth frame is expanded to three horizontal single-channel lines of resized depth frame such that the height, H’ will be increased to 3 times, 3H’, where the reduced depth region could be RDT or RDB and RDLT, RDLB, RDRT, or RDRB region specified in Figure D-X1 and Figure D-X3, respectively.

if centralized_texture-depth_packing_content_interpretation_type is equal to 1 or 3, one vertical line of reduced depth region is expanded to three vertical lines of resized depth region such that the width W’ will be increased to 3 times, 3W’, where the reduced depth region could be RDL or RDR and RDTL, RDBL, RDTR, or RDBR region specified in Figure D-X2 and Figure D-X4, respectively.

texture_sampling_type indicates that the method of the texture sampling is used by an encoder to indicate to a decoder as specified in Table D-X3.

Table D-X3 – Definition of texture_sampling_type

| Value | Interpretation |
| --- | --- |
| 0 | Encoder using bilinear method to downscale the vertical or horizontal resolution of texture frame |
| 1 | Encoder using bicubic method to downscale the vertical or horizontal resolution of texture frame |

NOTE 1–When centralized_texture-depth_packing_content_interpretation_type equal to 0 or 2, texture_sampling_type equal to 0 or 1, the texture frame is down scale the vertical resolution by using bilinear or bicubic method. When centralized_texture-depth_packing_content_interpretation_type equal to 1 or 3, texture_sampling_type equal to 0 or 1, the texture frame is down scaled in the horizontal resolution by using bilinear or bicubic method.

Values of texture_sampling_type that do not appear in Table D-X3 are reserved for future specification by ITU-T |ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore centralized texture-depth packing SEI messages that contain reserved values of texture_sampling_type.

depth_sampling_type indicates that the method of the depth sampling is used by an encoder to indicate to a decoder as specified in Table D-X4.

Table D-X4– Definition of depth_sampling_type

| Value | Interpretation |
| --- | --- |
| 0 | Encoder using bilinear method to downscale the vertical or horizontal resolution of depth frame |
| 1 | Encoder using bicubic method to downscale the vertical or horizontal resolution of depth frame |

NOTE 2–When centralized_texture-depth_packing_content_interpretation_type equal to 0 or 2, depth_sampling_type equal to 0 or 1, the depth frame is down scale the vertical resolution by using bilinear or bicubic method. When centralized_texture-depth_packing_content_interpretation_type equal to 1 or 3, depth_sampling_type equal to 0 or 1, the depth frame is down scaled in the horizontal resolution by using bilinear or bicubic method.

Values of depth_sampling_type that do not appear in Table D-X4 are reserved for future specification by ITU-T |ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore centralized texture-depth packing SEI messages that contain reserved values of depth_sampling_type.

depth_chroma_sampling_type indicates that the depth chroma sampling method from 444 to 422 or 420 format is used by an encoder to indicate to a decoder as specified in Table D-X5.

Table D-X5– Definition of depth_chroma_sampling_type

| Value | Interpretation |
| --- | --- |
| 0 | Encoder using the direct sample to convert the chroma sample from 444 to 422 or 420 format according to marked chroma sample location. |
| 1 | Encoder using the mean of two (four) samples to convert the chroma sample from 444 to 422 (420) format. |

Values of depth_chroma_sampling_type that do not appear in Table D-X5 are reserved for future specification by ITU-T |ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore centralized texture-depth packing SEI messages that contain reserved values of depth_chroma_sampling_type.

centralized_texture_depth_packing_persistence_flag specifies the persistence of the centralized texture-depth packing arrangement SEI message.

centralized_texture-depth_packing_persistence_flag equal to 0 specifies that the centralized texture-depth packing arrangement SEI message applies to the current decoded frame only.

centralized_texture-depth_packing_persistence_flag equal to 1 specifies that the centralized texture-depth packing arrangement SEI message persists in output order until any of the following conditions are true:

–	A new CVS begins.

–	The bitstream ends.

–	A frame in an access unit containing a centralized texture-depth packing SEI message with the same

Add a new Subclause D.3.X.1 “Reconstruction of the Centralized texture-depth packing” as follows, with " D.3.X.1 " replaced with an appropriate subclause number in the sequence of subclauses present in the text.

- D.3.X.1 Reconstruction of texture and depth information from the CTDP frame

The following describes the reconstruction of texture and depth information from CTDP frame by applying the centralized texture-depth packing SEI message syntax and semantics:

If the current decoded picture is not 4:4:4 colour YCbCr format, it should be converted to 4:4 4 colour YCbCr format. Let W and H be the width and height as pic_width_in_luma_samples and pic_height_in_luma_samples, respectively, of the decoded frame output from the decoder in units of luma and chroma samples.

When centralized_texture-depth_packing_content_interpretation_type equal to 0.

- According to depth_sampling_factor, the RDT, RT, and RDB regions from top to bottom in Figure D-X1 are extracted from the decoded picture by horizontal splitting.

- According to texture_sampling_type, the texture T is upsampled from reduced texture RT by a factor of H:HRT by bilinear interpolation if texture_sampling_type equal to 0 or by bicubic interpolation if texture_sampling_type equal to 1, where HRT stated in (D-X7) is determined by depth_sampling_factor.

- A color-packed depth frame RD is reconstructed by the combination of two, RDT and RDB, reduced depth regions if depth_spatial_flipping_flag equal to 0 or the combination of vertical spatial flips of RDT and RDB regions if depth_spatial_flipping_flag equal to 1. The colored RD is reconstructed in order of RDT, and RDB from top to bottom.

- According to depth_chroma_sampling_type, YCbCr colour 422 or 420 format is converted to YCbCr colour 444 format.

- The YCbCr colour plane of the color-packed RD is converted to RGB colour plane by using inverse colour format conversion by

RGB=1.1644−0.00011.59601.1644−0.3917−0.8131.16442.0173−0.0001YCbCr−16128128.                        	(D-X11)

- According to depth_subpixel_arrangement_type, the three depth pixels of resized RD are rearranged from one color depth pixel, which is with R, G, B subpixel channels in vertical direction of reduced depth regions, to obtain the resized depth frame.

- According to depth_sampling_type, the depth frame D is upsampled in the vertical direction from reduced depth RD with a factor of H:6HRD by bilinear interpolation if depth_sampling_type equal to 0 or by bicubic interpolation if depth_sampling_type equal to 1.

When centralized_texture-depth_packing_content_interpretation_type equal to 1.

- According to depth_sampling_factor, the RDL, RT, and RDR regions from left to right in Figure D-X2 are obtained from the decoded picture by vertical splitting.

- According to texture_sampling_type, the texture T is upsampled from reduced texture RT by a factor of W:WRT by bilinear interpolation if texture_sampling_type equal to 0 or by bicubic interpolation if texture_sampling_type equal to 1, where WRT stated in (D-X8) is determined by depth_sampling_factor.

- A color-packed depth frame RD is reconstructed by the combination of two, RDL and RDR reduced depth regions if depth_spatial_flipping_flag equal to 0 or the combination of horizontal spatial flips of RDL and RDR regions if depth_spatial_flipping_flag equal to 1. The colored RD is reconstructed in order of RDL, and RDR from left to right.

- According to depth_chroma_sampling_type, YCbCr colour 422 or 420 format is converted to YCbCr colour 444 format.

- The YCbCr colour plane of color-packed RD is converted to RGB colour plane by using inverse colour format conversion stated in (D-X11).

- According to depth_subpixel_arrangement_type, the three depth pixels of colored RD are rearranged from one color-depth pixel, which is with depth R, G, B subpixel channels in the horizontal direction to obtain the resized depth frame.

- According to depth_sampling_type, the depth frame D is upsampled in the horizontal direction from the resized depth frame with a factor of W:6WRD by bilinear interpolation if depth_sampling_type equal to 0 or by bicubic interpolation if depth_sampling_type equal to 1.

When centralized_texture-depth_packing_content_interpretation_type equal to 2.

- According to depth_sampling_factor, the RDLT, RDRT, RTL, RTR, RDLB, and RDRB regions in Figure D-X3 are extracted from the decoded picture by vertical and horizontal splitting.

- According to texture_sampling_type, two, leftview and rightview textures, TL and TR are upsampled from reduced texture regions, RTL and RTR with a factor of H:HRT in the vertical direction and a factor of 2:1 in the horizontal direction by bilinear interpolation if texture_sampling_type equal to 0 or by bicubic interpolation if texture_sampling_type equal to 1, where HRT stated in (D-X9) is determined by depth_sampling_factor.

- Two color-packed depth frames RDL and RDR are respectively reconstructed by combinations of two, RDLT and RDLB regions and two, RDRT and RDRB regions if depth_spatial_flipping_flag equal to 0 or the combinations of vertical spatial flips of RDLT and RDLB regions and RDRT and RDRB regions if depth_spatial_flipping_flag equal to 1. The colored RDL and RDR are reconstructed in order of RDLT and RDLB regions and RDRT and RDRB regions from top to bottom.

- According to depth_chroma_sampling_type, YCbCr colour 422 or 420 format is converted to YCbCr colour 444 format.

- The YCbCr colour planes of color-packed RDL and RDR are converted into visible RGB colour planes by using inverse color format conversion stated in (D-X11).

- According to depth_subpixel_arrangement_type, the three depth pixels of colored RDL and RDR are rearranged from one color-depth pixel, which is with depth R, G, B subpixel channels to obtain the reduced RDL and RDR in vertical direction.

- According to depth_sampling_type, the left and right depth frames DL and DR are upsampled from reduced depths RDL and RDR by a factor of H:6HRD in vertical direction and a factor of 2:1 in horizontal direction by bilinear interpolation if depth_sampling_type equal to 0 or by bicubic interpolation if depth_sampling_type equal to 1.

When centralized_texture-depth_packing_content_interpretation_type equal to 3.

- According to depth_sampling_factor, the RDTL, RDBL, RTT, RTB, RDTR, and RDBR regions in Figure D-X4 are obtained from the decoded picture by horizontal and vertical splitting.

- According to texture_sampling_type, the top and bottom textures, TT and TB are upsampled from reduced textures RTT and RTB and by a factor of W:WRT in horizontal direction and a factor of 2:1 in vertical direction by bilinear interpolation if texture_sampling_type equal to 0 or by bicubic interpolation if texture_sampling_type equal to 1, where WRT stated in (D-X10) is determined by depth_sampling_factor.

- Two color packed depth frames RDT and RDB are reconstructed by combinations of RDTL and RDTR regions and RDBL and RDBR regions if depth_spatial_flipping_flag equal to 0 or the combinations of horizontal spatial flips of RDTL and RDTR regions and RDBL and RDBR regions if depth_spatial_flipping_flag equal to 1, respectively. The colored RDT and RDB are reconstructed in order of RDTL and RDTR regions and RDBL and RDBR regions from left to right.

- According to depth_chroma_sampling_type, YCbCr colour 422 or 420 format is converted to YCbCr colour 444 format.

- The YCbCr colour planes of color-packed RDT and RDB are converted into visible RGB colour planes by using inverse color format conversion stated in (D-X11).

- According to depth_subpixel_arrangement_type, the three depth pixels of colored RDT and RDB are rearranged from one color-depth pixel, which is with depth R, G, B subpixel channels to obtain the reduced RDT and RDB in horizontal direction.

- According to depth_sampling_factor, the top and bottom depth frames DT and DB are respectively upsampled from reduced depths RDT and RDB by a factor of W:6WRD in horizontal direction and a factor of 2:1 in vertical direction by bilinear interpolation if depth_sampling_type equal to 0 or by bicubic interpolation if depth_sampling_type equal to 1.

# Performance Measurements

To verify CTDP coding capability, we compare the coding performances of resized texture ratios of 3/4, 5/6, and 11/12 with CTDP TB and CTDP LR types coded by HEVC HM13.0 to those of 1-view plus 1-depth coded by 3D-HEVC HTM8.0. Besides, we also compare the coding performances of resized texture ratios of 3/4, 5/6, and 11/12 with CTDP 2TB and CTDP 2LR types coded by HEVC HM13.0 to those of 2-view plus 2-depth coded by 3D-HEVC HTM8.0.The experiments executed on HM13.0 and HTM8.0 are both with compression quality parameters are 27, 32, 37, and 42. We adopted two 3D video sequences, which are Poznan Hall (1920x1088), and Balloons (1024x768) to conduct the objective measurements.

Tables 1 and 2 respectively show the CTDP-TB coding performances of the averaged BDPSNR in dB and the averaged BDBR in % under All Intra (ai), Low Delay (ldp), and Random Access (ra) test conditions. For comparisons, the coding performances of texture and depth information are separately presented. We calculate the PSNR of reconstructed texture and depth frames to the original view and original depth ones, respectively. To calculate BDPSNR and BDBR, with respect to 1-view plus 1-depth format, are exhibited where the bitstreams are the consuming bits to restore reconstructed the texture and depth frames, respectively. In order to compare the view synthesized performances, the depacked texture and depth with corresponding camera parameters are used to generate a new viewpoint image through the depth image based rendering (DIBR), where we fill the holes by directly extending nearby background. Tables 3 and 4 respectively also show the CTDP-LB coding performances with respect to 1-view plus 1-depth format.

Table 1. Average BDPSNR (dB) results for CTDP TB type and 1-view plus 1-depth

Table 2. Average BDBR (%) results for CTDP-TB and 1-view plus 1-depth

Table 3. Average BDPSNR (dB) results for CTDP-LR and 1-view plus 1-depth

Table 4. Average BDBR (%) results for CTDP-LR and 1-view plus 1-depth

We also compare the coding performances of texture- 5/6, and 11/12 CTDP formats coded by HEVC HM13.0 to the coding performances of 2-view and 2-depth coded by 3D-HEVC HTM8.0. For the view synthesis, 3D-HEVC HTM8.0 and HEVC HM 13.0 uses VSRS to generate the virtual view. Tables 5 and 6 also show the performances of BDPSNR and BDBR with same coding conditions as Tables 1 and 2.

Table 5. Average BDPSNR (dB) results for CTDP-2TB and 2-view plus 2-depth

Table 6. Average BDBR (%) results for CTDP-2TB and 2-view plus 2-depth

We found that the coding performances of the proposed CTDP formats coded by HEVC HM13.0 are slightly worse than those of 1view plus 1 depth and 2 view plus 2 depth coded by 3D-HEVC HTM8.0. It is noted that we did not perform any depth enhancement method in the enlisted simulation results for all simulations.

# Technology Adoptions

In order to promote the CTDP formats, we have signed two Memorandum of Understandings (MOUs) with Chunghwa Telecom (CHT) Company and Hsin Yeong An (HYA) Cable TV Company, while the former with about 1.4 million customers is a national-wise company in Taiwan and the later with about 145 thousand customers is a local cable company near the National Cheng Kung University.

For CHT Company, the signed agreement in English is shown in Fig. 3.1(a). The test and adoption plan with time lines are designed as the following:

1. 03/01/2015: Propose and confirm the test and adoption detailed plan

2. 08/01/2015: Test of downloaded 3D CTDP services with computer clients

(Internal test in CHT and TOUCH Center only)

3. 05/01/2016: Services of 3D MOD services for computer clients

(Charge and open to the public and final CTDP format selection)

4. 05/01/2017: Test of 3D MOD services through Set-top-Boxes with separated CTDP depacking module (Open for selected users)

5. 05/01/2018：Test of 3D MOD services through Set-top-Boxes with Integrated CTDP depacking module (charged 3D services)

For HYA Cable TV Company, the signed agreement in English is shown in Fig. 3.1(b). The test and adoption plan with time lines are designed as the following:

1. 03/01/2015: Propose and confirm the test and adoption detailed plan

2. 08/01/2015: Test of downloaded 3D CTDP services with computer clients

(Internal test in HYA and TOUCH Center only)

3. 05/01/2016: Services of 3D CATV services for computer clients

(Charge and open to the public and final CTDP format selection)

4. 05/01/2017: Test of 3D CATV services through Set-top-Boxes with separated CTDP depacking module (Open for selected users)

5. 05/01/2018：Test of 3D CATV services through Set-top-Boxes with Integrated CTDP depacking module (charged 3D services)

(a) ChungHwa Telecom (CHT) Company               (b) Hsin Yeong An (HYA) Cable TV Company

Figure 3.1: Two signed adoption agreements by CHT Company and HYA CATV Company

# Conclusions

This contribution proposes modified CTDP SEI Message which is an update of the prior contribution JCT3V-J0108, by adding two-view plus two-depth type in centralized texture-depth packing content interpretation type. The additional experiment results are also providing in this contribution. The objective measurements of the proposed texture-3/4, texture-5/6 and texture-11/12 CTDP format with LR, TB, 2LR 2TB configurations are comparing to those of the one-view plus one-depth and two-view plus two-depth packing format by 3D-HEVC.

# References

[1]	J.-F. Yang, K.-Y. Liao. H.-M. Wang, and Y.-H. Hu, Centralized Texture-Depth Packing (CTDP) SEI Message Syntax, Document no. JCT3V-J0108, Jan. 2013.

[2]	J.-F. Yang, K.-Y. Liao. H.-M. Wang, and Y.-H. Hu, Additional Definitions of FPA SEI Message for Inclusion of Centralized Color-Depth Packing (CCDP) Formats, Document no. JCTVC-S0031, Jan. 2013.

# Patent rights declaration(s)

National Cheng Kung University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).