| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Geneva, CH, 12–18 Feb. 2015 | Document: JCT3V-K0030 |
| --- | --- |

| Title: | Results of Texture and Depth View Packing SEI message integration in ATM13.3 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Takanori Senoh, Koki Wakunami, Yasuyuki Ichihashi, Hisayuki Sasaki, Ryutaro Oi, Kenji Yamamoto Masayuki Tanimoto | Tel: Email: | {senoh, k.wakunami, y-ichihashi, sasaki, oi.ryutaro, k.yamamoto}@nict.go.jp tanimototentative3@yahoo.co.jp |
| Source: | NICT, NISRI |  |  |

_____________________________

# Abstract

This contribution reports revised results on the integration of texture and depth view packing SEI message in ATM reference software.

# Introduction

At the 10th JCT-3V meeting, software integration results were reported on the texture and depth view packing SEI message in ATM13.1 (JCT3V-J0062). After that, it was found the bit rate calculation in report.c in ATM13.1 did not work correctly when this SEI message uses different number of views for texture and depth map (ex. 2 for texture and 1 for depth). Also a bug in the view synthesis process was corrected, which had been detecting occlusion hole area wider than actual. This contribution reports correct results with the ATM13.3. The bit rates were lower than before and the artifacts were reduced. The results and the software are attached to this contribution.

# Experimental results

The encoding, decoding and view-synthesis processes are same as reported in JCT3V-J0062. The SEI message decoder in ATM13.3 outputs GVDViewSynth.cfg file, GVDCameraParam.txt file and GVDCameraParam.CSV file. GVDViewSynth.cfg file contains parameter values for depth_type, num_residual_texture_views_minus1, residual_depth_flag, and other parameter values used for the view synthesis. GVDCameraParam.txt file contains parameter values for intrinsic camera parameters (focal length, principal point co-ordinates) and extrinsic camera parameters (rotation matrix and x-translation) for view synthesis. GVDCameraParam.CSV file contains parameter values for Znear, Zfar and x-translation for all views.

Table 1 shows the test results for 3-view case. The enc time includes global depth generation, its encoding, residual view generation and encoding of base view, residual view and global depth map. Ren time includes the generation of eight views (view 0, 0.25. 0.5, 0.75, 1.25, 1.5, 1.75 and 2).

Table 1 Summary of GVD performance referring ATMv.13.3 HP mode for 3-view case.

Figure 1 Summary of GVD performance comparing to ATMv.13.3 HP mode for 3-view case.

# Patent rights declaration(s)

NICT (National Institute of Information and Communications Technology) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

NISRI (Nagoya Industrial Science Research Institute) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).