| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Geneva, CH, 12–18 Feb. 2015 | Document: JCT3V-K0031 |
| --- | --- |

| Title: | 3D-CE1: Segmental prediction in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang1, Jicheng An1, Xianguo Zhang1, Han Huang1, Jian-Liang Lin2, Shawmin Lei2 1 MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Kai Zhang +86-010-56900888 ext. 83890 {kai.zhang, jicheng.an, xianguo.zhang, h.huang, jl.lin, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents the results of CE1 in which a segmental prediction method for Inter-SDC and Intra-SDC is proposed in depth coding. Besides the current Inter-SDC/Intra-SDC mode, a segmental mode with 2 segments is added. For each segment, a single depth value is derived and assigned to all reconstructed samples within that segment. The experimental results reportedly show that the proposed method achieves 0.5% BD-rate saving for the synthesized views in the common test condition. And the complexity analysis reportedly shows that the proposed method does not increase decoding complexity in the worst case.

# Introduction

Inter-SDC/Intra-SDC is special prediction modes for depth coding in 3D-HEVC [1]. When Inter-SDC/Intra-SDC is applied, a normal inter-prediction/intra-prediction is conducted first and a coded offset is added to all prediction samples to derive the reconstructed sample values.

In a depth picture, the foreground and background usually hold very different brightness with sharp edges between them as an example demonstrated in Fig. 1. Because the coding process loses high-frequency information generally, the quality of reconstruction is deteriorated more when there are acute pixel value changes. The coding efficiency will be reduced when such reconstructed pixels are referred to as the prediction samples.

Fig. 1 A block with two segments with sharp sample value changes.

# Proposed method

In this contribution, a segmental prediction method for Inter-SDC (segInter-SDC) is proposed. For a CU which is coded with Inter-SDC mode, a flag is signaled to indicate whether to apply the normal Inter-SDC or the segInter-SDC. When the segInter-SDC is enabled, two steps are applied to obtain the reconstructed block from the prediction block.

In the first step, samples in the current block are classified into two segments. A sample is classified as segment 0 if its value is smaller than threshold T which is the average value of four corner samples in the prediction block as depicted in Fig. 2. Otherwise, it is classified as segment 1.

In the second step, a single value is derived for each segment in the reconstructed block. A single value denoted as V is calculated as V = E + O, where E is an estimated value for the segment of the prediction block and O is an offset transmitted to the decoder. When depth lookup table (DLT) is available, O is coded with DLT. An example of segInter-SDC for an 8x8 block is illustrated in Fig. 3.

E is calculated as the sample value appearing most often in the segment of the prediction block. In a simplified method, only one out of 4 samples are used to derive E as demonstrated in Fig. 4. E is defined as the sample value in black appearing most often in the segment of the prediction block. If no sample in a segment is colored in black, E is set to be 128 in default.

The segmental prediction method is also extended to Intra-SDC, termed as segIntra-SDC. The segIntra-SDC is performed in the same way as segInter-SDC except that the prediction block is produced by intra-prediction.

Fig. 2 Four corner samples (black) in an 8x8 block.

Fig. 3 An example of segInter-SDC with 2 segments for an 8x8 block.

Fig. 4 Samples (black) used to calculated E in an 8x8 block.

# Experimental results

The proposed method is integrated on HTM-13.0 [2]. Two tests are conducted under the common test condition (CTC) [3]. In test1, the segmental prediction method is implemented. In test2, the simplified method which only one uses out of 4 samples to derive E is applied. Table 1 and Table 2 show the results of test1 and test2 respectively. In both tests, the results show that the segmental prediction method can achieve 0.5% BD-rate saving for the synthesized views.

Table 1.  Performance for the test 1 (CTC)

Table 2.  Performance for the test 2 (CTC)

# Complexity analysis

The experimental results show that the proposed method increases the encoding time about 4% and does not increase the decoding time. A more detailed complexity analysis is presented below.

## Encoder analysis

### Encoder strategies for normal Inter-SDC/Intra-SDC and segInter-SDC/segIntra-SDC are analyzed.

### Normal Inter-SDC

Normal Inter-SDC mode is checked five times in the RDO process. An estimated offset K for a segment is calculated by subtracting the average value of the original sample and the prediction value of the segment. Five RDO checks are conducted with the offset equal to K-2, K-1, K, K+1 and K+2 respectively.

### Normal Intra-SDC

Normal Intra-SDC mode is checked six times in the RDO process. Besides the five RDO checks with the offset equal to K-2, K-1, K, K+1 and K+2, an additional RDO check is conducted with the offsets for all segments equal to 0.

### segInter-SDC/segIntra-SDC

segInter-SDC/segIntra-SDC mode is checked only twice in the RDO process. Comparing to the normal Intra-SDC mode, only the RDO check with the offset equal to K and that with the offsets for all segments equal to 0 are conducted. The RDO process for any other mode is not changed.

## Decoder analysis

### Bandwidth

The segmental prediction method only requires the samples in the prediction block; therefore, the proposed method does not increase the bandwidth.

### Operation numbers

Fig5. Decoding procedure for different modes

Different modes have different decoding procedure with different complexity as shown in Fig. 5. In practice, we only focus on the critical path which determine the decoding time in the worst case. In the following analysis, the segmental prediction mode is compared with other modes to determine whether it will form a new critical path for depth decoding.

Table 3 shows the complexity comparison between DMM4, non-skip inter mode and the segmental prediction mode. The segmental prediction mode requires much less operations because it does not involve transform or quantization. The comparison reveals that the segmental prediction mode is not the most complicated mode so it will not increase the decoding time in the worst case.

At the Strasbourg meeting, some experts showed a concern about the complexity without considering transform/quantization. Table 4 shows the complexity comparison between DMM 4 and the segmental prediction mode without considering transform. It can be seen that the segmental prediction mode in test 1 requires more operations than DMM4. But the segmental prediction mode in test 2 requires fewer operations than DMM4.

Table 3.	Complexity comparison

Table 4. Complexity comparison without considering transform.

# Conclusion

In the proposed segmental prediction mode, a single reconstructed sample value is derived and assigned to all reconstructed samples within a segment. The experimental results show that proposed method can achieve 0.5% BD-rate saving for the synthesized views in the common test condition. The complexity analysis also shows that the segmental prediction method does not increase the decoding complexity in the worst case.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 6,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-J1001, July. 2014.

- HTM-13.0, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-13.0.

- K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-G1100, Jan. 2014.