| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Geneva, CH, 12 – 18 February 2015 | Document: JCT3V-K0035_WD |
| --- | --- |

| Title: | Proposed text of 3D-HEVC Working Draft for JCT3V-K0035 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document provides the proposed text of 3D-HEVC Working Draft. The deletion is indicated in strikethrough.

I.8.3.9	Derivation process for the target reference index for residual prediction

This process is invoked when the current slice is a P or B slice.

Let currPic be the current picture.

The variables RpRefIdxL0 and RpRefIdxL1 are set equal to −1, and the variables RpRefPicAvailFlagL0 and RpRefPicAvailFlagL1 are set equal to 0.

The following applies for X in the range of 0 to 1, inclusive:

- When X is equal to 0 or the current slice is a B slice the following applies:

  - The variable pocDiff is set equal to 215 − 1.

  - For i in the range of 0 to num_ref_idx_lX_active_minus1, inclusive, the following applies:

    - The variable currPocDiff is set equal to Abs( PicOrderCnt( RefPicListX[ i ] ) − PicOrderCntVal ).

    - When currPocDiff is not equal to 0 and currPocDiff is less than pocDiff, the following applies:

        - pocDiff = currPocDiff		(Error: Reference source not found−1)

        - RpRefIdxLX = i		(Error: Reference source not found−2)

        - RpRefPicAvailFlagLX = 1		(Error: Reference source not found−3)

- The variable RpRefPicAvailFlag is derived as specified in the following:

  - RpRefPicAvailFlag = ( RpRefPicAvailFlagL0  | |  RpRefPicAvailFlagL1 )  &&  DispAvailFlag	(Error: Reference source not found−4)

…

I.7.4.9.5.2	Coding unit extension semantics

…

The variable rpEnableFlag is derived as specified in the following:

  - rpEnableFlag = IvResPredFlag  &&  RpRefPicAvailFlag  &&
	( CuPredMode[ x0 ][ y0 ]  !=  MODE_INTRA )  &&  ( PartMode  = =  PART_2Nx2N )	(Error: Reference source not found−5)

iv_res_pred_weight_idx[ x0 ][ y0 ] specifies the index of the weighting factor used for residual prediction. iv_res_pred_weight_idx[ x0 ][ y0 ] equal to 0 specifies that residual prediction is not used for the current coding unit. iv_res_pred_weight_idx[ x0 ][ y0 ] not equal to 0 specifies that residual prediction is used for the current coding unit. When not present, the value of iv_res_pred_weight_idx[ x0 ][ y0 ] is inferred to be equal to 0.

When DispAvailFlag is equal to 0, iv_res_pred_weight_idx[ x0 ][ y0 ] shall be equal to 0.

…