| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Geneva, CH, 12 – 18 February 2015 | Document: JCT3V-K0036 |
| --- | --- |

| Title: | Position Derivation for Shifted-IVMC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In HEVC and its extensions, in order to derive a neighboring block, the immediately neighboring pixels to the current block are typically used. However, currently the shifted-IVMC does not use the immediately adjacent pixel to derive the bottom-right block, but uses the position misplaced by (1, 1) pixels to the bottom-right pixel. To fix this problem, this contribution proposes to use the immediately bottom-right pixel for the shifted-IVMC. The proposed method does not change the coding performance.

# Introduction

In order to derive a neighboring block, the immediately neighboring pixels are typically used in HEVC and its extension. For example, for deriving spatial and temporal merge candidates in HEVC and 3D-HEVC, the immediately adjacent pixels are used as shown in Figure 1.

Figure 1. Neighboring pixels for deriving merge candidates

However, in the current 3D-HEVC, the shifted IVMC does not use the immediately adjacent pixel to derive the bottom-right block, but uses the pixel position misplaced by (1, 1) pixels to the bottom-right pixe as shown in Figure 2. Currently the pixel position used for the shifted-IVMC is derived by the relative distance of the center pixel position used for the IVMC. A few meetings ago, the center pixel position was changed from the top-left center to the bottom-right center, but the position calculation for the shifted-IVMC was not changed. This is the reason why the pixel position is misaligned.

Figure 2. Neighboring pixel position for IVMC and Shifted-IVMC

# Proposed Method

In order to fix this problem, it is proposed to use the immediately bottom-right pixel position as shown in Figure 3 for the shifted-IVMC to derive the motion information of bottom-right block of the corresponding block rather than to use the misaligned pixel position.

Figure 3. Proposed neighboring pixel position for Shifted-IVMC

The decoding process for inter-view predicted and disparity information merge candidate can be changed as follows. The deletions are indicated in strikethrough.

I.8.5.3.2.11 Derivation process for inter-view predicted and disparity information merge candidates … When DepthFlag is equal to 0 and ic_flag[ xPb ][ yPb ] is equal to 0, the shifted temporal inter-view motion vector merging candidate is derived by the following ordered steps. - For the prediction list indication X being 0 and 1 the following applies: - The derivation process for an inter-view predicted motion vector candidate as specified in clause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, the prediction list indication X, the view order index RefViewIdx[ xPb ][ yPb ], and the disparity vector MvRefinedDisp[ xPb ][ yPb ] + ( nPbW *2 + 4, nPbH *2 + 4 ) as inputs, and the outputs are the flag availableFlagLXIVShift, the motion vector mvLXIVShift and the reference index refIdxLXIVShift. - The availability flag availableFlagIVShift, and the prediction utilization flags predFlagL0IVShift and predFlagL1IVShift are derived by - availableFlagIVShift = availableFlagL0IVShift ¦ ¦ availableFlagL1IVShift (Error: Reference source not found−1) - predFlagL0IVShift = availableFlagL0IVShift (Error: Reference source not found−2) - predFlagL1IVShift = availableFlagL1IVShift (Error: Reference source not found−3) …

# Experimental Results

The simulation was carried out based on the common test condition [2] and HTM 13 reference software. Table 1 shows the performance comparison results between the current and the proposed method. The proposed method does not change the coding performance.

Table 1. Performance comparison results between the CTC anchor and the proposed method

# Conclusion

In this contribution, it is proposed to use the immediately bottom-right pixel position for the shifted-IVMC to derive the motion information of the bottom-right block rather than to use misaligned pixel position. The proposed method does not change the coding performance. And it is recommended to adopt the proposed fix into next 3D-HEVC draft text.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 6,” JCT3V-J1001, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Oct. 2014, Strasbourg, FR.

[2] D. Rusanovskyy, K. Mueller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-G1100, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Jan. 2014, San Jose, USA.