| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Geneva, CH, 12–18 Feb. 2015 | Document: JCT3V-K0038_v2 |
| --- | --- |

| Title: | Redundant signaling removal in SPS 3D extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sunmi Yoo Junghak Nam Sehoon Yea 19, Yangjaedaero-11gil Seocho-gu, Seoul 137-893 | Tel: Email: | +82-2-6912-6477 sunmi.yoo@lge.com junghak.nam@lge.com sehoon.yea@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In this contribution, an efficient method of the coding tool on/off signaling in the SPS 3D extension is proposed. At the last meeting, the coding tool activation signaling flags for each layer have been moved from VPS to SPS 3D extension. Although they are moved into the sequence parameter set, they still receive all coding tool flags for a sequence. Since it can be identified whether the current SPS is for the texture or depth sequence, by referring to its corresponding VPS, it is unnecessary for some flags to be transmitted. Therefore, it is proposed to receive only required coding tool switches according to the depth flag in the corresponding VPS.

# Problem Statement

As mentioned above, currently the coding tool activation flags are transmitted in the SPS 3D extension. However, even though the decoder can recognize if its sequence is texture or depth during the SPS parsing process, the coding tool flags for both texture and depth are transmitted. By using depth flag from the corresponding VPS, the redundant information for the current sequence can be removed.

# Proposed text

## Flag array method

- I.7.3.2.2.4	Sequence parameter set 3d extension syntax

| sps_3d_extension( ) { | Descriptor |
| --- | --- |
| iv_mv_pred_flag[ DepthFlag ] | u(1) |
| iv_mv_scaling_flag[ DepthFlag ] | u(1) |
| if( DepthFlag = = 0 ) { |  |
| log2_sub_pb_size_minus3[ DepthFlag ] | ue(v) |
| iv_res_pred_flag[ DepthFlag ] | u(1) |
| depth_refinement_flag[ DepthFlag ] | u(1) |
| view_synthesis_pred_flag[ DepthFlag ] | u(1) |
| depth_based_blk_part_flag[ DepthFlag ] | u(1) |
| } else { |  |
| mpi_flag[ DepthFlag ] | u(1) |
| log2_mpi_sub_pb_size_minus3[ DepthFlag ] | ue(v) |
| intra_contour_flag[ DepthFlag ] | u(1) |
| intra_sdc_wedge_flag[ DepthFlag ] | u(1) |
| qt_pred_flag[ DepthFlag ] | u(1) |
| inter_sdc_flag[ DepthFlag ] | u(1) |
| intra_single_flag[ DepthFlag ] | u(1) |
| } |  |
| } |  |
| } |  |

- I.7.4.3.2.4	Sequence parameter set 3d extension semantics

The variable DepthFlag is set equal to VpsDepthFlag[ nuh_layer_id ].

iv_mv_pred_flag equal to 1 specifies that inter-view motion parameter prediction may be used in the decoding process for CVSs referring to the SPS. iv_mv_pred_flag equal to 0 specifies that inter-view motion parameter prediction is not used in the decoding process for CVSs referring to the SPS. When not present, the value of iv_mv_pred_flag is inferred to be equal to 0.

iv_mv_scaling_flag equal to 1 specifies that motion vectors used for inter-view prediction in the decoding process for CVSs referring to the SPS may be scaled based on ViewId values. iv_mv_scaling_flag equal to 0 specifies that motion vectors used for inter-view prediction in the decoding process for CVSs referring to the SPS are not scaled based on ViewId values. When not present, the value of iv_mv_scaling_flag is inferred to be equal to 0.

log2_sub_pb_size_minus3[ DepthFlag ] is used to derive the value of the variable SubPbSize that is used in the decoding of prediction units using the inter-view merge candidate. When not present, the value of log2_sub_pb_size_minus3[ DepthFlag ] and log2_sub_pb_size_minus3[ !DepthFlag ]  are inferred to be equal to ( CtbLog2SizeY − 3 ). The value of log2_sub_pb_size_minus3[ DepthFlag ] and log2_sub_pb_size_minus3[ !DepthFlag ] shall be in the range of ( MinCbLog2SizeY − 3 ) to ( CtbLog2SizeY − 3 ), inclusive.

iv_res_pred_flag[ DepthFlag ] equal to 1 specifies that inter-view residual prediction may be used the decoding process for CVSs referring to the SPS. iv_res_pred_flag[ DepthFlag ] equal to 0 specifies that inter-view residual prediction is not used the decoding process for CVSs referring to the SPS. When not present, the value of iv_res_pred_flag[ DepthFlag ] and iv_res_pred_flag[ !DepthFlag ] are inferred to be equal to 0.

view_synthesis_pred_flag[ DepthFlag ] equal to 1 specifies that view synthesis prediction merge candidates may be used for CVSs referring to the SPS. view_synthesis_pred_flag[ DepthFlag ] equal to 0 specifies that view synthesis prediction merge candidates is not used the decoding process for CVSs referring to the SPS. When not present, the value of view_synthesis_pred_flag[ DepthFlag ] and view_synthesis_pred_flag[ !DepthFlag ] are inferred to be equal to 0.

depth_based_blk_part_flag[ DepthFlag ] equal to 1 specifies that depth based block partitioning is used in the decoding process for CVSs referring to the SPS. depth_based_blk_part_flag[ DepthFlag ] equal to 0 specifies that depth based block partitioning is not used in the decoding process for CVSs referring to the SPS. When not present, the value of depth_based_blk_part_flag[ DepthFlag ] and depth_based_blk_part_flag[ !DepthFlag ] are inferred to be equal to 0.

depth_refinement_flag[ DepthFlag ] equal to 1 specifies that depth layers may be used in the derivation process for a disparity vector in the decoding process . depth_refinement_flag[ DepthFlag ] equal to 0 specifies that depth components are not used in the derivation process for a disparity vector the decoding process for CVSs referring to the SPS. When not present, the value of depth_refinement_flag[ DepthFlag ] and depth_refinement_flag[ !DepthFlag ] are inferred to be equal to 0.

mpi_flag[ DepthFlag ] equal to 1 specifies that motion parameter inheritance may be used in the decoding process of the layer for CVSs referring to the SPS. mpi_flag[ DepthFlag ] equal to 0 specifies that motion parameter inheritance is not used in the decoding process of the layer for the CVSs referring to the SPS. When not present, the value of mpi_flag[ DepthFlag ] and mpi_flag[ !DepthFlag ] are inferred to be equal to 0.

log2_mpi_sub_pb_size_minus3[ DepthFlag ] is used for the derivation of the variable MpiSubPbSize that is used in the decoding of prediction units using the texture merge candidate. The value of log2_mpi_sub_pb_size_minus3[ DepthFlag==1 ] shall be in the range of ( MinCbLog2SizeY − 3 ) to ( CtbLog2SizeY − 3 ), inclusive.

intra_contour_flag[ DepthFlag ] equal to 1 specifies that the intra prediction mode INTRA_DEP_CONTOUR might be used in the decoding process for CVSs referring to the SPS. intra_contour_flag[ DepthFlag ] equal to 0 specifies that the intra prediction mode INTRA_DEP_CONTOUR is not used in the decoding process for CVSs referring to the SPS. When not present, intra_contour_flag[ DepthFlag ] and intra_contour_flag[ !DepthFlag ] are inferred to be equal to 0.

intra_sdc_wedge_flag[ DepthFlag ] equal to 1 specifies that SDC and the depth intra prediction mode INTRA_DEP_WEDGE might be used in the decoding process for coding units in intra prediction mode of CVSs referring to the SPS. intra_sdc_wedge_flag[ DepthFlag ] equal to 0 specifies that SDC and the depth intra prediction mode INTRA_DEP_WEDGE is not used in the decoding process for coding units in intra prediction mode of CVSs referring to the SPS. When not present, intra_sdc_wedge_flag[ DepthFlag ] and intra_sdc_wedge_flag[ !DepthFlag ] are inferred to be equal to 0.

qt_pred_flag[ DepthFlag ] equal to 1 specifies that prediction of a limited quadtree is used in the decoding process of CVSs referring to the SPS. qt_pred_flag[ DepthFlag ] equal to 0 specifies that prediction of a limited quadtree is not used in the decoding process for CVSs referring to the SPS. When not present, the value of qt_pred_flag[ DepthFlag ] and the qt_pred_flag[ !DepthFlag ] are inferred to be equal to 0.

inter_sdc_flag[ DepthFlag ] equal to 1 specifies that SDC might be used in the decoding process for coding units in inter prediction mode of the layer for CVSs referring to the SPS. inter_sdc_flag[ DepthFlag ] equal to 0 specifies that SDC is not used in the decoding process for coding units in inter prediction mode of the layer for CVSs referring to the SPS. When not present, inter_sdc_flag[ DepthFlag ] and inter_sdc_flag[ !DepthFlag ] are inferred to be equal to 0.

intra_single_flag[ DepthFlag ] equal to 1 specifies that the single depth mode might be used in the decoding process of CVSs referring to the SPS. intra_single_flag[ DepthFlag ] equal to 0 specifies that the single depth mode is not used in the decoding process for CVSs referring to the SPS. When not present, intra_single_flag[ DepthFlag ] and intra_single_flag[ !DepthFlag ] are inferred to be equal to 0.

…

- I.7.4.7.1	General slice segment header semantics

- …

The variables IvMvPredFlag, IvMvScalingFlag, SubPbSize, IvResPredFlag, ViewSynthesisPredFlag, DepthBasedBlkPartFlag, DepthRefinementFlag, MpiSubPbSize, IntraContourFlag, IntraSdcWedgeFlag, QtPredFlag, InterSdcFlag and DisparityDerivationFlag are derived as specified in the following:

  - IvMvPredFlag = ( nuh_layer_id > 0 )  &&
	NumRefListLayers[ nuh_layer_id ] > 0  &&  iv_mv_pred_flag	(Error: Reference source not found−1)

  - IvMvScalingFlag = ( nuh_layer_id > 0 )  &&  iv_mv_scaling_flag		(Error: Reference source not found−2)

  - SubPbSize  = 1  <<  ( nuh_layer_id > 0 ? log2_sub_pb_size_minus3[ DepthFlag ] + 3  : CtbLog2SizeY )		(Error: Reference source not found−3)

  - IvResPredFlag = ( nuh_layer_id > 0 )  &&
	NumRefListLayers[ nuh_layer_id ] > 0  &&  iv_res_pred_flag[ DepthFlag ]	(Error: Reference source not found−4)

  - ViewSynthesisPredFlag = ( nuh_layer_id > 0 )  &&  NumRefListLayers[ nuh_layer_id ] > 0  &&
	view_synthesis_pred_flag[ DepthFlag ]  &&  depthOfRefViewsAvailFlag	(Error: Reference source not found−5)

  - DepthBasedBlkPartFlag = ( nuh_layer_id > 0 )  &&
	depth_based_blk_part_flag[ DepthFlag ]  &&  depthOfRefViewsAvailFlag	(Error: Reference source not found−6)

  - DepthRefinementFlag = ( nuh_layer_id > 0 )  &&
	depth_refinement_flag[ DepthFlag ]  &&  depthOfRefViewsAvailFlag	(Error: Reference source not found−7)

  - MpiFlag = ( nuh_layer_id > 0 )  &&  mpi_flag[ DepthFlag ]  &&  textOfCurViewAvailFlag		(Error: Reference source not found−8)

  - MpiSubPbSize = 1  <<  ( log2_mpi_sub_pb_size_minus3[ DepthFlag ] + 3 )		(Error: Reference source not found−9)

  - IntraContourFlag = ( nuh_layer_id > 0 )  &&  
	intra_contour_flag[ DepthFlag ]  &&  textOfCurViewAvailFlag	(Error: Reference source not found−10)

  - IntraSdcWedgeFlag = ( nuh_layer_id > 0 )  &&  intra_sdc_wedge_flag[ DepthFlag ]		(Error: Reference source not found−11)

  - QtPredFlag = ( nuh_layer_id > 0 )  &&  qt_pred_flag[ DepthFlag ]  &&  textOfCurViewAvailFlag	(Error: Reference source not found−12)

  - InterSdcFlag = ( nuh_layer_id > 0 )  &&  inter_sdc_flag[ DepthFlag ]		(Error: Reference source not found−13)

  - IntraSingleFlag = ( nuh_layer_id > 0 )  &&  intra_single_flag[ DepthFlag ]		(Error: Reference source not found−14)

  - DisparityDerivationFlag = IvMvPredFlag | |  IvResPredFlag  | |
	ViewSynthesisPredFlag  | | DepthBasedBlkPartFlag	(Error: Reference source not found−15)

  - …

## Single flag method

It is proposed to use single flags instead of flag arrays which are distinguished by a depth flag, because sequence parameter set is referred by a sequence, and it is not used for both texture and depth sequences at the same time.

- I.7.3.2.2.4	Sequence parameter set 3d extension syntax

| sps_3d_extension( ) { | Descriptor |
| --- | --- |
| iv_mv_pred_flag | u(1) |
| iv_mv_scaling_flag | u(1) |
| if( DepthFlag = = 0 ) { |  |
| log2_sub_pb_size_minus3 | ue(v) |
| iv_res_pred_flag | u(1) |
| depth_refinement_flag | u(1) |
| view_synthesis_pred_flag | u(1) |
| depth_based_blk_part_flag | u(1) |
| } else { |  |
| mpi_flag | u(1) |
| log2_mpi_sub_pb_size_minus3 | ue(v) |
| intra_contour_flag | u(1) |
| intra_sdc_wedge_flag | u(1) |
| qt_pred_flag | u(1) |
| inter_sdc_flag | u(1) |
| intra_single_flag | u(1) |
| } |  |
| } |  |

- I.7.4.3.2.4	Sequence parameter set 3d extension semantics

The variable DepthFlag is set equal to VpsDepthFlag[ nuh_layer_id ].

iv_mv_pred_flag equal to 1 specifies that inter-view motion parameter prediction may be used in the decoding process for CVSs referring to the SPS. iv_mv_pred_flag equal to 0 specifies that inter-view motion parameter prediction is not used in the decoding process for CVSs referring to the SPS. When not present, the value of iv_mv_pred_flag is inferred to be equal to 0.

iv_mv_scaling_flag equal to 1 specifies that motion vectors used for inter-view prediction in the decoding process for CVSs referring to the SPS may be scaled based on ViewId values. iv_mv_scaling_flag equal to 0 specifies that motion vectors used for inter-view prediction in the decoding process for CVSs referring to the SPS are not scaled based on ViewId values. When not present, the value of iv_mv_scaling_flag is inferred to be equal to 0.

log2_sub_pb_size_minus3 is used to derive the value of the variable SubPbSize that is used in the decoding of prediction units using the inter-view merge candidate. When not present, the value of log2_sub_pb_size_minus3 is inferred to be equal to ( CtbLog2SizeY − 3 ). The value of log2_sub_pb_size_minus3 shall be in the range of ( MinCbLog2SizeY − 3 ) to ( CtbLog2SizeY − 3 ), inclusive.

iv_res_pred_flag equal to 1 specifies that inter-view residual prediction may be used the decoding process for CVSs referring to the SPS. iv_res_pred_flag equal to 0 specifies that inter-view residual prediction is not used the decoding process for the CVSs referring to the SPS. When not present, the value of iv_res_pred_flag is inferred to be equal to 0.

view_synthesis_pred_flag equal to 1 specifies that view synthesis prediction merge candidates may be used in the decoding process for CVSs referring to the SPS. view_synthesis_pred_flag equal to 0 specifies that view synthesis prediction merge candidates is not used the decoding process for CVSs referring to the SPS. When not present, the value of view_synthesis_pred_flag is inferred to be equal to 0.

depth_based_blk_part_flag equal to 1 specifies that depth based block partitioning is used in the decoding process for CVSs referring to the SPS. depth_based_blk_part_flag equal to 0 specifies that depth based block partitioning is not used in the decoding process for CVSs referring to the SPS. When not present, the value of depth_based_blk_part_flag is inferred to be equal to 0.

depth_refinement_flag equal to 1 specifies that depth layers may be used in the derivation process for a disparity vector in the decoding process for CVSs referring to the SPS. depth_refinement_flag[ d ] equal to 0 specifies that depth components are not used in the derivation process for a disparity vector the decoding process for CVSs referring to the SPS. When not present, the value of depth_refinement_flag is inferred to be equal to 0.

mpi_flag equal to 1 specifies that motion parameter inheritance may be used in the decoding process for CVSs referring to the SPS. mpi_flag[ d ] equal to 0 specifies that motion parameter inheritance is not used in the decoding process for CVSs referring to the SPS. When not present, the value of mpi_flag is inferred to be equal to 0.

log2_mpi_sub_pb_size_minus3 is used for the derivation of the variable MpiSubPbSize that is used in the decoding of prediction units using the texture merge candidate. The value of log2_mpi_sub_pb_size_minus3 shall be in the range of ( MinCbLog2SizeY − 3 ) to ( CtbLog2SizeY − 3 ), inclusive.

intra_contour_flag equal to 1 specifies that the intra prediction mode INTRA_DEP_CONTOUR might be used in the decoding process for CVSs referring to the SPS. intra_contour_flag[ d ] equal to 0 specifies that the intra prediction mode INTRA_DEP_CONTOUR is not used in the decoding process for CVSs referring to the SPS. When not present, intra_contour_flag is inferred to be equal to 0.

intra_sdc_wedge_flag equal to 1 specifies that SDC and the depth intra prediction mode INTRA_DEP_WEDGE might be used in the decoding process for coding units in intra prediction mode of CVSs referring to the SPS. intra_sdc_wedge_flag[ d ] equal to 0 specifies that SDC and the depth intra prediction mode INTRA_DEP_WEDGE is not used in the decoding process for coding units in intra prediction mode of CVSs referring to the SPS. When not present, intra_sdc_wedge_flag is inferred to be equal to 0.

qt_pred_flag equal to 1 specifies that prediction of a limited quadtree is used in the decoding process for CVSs referring to the SPS. qt_pred_flag equal to 0 specifies that prediction of a limited quadtree is not used in the decoding process for CVSs referring to the SPS. When not present, the value of qt_pred_flag is inferred to be equal to 0.

inter_sdc_flag equal to 1 specifies that SDC might be used in the decoding process for coding units in inter prediction mode of CVSs referring to the SPS. inter_sdc_flag equal to 0 specifies that SDC is not used in the decoding process for coding units in inter prediction mode of CVSs referring to the SPS. When not present, inter_sdc_flag is inferred to be equal to 0.

intra_single_flag equal to 1 specifies that the single depth mode might be used in the decoding process for CVSs referring to the SPS. intra_single_flag equal to 0 specifies that the single depth mode is not used in the decoding process for CVSs referring to the SPS. When not present, intra_single_flag is inferred to be equal to 0.

…

- I.7.4.7.1	General slice segment header semantics

…

The variables IvMvPredFlag, IvMvScalingFlag, SubPbSize, IvResPredFlag, ViewSynthesisPredFlag, DepthBasedBlkPartFlag, DepthRefinementFlag, MpiSubPbSize, IntraContourFlag, IntraSdcWedgeFlag, QtPredFlag, InterSdcFlag and DisparityDerivationFlag are derived as specified in the following:

  - IvMvPredFlag = ( nuh_layer_id > 0 )  &&
	NumRefListLayers[ nuh_layer_id ] > 0  &&  iv_mv_pred_flag[ DepthFlag ]	(Error: Reference source not found−16)

  - IvMvScalingFlag = ( nuh_layer_id > 0 )  &&  iv_mv_scaling_flag[ DepthFlag ]		(Error: Reference source not found−17)

  - SubPbSize  = 1  <<  ( nuh_layer_id > 0 ? log2_sub_pb_size_minus3 + 3  : CtbLog2SizeY )		(Error: Reference source not found−18)

  - IvResPredFlag = ( nuh_layer_id > 0 )  &&
	NumRefListLayers[ nuh_layer_id ] > 0  &&  iv_res_pred_flag	(Error: Reference source not found−19)

  - ViewSynthesisPredFlag = ( nuh_layer_id > 0 )  &&  NumRefListLayers[ nuh_layer_id ] > 0  &&
	view_synthesis_pred_flag  &&  depthOfRefViewsAvailFlag	(Error: Reference source not found−20)

  - DepthBasedBlkPartFlag = ( nuh_layer_id > 0 )  &&
	depth_based_blk_part_flag  &&  depthOfRefViewsAvailFlag	(Error: Reference source not found−21)

  - DepthRefinementFlag = ( nuh_layer_id > 0 )  &&
	depth_refinement_flag  &&  depthOfRefViewsAvailFlag	(Error: Reference source not found−22)

  - MpiFlag = ( nuh_layer_id > 0 )  &&  mpi_flag  &&  textOfCurViewAvailFlag		(Error: Reference source not found−23)

  - MpiSubPbSize = 1  <<  ( log2_mpi_sub_pb_size_minus3 + 3 )		(Error: Reference source not found−24)

  - IntraContourFlag = ( nuh_layer_id > 0 )  &&  
	intra_contour_flag  &&  textOfCurViewAvailFlag	(Error: Reference source not found−25)

  - IntraSdcWedgeFlag = ( nuh_layer_id > 0 )  &&  intra_sdc_wedge_flag		(Error: Reference source not found−26)

  - QtPredFlag = ( nuh_layer_id > 0 )  &&  qt_pred_flag  &&  textOfCurViewAvailFlag	(Error: Reference source not found−27)

  - InterSdcFlag = ( nuh_layer_id > 0 )  &&  inter_sdc_flag		(Error: Reference source not found−28)

  - IntraSingleFlag = ( nuh_layer_id > 0 )  &&  intra_single_flag		(Error: Reference source not found−29)

  - DisparityDerivationFlag = IvMvPredFlag | |  IvResPredFlag  | |
	ViewSynthesisPredFlag  | | DepthBasedBlkPartFlag	(Error: Reference source not found−30)

…

# Patent rights declaration

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).