| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Geneva, CH, 12–18 Feb. 2015 | Document: JCT3V-K0042 |
| --- | --- |

| Title: | Test Moel of K0042 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Takeshi Tsukuba 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520, JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

#### Mode 1: Explicit Wedgelet Signalization

The basic principle of this mode is to find the best matching Wedgelet partition at the encoder and transmit the partition information in the bitstream. At the decoder the signal of the block is reconstructed using the transmitted partition information.

The Wedgelet partition information for this mode is not predicted. At the encoder, a search over a set of Wedgelet partitions is carried out using the original depth signal of the current block as a reference. During this search, the Wedgelet partition that yields the minimum distortion between the original signal and the Wedgelet approximation is selected. The resulting prediction signal is then evaluated using the conventional mode decision process.

Each Wedgelet partition is decided by a starting position and ending position of the pixels along the boundary of the current block. In order to reduce the number of Wedgelet partitions thus the storage for the table storing the Wedgelet partitions, constraints on the starting and ending point positions of Wedgelet partitions apply. In general, Wedgelet partitions can be classified into two categories and six directions. The two categories are namely adjacent edge partitions and opposite edge partitions. The adjacent edge partitions have starting position and ending position belonging to adjacent vertical and horizontal boundary lines of the current PU, while opposite edge partitions have starting position and ending position belonging to parallel boundary lines of the current PU. As shown in Figure 1, example adjacent edge partitions are indicated with a wedgeOri ranging from 0 to 3, inclusive, while the opposite edge partitions are indicated with a wedgeOri equal to 4 or 5. For an adjacent edge partition, a corner point of the Wedgelet partition is defined to be the sample that is among the four corners of the current block and has the closet distance to the starting and ending positions. For the opposite edge partitions, the corner point is set to the tom-left and tom-right corner of the PU for wedgeOri equal to 4 and 5 respectively. For Wedgelet partitions that are adjacent edge partitions, starting and ending positions must be even (relative to the corresponding corner point). For any Wedgelet partition that is an opposite edge partition, its starting position must have an even number distance to the corner point. As shown in Figure 1, the solid dots represent the pixel positions that can be chosen as starting (denoted as S in the figure) or ending positions (denoted as E in the figure) for each Wedgelet type (with wedgeOri ranging from 0 to 5, inclusive). To specify symmetric partitions, rotation by 90 degree is utilized as shown in Figure 2. In rotation, reversing is also utilized for efficiency. Specifically, each of adjacent edge partitions with a wedgeOri equal to 1, 2 and 3 is generated by rotating and reversing one of the adjacent edge partitions with a wedgeOri equal to 0, 1 and 2, respectively. Likewise, each of opposite edge partitions with a wedgeOri equal to 5 is generated by rotating and reversing one of the opposite edge partitions with a wedgeOri equal to 4. Note: For each Wedgelet type (with a wedgeOri ranging from 1, 2, 3, and 5, inclusive), Wedgelet partitions are generated from other partitions, but those can be also defined with the corresponding starting and ending positions as shown in Figure 1.

Figure 1: Selective Wedgelet Partitions.

Figure 2: Symmetric Partitions