| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Geneva, CH, 12–18 Feb. 2015 | Document: JCT3V-K0043 |
| --- | --- |

| Title: | Text of IVMC simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Takeshi Tsukuba 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520, JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

- Text change

I.8.5.3.2.11 Derivation process for inter-view predicted and disparity information merge candidates

This process is not invoked when IvMvPredFlag is equal to 0 and MpiFlag is equal to 0.

Inputs to this process are:

- a luma location ( xPb, yPb ) of the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- two variables nPbW and nPbH specifying the width and the height of the current luma prediction block,

Outputs of this process are (with X being 0 or 1, respectively)

- the availability flags availableFlagIV, availableFlagIVShift and availableFlagDI specifying whether the inter-view merge candidates are available,

- the reference indices refIdxLXIV, refIdxLXIVShift and refIdxLXDI,

- the prediction list utilization flags predFlagLXIV, predFlagLXIVShift and predFlagLXDI,

- the motion vectors mvLXIV, mvLXIVShift and mvLXDI.

The availability flags availableFlagIV, availableFlagIVShift and availableFlagDI are set equal to 0, and for X in the range of 0 to 1, inclusive, the variables predFlagLXIV, predFlagLXIVShift, predFlagLXDI are set equal to 0, the variables refIdxLXIV, refIdxLXIVShift and refIdxLXDI are set equal to −1, and both components of mvLXIV, mvLXIVShift and mvLXDI are set equal to 0.

When ic_flag[ xPb ][ yPb ] is equal to 0, the temporal inter-view motion vector merging candidate is derived by the following ordered steps.

  - Depending on dbbp_flag[ xPb ][ yPb ], the following applies:

    - If dbbp_flag[ xPb ][ yPb ] is equal to 0, the derivation process for inter layer predicted sub prediction block motion vector candidates as specified in clause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, the view order index RefViewIdx[ xPb ][ yPb ], the disparity vector MvRefinedDisp[ xPb ][ yPb ], and the merge candidate indicator N being equal to IV, as inputs, and the outputs are, with X being in the range of 0 to 1, inclusive, the flag availableFlagLXIV, the motion vector mvLXIV and the reference index refIdxLXIV.

    - Otherwise (dbbp_flag[ xPb ][ yPb ] is not equal to 0), the derivation process for an inter-view predicted motion vector candidate as specified in clause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, offsetFlag being equal to 0, the prediction list indication X, the view order index RefViewIdx[ xPb ][ yPb ], and the disparity vector MvRefinedDisp[ xPb ][ yPb ] as inputs, and the outputs are the flag availableFlagLXIV, the motion vector mvLXIV and the reference index refIdxLXIV.

  - The availability flag availableFlagIV, and the prediction utilization flags predFlagL0IV and predFlagL1IV are derived by

      - availableFlagIV = availableFlagL0IV  | |  availableFlagL1IV	(Error: Reference source not found−1)

      - predFlagL0IV = availableFlagL0IV		(Error: Reference source not found−2)

      - predFlagL1IV = availableFlagL1IV		(Error: Reference source not found−3)

When DepthFlag is equal to 0 and ic_flag[ xPb ][ yPb ] is equal to 0, the shifted temporal inter-view motion vector merging candidate is derived by the following ordered steps.

  - For the prediction list indication X being 0 and 1 the following applies:

    - The derivation process for an inter-view predicted motion vector candidate as specified in clause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, offsetFlag being equal to 1, the prediction list indication X, the view order index RefViewIdx[ xPb ][ yPb ], and the disparity vector MvRefinedDisp[ xPb ][ yPb ] + ( nPbW *2 + 4, nPbH *2 + 4 ) as inputs, and the outputs are the flag availableFlagLXIVShift, the motion vector mvLXIVShift and the reference index refIdxLXIVShift.

  - The availability flag availableFlagIVShift, and the prediction utilization flags predFlagL0IVShift and predFlagL1IVShift are derived by

      - availableFlagIVShift = availableFlagL0IVShift  | |  availableFlagL1IVShift	(Error: Reference source not found−4)

      - predFlagL0IVShift = availableFlagL0IVShift		(Error: Reference source not found−5)

      - predFlagL1IVShift = availableFlagL1IVShift		(Error: Reference source not found−6)

When DepthFlag is equal to 0, the disparity information motion vector merging candidate is derived by the following ordered steps.

  - For the prediction list indication X being 0 and 1 the following applies:

    - The derivation process for a disparity information motion vector candidate as specified in clause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, the view order index RefViewIdx[ xPb ][ yPb ], the disparity vector MvRefinedDisp[ xPb ][ yPb ], and the prediction list indication X, as inputs, and the outputs are the flag availableFlagLXDI, the motion vector mvLXDI, and the reference index refIdxLXDI.

  - The availability flag availableFlagDI, and the prediction utilization flags predFlagL0DI and predFlagL1DI are derived by

      - availableFlagDI = availableFlagL0DI  | |  availableFlagL1DI 	(Error: Reference source not found−7)

      - predFlagL0DI = availableFlagL0DI		(Error: Reference source not found−8)

      - predFlagL1DI = availableFlagL1DI		(Error: Reference source not found−9)

I.8.5.3.2.12 Derivation process for an inter-view predicted motion vector candidate

This process is not invoked when IvMvPredFlag is equal to 0.

Inputs to this process are:

- a luma location ( xPb, yPb ) of the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- two variables nPbW and nPbH specifying the width and the height of the current luma prediction block,

- a flag offsetFlag

- a prediction list indication X,

- a reference view index refViewIdx,

- a disparity vector mvDisp.

Outputs of this process are:

- a flag availableFlagLXInterView specifying whether the temporal inter-view motion vector candidate is available,

- a temporal inter-view motion vector candidate mvLXInterView,

- a reference index refIdxLX specifying a reference picture in the reference picture list RefPicListLX.

The flag availableFlagLXInterView is set equal to 0, the variable refIdxLX is set equal to −1, and both components of mvLXInterView are set equal to 0.

When X is equal to 1 and the current slice is not a B slice the whole decoding process specified in this clause terminates.

The reference layer luma location ( xRef, yRef ) is derived as specified in the following:

  - xRefFull = xPb + ( offsetFlag ? nPbW + 1: ( nPbW  >>  1 ) )+ ( ( mvDisp[ 0 ] + 2 )  >>  2 )	(Error: Reference source not found−10)

  - yRefFull = yPb + ( offsetFlag ? nPbH + 1: ( nPbH  >>  1 ) ) + ( ( mvDisp[ 1 ] + 2 )  >>  2 )	(Error: Reference source not found−11)

  - xRef = Clip3( 0, PicWidthInSamplesL − 1, ( xRefFull  >>  3 )  <<  3 )	(Error: Reference source not found−12)

  - yRef = Clip3( 0, PicHeightInSamplesL − 1, ( yRefFull  >>  3 )  <<  3 )	(Error: Reference source not found−13)

NOTE – The vector mvDisp is given in units of quarter luma samples, independent of the value of DepthFlag.

The variable ivRefPic is set equal to the picture with ViewIdx equal to refViewIdx in the current access unit.

The variable ivRefPb specifies the luma prediction block covering the luma location ( xRef, yRef ) inside the inter-view reference picture specified by ivRefPic.

The luma location ( xIvRefPb, yIvRefPb ) is set equal to the top-left sample of the inter-view reference luma prediction block specified by ivRefPb relative to the top-left luma sample of the inter-view reference picture specified by ivRefPic.

When ivRefPb is not coded in an intra prediction mode, the following applies, for Y in the range of X to ( 1 − X ), inclusive:

- The variables refPicListLYIvRef, predFlagLYIvRef[ x ][ y ], mvLYIvRef[ x ][ y ], and refIdxLYIvRef[ x ][ y ] are set equal to the corresponding variables of the inter-view reference picture specified by ivRefPic, RefPicListLY, PredFlagLY[ x ][ y ], MvLY[ x ][ y ], and RefIdxLY[ x ][ y ], respectively.

- When predFlagLYIvRef[ xIvRefPb ][ yIvRefPb ] is equal to 1, the following applies for each i from 0 to num_ref_idx_lX_active_minus1, inclusive:

  - When PicOrderCnt( refPicListLYIvRef[ refIdxLYIvRef[ xIvRefPb ][ yIvRefPb ] ] ) is equal to PicOrderCnt( RefPicListLX[ i ] ) and availableFlagLXInterView is equal to 0, the following applies:

      - availableFlagLXInterView = 1		(Error: Reference source not found−14)

      - mvLXInterView = mvLYIvRef[ xIvRefPb ][ yIvRefPb ]	(Error: Reference source not found−15)

      - refIdxLX = i		(Error: Reference source not found−16)

I.8.5.3.2.16 Derivation process for inter layer predicted sub prediction block motion vector candidates

Inputs to this process are:

- a luma location ( xPb, yPb ) of the top-left sample of the current prediction luma block relative to the top-left luma sample of the current picture,

- two variables nPbW and nPbH specifying the width and the height of the current prediction block,

- a reference view index refViewIdx,

- a disparity vector mvDisp,

- a merge candidate indicator N.

Outputs of this process are:

- the flags availableFlagLXN, with X in the range of 0 to 1, inclusive, specifying whether the inter-view or inter-component predicted merging candidate is available,

- the inter-view or inter-component predicted merging candidate mvLXN, with X in the range of 0 to 1, inclusive,

- the reference index refIdxLXN, with X in the range of 0 to 1, inclusive, specifying a reference picture in the reference picture list RefPicListLX.

For X in the range of 0 to 1, inclusive, the following applies:

–	The flag availableFlagLXN is set equal to 0.

–	The motion vector mvLXN is set equal to ( 0, 0 ).

–	The reference index refIdxLXN is set equal to −1.

The variables minSize, nSbW and nSbH are derived as:

  - minSize = DepthFlag ? MpiSubPbSize : SubPbSize		(Error: Reference source not found−17)

  - nSbW = ( nPbW % SubPbSize  !=  0 | |  nPbH % SubPbSize  !=  0 ) ? nPbW : minSize	(Error: Reference source not found−18)

  - nSbH = ( nPbW % SubPbSize  !=  0 | |  nPbH % SubPbSize  !=  0 ) ? nPbH : minSize	(Error: Reference source not found−19)

Depending on DepthFlag, the following applies:

- If DepthFlag is equal to 0, for X in the range of 0 to 1, inclusive, the derivation process for a temporal inter-view motion vector candidate as specified in clause Error: Reference source not found is invoked with the luma location ( xPb + ( ( nPbW / nSbW ) / 2 ) * nSbW, yPb + ( ( nPbH / nSbH ) / 2 ) * nSbH ), the variables nSbW and nSbH, offsetFlag being equal to 0, the prediction list indication X, the view order index refViewIdx, and the disparity vector mvDisp as inputs, and the outputs are the flag availableFlagLXN, the motion vector mvLXN, and the reference index refIdxLXN.

- Otherwise (DepthFlag is equal to 1), the derivation process for a texture merging candidate as specified in clause Error: Reference source not found is invoked with the luma location ( xPb + ( ( nPbW / nSbW ) / 2 ) * nSbW, yPb + ( ( nPbH / nSbH ) / 2 ) * nSbH ), and the variables nSbW and nSbH as inputs, and the outputs are the flags availableFlagT, predFlagLXN, the reference index refIdxLXN, and the motion vector mvLXN (with X being replaced by 0 and 1).

- When availableFlagL0N or availableFlagL1N is equal to 1, the following applies:

- For yBlk in the range of 0 to ( nPbH / nSbH − 1 ), inclusive, the following applies:

  - For xBlk in the range of 0 to ( nPbW / nSbW − 1 ), inclusive, the following applies:

    - If DepthFlag is equal to 0, for X in the range of 0 to 1, inclusive, the derivation process for an inter-view predicted motion vector candidate as specified in clause Error: Reference source not found is invoked with the luma location ( xPb + xBlk * nSbW, yPb + yBlk * nSbH ), the variables nSbW and nSbH, offsetFlag being equal to 0, the prediction list indication X, the view order index refViewIdx, and the disparity vector mvDisp as inputs, and the outputs are the flag spPredFlagLX[ xBlk ][ yBlk ], the motion vector spMvLX[ xBlk ][ yBlk ] and the reference index spRefIdxLX[ xBlk ][ yBlk ].

    - Otherwise (DepthFlag is equal to 1), the derivation process for a texture merging candidate as specified in clause Error: Reference source not found is invoked with the luma location ( xPb + xBlk * nSbW, yPb + yBlk * nSbH ), and the variables nSbW and nSbH as inputs, and the outputs are the flags availableFlagT, spPredFlagLX[ xBlk ][ yBlk ], the reference index spRefIdxLX[ xBlk ][ yBlk ], and the motion vector spMvLX[ xBlk ][ yBlk ] (with X being replaced by 0 and 1).

    - When spRefIdxL0[ xBlk ][ yBlk ] and spRefIdxL1[ xBlk ][ yBlk ] are both equal to −1, the following applies for X in the range of 0 to 1, inclusive:

        - spMvLX[ xBlk ][ yBlk ] = mvLXN		(Error: Reference source not found−20)

        - spRefIdxLX[ xBlk ][ yBlk ] = refIdxLXN		(Error: Reference source not found−21)

        - spPredFlagLX[ xBlk ][ yBlk ] = availableFlagLXN	(Error: Reference source not found−22)

    - When ( nSbW + nSbH ) is equal to 12, and both spPredFlagL0[ xBlk ][ yBlk ] and spPredFlagL1[ xBlk ][ yBlk ] are equal to 1, the following applies:

        - spMvL1[ xBlk ][ yBlk ] = 0		(Error: Reference source not found−23)

        - spRefIdxL1[ xBlk ][ yBlk ] = −1		(Error: Reference source not found−24)

        - spPredFlagL1[ xBlk ][ yBlk ] = 0		(Error: Reference source not found−25)

- For use in derivation processes of variables invoked later in the decoding process, the following assignments are made for x = 0.. nPbW − 1 and y = 0.. nPbH − 1:

  - The variable SubPbPartSizeN is derived as specified in following:

      - SubPbPartSizeN[ xPb + x ][ yPb + y ] = ( nSbW, nSbH )	(Error: Reference source not found−26)

  - For X in the range of 0 to 1, inclusive, the following applies:

    - The variables SubPbPredFlagLX, SubPbMvLX and SubPbRefIdxLX are derived as specified in following:

        - SubPbPredFlagLXN[ xPb + x ][ yPb + y ] = spPredFlagLX[ x / nSbW ][ y / nSbW ]	(Error: Reference source not found−27)

        - SubPbMvLXN[ xPb + x ][ yPb + y ] = spMvLX[ x / nSbW ][ y / nSbW ]	(Error: Reference source not found−28)

        - SubPbRefIdxLXN[ xPb + x ][ yPb + y ] = spRefIdxLX[ x / nSbW ][ y / nSbW ]	(Error: Reference source not found−29)

    - The derivation process for chroma motion vectors in clause 8.5.3.2.10 is invoked with SubPbMvLXN[ xPb + x ][ yPb + y ] as input and the output is SubPbMvCLXN[ xPb + x ][ yPb + y ].