| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Geneva, CH, 12–18 Feb. 2015 | Document: JCT3V-K0043 |
| --- | --- |

| Title: | On inter-view shift candidate |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Takeshi Tsukuba 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520, JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

The position of inter-view (IV) candidate and inter-view shift (IVShift) candidate are defined as center and bottom right (BR) respectively. However the BR position of IVShift is oddly derived in the current specification by adding (blocksize * 2 + 4) to the disparity vector. We propose to provide a simplified derivation process, which just adds blocksize + K to the PU position. The proposed method (test1) doesn’t change the coding results.

# Introduction

The position of inter-view shift (IVShift) candidate is derived adding (“blockSize * 2 + 4”) to the disparity vector. The modified disparity vector is shifted by 2 and added by blockSize >> 1 to get final “blockSize + 1” position.

The following shows the current spec’s operation, (DV +”blockSize * 2 + 4”), can be translated to more straightforward one, (pos + “blockSize + 1”):

posIVShift = posPU + (blockSize>>1) + (DV + blockSize * 2 + 4 + 2) >> 2     (current spec)

= posPU + (blockSize>>1) + (blockSize * 2 + 4) >> 2 + (DV + 2) >> 2

= posPU + (blockSize>>1) + 2*(blockSize + 2) >> 2 + (DV + 2) >> 2

= posPU + (blockSize>>1) + (blockSize + 2) >> 1 + (DV + 2) >> 2

= posPU + (blockSize>>1) + (blockSize >> 1) + (2 >> 1) + (DV + 2) >> 2

= posPU + blockSize + 1 + (DV + 2) >> 2                       (proposal test1)

We claim that the current derivation doesn’t make sense (do the same thing complicatedly), which may bring confusions.

Current spec:

    - The derivation process for an inter-view predicted motion vector candidate as specified in clause I.8.5.3.2.12 is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, the prediction list indication X, the view order index RefViewIdx[ xPb ][ yPb ], and the disparity vector MvRefinedDisp[ xPb ][ yPb ] + ( nPbW *2 + 4, nPbH *2 + 4 ) as inputs, and the outputs are the flag availableFlagLXIVShift, the motion vector mvLXIVShift and the reference index refIdxLXIVShift.

the above corresponds to

  - xRefFull = xPb + ( nPbW  >>  1 ) + ( ( mvDisp[ 0 ] + 2 + nPbW * 2 + 4)  >>  2 )	(I-153)

  - yRefFull = yPb + ( nPbH  >>  1 ) + ( ( mvDisp[ 1 ] + 2+ nPbH * 2 + 4 )  >>  2 )	(I-154)

# Proposal

We propose to define a inter-view shift offset (nPbW + 1, nPbH + 1) like the current inter-view offset  (nPbW>>1, nPbH>>1). The offset is switched by a new flag offsetFlag in the text.

Proposed text (part):

    - The derivation process for an inter-view predicted motion vector candidate as specified in clause I.8.5.3.2.12 is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, offsetFlag being equal to 1, the prediction list indication X, the view order index RefViewIdx[ xPb ][ yPb ], and the disparity vector MvRefinedDisp[ xPb ][ yPb ] + ( nPbW *2 + 4, nPbH *2 + 4 ) as inputs, and the outputs are the flag availableFlagLXIVShift, the motion vector mvLXIVShift and the reference index refIdxLXIVShift.

  - xRefFull = xPb + ( offsetFlag? nPbW + K : ( nPbW  >>  1 ) ) + ( ( mvDisp[ 0 ] + 2)  >>  2 )	(I-153)

  - yRefFull = yPb + ( offsetFlag? nPbH + K: ( nPbH  >>  1 ) ) + ( ( mvDisp[ 1 ] + 2)  >>  2 )	(I-154)

[test1] K = 1

  - xRefFull = xPb + ( offsetFlag? nPbW + 1 : ( nPbW  >>  1 ) ) + ( ( mvDisp[ 0 ] + 2)  >>  2 )	(I-153)

  - yRefFull = yPb + ( offsetFlag? nPbH + 1: ( nPbH  >>  1 ) ) + ( ( mvDisp[ 1 ] + 2)  >>  2 )	(I-154)

[test2] K = 0

  - xRefFull = xPb + ( offsetFlag? nPbW : ( nPbW  >>  1 ) ) + ( ( mvDisp[ 0 ] + 2)  >>  2 )	(I-153)

  - yRefFull = yPb + ( offsetFlag? nPbH: ( nPbH  >>  1 ) ) + ( ( mvDisp[ 1 ] + 2)  >>  2 )	(I-154)

# Simulation results

Proposal (Test 1) doesn’t change the coding results. It can be considered text clean up.

Test 1 (K = 1) on CTC

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.5% | 99.3% | 100.8% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 97.4% | 94.3% | 96.6% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.2% | 101.0% | 99.4% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 99.8% | 99.1% | 98.5% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 99.9% | 98.7% | 99.2% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 99.9% | 100.4% | 99.8% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 99.5% | 98.3% | 97.9% |
| Shark | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 99.5% | 100.8% | 99.7% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 99.4% | 98.2% | 98.9% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 99.7% | 99.5% | 99.0% |
| average | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 99.6% | 99.0% | 99.0% |

Proposal (Test 2) shows very small loss in synthesis.

Test 2 (K = 0) on CTC

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.00% | 0.01% | 0.05% | 100.4% | 100.0% | 100.9% |
| Kendo | 0.0% | 0.0% | 0.1% | 0.00% | 0.00% | 0.00% | 100.0% | 96.5% | 101.0% |
| Newspaper_CC | 0.0% | -0.1% | 0.1% | 0.01% | 0.01% | 0.05% | 100.5% | 101.2% | 99.3% |
| GT_Fly | 0.0% | 0.1% | 0.0% | 0.00% | 0.00% | 0.00% | 99.0% | 99.3% | 99.4% |
| Poznan_Hall2 | 0.0% | 0.0% | -0.2% | -0.04% | -0.03% | -0.04% | 100.1% | 100.4% | 100.6% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | -0.01% | -0.01% | -0.01% | 100.0% | 101.6% | 100.1% |
| Undo_Dancer | 0.0% | -0.1% | 0.0% | 0.00% | 0.00% | 0.05% | 99.8% | 100.6% | 100.2% |
| Shark | 0.0% | -0.1% | 0.0% | 0.00% | 0.01% | -0.01% | 100.4% | 100.7% | 98.4% |
| 1024x768 | 0.0% | 0.0% | 0.1% | 0.00% | 0.01% | 0.04% | 100.3% | 99.3% | 100.4% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | -0.01% | -0.01% | 0.00% | 99.8% | 100.5% | 99.7% |
| average | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.01% | 100.0% | 100.0% | 100.0% |

# Conclusion

It is recommended to adopt the proposed clean up in 3D-HEVC and HTM.

# Patent rights declaration(s)

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).