| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Geneva, CH, 12–18 Feb. 2015 | Document: JCT3V-K0044 |
| --- | --- |

| Title: | Clean up of 3D-HEVC specification |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Takeshi Tsukuba 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520, JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

Before 3D-HEVC finalization, specification cleanup should be completed. This contribution proposes several modifications to remove and fix unreasonable process for clean up.

# Proposal changes

## [P1:TEXT] SubPU / MPI Size bugfix

Sub-block and MPI is defined as the common procedure in both specification and software. However the following nSbW and nSbH are differently specified in the specification. Both MPI and Sub-block should use the same logic as implemented in the HTM software.

I.8.5.3.2.16 Derivation process for inter layer predicted sub prediction block motion vector candidates

The variables minSize, nSbW and nSbH are derived as:

  - minSize = DepthFlag ? MpiSubPbSize : SubPbSize		(I-180)

  - nSbW = ( nPbW % SubPbSize minSize  !=  0 | |  nPbH % SubPbSize minSize  !=  0 ) ? nPbW : minSize	(I-181)

  - nSbH = ( nPbW % SubPbSize minSize  !=  0 | |  nPbH % SubPbSize minSize  !=  0 ) ? nPbH : minSize	(I-182)

## [P2:TEXT&SW] unused bi-prediction restriction in SubPU

According to the SubPU size derivation, nSbW x nSbH can be only 8x8, 16x16,32x32 (square only). In other words, sub-block size (nSbW x nSbH) cannot be 4x8 or 8x4. Thus, the bi-prediction restriction in 4x8 or 8x4 is not used. We propose to remove the unused specification.

    - When ( nSbW + nSbH ) is equal to 12, and both spPredFlagL0[ xBlk ][ yBlk ] and spPredFlagL1[ xBlk ][ yBlk ] are equal to 1, the following applies:

        - spMvL1[ xBlk ][ yBlk ] = 0		(I-186)

        - spRefIdxL1[ xBlk ][ yBlk ] = −1		(I-187)

        - spPredFlagL1[ xBlk ][ yBlk ] = 0		(I-188)

For record, we‘ve implemented and simulated the proposed change and confirmed the results didn’t change as shown in Table.

Table CTC results with the proposed fix

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 100.9% | 98.2% | 99.2% |
| Kendo | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 99.9% | 95.7% | 98.8% |
| Newspaper_CC | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 100.3% | 98.4% | 99.1% |
| GT_Fly | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 100.0% | 98.5% | 99.8% |
| Poznan_Hall2 | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 100.2% | 98.2% | 100.9% |
| Poznan_Street | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 100.1% | 101.0% | 100.8% |
| Undo_Dancer | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 99.9% | 99.5% | 100.2% |
| Shark | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 99.8% | 98.6% | 98.9% |
| 1024x768 | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 100.4% | 97.5% | 99.0% |
| 1920x1088 | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 100.0% | 99.1% | 100.1% |
| average | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 100.1% | 98.5% | 99.7% |

## [P3:TEXT] IntraSdcWedgeFlag and IntraContourFlag

When either of IntraSdcWegeFlag or IntraContourFlag is equal to 1, depth_intra_mode_flag can be immediately decided without decoding the syntax element. i.e. If IntraSdcWegeFlag is 1, depth_intra_mode_flag is 0, if IntraContourFlag is equal to 1, then depth_intra_mode_flag is equal to 1). However the current specification decode depth_intra_mode_flag regardless of that condition.

We propose to signall depth_intra_mode_flag only when both IntraSdcWegeFlag and IntraContourFlag is equal to 1.

I.7.3.8.5.1

| intra_mode_ext( x0 , y0 , log2PbSize ) { | Descriptor |
| --- | --- |
| if( log2PbSize < 6 ) |  |
| dim_not_present_flag[ x0 ][ y0 ] | ae(v) |
| if ( !dim_not_present_flag[ x0 ][ y0 ] && IntraSdcWedgeFlag && IntraContourFlag ) |  |
| depth_intra_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_WEDGE ) |  |
| wedge_full_tab_idx[ x0 ][ y0 ] | ae(v) |
| } |  |

depth_intra_mode_flag[ x0 ][ y0 ] is used to specify the depth intra mode of the current prediction unit. When not present, the value of depth_intra_mode_flag[ x0 ][ y0 ] is inferred to be equal to !IntraSdcWedgeFlag | | IntraContourFlag

When IntraContourFlag is equal to 0 or nal_unit_type is equal to BLA_W_LP, BLA_W_RADL, BLA_N_LP, IDR_W_RADL or IDR_N_LP, it is a requirement of bitstream conformance, that depth_intra_mode_flag is equal to 0.

The variable DepthIntraMode[ x0 ][ y0 ] is derived as specified in the following:

  - DepthIntraMode[ x0 ][ y0 ] = dim_not_present_flag[ x0 ][ y0 ] ? −1 : depth_intra_mode_flag[ x0 ][ y0 ]	(Error: Reference source not found−1)

Table I-4 Specification of DepthIntraMode and associated name

| DepthIntraMode | Associated name |
| --- | --- |
| −1 | INTRA_DEP_NONE |
| 0 | INTRA_DEP_WEDGE |
| 1 | INTRA_DEP_CONTOUR |

## [P4:TEXT&SW]SPS present flag

In SPS, texture and depth flag are always sent. If SPS is not shared in texture and depth, this means unused syntax sending. Although the bit cost may be negligible, this kind of unused syntax sending should be avoided.

Note: SPS would not be shared if different parameters in such conformance window, sps scaling list or amp enable, sao, loop filter parameter is used in text and depth.

Note; Too many presentation flags to prevent unused syntax is bad. But this is not the case (only one present flag should not be a problem) .

| sps_3d_extension( ) { | Descriptor |
| --- | --- |
| for( d = 0; d <= 1; d++ ) { |  |
| 3d_sps_param_present_flag[ d ] | u(1) |
| if (3d_sps_param_present_flag[ d ]){ |  |
| iv_mv_pred_flag[ d ] | u(1) |
| iv_mv_scaling_flag[ d ] | u(1) |
| if( d = = 0 ) { |  |
| log2_sub_pb_size_minus3[ d ] | ue(v) |
| iv_res_pred_flag[ d ] | u(1) |
| depth_refinement_flag[ d ] | u(1) |
| view_synthesis_pred_flag[ d ] | u(1) |
| depth_based_blk_part_flag[ d ] | u(1) |
| } else { |  |
| mpi_flag[ d ] | u(1) |
| log2_mpi_sub_pb_size_minus3[ d ] | ue(v) |
| intra_contour_flag[ d ] | u(1) |
| intra_sdc_wedge_flag[ d ] | u(1) |
| qt_pred_flag[ d ] | u(1) |
| inter_sdc_flag[ d ] | u(1) |
| intra_single_flag[ d ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |

3d_sps_param_present_flag [ d ] equal to 1 specifies the coding tool parameters are present in this SPS. 3d_sps_param_present_flag equal to 0 specifies the coding tool parameters are not present in this SPS.

iv_mv_pred_flag[ d ] equal to 1 specifies that inter-view motion parameter prediction may be used in the decoding process of the layer with nuh_layer_id equal to lId and VpsDepthFlag[ lId ] equal to d. iv_mv_pred_flag[ d ] equal to 0 specifies that inter-view motion parameter prediction is not used in the decoding process of the layer with nuh_layer_id equal to lId and VpsDepthFlag[ lId ] equal to d. When not present, the value of iv_mv_pred_flag[ d ] is inferred to be equal to 0.

…

# Conclusion

It is recommended to adopt the proposed clean ups in 3D-HEVC and HTM.

# Patent rights declaration(s)

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).