| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Geneva, CH, 12–18 Feb. 2015 | Document: JCT3V-K0045 |
| --- | --- |

| Title: | Bug Fix for Texture-dependent Depth Partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xianguo Zhang1, Kai Zhang1, Jicheng An1, Han Huang1, Jian-Liang Lin2, Shawmin Lei2 1MediaTek Beijing: Building 1-B, No. 6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {xianguo.zhang, kai.zhang, jicheng.an, h.huang, jl.lin, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

For the tool of texture-dependent depth partitioning (TDDP) in quad-tree limitation (QTL), the allowed partitions for current coded depth CU depend on the partition of the collocated texture block. For the coded depth CU, the vertical/horizontal partitions are allowed for a depth CU only when the collocated texture CU at the same quad-tree level is coded with vertical/horizontal or NxN partitions. However, in the current working draft, Draft Text 6, part of the part_mode binarization table is missed when the collocated texture block is coded with vertical or horizontal partition. This contribution provides the bug fix of the binarization table to match the reference software, HTM-13.

# Introduction

In 3D-HEVC [1], the vertical/horizontal partitions are allowed for a depth CU only when the collocated texture CU at the same quad-tree level is coded with vertical/horizontal or NxN partitions as illustrated in Fig. 1. When the collocated texture CU at the same quad-tree level is coded with horizontal partitions such as PART_2NxN, PART_2NxnU, or PART_2NxnD, the flag partPredIdc is set to 1 and only four partition candidates are allowed for the depth block, PART_2NxN, PART_2NxnU, PART_2NxnD and PART_2Nx2N. On the other hand, when the collocated texture CU at the same quad-tree level is coded with vertical partitions such as PART_Nx2N, PART_nLx2N, or PART_nRx2N, the flag partPredIdc is set to 2 and only four partition candidates are allowed for the depth block, PART_Nx2N, PART_nLx2N, or PART_nRx2N and PART_2Nx2N. When the collocated texture CU is coded with PART_NxN, the flag partPredIdc is set to 0 and all the partition candidates are allowed. Otherwise, when the collocated texture CU is coded with PART_2Nx2N, only the partition PART_2Nx2N is allowed for the depth block. Table 1 shows the possible partition candidate of current depth block according to the partition of the collocated texture block.

Figure 1.  The vertical/horizontal partitions for depth are allowed only if the collocated texture CU is coded with vertical/horizontal or NxN partition

Table 1. “Table I.3 Name association to prediction mode and partitioning type”

| - CuPredMode[ x0 ][ y0 ] | - part_mode | - partPredIdc | - IntraSplitFlag | PartMode |
| --- | --- | --- | --- | --- |
| - MODE_INTRA | - 0 | - - | - 0 | - PART_2Nx2N |
|  | - 1 | - - | - 1 | - PART_NxN |
| - MODE_INTER | - 0 | - - | - 0 | PART_2Nx2N |
|  | - 1 | - 0 | - 0 | PART_2NxN |
|  | - 1 | - 1 | - 0 | PART_2NxN |
|  | - 1 | - 2 | - 0 | PART_Nx2N |
|  | - 2 | - 0 | - 0 | PART_Nx2N |
|  | - 2 | - 1 | - 0 | PART_2NxnU |
|  | - 2 | - 2 | - 0 | PART_nLx2N |
|  | - 3 | - 0 | - 0 | PART_NxN |
|  | - 3 | - 1 | - 0 | PART_2NxnD |
|  | - 3 | - 2 | - 0 | PART_nRx2N |
|  | - 4 | - - | - 0 | PART_2NxnU |
|  | - 5 | - - | - 0 | PART_2NxnD |
|  | - 6 | - - | - 0 | PART_nLx2N |
|  | - 7 | - - | - 0 | PART_nRx2N |

# Problem

However, in the current working draft text 6, part of the part_mode binarization table is missed when the flag partPredIdc is equal to 1 or 2. This contribution fixes the binarization Table 9-34 to match the reference software, HTM-13 [2].

# Proposed Text Changes

The added part is highlighted in yellow color.

- I.9.3.3.6 Binarization process for part_mode

The specifications in subclause 9.3.3.6 apply with the following modifications.

Table 9-34 – Binarization for part_mode

| CuPredMode[ xCb ][ yCb ] | part_mode | partPredIdc | PartMode | Bin string |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | log2CbSize > MinCbLog2SizeY |  | log2CbSize == MinCbLog2SizeY |  |
|  |  |  |  | !amp_enabled_flag | amp_enabled_flag | log2CbSize = = 3 | log2CbSize > 3 |
| MODE_INTER | 0 | 0,1,2 | PART_2Nx2N | 1 | 1 | 1 | 1 |
|  | 1 | 0 | PART_2NxN | 01 | 011 | 01 | 01 |
|  | 2 | 0 | PART_Nx2N | 00 | 001 | 00 | 001 |
|  | 3 | 0 | PART_NxN | - | - | - | 000 |
|  | 4 | 0 | PART_2NxnU | - | 0100 | - | - |
|  | 5 | 0 | PART_2NxnD | - | 0101 | - | - |
|  | 6 | 0 | PART_nLx2N | - | 0000 | - | - |
|  | 7 | 0 | PART_nRx2N | - | 0001 | - | - |
|  | 1 | 1 | PART_2NxN | 0 | 01 | 0 | 0 |
|  | 2 | 1 | PART_2NxnU | - | 000 | - | - |
|  | 3 | 1 | PART_2NxnD | - | 001 | - | - |
|  | 1 | 2 | PART_Nx2N | 0 | 01 | 0 | 0 |
|  | 2 | 2 | PART_nLx2N | - | 000 | - | - |
|  | 3 | 2 | PART_nRx2N | - | 001 | - | - |

- I.7.4.9.5 Coding unit semantics

Table I.3 Name association to prediction mode and partitioning type

| - CuPredMode[ x0 ][ y0 ] | - part_mode | - partPredIdc | - IntraSplitFlag | PartMode |
| --- | --- | --- | --- | --- |
| - MODE_INTRA | - 0 | - 0 | - 0 | - PART_2Nx2N |
|  | - 1 | - 0 | - 1 | - PART_NxN |
| - MODE_INTER | - 0 | - 0,1,2 | - 0 | PART_2Nx2N |
|  | - 1 | - 0 | - 0 | PART_2NxN |
|  | - 1 | - 1 | - 0 | PART_2NxN |
|  | - 1 | - 2 | - 0 | PART_Nx2N |
|  | - 2 | - 0 | - 0 | PART_Nx2N |
|  | - 2 | - 1 | - 0 | PART_2NxnU |
|  | - 2 | - 2 | - 0 | PART_nLx2N |
|  | - 3 | - 0 | - 0 | PART_NxN |
|  | - 3 | - 1 | - 0 | PART_2NxnD |
|  | - 3 | - 2 | - 0 | PART_nRx2N |
|  | - 4 | - 0 | - 0 | PART_2NxnU |
|  | - 5 | - 0 | - 0 | PART_2NxnD |
|  | - 6 | - 0 | - 0 | PART_nLx2N |
|  | - 7 | - 0 | - 0 | PART_nRx2N |

The value of part_mode is restricted as follows:

- If CuPredMode[ x0 ][ y0 ] is equal to MODE_INTRA, part_mode shall be equal to 0 or 1.

- Otherwise (CuPredMode[ x0 ][ y0 ] is equal to MODE_INTER),

  - if partPredIdx is equal to 0, the following applies:

    - If log2CbSize is greater than MinCbLog2SizeY and amp_enabled_flag is equal to 1, part_mode shall be in the range of 0 to 2, inclusive, or in the range of 4 to 7, inclusive.

    - Otherwise, if log2CbSize is greater than MinCbLog2SizeY and amp_enabled_flag is equal to 0, or log2CbSize is equal to 3, part_mode shall be in the range of 0 to 2, inclusive.

    - Otherwise (log2CbSize is greater than 3 and less than or equal to MinCbLog2SizeY), the value of part_mode shall be in the range of 0 to 3, inclusive.

  - Othewise, the following applies:

    - If amp_enabled_flag is equal to 1, part_mode shall be in the range of 0 to 3.

    - Otherwise, part_mode shall be in the range of 0 to 1, inclusive.

# Patent rights declarations

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, "3D-HEVC Draft Text 6," Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG JCT3V-J1002, Nov. 2014

- HTM-13.0, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-13.0