| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Geneva, CH, 12–18 Feb. 2015 | Document: JCT3V-K0047 |
| --- | --- |

| Title: | Lookup Table Size Reduction for DMM1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xu Chen1, Ce Zhu2, Shu-Ying Ma2, Xiaozhen Zheng1, Yong-Bing Lin1, Jian-Hua Zheng1 1 HiSilicon technologies, Shenzhen, P. R. China, 518129 2 School of Electronic Engineering University of Electronic Science and Technology of China, Chengdu, P. R. China,611731 | Tel: Email: | +86-755-28425333 anderson.chen@huawei.com +86-1822-808-8676 eczhu@uestc.edu.cn +86-1898-199-6913 maya0313@126.com |
| Source: | HiSilicon Technologies, UESTC |  |  |

_____________________________

# Abstract

This contribution proposes three methods to make lookup table size reduction for DMM1. The first method proposed to downsample 8x8 wedgelet patterns for DMM1 to reduce the lookup table size. The second method to reuse wedgelet pattern of 16x16 block for 8x8 block instead of specifying 8x8 wedgelet pattern in order to reduce the storage size for wedgelet pattern in DMM1. The third one provides a method that can make more lookup table size reduction for 16x16 without downsample process. Experimental result reportedly shows that (0.01% / 0.00%, 0.03% / 0.01%, 0.00% / 0.00%) coding loss for synth PSNR vs. total bitrate in the CTC / all-intra configuration, respectively. It is asserted that proposed methods can achieve storage size reduction by 8.98%, 27.1% and 2.12%, respectively.

# Introduction

In the current 3D-HEVC[1], DMM1 mode constructs the lookup table for blocks of different size. In order to reduce the size of lookup table, 16x16 block choose the wedgelet pattern with different segments partially by down sampling[2], and 32x32 block reuses wedgelet pattern of 16x16 block[3]. The lookup table size requires about 180k-bits storage as listed in Table 1.

Table 1. Table size for wedgelet pattern in the current HTM13.0

| PU size | Number of wedgelet pattern | Bins for decoding the index | Number of bits for one wedgelet pattern | Total Table Size(bits) | Accumulated Table Size (bits) | bits ratio [%] |
| --- | --- | --- | --- | --- | --- | --- |
| 4×4 | 86 | 7 | 16 | 1,376 | 1,376 | 0.70%=1376/180960 |
| 8×8 | 766 | 10 | 64 | 49,024 | 50,400 | 27.1%=49024/180960 |
| >=16×16 | 510 | 9 | 256 | 130,560 | 180,960 | 72.2%=130560/180960 |

# Proposed Methods

In this contribution, three methods are proposed to reduce lookup table size reduction for DMM1. The proposed methods are described as follows.

## Downsample 8x8 wedgelet patterns

In order to reduce the lookup table size further, we propose to downsample the lookup table size for PU=8x8. The proposed method is illustrated as Fig. 1 and described as follows:

- For the wedgelet patterns with starting point and ending point located in adjacent or opposite edges: with even-x-or-y-coordinate starting points are kept.

Figure 1. one categorie of wedgelet segmentations

The number of wedgelet patterns with this proposed method for PU=8x8 is shown in Table 2. As shown in the table, this proposed method could achieve 8.98% total table size saving.

Table 2. Table size for wedgelet pattern with proposed method 2.1

| PU size | Number of wedgelet pattern | Bins for decoding the index | Number of bits for one wedgelet pattern | Total Table Size(bits) | Accumulated Table Size (bits) | Table size saving [%] |
| --- | --- | --- | --- | --- | --- | --- |
| 4×4 | 86 | 7 | 16 | 1,376 | 1,376 | (180,960-164,704) /180,960 = 8.98% |
| 8×8 | 512 | 9 | 64 | 32,768 | 34,144 |  |
| >=16×16 | 510 | 9 | 256 | 130,560 | 164,704 |  |

## Reuse wedgelet pattern of 16x16 block for 8x8 block

In order to reduce the lookup table size further, we propose to down sampling the wedgelet pattern of 16x16 block for 8x8 block as shown in eq.1.

- The variable wedgePattern[ x ][ y ] with x, y =0..nTbS − 1, specifying a binary partition pattern is derived as follows:

- If Log2( nTbS ) is equal to 3,

- wedgePattern[ x ][ y ] =

- WedgePatternTable[ Log2( nTbS ) + 1 ][ wedge_full_tab_idx[ xTb ][ yTb ] ][ (x <<1)+xoffset][ (y<<1) +yoffset]    (eq.1)

- Otherwise if Log2( nTbs ) is equal to 5,

- wedgePattern[ x ][ y ] = 
WedgePatternTable[ Log2( nTbS ) – 1 ][ wedge_full_tab_idx[ xTb ][ yTb ] ][ x >> 1 ][ y >> 1 ]	            (eq.2)

- Otherwise,

- wedgePattern[ x ][ y ] = WedgePatternTable[ Log2( nTbS ) ][ wedge_full_tab_idx[ xTb ][ yTb ] ][ x ][ y ]	    (eq.3)

- where xoffset and yoffset can be 0 or 1.We assume that xoffset=0, xoffset=1.

The information of wedgelet patterns with this proposed method is shown in Table 3. As can be seen, this proposed method could achieve 27.1% total table size saving. For 16x16 block and 8x8 block, the number of wedgelet pattern and the bins for decoding the wedgelet index are the same. Because the wedgelet pattern for 8x8 block is generated by down sampling the 16x16 block.

Table 3. Table size for wedgelet pattern with proposed method 2.2

| PU size | Number of wedgelet pattern | Bins for decoding the index | Number of bits for one wedgelet pattern | Total Table Size(bits) | Accumulated Table Size (bits) | Table size saving [%] |
| --- | --- | --- | --- | --- | --- | --- |
| 4×4 | 86 | 7 | 16 | 1,376 | 1,376 | 27.1%=49024/180960 |
| 8×8 | 510 | 9 | - | - | - |  |
| >=16×16 | 510 | 9 | 256 | 130,560 | 131,936 |  |

## Lookup table size reduction without downsample

In order to reduce the lookup table size further and simplified decoding process, we propose to modify patIdenticalFlag condition in wedgelet pattern list insertion process.

Figure 2. Example of patIdenticalFlag setting for wedgelet pattern

In current wedgelet pattern list insertion process, patIdenticalFlag set 0 when wedgePattern[ x ][ y ] is not equal to WedgePatternTable[ log2BlkSize ][ k ][ x ][ y ], k = 0..NumWedgePattern[ log2BlkSize ] – 1. In proposed design, patIdenticalFlag set 0 when the number of case when wedgePattern[ x ][ y ] not equal to WedgePatternTable [ log2BlkSize ][ k ][ x ][ y ] is larger than transform block size - 9 in wedgelet pattern list insertion process. As Figure 2 shows, patIdenticalFlag set 1 as the number of case when wedgePattern [ x ][ y ] not equal to WedgePatternTable[ log2BlkSize ][ k ][ x ][ y ] is 2 (green ones).

The number of wedgelet patterns with this proposed simplification for PU=16x16 is shown in Table 4. As shown in the table, this proposed method could achieve 2.12% total table size saving.

Table 4. Table size for wedgelet pattern with proposed method 2.3

| PU size | Number of wedgelet pattern | Bins for decoding the index | Number of bits for one wedgelet pattern | Total Table Size(bits) | Accumulated Table Size (bits) | Table size saving [%] |
| --- | --- | --- | --- | --- | --- | --- |
| 4×4 | 86 | 7 | 16 | 1,376 | 1,376 | (180,960-177,120) /180,960 = 2.12% |
| 8×8 | 766 | 10 | 64 | 49,024 | 50,400 |  |
| >=16×16 | 495 | 9 | 256 | 126,720 | 177,120 |  |

- Text Modification of mehods

The newly added parts compared to working draft are highlighted in yellow and the removed parts are marked with strikethrough.

## Downsample 8x8 wedgelet patterns

- I.8.4.4.2.11  Specification of table WedgePatternTable

    - The variables sizeScaleS and sizeScaleE are derived as specified in the following:

        - sizeScaleS = ( log2BlkSize > 32 ) ? 2 : 1		(Error: Reference source not found−1)

        - sizeScaleE = ( wedgeOri < 4  &&  log2BlkSize > 3 ) ? : 2 : 1	(Error: Reference source not found−2)

## Reuse wedgelet pattern of 16x16 block for 8x8 block

- I.8.4.4.2.11  Specification of table WedgePatternTable

NOTE 1 – Tables and values resulting from the processes specified in the following are independent of any information contained in the bitstream. Therefore the derivation process described in this clause can be invoked once as part of the initialization of the decoding process. Alternatively, the tables and values can be stored within the decoder (read-only) memory as fixed look-up tables, such that the derivation process described in this clause does not need to be implemented in the decoder at all.

The list WedgePatternTable[ log2BlkSize ] of binary partition patterns of size ( 1  <<  log2BlkSize )x( 1  <<  log2BlkSize ), the variable NumWedgePattern[ log2BlkSize ] specifying the number of binary partition patterns in list WedgePatternTable[ log2BlkSize ] are derived as specified in the following:

For log2BlkSize ranging fromis 2 toor 4, inclusive, the following applies:

- The variable resShift is set equal to ( log2BlkSize  = =  4 ) ? 0 : 1.

- The variable wBlkSize is set equal to ( 1  <<  ( log2BlkSize + resShift ) )

- For wedgeOri in the range of 0 to 5, inclusive, the following ordered steps apply.

- Depending on wedgeOri the variables xPosS, yPosS, xPosE, yPosE, xIncS, yIncS, xIncE, yIncE are derived as specified in Table Error: Reference source not found.1.

Table Error: Reference source not found.1 – Specification of xPosS, yPosS, xPosE, yPosE, xIncS, yIncS, xIncE, yIncE

| wedgeOri | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| xPosS | 0 | wBlkSize − 1 | wBlkSize − 1 | 0 | 0 | wBlkSize − 1 |
| yPosS | 0 | 0 | wBlkSize − 1 | wBlkSize − 1 | 0 | 0 |
| xPosE | 0 | wBlkSize − 1 | wBlkSize− 1 | 0 | 0 | 0 |
| yPosE | 0 | 0 | wBlkSize − 1 | wBlkSize − 1 | wBlkSize − 1 | 0 |
| xIncS | 1 | 0 | −1 | 0 | 1 | 0 |
| yIncS | 0 | 1 | 0 | −1 | 0 | 1 |
| xIncE | 0 | −1 | 0 | 1 | 1 | 0 |
| yIncE | 1 | 0 | −1 | 0 | 0 | 1 |

- The variables sizeScaleS and sizeScaleE are derived as specified in the following:

- sizeScaleS = ( log2BlkSize > 3 ) ? 2 : 1		(Error: Reference source not found−3)

- sizeScaleE = ( wedgeOri < 4  &&  log2BlkSize > 3 ) ? : 2 : 1	(Error: Reference source not found−4)

- For m in the range of 0 to wBlkSize / sizeScaleS − 1, inclusive, the following applies:

- For n in the range of 0 to wBlkSize / sizeScaleE − 1, inclusive, the following applies:

- The Wedgelet pattern generation process as specified in clause Error: Reference source not found is invoked with patternSize being equal to ( 1  <<  log2BlkSize ), the variable resShift, variable wedgeOri, xS being equal to ( xPosS + m * sizeScaleS * xIncS ), yS being equal to ( yPosS + m * sizeScaleS * yIncS ), xE being equal to ( xPosE + n * sizeScaleE * xIncE ) and yE being equal to ( yPosE + n * sizeScaleE * yIncE ) as inputs, and the output is the binary array curWedgePattern.

- The wedgelet pattern list insertion process as specified in clause  is invoked with log2BlkSize, and the binary partition pattern curWedgePattern as inputs.

NumWedgePattern[ 5 ] and NumWedgePattern[ 3 ] is set equal to NumWedgePattern[ 4 ].

For k = 0..NumWedgePattern[ 5 ] − 1 the following applies:

- For x, y = 0..( 1  <<  5 ) − 1 the following applies:

- WedgePatternTable[ 5 ][ k ][ x ][ y ] = WedgePatternTable[ 4 ][ k ][ x  >>  1 ][ y  >>  1 ]

For k = 0..NumWedgePattern[ 3 ] − 1 the following applies:

- For x, y = 0..( 1  <<  3 ) − 1 the following applies:

WedgePatternTable[ 3 ][ k ][ x ][ y ] = WedgePatternTable[ 4 ][ k ][ (x  <<  1)+0 ][ (y  <<  1)+1 ]

NOTE 2 – WedgePatternTable[ 5 ] and WedgePatternTable [ 3 ] might be derived on-the-fly in the decoding process from WedgePatternTable[ 4 ].

## Lookup table size reduction without downsample

- I.8.4.4.2.11  Specification of table WedgePatternTable

    - The variables sizeScaleS and sizeScaleE are derived as specified in the following:

        - sizeScaleS = ( log2BlkSize > 3 ) ? 2 : 1		(Error: Reference source not found−5)

        - sizeScaleE = ( wedgeOri < 4  &&  log2BlkSize > 3 ) ? : 2 : 1	(Error: Reference source not found−6)

    - For m in the range of 0 to wBlkSize / sizeScaleS − 1, inclusive, the following applies:

      - For n in the range of 0 to wBlkSize / sizeScaleE − 1, inclusive, the following applies:

        - The Wedgelet pattern generation process as specified in clause Error: Reference source not found is invoked with patternSize being equal to ( 1  <<  log2BlkSize ), the variable resShift, variable wedgeOri, xS being equal to ( xPosS + m * sizeScaleS * xIncS ), yS being equal to ( yPosS + m * sizeScaleS * yIncS ), xE being equal to ( xPosE + n * sizeScaleE * xIncE ) and yE being equal to ( yPosE + n * sizeScaleE * yIncE ) as inputs, and the output is the binary array curWedgePattern.

        - The wedgelet pattern list insertion process as specified in clause  is invoked with log2BlkSize, and the binary partition pattern curWedgePattern as inputs.

- I.8.4.4.2.11.2 Wedgelet pattern list insertion process

- The variable validPatternFlag is set equal to 0 and the following applies:.

- For x, y = 0..( 1  <<  log2BlkSize ) − 1 the following applies:

    - When wedgePattern[ x ][ y ] is not equal to wedgePattern[ 0 ][ 0 ], the flag validPatternFlag is set to 1.

  - For k = 0..NumWedgePattern[ log2BlkSize ] − 1 the following applies:

    - The flag patIdenticalFlag is set equal to 1.

    - For x, y = 0..( 1  <<  log2BlkSize ) − 1 the following applies:

      - When the number of the case that wedgePattern[ x ][ y ] is not equal to WedgePatternTable [ log2BlkSize ][ k ] [ x ][ y ] is larger than ( log2BlkSize > 3 ) ? 7 : 0, patIdenticalFlag is set to 0.

    - When patIdenticalFlag is equal to 1, validPatternFlag is set to 0.

- Experimental Results

The proposed methods were implemented on top of HTM-13.0[4]. All the test results are evaluated in random access (CTC) and All-Intra configurations[5], respectively. The results are summarized in Table 5 - 10.

Table 5 shows the proposed method 2.1 introduces a negligible coding loss (0.01% loss for synthesized views).

Table 5. Comparison results between CTC anchor and Proposed Method 2.1

|  | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 99.9% | 100.3% | 100.6% |
| Kendo | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 100.7% | 101.9% | 101.6% |
| Newspaper_CC | -0.2% | 0.0% | 0.0% | 0.0% | 0.0% | 99.8% | 98.2% | 97.7% |
| GT_Fly | 0.2% | 0.0% | 0.0% | 0.0% | 0.0% | 101.0% | 102.7% | 101.2% |
| Poznan_Hall2 | 0.3% | 0.1% | 0.1% | 0.1% | 0.0% | 100.1% | 99.3% | 101.9% |
| Poznan_Street | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 103.4% | 99.3% | 101.3% |
| Undo_Dancer | -0.1% | 0.2% | 0.0% | 0.0% | 0.0% | 103.6% | 99.9% | 102.6% |
| Shark | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 101.1% | 101.1% | 101.8% |
| 1024x768 | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 100.1% | 100.1% | 100.0% |
| 1920x1088 | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 101.8% | 100.4% | 101.7% |
| average | 0.0% | 0.0% | 0.01% | 0.01% | 0.01% | 101.2% | 100.3% | 101.1% |

Table 6 shows proposed method 2.1 provides no coding loss (0.00% loss for AI).

Table 6. Comparison results between AI anchor and Proposed Method 2.1

|  | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.2% | 99.2% | 98.8% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.0% | 100.3% | 99.3% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.3% | 97.1% | 98.8% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.5% | 96.9% | 95.1% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.2% | 101.4% | 99.0% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.8% | 97.3% | 100.3% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.9% | 106.6% | 100.3% |
| Shark | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.4% | 98.0% | 102.0% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.2% | 98.9% | 99.0% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.8% | 100.1% | 99.4% |
| average | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 99.9% | 99.6% | 99.2% |

Table 7 shows the proposed method 2.2 introduces a negligible coding loss (0.03% loss for synthesized views).

Table 7. Comparison results between CTC anchor and Proposed Method 2.2

|  | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.1% | 0.0% | -0.01% | 0.01% | 0.06% | 100.4% | 100.0% | 94.0% |
| Kendo | -0.1% | 0.2% | 0.03% | 0.03% | 0.04% | 98.5% | 101.4% | 102.0% |
| Newspaper_CC | -0.1% | -0.1% | -0.05% | -0.02% | 0.05% | 98.0% | 106.0% | 99.0% |
| GT_Fly | 0.0% | 0.0% | 0.01% | 0.00% | 0.06% | 100.9% | 102.2% | 104.5% |
| Poznan_Hall2 | 0.2% | 0.0% | 0.05% | 0.06% | 0.00% | 101.2% | 102.9% | 98.8% |
| Poznan_Street | -0.1% | 0.1% | 0.00% | -0.01% | -0.04% | 100.2% | 102.6% | 104.1% |
| Undo_Dancer | -0.2% | 0.0% | -0.03% | -0.03% | 0.04% | 98.1% | 99.1% | 97.2% |
| Shark | 0.0% | 0.1% | 0.00% | -0.02% | 0.03% | 99.2% | 100.4% | 99.5% |
| 1024x768 | -0.1% | 0.0% | -0.01% | 0.01% | 0.05% | 99.0% | 102.5% | 98.3% |
| 1920x1088 | 0.0% | 0.0% | 0.01% | 0.00% | 0.02% | 99.9% | 101.4% | 100.8% |
| average | 0.0% | 0.0% | 0.00% | 0.00% | 0.03% | 99.6% | 101.8% | 99.9% |

Table 8 shows proposed method 2.2 provides a negligible coding loss (0.01% loss for AI).

Table 8. Comparison results between AI anchor and Proposed Method 2.2

|  | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.00% | 0.00% | -0.01% | 99.0% | 101.0% | 105.1% |
| Kendo | 0.0% | 0.0% | 0.00% | 0.00% | -0.01% | 101.1% | 109.8% | 104.3% |
| Newspaper_CC | 0.0% | 0.0% | 0.00% | 0.00% | 0.02% | 100.0% | 102.2% | 103.0% |
| GT_Fly | 0.0% | 0.0% | 0.00% | -0.01% | 0.01% | 102.5% | 106.7% | 109.5% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.00% | 0.00% | -0.01% | 99.4% | 96.0% | 99.8% |
| Poznan_Street | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 101.6% | 108.7% | 109.7% |
| Undo_Dancer | 0.0% | 0.0% | 0.00% | 0.00% | 0.01% | 101.3% | 96.9% | 95.4% |
| Shark | 0.0% | 0.0% | 0.00% | -0.01% | 0.06% | 102.6% | 106.3% | 111.2% |
| 1024x768 | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.0% | 104.3% | 104.1% |
| 1920x1088 | 0.0% | 0.0% | 0.00% | 0.00% | 0.02% | 101.5% | 102.9% | 105.1% |
| average | 0.0% | 0.0% | 0.00% | 0.00% | 0.01% | 100.9% | 103.4% | 104.7% |

Table 9 shows the proposed method 2.3 introduces no coding loss (0.00% loss for synthesized views).

Table 9. Comparison results between CTC anchor and Proposed Method 2.3

|  | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.13% | 0.01% | -0.02% | 0.03% | -0.04% | 100.0% | 92.5% | 95.7% |
| Kendo | 0.00% | 0.03% | 0.01% | 0.02% | -0.06% | 99.6% | 94.1% | 99.7% |
| Newspaper_CC | -0.06% | -0.15% | -0.03% | 0.01% | 0.06% | 100.2% | 95.6% | 97.9% |
| GT_Fly | -0.02% | -0.15% | -0.01% | -0.03% | 0.03% | 99.5% | 95.9% | 99.4% |
| Poznan_Hall2 | 0.02% | 0.13% | 0.04% | 0.03% | -0.05% | 98.5% | 97.5% | 98.9% |
| Poznan_Street | -0.06% | 0.08% | 0.01% | 0.01% | -0.04% | 99.0% | 95.6% | 93.7% |
| Undo_Dancer | -0.16% | 0.16% | -0.01% | -0.01% | 0.02% | 98.3% | 94.7% | 94.8% |
| Shark | 0.08% | 0.18% | 0.02% | 0.03% | 0.09% | 98.1% | 95.5% | 98.3% |
| 1024x768 | -0.06% | -0.04% | -0.01% | 0.02% | -0.01% | 99.9% | 94.1% | 97.8% |
| 1920x1088 | -0.03% | 0.08% | 0.01% | 0.01% | 0.01% | 98.7% | 95.8% | 97.0% |
| average | -0.04% | 0.04% | 0.00% | 0.01% | 0.00% | 99.1% | 95.2% | 97.3% |

Table 10 shows proposed method 2.3 provides no coding loss (0.00% loss for AI).

Table 10. Comparison results between AI anchor and Proposed Method 2.3

|  | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 97.7% | 104.6% | 102.9% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.7% | 100.1% | 97.7% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 97.9% | 109.4% | 103.3% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 98.7% | 106.1% | 105.0% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 98.0% | 96.3% | 102.1% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 97.9% | 101.0% | 98.5% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 96.0% | 93.2% | 92.9% |
| Shark | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.0% | 111.6% | 102.4% |
| 1024x768 | 0.00% | 0.00% | 0.00% | 0.01% | 0.01% | 98.5% | 104.7% | 101.3% |
| 1920x1088 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 98.1% | 101.7% | 100.2% |
| average | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 98.2% | 102.8% | 100.6% |

- Conclusions

This contribution proposes three methods to make lookup table size reduction for DMM1. It is asserted that proposed methods can achieve storage size reduction by 8.98%, 27.1% and 2.12% with 0.01% / 0.00%, 0.03% / 0.01%, 0.00% / 0.00% coding loss for synth PSNR vs. total bitrate in the CTC / all-intra configuration, respectively.

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, "3D-HEVC Draft Text 6," Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG JCT3V-J1001-v6, Oct. 2014

[2] X. Zhang, K. Zhang, J. An, H. Huang, J. Lin, S. Lei, On Lookup Table Size Reduction for DMM1, JCT3V-J0035, Oct., 2014.

[3] Tomohiro Ikai, Lookup table size reduction in DMM1, JCT3V-I0110, Jul., 2014.

[4] HTM-13.0, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-13.0

[5] D. Rusanovksyy, A. Vetro, and K. Müller, "Common Test Conditions for 3D Video Extensions Development," Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG JCT3V-E1100, July, 2013.

# Patent rights declaration(s)

HiSilicon Technologies, UESTC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).