| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Geneva, CH, 12–18 Feb. 2015 | Document: JCT3V-K0048 |
| --- | --- |

| Title: | Clean up on DBBP location derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xu Chen, Xiaozhen Zheng, Yong-Bing Lin, Jian-Hua Zheng HiSilicon technologies, Shenzhen, P. R. China, 518129 | Tel: Email: | +86-755-28425333 anderson.chen@huawei.com |
| Source: | HiSilicon Technologies |  |  |

_____________________________

# Abstract

In HTM-13.0[1], location of top-left luma sample of the corresponded depth block is calculated by the top-left luma sample of current block with limitation of Clip3(0, PictureWidth(Heigh) - BlockWidth(Heigh), xTb(yTb) + ((mvDisp[ 0 ]+2) >>2)). However, the text of 3D-HEVC[2] set the location of top-left luma sample of the corresponded depth block with limitation of Clip3(0, PictureWidth(Heigh) - 1, xTb(yTb) + ((mvDisp[ 0 ]+2) >>2)). The contribution set the alignment and experimental result reportedly shows that minor coding gain -0.05% for synth PSNR vs. total bitrate in the CTC. Also, this contribution proposes a method to simplify luma location calculation of top-left sample of depth block for DBBP. Experimental result reportedly shows that no coding loss (0.00% / 0.00%) for synth PSNR vs. total bitrate in the CTC with DepthRefinementflag on/off configuration, respectively.

# Introduction

In the current 3D-HEVC[2], the location of top-left luma sample of the corresponded depth block is determined by picture size, disparity vector, location and size of top-left luma sample of the current luma coding block with clip process.

# Proposed Methods

## Text alignment with limitation

We propose to make the alignment with HTM and the text of 3D-HEVC that the location (xTL, yTL) of top-left luma sample of the corresponded depth block with the limitation of (iPictureWidth – 1, iPictureHeight - 1).

## Simplification of location calculation

We propose to set the y coordinate of top-left luma sample of the corresponded depth block equal to the y coordinate of top-left luma sample of the current luma coding block. The proposed methods are described as follows.

Int depthPosX = Clip3(0,   iPictureWidth - 1,  iBlkX + ((cDv.getHor()+2)>>2));

Int depthPosY = Clip3(0,   iPictureHeight - 1, iBlkY + ((cDv.getVer()+2)>>2)) iBlkY;

- Text Modification of method 2.2

The newly added parts compared to working draft are highlighted in yellow and the removed parts are marked with strikethrough.

- I.8.5.5.2 Derivation process for a disparity sample array

Let refDepPels be an array of reconstructed depth samples refDepPic. The luma location ( xTL, yTL ) of top-left luma sample of a block in refDepPels is derived by

- xTL = xP + ( ( mvDisp[ 0 ] + 2 )  >>  2 )	                                       (Error: Reference source not found−1)

yTL = dbbp_flag[ xP ][ yP ] ? yP : yP + ( ( mvDisp[ 1 ] + 2 )  >>  2 )	            (Error: Reference source not found−2)

- Experimental Results

The proposed methods were implemented on top of HTM-13.0. All the test results are evaluated in random access (CTC) [2]. The results are summarized in Table 1 - 4.

Table 1 shows the proposed method 2.1 provides minor coding gain (0.05% gain for synthesized views).

Table 1. Comparison results between CTC anchor and Proposed Method 2.1

|  | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.01% | 0.00% | 0.00% | 0.02% | 100.7% | 100.8% | 100.6% |
| Kendo | 0.00% | 0.02% | 0.00% | 0.00% | 0.01% | 100.7% | 99.5% | 100.5% |
| Newspaper_CC | 0.00% | -0.01% | 0.00% | 0.01% | 0.02% | 100.4% | 99.5% | 100.0% |
| GT_Fly | 0.00% | -0.02% | 0.00% | 0.00% | 0.00% | 100.0% | 100.6% | 100.1% |
| Poznan_Hall2 | 0.00% | 0.02% | 0.00% | 0.00% | 0.00% | 99.6% | 98.3% | 100.6% |
| Poznan_Street | 0.00% | 0.00% | 0.00% | 0.00% | -0.42% | 100.3% | 99.0% | 99.7% |
| Undo_Dancer | 0.00% | 0.05% | 0.00% | 0.00% | 0.00% | 100.4% | 101.5% | 102.3% |
| Shark | 0.00% | -0.02% | 0.00% | 0.00% | 0.00% | 100.7% | 102.9% | 100.5% |
| 1024x768 | 0.00% | 0.00% | 0.00% | 0.00% | 0.02% | 100.6% | 99.9% | 100.4% |
| 1920x1088 | 0.00% | 0.01% | 0.00% | 0.00% | -0.08% | 100.2% | 100.5% | 100.6% |
| average | 0.00% | 0.00% | 0.00% | 0.00% | -0.05% | 100.4% | 100.3% | 100.5% |

Table 2-3 shows the proposed method 2.2 provides no coding loss (0.00% loss for synthesized views).

Table 2. Comparison results between CTC anchor and Proposed Method 2.2

|  | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100.4% | 100.6% | 98.4% |
| Kendo | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100.6% | 97.8% | 100.0% |
| Newspaper_CC | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100.5% | 99.8% | 100.2% |
| GT_Fly | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100.4% | 99.1% | 100.1% |
| Poznan_Hall2 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99.8% | 98.9% | 99.3% |
| Poznan_Street | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100.1% | 98.6% | 98.2% |
| Undo_Dancer | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100.1% | 101.2% | 101.1% |
| Shark | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100.2% | 99.4% | 98.7% |
| 1024x768 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100.5% | 99.4% | 99.5% |
| 1920x1088 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100.1% | 99.4% | 99.5% |
| average | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100.3% | 99.4% | 99.5% |

Table 3. Comparison results between CTC anchor and Proposed Method 2.2(Depthrefinementflag Off)

|  | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.07% | 0.07% | 0.00% | 0.00% | 0.06% | 100.1% | 99.7% | 99.1% |
| Kendo | -0.20% | 0.06% | -0.02% | -0.02% | -0.02% | 100.1% | 101.2% | 100.6% |
| Newspaper_CC | 0.05% | -0.07% | -0.02% | -0.06% | -0.02% | 100.4% | 99.9% | 101.0% |
| GT_Fly | 0.05% | -0.08% | 0.00% | 0.00% | 0.00% | 99.9% | 98.2% | 97.3% |
| Poznan_Hall2 | -0.11% | 0.03% | -0.02% | -0.04% | -0.07% | 99.8% | 99.5% | 100.6% |
| Poznan_Street | -0.01% | 0.11% | 0.01% | 0.00% | 0.02% | 102.1% | 99.1% | 103.6% |
| Undo_Dancer | -0.05% | -0.10% | -0.02% | -0.02% | 0.02% | 102.1% | 99.6% | 100.0% |
| Shark | -0.01% | -0.02% | 0.00% | 0.00% | 0.01% | 98.4% | 97.9% | 93.7% |
| 1024x768 | -0.07% | 0.02% | -0.01% | -0.03% | 0.01% | 100.2% | 100.2% | 100.2% |
| 1920x1088 | -0.03% | -0.01% | -0.01% | -0.01% | -0.01% | 100.5% | 98.9% | 99.1% |
| average | -0.04% | 0.00% | -0.01% | -0.02% | 0.00% | 100.4% | 99.4% | 99.5% |

Table 4 shows the proposed method 2.1+2.2 provides no coding loss (0.00% loss for synthesized views).

Table 4. Comparison results between CTC anchor and Proposed Method 2.1+2.2(Depthrefinementflag Off)

|  | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.07% | 0.08% | 0.00% | 0.00% | 0.05% | 99.2% | 97.9% | 100.6% |
| Kendo | -0.20% | 0.06% | -0.02% | -0.03% | -0.03% | 98.2% | 101.3% | 99.4% |
| Newspaper_CC | 0.05% | -0.08% | -0.02% | -0.06% | -0.02% | 100.8% | 104.7% | 101.8% |
| GT_Fly | 0.05% | 0.03% | 0.00% | 0.00% | 0.00% | 101.6% | 102.7% | 103.3% |
| Poznan_Hall2 | -0.11% | 0.03% | -0.02% | -0.04% | -0.08% | 99.3% | 101.6% | 95.6% |
| Poznan_Street | -0.01% | 0.08% | 0.01% | 0.00% | 0.02% | 99.5% | 96.1% | 96.3% |
| Undo_Dancer | -0.05% | -0.07% | -0.01% | -0.01% | 0.02% | 101.5% | 96.4% | 101.9% |
| Shark | -0.01% | -0.03% | 0.00% | 0.00% | 0.01% | 98.4% | 97.9% | 94.1% |
| 1024x768 | -0.07% | 0.02% | -0.01% | -0.03% | 0.00% | 99.4% | 101.3% | 100.6% |
| 1920x1088 | -0.03% | 0.01% | 0.00% | -0.01% | -0.01% | 100.1% | 98.9% | 98.2% |
| average | -0.04% | 0.01% | -0.01% | -0.02% | 0.00% | 99.8% | 99.8% | 99.1% |

- Conclusions

This contribution proposes 2 methods for DBBP clean up and calculation simplification. Experimental results reportedly shows that the clean up has minor coding gain (-0.05%) and no coding loss (0.00%) for the simplification of depth sample location calculation for synth PSNR vs. total bitrate in the CTC configuration, respectively.

# References

[1] HTM-13.0, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-13.0

[2] G. Tech, K. Wegner, Y. Chen, S. Yea, "3D-HEVC Draft Text 6," Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG JCT3V-J1001-v6, Oct. 2014

[3] D. Rusanovksyy, A. Vetro, and K. Müller, "Common Test Conditions for 3D Video Extensions Development," Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG JCT3V-E1100, July, 2013.

# Patent rights declaration(s)

HiSilicon Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).