| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Geneva, CH, 12–18 Feb. 2015 | Document: JCT3V-K0053 |
| --- | --- |

| Title: | Bug fix of the 3D-HEVC specification text on the scaling of TMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ying Chen 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

It is asserted that the current 3D-HEVC text has a minor bug for the scaling of the motion vector during temporal motion vector prediction. The current text has two issues: 1) there is a case that the disparity motion vectors are scaled based on POC difference when the high-level syntax indicates there is no scaling based on view identifier values. 2) when a motion vector is pointing to a long-term (temporal) reference picture, it may be scaled. These two issues are reported to be not aligned with the design intent thus a bug fix in the 3D-HEVC text is proposed to resolve them.

# Introduction

The major design principles related to motion vector scaling (as part of temporal motion vector prediction) in 3D-HEVC are summarized as follows:

- For alignment with MV-HEVC (thus also HEVC version 1), an inter-view reference picture is marked as long-term;

- Scaling of motion vectors corresponding to long-term reference pictures (including inter-view reference pictures) based on POC distance is disabled to avoid the cases when POC distance becomes equal to 0;

- If a high-level syntax flag (IvMvScalingFlag) further indicates view identifier difference based scaling applies, disparity motion vectors are scaled based on view identifier difference.

- A motion vector corresponding to a long-term reference picture (real temporal reference) may be predicted by another motion vector corresponding to an inter-view reference picture (also marked as long-term) without scaling, same as in MV-HEVC (and hypothetically in HEVC version 1).

- A motion vector corresponding to a long-term reference picture (real temporal reference) may be predicted by another motion vector corresponding to a long-term reference picture (real temporal reference) without scaling, same as in HEVC version 1.

The current 3D-HEVC has the following issues:

- When IvMvScalingFlag is equal to 0 (thus no view identifier based prediction) and the relevant motion vectors (the one from the temporal candidate and the target one) are both associated with long-term reference pictures, POC based scaling applies.

- When IvMvScalingFlag is equal to 1 and the relevant motion vectors are both associated with long-term reference pictures and not both associated with inter-view reference pictures, view identifier based scaling applies.

In both of the above two cases, by the design principles as mentioned above, scaling should be just simply avoided. The proposed change doesn’t have an impact to CTC results.

The bug fix for the 3D-HEVC text (with change marks) is provided in the associated working draft text.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).