| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Geneva, CH, 12–18 Feb. 2015 | Document: JCT3V-K0068 |
| --- | --- |

| Title: | 3D-CE2: Crosscheck of complexity reduction on illumination compensation for 3D-HEVC - Enabling 4x4 chroma and disabling bi-prediction (JCT3V-K0032) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Shinya Shimizu, and Shiori Sugimoto 1-1, Hikari-no-Oka, Yokosuka, Kanagawa, JAPAN | Tel: Email: | +81-46-859-2703 shimizu.shinya@lab.ntt.co.jp sugimoto.shiori@lab.ntt.co.jp |
| Source: | NTT Corporation |  |  |

_____________________________

# Abstract

The purpose of this document is to crosscheck on the complexity reduction on illumination compensation (IC) for 3D-HEVC in JCT3V-K0032. This documents reports the crosscheck result on the combination of enabling 4x4 chroma IC and disabling bi-prediction for IC. The verification task was done successfully. No mismatch is identified under both CTC and IBP cases. The simulation result reportedly shows the average bitrate increase of 0.01% and 0.01% with 0.8% decoding runtime decrease for the coded and synthesized views, respectively.

# Introduction

The contribution JCT3V-K0032 proposed the complexity reduction on illumination compensation (IC) for 3D-HEVC. The contribution includes two aspects: enabling 4x4chroma IC and disabling bi-prediction for IC. This document reports the crosscheck results on the combination of these aspects. In this crosscheck report, the simulations are conducted with the same software used in [1].

# Crosscheck Results

The simlations are conducted on the common test condition [2]. The provided source code was compiled on 64-bit Windows 7 environment by using Microsoft Visual Studio 2010 Professional Edition, and the experiments were performed on the cluster of PCs with 64-bit Windows 7. Regarding the decoding time, we use the average decoding times of three decoding trials.

No mismatch is identified between encoder and decoder. The coding results matched with those provided by the proponents. The obtained results are reported in Error: Reference source not found. Detailed results are available in the attached EXCEL files.

TABLE 1: Results of crosscheck simulation

- CTC

- IBP

# Conclusions

The results obtained during this crosscheck match those provided by the proponents. Experimental result reportedly shows the average bitrate increase of 0.04% and 0.02% with 0.4% decoding runtime decrease0.01% and 0.01% with 0.8% decoding runtime decrease for the coded and synthesized views, respectively. Source codes were not reviewed.

# References

- X. Zhang, K. Zhang, J. An, H. Huang, J.-L. Lin, and S. Lei, “3D-CE2: Complexity reduction on illumination compensation for 3D-HEVC”, JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-K0032, February 2015, Geneva, CH.

- JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, “Common Test Conditions of 3DV Core Experiments”, JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-G1100, January 2014, San Jose, US.