| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Geneva, CH, 12–18 Feb. 2015 | Document: JCT3V-K0073 |
| --- | --- |

| Title: | On MFC plus Depth: Conformance Testing Update |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Peng Yin 432 Lakeside Dr. Sunnyvale CA 94085 USA Dong Tian MERL 201 Broadway Cambridge, MA 02139 | Tel: Email: | +1 408 330 3251 pyin@dolby.com tian@merl.com |
| Source: | Dolby Laboratories Inc. and MERL |  |  |

_____________________________

# Abstract

This document provides an update on draft 1 of new amendment to ITU-T Rec. H.264.1 | ISO/IEC 14496-4 adding conformance test of Multi-resolution Frame Compatible Stereo Coding with Depth Maps extension of AVC. The text of MFC+D conformance testing is updated based on USNB technical comments. There is no change on MFC+D conformance bitstreams.

In 10.6.5.7, add the following text at the end of the subclause:

A decoder that conforms to the MFC Depth High profile at a specific level shall be capable of decoding the specified bitstreams in Table-AMD45-1. A decoder that conforms to the MFC Depth High profile shall also be capable of decoding all bitstreams that are required to be decoded by a Main, High, Stereo High and MFC High profile decoder of the same level. In addition to the specified bitstreams in Table-AMD-45-1, a decoder that conforms to the MFC Depth High profile shall be capable of decoding the bitstreams in Tables 1, 2, 5 and 7 that correspond to these requirements.

After 10.6.6.37.12, add the following text:

- 10.6.6.36 Test bitstreams – MFC Depth High Profile

- 10.6.6.36.1 Test bitstream # MFCDDR-1

Specification: All slices are coded as I, P or B slices. Only the first picture is coded as an IDR access unit. Each view component contains only one slice. num_views_minus1 is equal to 1. NumDepthViews is equal to 2. The base layer texture view is in SbS frame compatible format. Both the texture view and depth view components are progressive. The coding order of texture view and depth view is specified as "T0D0D1T1", i.e., texture from view point 0, followed by depth from view point 0, followed by depth from view point 1, and followed by texture from view point 1. The width and the height of depth view components are equal to the texture view components. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T H.264 | ISO/IEC 14496-10.

Functional stage: Decoding of the base and enhancement texture view component and depth view component and reconstruction of the enhanced resolution stereo texture views.

Purpose: Check that the decoder can properly decode the base and enhancement texture view components and the same resolution depth view components, and reconstruct the enhanced resolution stereo texture views.

- 10.6.6.36.2 Test bitstream # MFCDDR-2

Specification: All slices are coded as I, P or B slices. Only the first picture is coded as an IDR access unit. Each view component contains only one slice. num_views_minus1 is equal to 1. NumDepthViews is equal to 2. The base layer texture view is in SbS frame compatible format. Both the texture view and depth view components are progressive. The coding order of texture view and depth view is specified as "T0D0D1T1", i.e., texture from view point 0, followed by depth from view point 0, followed by depth from view point 1, and followed by texture from view point 1. The width and the height of the depth view components are half of the texture view components. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T H.264 | ISO/IEC 14496-10.

Functional stage: Decoding of the base and enhancement texture view component and depth view component and reconstruction of the enhanced resolution stereo texture views.

Purpose: Check that the decoder can properly decode the base and enhancement texture view components and lower resolution depth view components, and reconstruct the enhanced resolution stereo texture views.

- 10.6.6.36.3 Test bitstream #MFCDFLD-1

Specification: All slices are coded as I, P or B slices. Only the first picture is coded as an IDR access unit. Each view component contains only one slice. num_views_minus1 is equal to 1. NumDepthViews is equal to 2. The base layer texture view is in SbS frame compatible format. The coding order of texture view and depth view is specified as "T0D0D1T1", i.e., texture from view point 0, followed by depth from view point 0, followed by depth from view point 1, and followed by texture from view point 1. . field_pic_flag is equal to 1 in texture view components. field_pic_flag is equal to 0 in depth view components. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T H.264 | ISO/IEC 14496-10.

Functional stage: Decoding of the base and enhancement interlaced texture view component and interlaced depth view component and reconstruction of the enhanced resolution stereo texture views.

Purpose: Check that the decoder can properly decode the interlaced base and enhancement texture view component and progressive depth components, and reconstruct the enhanced resolution stereo texture views.

- 10.6.6.36.4 Test bitstream #MFCDFLD-2

Specification: All slices are coded as I, P or B slices. Only the first picture is coded as an IDR access unit. Each view component contains only one slice. num_views_minus1 is equal to 1. NumDepthViews is equal to 2. The base layer texture view is in SbS frame compatible format. The coding order of texture view and depth view is specified as "T0D0D1T1", i.e., texture from view point 0, followed by depth from view point 0, followed by depth from view point 1, and followed by texture from view point 1. field_pic_flag is equal to 1 for both texture view components and depth view components. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T H.264 | ISO/IEC 14496-10.

Functional stage: Decoding of the base and enhancement interlaced texture view component and progressive depth view component and reconstruction of the enhanced resolution stereo texture views.

Purpose: Check that the decoder can properly decode the interlaced base and enhancement texture view component and interlaced depth view components, and reconstruct the enhanced resolution stereo texture views.

- 10.6.6.36.5 Test bitstream #MFCDTDC-1

Specification: All slices are coded as I, P or B slices. Only the first picture is coded as an IDR access unit. Each view component contains only one slice. num_views_minus1 is equal to 1. NumDepthViews is equal to 2. The base layer texture view is in SbS frame compatible format. Both the texture view and depth view components are progressive. The coding order of texture view and depth view is specified as "T0D0T1D1", i.e., texture from view point 0, followed by depth from view point 0, followed by texture from view point 1, followed by depth from view point 1. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T H.264 | ISO/IEC 14496-10.

Functional stage: Decoding of the base and enhancement texture view component and depth view component and reconstruction of the enhanced resolution stereo texture views.

Purpose: Check that the decoder can properly decode the base and enhancement texture view components and depth view components with various coding order, and reconstruct the enhanced resolution stereo texture views.

- 10.6.6.36.6 Test bitstream #MFCDTDC-2

Specification: All slices are coded as I, P or B slices. Only the first picture is coded as an IDR access unit. Each view component contains only one slice. num_views_minus1 is equal to 1. NumDepthViews is equal to 2. The base layer texture view is in SbS frame compatible format. Both the texture view and depth view components are progressive. The coding order of texture view and depth view is specified as "T0T1D0D1", i.e., texture from view point 0, followed by texture from view point 1, followed by depth from view point 0, followed by depth from view point 1. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T H.264 | ISO/IEC 14496-10.

Functional stage: Decoding of the base and enhancement texture view components and depth view component and reconstruction of the enhanced resolution stereo texture views.

Purpose: Check that the decoder can properly decode the base and enhancement texture view components and depth view components with various coding order, and reconstruct the enhanced resolution stereo texture views.

- 10.6.6.36.7 Test bitstream #MFCDMFC-1

Specification: All slices are coded as I, P or B slices. Only the first picture is coded as an IDR access unit. Each view component contains only one slice. num_views_minus1 is equal to 1. NumDepthViews is equal to 2. Both the texture view and depth view components are progressive. The coding order of texture view and depth view is specified as "T0D0D1T1", i.e., texture from view point 0, followed by depth from view point 0, followed by depth from view point 1, and followed by texture from view point 1. mfc_format_idc is set to 1, i.e., the base layer texture view is in TaB frame compatible format. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T H.264 | ISO/IEC 14496-10.

Functional stage: Decoding of the base and enhancement texture view component and depth view component and reconstruction of the enhanced resolution stereo texture views.

Purpose: Check that the decoder can properly decode the base and enhancement texture and depth view components with various base layer texture frame compatible format, and reconstruct the enhanced resolution stereo texture views.

Add the following after Table AMD45.1:

Table AMD45.1 – Bitstreams for the MFC Depth High profile

| Categories | Bitstream | Donated by | File Name | MFC Depth High | Level | Frame Rate (Frame/Sec) |
| --- | --- | --- | --- | --- | --- | --- |
| Depth Resolution | MFCDDR-1 | Dolby | MFCDDR-1 | X | 4.1 | 25p |
|  | MFCDDR-2 | Dolby | MFCDDR-2 | X | 4.1 | 25p |
| Interlaced Coding Tools | MFCDFLD-1 | Dolby | MFCDFLD-1 | X | 4.1 | 50i |
|  | MFCDFLD-2 | Dolby | MFCDFLD-2 | X | 4.1 | 50i |
| Texture and Depth Coding Order | MFCDTDC-2 | Dolby | MFCDTDC-2 | X | 4.1 | 25p |
|  | MFCDTDC-2 | Dolby | MFCDTDC-2 | X | 4.1 | 25p |
| MFC format | MFCDMFC-1 | Dolby | MFCDMFC-1 | X | 4.1 | 25p |

Attach the corresponding electronic attachment.