| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Warsaw, PL, 20–26 June 2015 | Document: JCT3V-L0025r2 |
| --- | --- |

| Title: | Draft of Texture/Depth View Packing SEI Message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document of JCT-3V |  |  |
| Purpose: | Draft Text |  |  |
| Author(s) or Contact(s): | Takanori Senoh NICT Ying Chen Qualcomm Miska M. Hannuksela Nokia Jens-Rainer Ohm RWTH Aachen University Gary J. Sullivan Microsoft | Email: Email: Email: Email: Email: | senoh@nict.go.jp cheny@qti.qualcomm.com miska.hannuksela@nokia.com ohm@ient.rwth-aachen.de garysull@microsoft.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This amendment defines a texture and depth view packing SEI message.

In Table D.1, add the following after “depth_view_info ( payloadSize )”:

| else if( payloadType = = 181 ) |  |  |
| --- | --- | --- |
| texture_and_depth_view_packing( payloadSize ) /* specified in Annex J */ | 5 |  |

Add a new subclause J.13.1.2 “Texture and depth view packing SEI message syntax” as follows:

      - Texture and depth view packing SEI message syntax

| texture_and_depth_view_packing( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| depth_type | 5 | ue(v) |
| if( depth_type = = 0 ) { |  |  |
| num_constituent_views_gvd_minus1 | 5 | ue(v) |
| depth_present_gvd_flag | 5 | u(1) |
| z_gvd_flag | 5 | u(1) |
| intrinsic_param_gvd_flag | 5 | u(1) |
| rotation_gvd_flag | 5 | u(1) |
| translation_gvd_flag | 5 | u(1) |
| if( z_gvd_flag ) |  |  |
| for( i = 0; i <= num_constituent_views_gvd_minus1 + 1; i++ ) { |  |  |
| sign_gvd_z_near_flag[ i ] | 5 | u(1) |
| exp_gvd_z_near[ i ] | 5 | u(7) |
| man_len_gvd_z_near_minus1[ i ] | 5 | u(5) |
| man_gvd_z_near[ i ] | 5 | u(v) |
| sign_gvd_z_far_flag[ i ] | 5 | u(1) |
| exp_gvd_z_far[ i ] | 5 | u(7) |
| man_len_gvd_z_far_minus1[ i ] | 5 | u(5) |
| man_gvd_z_far[ i ] | 5 | u(v) |
| } |  |  |
| if( intrinsic_param_gvd_flag ) { |  |  |
| prec_gvd_focal_length | 5 | ue(v) |
| prec_gvd_principal_point | 5 | ue(v) |
| } |  |  |
| if( rotation_gvd_flag ) |  |  |
| prec_gvd_rotation_param | 5 | ue(v) |
| if ( translation_gvd_flag ) |  |  |
| prec_gvd_translation_param | 5 | ue(v) |
| for( i = 0; i <= num_constituent_views_gvd_minus1 + 1; i++ ) { |  |  |
| if( intrinsic_param_gvd_flag ) { |  |  |
| sign_gvd_focal_length_x[ i ] | 5 | u(1) |
| exp_gvd_focal_length_x[ i ] | 5 | u(6) |
| man_gvd_focal_length_x[ i ] | 5 | u(v) |
| sign_gvd_focal_length_y[ i ] | 5 | u(1) |
| exp_gvd_focal_length_y[ i ] | 5 | u(6) |
| man_gvd_focal_length_y[ i ] | 5 | u(v) |
| sign_gvd_principal_point_x[ i ] | 5 | u(1) |
| exp_gvd_principal_point_x[ i ] | 5 | u(6) |
| man_gvd_principal_point_x[ i ] | 5 | u(v) |
| sign_gvd_principal_point_y[ i ] | 5 | u(1) |
| exp_gvd_principal_point_y[ i ] | 5 | u(6) |
| man_gvd_principal_point_y[ i ] | 5 | u(v) |
| } |  |  |
| if( rotation_gvd_flag ) |  |  |
| for ( j = 0; j <= 2; j++ ) /* row */ |  |  |
| for( k = 0; k <= 2; k++ ) { /* column */ |  |  |
| sign_gvd_r[ i ][ j ][ k ] | 5 | u(1) |
| exp_gvd_r[ i ][ j ][ k ] | 5 | u(6) |
| man_gvd_r[ i ][ j ][ k ] | 5 | u(v) |
| } |  |  |
| if ( translation_gvd_flag ) { |  |  |
| sign_gvd_t_x[ i ] | 5 | u(1) |
| exp_gvd_t_x[ i ] | 5 | u(6) |
| man_gvd_t_x[ i ] | 5 | u(v) |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

Add a new subclause J.13.2.2 “Texture and depth view packing SEI message semantics” as follows, with “J-xx” replaced with the appropriate table number in the sequence of tables present in the text.

      - Texture and depth view packing SEI message semantics

The texture and depth view packing SEI message indicates that view components of one output view contain samples of multiple distinct spatially packed constituent pictures. The related output view is not suitable to be displayed directly. This SEI message can be used after the decoder output to rearrange the samples to produce additional views that are appropriate for display or to server for other purposes (which are outside the scope of this Recommendation | International Standard).

Texture and depth view referred by this SEI message conform to the specifications in Annex I and J. They are 8 bit, 4:2:0 sampled. The depth_representation_type is defined in the depth_representation_info SEI message which shall be always present. Depth view is only carried in luma component.

The texture and depth view packing SEI message, when present, shall be associated with an IDR access unit. The information signaled in this SEI message applies to all the access units in the coded video sequence. Also, num_anchor_refs_l0[ i ], num_anchor_refs_l1[ i ], num_non_anchor_refs_l0[ i ] and num_non_anchor_refs_l1[ i ] shall be equal to 0 in the active SPS RBSP syntax structure for the coded video sequence and avc_3d_extension_flag shall be 0 in all VCL NAL units of the coded video sequence.

NOTE 1 – These constraints disable inter-view and inter-component prediction.

In such a coded video sequence, there shall be two output views, wherein the base view is a complete view and a non-base view, which contains a packing arrangement of 1-4 sub-sampled pictures of additional views. Such sub-sampled pictures are denoted as constituent pictures here and after.

NOTE 2 – The view_id of the non-base view is not used for the view output in the context of this SEI message.

All constituent pictures shall have a width and a height equal to ( (pic_width_in_mbs_minus1 + 1) * 8 ) and ( (pic_height_in_map_unit_minus1 + 1) * 8 ) in luma samples, respectively . frame_mbs_only_flag shall be equal to 1. The variable i, with a value from 0 to num_constituent_views_gvd_minus1 + 1 inclusive, indicates the location of the constituent pictures in the non-base texture view as specified in Table J-xx. i equal to 0 indicates the base texture view. i greater than 0 indicates the constituent texture view.

- Table J-xx – Locations of the top-left luma samples of constituent pictures packed in a non-base texture view relative to the top-left luma sample of this picture

| Constituent picture index i | Location of the top-left luma sample in a non-base texture view |
| --- | --- |
| 1 | (0, 0) |
| 2 | (0, (pic_height_in_map_unit_minus1 + 1) * 8 ) |
| 3 | ((pic_width_in_mbs_minus1 + 1) * 8, 0) |
| 4 | ((pic_width_in_mbs_minus1 + 1) * 8, (pic_height_in_map_unit_minus1 + 1) * 8) |

The decoded depth views have the same structure as the decoded texture views, namely they are base depth view (i = 0) and non-base depth view. The non-base depth view contains up to four constituent depth pictures (i = 1,…4) for the constituent views. The constituent depth pictures are packed in the non-base depth view in the same arrangement as the constituent texture pictures shown in Table J-xx.

depth_type shall be equal to 0. depth_type is equal to 0 indicates Global View and Depth (GVD) information are present in this SEI. Other values are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this Specification. Decoders shall ignore texture and depth view packing SEI messages in which such other values are present.

num_constituent_views_gvd_minus1 plus 1 specifies the number of constituent texture pictures packed into each texture component of the non-base view. num_constituent_views_gvd_minus1 shall be in the range of 0 to 3, inclusive.

depth_present_gvd_flag equal to 1 specifies that constituent depth pictures are packed into the depth components of the non-base view, with a packing scheme that is identical to the one used for the constituent texture pictures. depth_present_gvd_flag equal to 0 specifies that the depth component of the non-base view is not present.

Each constituent depth picture in the depth component of non-base view is associated with a constituent picture in the texture component of non-base view in the same relative location. The number of the coded depth views in the coded video sequence shall be equal to 1 + depth_present_gvd_flag. The depth component of the base view shall always be present, independent of the value of depth_present_gvd_flag.

NOTE 3 – The following SEI message parameters can be used along with the depth maps to project samples from the base view into the co-ordinates of constituent views such that reconstructed views can be generated by combining projected samples and samples from the constituent views.

The function binToFp( s, e, n, v ) is specified as follows:

  - binToFp( s, e, n, v ) = (−1)s * ( e  = =  0 ? ( 2−( 30 + v ) * n ) : ( 2e − 31 * ( 1 + n  2v ) ) )

NOTE 4 – The above specification is similar to what is found in IEC 60559:1989, Binary floating-point arithmetic for microprocessor systems.

z_gvd_flag equal to 1 indicates the presence of the syntax elements sign_gvd_z_near_flag[ i ], exp_gvd_z_near[ i ],  man_len_gvd_z_near_minus1[ i ], man_gvd_z_near[ i ], sign_gvd_z_far_flag[ i ], exp_gvd_z_far[ i ], man_len_gvd_z_far_minus1[ i ], and man_gvd_z_far[ i ], for i in the range of 0 to num_constituent_views_gvd_minus1 + 1 inclusive. z_gvd_flag equal to 0 indicates that these syntax elements are not present.

intrinsic_param_gvd_flag equal to 1 indicates the presence of intrinsic camera parameter syntax elements. intrinsic_param_gvd_flag equal to 0 indicates that these syntax elements are not present.

rotation_gvd_flag equal to 1 indicates the presence of rotation camera parameter syntax elements. rotation_gvd_flag equal to 0 indicates that these syntax elements are not present. When rotation_gvd_flag is 0, a default rotation camera parameter of a unit matrix value is inferred.

translation_gvd_flag equal to 1 indicates the presence of horizontal translation camera parameter syntax elements. translation_gvd_flag equal to 0 indicates that these syntax elements are not present.

sign_gvd_z_near_flag[ i ] equal to 0 indicates that the sign of the nearest depth value of the i-th camera is positive. sign_gvd_z_near[ i ] equal to 1 indicates that the sign of the nearest depth value of the i-th camera is negative.

exp_gvd_z_near[ i ] specifies the exponent part of the nearest depth value of the i-th camera. The value of exp_gvd_z_near[ i ] shall be in the range of 0 to 126, inclusive. The value 127 is reserved for future use by ITU T | ISO/IEC. When exp_gvd_z_near[ i ] is equal to 127, the value of zNear[ i ] is unspecified.

man_len_gvd_z_near_minus1[ i ] + 1 specifies the length in bits of the mantissa of the nearest depth value of the i-th camera. The value of man_len_gvd_z_near_minus1 [ i ] shall be in the range of 0 to 31, inclusive.

man_gvd_z_near[ i ] specifies the mantissa part of the nearest depth value of the i-th camera. The length of man_gvd_z_near[ i ] syntax elements is man_len_gvd_z_near_minus1[ i ] + 1 bits.

When exp_gvd_z_near[ i ] is not equal to 127, zNear[ i ] is set equal to binToFp( sign_gvd_z_near_flag[ i ], exp_gvd_z_near[ i ], man_gvd_z_near[ i ], man_len_gvd_z_near_minus1[ i ] + 1 ).

sign_gvd_z_far_flag[ i ] equal to 0 indicates that the sign of the farthest depth value of the i-th camera is positive. sign_gvd_z_far_flag[ i ] equal to 1 indicates that the sign of the farthest depth value of the i-th camera is negative.

exp_gvd_z_far[ i ] specifies the exponent part of the farthest depth value of the i-th camera. The value of exp_gvd_z_far[ i ] shall be in the range of 0 to 126, inclusive. The value 127 is reserved for future use by ITU T | ISO/IEC. When exp_gvd_z_far[ i ] is equal to 127, the value of zFar[ i ] is unspecified.

man_len_gvd_z_far_minus1[ i ] + 1 specifies the length in bits of the mantissa of the farthest depth value of the i-th camera. The value of man_len_gvd_z_far_minus1[ i ] shall be in the range of 0 to 31, inclusive.

man_gvd_z_far[ i ] specifies the mantissa part of the farthest depth value of the i-th camera. The length of man_gvd_z_far[ i ] syntax elements is man_len_gvd_z_far_minus1[ i ] + 1 bits.

When exp_gvd_z_far[ i ] is not equal to 127, zFar[ i ] is set equal to binToFp( sign_gvd_z_far_flag[ i ], exp_gvd_z_far[ i ], man_gvd_z_far[ i ], man_len_gvd_z_far_minus1[ i ] + 1 ).

prec_gvd_focal_length specifies the exponent of the maximum allowable truncation error for focalLengthX[ i ] and focalLengthY[ i ] as given by 2−prec_gvd_focal_length. The value of prec_gvd_focal_length shall be in the range of 0 to 31, inclusive.

prec_gvd_principal_point specifies the exponent of the maximum allowable truncation error for principalPontX[ i ] and principalPointY[ i ] as given by 2−prec_gvd_principal_point. The value of prec_gvd_principal_point shall be in the range of 0 to 31, inclusive.

prec_gvd_rotation_param specifies the exponent of the maximum allowable truncation error for r[ i ][ j ][ k ] as given by 2−pre_gvdc_rotation_param. The value of prec_gvd_rotation_param shall be in the range of 0 to 31, inclusive.

prec_gvd_translation_param specifies the exponent of the maximum allowable truncation error for tX[ i ] as given by 2−prec_gvd_translation_param. The value of prec_gvd_translation_param shall be in the range of 0 to 31, inclusive.

sign_gvd_focal_length_x[ i ] equal to 0 indicates that the sign of the focal length of the i-th camera in the horizontal direction is positive. sign_gvd_focal_length_x[ i ] equal to 1 indicates that the sign of the focal length of the i-th camera in the horizontal direction is negative.

exp_gvd_focal_length_x[ i ] specifies the exponent part of the focal length of the i-th camera in the horizontal direction. The value of exp_gvd_focal_length_x[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU T | ISO/IEC. When exp_gvd_focal_length_x[ i ] is equal to 63, the value of focal length of the horizontal direction for the i-th camera is unspecified.

man_gvd_focal_length_x[ i ] specifies the mantissa part of the focal length of the i-th camera in the horizontal direction. The length v of the man_gvd_focal_length_x[ i ] syntax element is determined as follows:

- If exp_gvd_focal_length_x[ i ] is equal to 0, the length v is set equal to Max( 0, prec_gvd_focal_length − 30 ).

Otherwise (exp_gvd_focal_length_x[ i ] is in the range of 1 to 62, inclusive), the length v is Max( 0, exp_gvd_focal_length_x[ i ] + prec_gvd_focal_length − 31 ).

- When exp_gvd_focal_length_x[ i ] is not equal to 63, the variable focalLengthX[ i ] is set equal to binToFp( sign_gvd_focal_length_x[ i ], exp_gvd_focal_length_x[ i ], man_gvd_focal_length_x[ i ], v ).

sign_gvd_focal_length_y[ i ] equal to 0 indicates that the sign of the focal length of the i-th camera in the vertical direction is positive. sign_gvd_focal_length_y[ i ] equal to 1 indicates that the sign of the focal length of the i-th camera in the vertical direction is negative.

exp_gvd_focal_length_y[ i ] specifies the exponent part of the focal length of the i-th camera in the vertical direction. The value of exp_gvd_focal_length_y[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU T | ISO/IEC. When exp_gvd_focal_length_y[ i ] is equal to 63, the value of focal length of the vertical direction is unspecified.

man_gvd_focal_length_y[ i ] specifies the mantissa part of the focal length of the i-th camera in the vertical direction.

The length v of the man_gvd_focal_length_y[ i ] syntax element is determined as follows:

- If exp_gvd_focal_length_y[ i ] is equal to 0, the length v is set equal to Max( 0, prec_gvd_focal_length − 30 ).

- Otherwise (exp_gvd_focal_length_y[ i ] is in the range of 1 to 62, inclusive), the length v is set equal to Max( 0, exp_gvd_focal_length_y[ i ] + prec_gvd_focal_length − 31 ).

- When exp_gvd_focal_length_y[ i ] is not equal to 63, the variable focalLengthY[ i ] is set equal to binToFp( sign_gvd_focal_length_y[ i ], exp_gvd_focal_length_y[ i ], man_gvd_focal_length_y[ i ], v ).

sign_gvd_principal_point_x[ i ] equal to 0 indicates that the sign of the principal point of the i-th camera in the horizontal direction is positive. sign_gvd_principal_point_x[ i ] equal to 1 indicates that the sign of the principal point of the i-th camera in the horizontal direction is negative.

exp_gvd_principal_point_x[ i ] specifies the exponent part of the principal point of the i-th camera in the horizontal direction. The value of exp_gvd_principal_point_x[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU T | ISO/IEC. When exp_gvd_principal_point_x[ i ] is equal to 63, the value of principal point in the horizontal direction for the i-th camera is unspecified.

man_gvd_principal_point_x[ i ] specifies the mantissa part of the principal point of the i-th camera in the horizontal direction The length v of the man_gvd_principal_point_x[ i ] syntax element in units of bits is determined as follows:

- If exp_gvd_principal_point_x[ i ] is equal to 0, the length v is set equal to Max( 0, prec_gvd_principal_point − 30 ).

- Otherwise (exp_gvd_principal_point_x[ i ] is in the range of 1 to 62, inclusive), the length v is set equal to Max( 0, exp_gvd_principal_point_x[ i ] + prec_gvd_principal_point − 31 ).

When exp_gvd_principal_point_x[ i ] is not equal to 63, the variable principalPointX[ i ] is set equal to binToFp( sign_gvd_principal_point_x[ i ], exp_gvd_principal_point_x[ i ], man_gvd_principal_point_x[ i ], v ).

sign_gvd_principal_point_y[ i ] equal to 0 indicates that the sign of the principal point of the i-th camera in the vertical direction is positive. sign_gvd_principal_point_y[ i ] equal to 1 indicates that the sign of the principal point of the i-th camera in the vertical direction is negative.

exp_gvd_principal_point_y[ i ] specifies the exponent part of the principal point of the i-th camera in the vertical direction. The value of exp_gvd_principal_point_y[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU T | ISO/IEC. When exp_gvd_principal_point_y[ i ] is equal to 63, the value of principal point in the vertical direction for the i-th camera is unspecified.

man_gvd_principal_point_y[ i ] specifies the mantissa part of the principal point of the i-th camera in the vertical direction The length v of the man_gvd_principal_point_y[ i ] syntax element in units of bits is determined as follows:

- If exp_gvd_principal_point_y[ i ] is equal to 0, the length v is set equal to Max( 0, prec_gvd_principal_point − 30 ).

- Otherwise (exp_gvd_principal_point_y[ i ] is in the range of 1 to 62, inclusive), the length v is set equal to Max( 0, exp_gvd_principal_point_y[ i ] + prec_gvd_principal_point − 31 ).

- When exp_gvd_principal_point_y[ i ] is not equal to 63, the variable principalPointY[ i ] is set equal to binToFp( sign_gvd_principal_point_y[ i ], exp_gvd_principal_point_y[ i ], man_gvd_principal_point_y[ i ], v ).

sign_gvd_r[ i ][ j ][ k ] equal to 0 indicates that the sign of (j, k) component of the rotation matrix for the i-th camera is positive. sign_gvd_r[ i ][ j ][ k ] equal to 1 indicates that the sign of (j, k) component of the rotation matrix for the i-th camera is negative.

exp_gvd_r[ i ][ j ][ k ] specifies the exponent part of (j, k) component of the rotation matrix for the i-th camera. The value of exp_gvd_r[ i ][ j ][ k ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU T | ISO/IEC. When exp_gvd_r[ i ][ j ][ k ] is equal to 63, the value of rotation matrix is unspecified.

man_gvd_r[ i ][ j ][ k ] specifies the mantissa part of (j, k) component of the rotation matrix for the i-th camera.

The length v of the man_gvd_r[ i ][ j ][ k ] syntax element in units of bits is determined as follows:

- If exp_gvd_r[ i ] is equal to 0, the length v is set equal to Max( 0, prec_gvd_rotation_param − 30 ).

- Otherwise (exp_gvd_r[ i ] is in the range of 1 to 62, inclusive), the length v is set equal to Max( 0, exp_gvd_r[ i ] + prec_gvd_rotation_param − 31 ).

- When exp_gvd_r[ i ][ j ][ k ] is not equal to 63, the variable r[ i ][ j ][ k ] is set equal to binToFp( sign_gvd_r[ i ][ j ][ k ], exp_gvd_r[ i ][ j ][ k ], man_gvd_r[ i ][ j ][ k ], v ).

The rotation matrix R[ i ] for i-th camera is represented as follows:

r[i][0][0]r[i][0][1]r[i][0][2]r[ i ][ 1 ][ 0 ]r[i][1][1]r[i][1][2]r[ i ][ 2 ][ 0 ]r[i][2][1]r[i][2][2] 	(J-XX)

sign_gvd_t_x[ i ] equal to 0 indicates that the sign of the horizontal component of the translation vector for the i-th camera is positive. sign_gvd_t_x[ i ] equal to 1 indicates that the sign of the horizontal component of the translation vector for the i-th camera is negative.

exp_gvd_t_x[ i ] specifies the exponent part of the horizontal component of the translation vector for the i-th camera. The value of exp_gvd_t_x[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU T | ISO/IEC. When exp_gvd_t_x[ i ] is equal to 63, the value of the translation vector is unspecified.

man_gvd_t_x[ i ] specifies the mantissa part of the horizontal component of the translation vector for the i-th camera.

The length v of the man_gvd_t_x[ i ] syntax element in units of bits is determined as follows:

- If exp_gvd_t_x[ i ] is equal to 0, the length v is set equal to Max( 0, prec_gvd_translation_param − 30 ).

- Otherwise (exp_gvd_t_x[ i ] is in the range of 1 to 62, inclusive), the length v is set equal to Max( 0, exp_gvd_t_x[ i ] + prec_gvd_translation_param − 31 ).

When exp_gvd_t_x[ i ] is not equal to 63, the variable tX[ i ] is set equal to binToFp( sign_gvd_t_x[ i ], exp_gvd_t_x[ i ], man_gvd_t_x[ i ], v ).