| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: San Diego, USA, 22–26 Feb. 2016 | Document: JCT3V-N0021 |
| --- | --- |

| Title: | Centralized Texture-Depth Packing SEI Message for H.264/AVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V – Revision of JCT3V-M0021 |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jar-Ferr Yang Hung-Ming Wang Chiu-Yu Chen Ting-Ann Chang Department of Electrical Engineering, National Cheng Kung University 1 University Road, Tainan, Taiwan 701 | Tel: Email: | +886-6-2757575#62362 jefyang@mail.ncku.edu.tw |
| Source: | National Cheng Kung University (NCKU), 1 University Road, Tainan, Taiwan 701 |  |  |

_____________________________

# Abstract

This contribution modifies the prior contribution, JCT3V-M0021 [1] “Centralized Texture-Depth Packing (CTDP) SEI Message for H.264/AVC”, by removing unprecise syntax terms, polishing the descriptions of syntax structures. In the syntax of H.264/AVC SEI message, quarter_reduced_depth_line_number, which can precisely specify the downsampling factors of depth and texture, is replaced of all the unprecise syntax terms. The selections of quarter_reduced_depth_line_numbers for HD (1920x1080) and UHD (3840x2160) formats are also enlisted as examples. Although the prior study and experiments are conducted based on the HEVC kernel, it can still be applicable to H.264/AVC. Before the available of 3D broadcasting systems, we believe that the proposed CTDP formats with SEI Message could help to deliver 3D videos in the current 2D broadcasting systems simply and efficiently.

# Centralized_texture-depth_packing SEI Message Syntax

In Subclause D.1. - SEI payload syntax, adds “centralized_texture-depth_packing (payloadSize)” :

| else if( payloadType == X ) | Descriptor |
| --- | --- |
| centralized_texture-depth_packing ( payloadSize ) /* specified in Annex D */ |  |

where “X” is an appropriate payloadType number in the sequence of defined payloadType numbers.

Add a new Subclause D.1.X “centralized_texture-depth_packing SEI message syntax”, where. X will be replaced with an appropriate subclause number in the sequence of subclauses present in the text.

D.1.X	Centralized texture-depth packing (CTDP) SEI message syntax

| centralized_texture-depth_packing( payloadSize ) { | Descriptor |
| --- | --- |
| centralized_texture-depth_packing_cancel_flag | u(1) |
| if( ! centralized_texture-depth_packing_cancel_flag ) { |  |
| centralized_texture-depth_packing_content_interpretation_type | u(3) |
| quarter_reduced_depth_line_number | u(10) |
| depth_spatial_flipping_flag | u(1) |
| depth_chroma_sampling_type | u(3) |
| depth_subpixel_arrangement_type | u(2) |
| baseline_dist_flag | u(1) |
| focal_length_flag | u(1) |
| z_near_flag | u(1) |
| z_far_flag | u(1) |
| d_min_flag | u(1) |
| d_max_flag | u(1) |
| depth_representation_type | ue(v) |
| if( baseline_dist_flag ) |  |
| view_syn_rep_info_element( BdistanceSign, BdistanceExp, BdistanceMantissa, BdistanceManLen ) |  |
| if( focal_length_pixel_flag ) |  |
| view_syn_rep_info_element( FlengthSign, FlengthExp, FlengthMantissa, FlengthManLen ) |  |
| if( z_near_flag ) |  |
| view_syn_rep_info_element( ZNearSign, ZNearExp, ZNearMantissa, ZNearManLen ) |  |
| if( z_far_flag ) |  |
| view_syn_rep_info_element ( ZFarSign, ZFarExp, ZFarMantissa, ZFarManLen ) |  |
| if( d_min_flag ) |  |
| view_syn_rep_info_element ( DMinSign, DMinExp, DMinMantissa, DMinManLen ) |  |
| if( d_max_flag ) |  |
| view_syn_rep_info_element ( DMaxSign, DMaxExp, DMaxMantissa, DMaxManLen ) |  |
| if( depth_representation_type = = 3 ) { |  |
| depth_nonlinear_representation_num_minus1 | ue(v) |
| for( i = 1; i <= depth_nonlinear_representation_num_minus1 + 1; i++ ) |  |
| depth_nonlinear_representation_model[ i ] |  |
| centralized_texture-depth_packing_persistence_flag | u(1) |
| } |  |
| } |  |

Add a new Subclause D.2.X “Centralized texture-depth packing SEI message semantics” where X will be replaced with an appropriate subclause number in the sequence of subclauses present in the text. In the following Subclause D.2.X, the numbers of tables, figures, and equations started from Table D-X1, Figure D-X1, D-X1 will be also replaced with an appropriate numbers in the sequence of tables, figures, and equations present in the text, respectively.

- D.2.X Centralized texture-depth packing SEI message semantics

This SEI message informs the decoder that the output cropped decoded picture contains samples of multiple distinct spatially packed constituent frames that are packed into one frame using a centralized texture-depth packing arrangement. This information can be used by the decoder to appropriately rearrange the samples and process the samples of the constituent frames appropriately for display or other purposes (which are outside the scope of this Specification).

When general_non_packed_constraint_flag is equal to 1 in the active SPS for the current layer, there shall be no centralized texture-depth packing SEI message applicable for any picture of the CLVS of the current layer.

When a frame packing arrangement SEI message or a segmented rectangular frame packing arrangement SEI message is applicable for any picture of the CLVS of the current layer, there shall be no centralized texture-depth packing SEI messages applicable for any picture of the CLVS of the current layer.

centralized_texture-depth_packing_cancel_flag equal to 1 indicates that the centralized texture-depth packing arrangement SEI message cancels the persistence of any previous the centralized texture-depth packing in output order. centralized_texture-depth_packing_cancel_flag equal to 0 indicates that indicates that centralized texture-depth packing follows.

centralized_texture-depth_packing_content_interpretation_type indicates the intended interpretation of the constituent frames as specified in Table D-X1.

Table D-X1 – Definition of centralized_texture-depth_packing_content_interpretation_type

| Value | Interpretation |
| --- | --- |
| 0 | Indicates the CTDP-1TB type that the CTDP packed frame, which provides one texture-plus-depth view, is arranged in the order of reduced_depth_top (RDT), resized_texture (RT), and reduced_depth_bottom (RDB) regions from top to bottom as illustrated in Figure D-X1. |
| 1 | Indicates the CTDP-1LR type that the CTDP packed frame is arranged in the order of reduced_depth_left (RDL), resized_texture (RT), and reduced_depth_right (RDR) regions from left to right as illustrated in Figure D-X2. |
| 2 | Indicates the CTDP-2TB type that the CTDP packed frame, which provides two texture-plus-depth views, is arranged in the order of reduced_depth_leftview_top (RDLT), resized_texture_leftview (RTL), and reduced_depth_leftview_bottom (RDLB) regions in the left-half frame and reduced_depth_rightview_top (RDRT), resized_texture_rightview (RTR), and reduced_depth_rightview_bottom (RDRB) regions in the right-half frame from top to bottom as illustrated in Figure D-X3. |
| 3 | Indicates the CTDP-2LR type that the CTDP packed frame, which provides two texture-plus-depth views, is arranged in the order of reduced_depth_topview_left (RDTL), resized_texture_topview (RTT), and reduced_depth_topview_right (RDTR) regions in the top-half frame and reduced_depth_bottomview_left (RDBL), resized_texture_bottomview (RTB), and reduced_depth_bottomview_right (RDBR) regions in the bottom-frame from left to right as illustrated in Figure D-X4. |

Values of centralized_texture-depth_packing_content_interpretation_type that do not appear in Table D-X1 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore centralized texture-depth packing SEI messages that contain reserved values of centralized_texture-depth_packing_content_interpretation_type.

quarter_reduced_depth_line_number indicates the half number of horizontal lines of the reduced depth top and reduced depth bottom regions if centralized_texture-depth_packing_content_interpretation_type is equal to 0 or 2. The quarter_reduced_depth_line_number defines the half number of vertical lines of the reduced depth left and right regions if centralized_texture-depth_packing_content_interpretation_type is equal to 1 or 3. The detailed descriptions of sizes of reduced depth and reduced texture regions according to centralized_texture-depth_packing_content_interpretation_type are stated as follows.

Let W and H be the width and height as pic_width_in_luma_samples and pic_height_in_luma_samples, respectively, of the decoded frame output from the decoder in units of luma samples.

If centralized_texture-depth_packing_content_interpretation_type is equal to 0 and 2, which respectively indicate CTDP-TB and CTDP-2TB packing types, the height of top and bottom half depth regions, RDT and RDB shown in Figure D-X1 and RDLT, RDRT, RDLB, and RDRB shown in Figure D-X3 is given as

HHD = 2*quarter_reduced_depth_line_number.                                      (D-X1)

The height of RT region shown in Figure D-X1 and RTL and RTR shown in Figure D-X3 is specified in units of frame luma samples as:

HRT = H  HHD *2.		(D-X2)

The guidelines for selecting the above quarter_reduced_depth_line_number are: 1) to be a divisible factor of H such that it will result in simple downsample and upsaming VLSI implementations; 2) to be less than H/16 such that the downsampling factor of texture frame is less than that of depth map to achieve good packing quality. For UHD (3840x2160) and HD (1920x1080) video formats for examples, the possible selections of horizontal quarter_reduced_depth_line_numbers and the corresponding HHD and HRT, which result in texture and depth vertical downsampling factors, are enlisted in Table DX-2.

Table D-X2 – Selections of horizontal quarter_reduced_depth_line_number for UHD and HD video formats

| Video Formats | quarter_reduced_ depth_ line_ number | HHD | HRT | Texture Vertical Downsampling factor | Depth Vertical Downsampling factor |
| --- | --- | --- | --- | --- | --- |
| UHD (H=2160) | 90 | 180 | 1800 | 5:6 | 1:2 |
|  | 60 | 120 | 1920 | 8:9 | 1:3 |
|  | 45 | 90 | 1980 | 11:12 | 1:4 |
| HD (H=1080) | 45 | 90 | 900 | 5:6 | 1:2 |
|  | 30 | 60 | 960 | 8:9 | 1:3 |

If centralized_texture-depth_packing_content_interpretation_type is equal to 1 and 3, which respectively indicate CTDP-LR and CTDP-2LR packing types, the width of the left and right half depth regions, RDL and RDR shown in Figure D-X2 and RDTL, RDTR, RDBL and RDBR shown in Figure D-X4 is given as,

WHD = 2*quarter_reduced_depth_line_number.                                       (D-X3)

The width of RT region shown in Figure D-X2 and RTT and RTB shown in Figure D-X4 in units of frame luma samples is specified as:

WRT = W WHD *2.		(D-X4)

The guidelines for determining the above quarter_reduced_depth_line_number are: 1) to be a divisible factor of W such that it will result in simple downsample and upsampling VLSI implementations; 2) to be less than W/16 such that the downsampling factor of texture frame is less than that of depth map to achieve good packing quality. For UHD (3840x2160) and HD (1920x1080) video formats as examples, the possible selections of vertical quarter_reduced_depth_line_numbers and the corresponding HHD and HRT, which result in texture and depth horizontal downsampling factors, are enlisted in Table DX-3.

Table D-X3 – Selections of vertical quarter_reduced_depth_line_number for UHD and HD video formats

| Video Formats | quarter_reduced_ depth_ line_ number | WHD | WRT | Texture Horizontal Downsampling factor | Depth Horizontal Downsampling factor |
| --- | --- | --- | --- | --- | --- |
| UHD (W=3840) | 160 | 320 | 3200 | 5:6 | 1:2 |
|  | 120 | 240 | 3360 | 8:9 | 1:3 |
|  | 80 | 160 | 3520 | 11:12 | 1:4 |
| HD (W=1920) | 80 | 160 | 1600 | 5:6 | 1:2 |
|  | 60 | 120 | 1680 | 8:9 | 1:3 |
|  | 40 | 80 | 1760 | 11:12 | 1:4 |

- When centralized_texture-depth_packing_content_interpretation_type is equal to 0, the centralized texture-depth packing frame is composed of three-resized rectangular regions identified as RDT, RT and RDB as illustrated in Figure D-X1. The height of RDT and RDB regions is specified as HHD in units of frame luma samples and the height of RT region is specified in units of frame luma samples as HRT. The width of RDT, RT and RDB regions is W in units of frame luma samples.

Followed the 7 reconstruction steps of texture and depth information stated in Subclause D.5.X, as illustrated in Figure D-X1, the CTDP frame is depacked to the depacked texture (T) and depacked depth (D), whose width and height are finally recovery back to W and H in units of frame luma samples, respectively.

Figure D-X1 Arrangement of the CTDP-1TB frame and its depacking steps if centralized_texture-depth_packing_content_interpretation_type is equal to 0

Figure D-X2 Arrangement of the CTDP-1LR frame and its depacking steps if centralized_texture-depth_packing_content_interpretation_type is equal to 1

- When centralized_texture-depth_packing_content_interpretation_type is equal to 1, the centralized texture-depth packing frame is composed of three-resized rectangular regions identified as RDL, RT, and RDR as illustrated in Figure D-X2. The width of RDL and RDR regions is specified as WHD in units of frame luma samples and the width of RT region in units of frame luma samples is specified as WRT. The height of RDL, RT and RDR regions is H in units of frame luma samples.

Followed the 7 reconstruction steps of texture and depth information stated in Subclause D.5.X, as illustrated in Figure D- X2, the CTDP frame is depacked to the depacked texture (T) and depacked depth (D), whose width and height are finally recovery back to W and H in units of frame luma samples, respectively.

- When centralized_texture-depth_packing_content_interpretation_type is equal to 2, the centralized texture-depth packing frame is composed of six-resized rectangular regions identified as RDLT, RDRT, RTL, RTR, RDLB, and RDRB as illustrated in Figure D-X3. The height of RDLT, RDRT, RDLB, and RDRB regions is specified as HHD in units of frame luma samples and the height of RTL and RTR regions in units of frame luma sample is specified as HRT. The width of RDLT, RTL, RDLB, RDRT, RTR and RDRB regions is W/2 in units of frame luma samples.

Followed the 7 reconstruction steps of texture and depth information stated in Subclause D.5.X, as illustrated in Figure D-X3, the CTDP frame is depacked to the depacked left texture (TL), the depacked right texture (TR), the depacked left depth (DL) and the depacked right depth (DR), whose width and height are finally recovery back to W and H in units of frame luma samples, respectively.

Figure D-X3 Arrangement of the CTDP-2TB frame and its depacking steps if centralized_texture-depth_packing_content_interpretation_type is equal to 2

- When centralized_texture-depth_packing_content_interpretation_type is equal to 3, the centralized texture-depth packing frame is composed of six-resized rectangular regions identified as RDTL, RDTR, RTT, RTB, RDBL, and RDBR as illustrated in Figure D-X4. The width of RDTL, RDTR, RDBL, and RDBR regions is specified as WHD in units of frame luma samples and The width of RTT and RTB regions in units of frame luma samples is specified as WRT. The height of RDTL, RTT, RDTR, RDBL, RTB and RDBR regions is H/2 in units of frame luma samples.

Followed the 7 reconstruction steps of texture and depth information stated in Subclause D.5.X, as illustrated in Figure D-X4, the CTDP frame is depacked to the depacked top texture (TT), the depacked bottom texture (TB), the depacked top depth (DT) and the depacked bottom depth (DB), whose width and height are finally recovery back to W and H in units of frame luma samples, respectively.

Figure D-X4 Arrangement of the CTDP-2LR frame and its depacking steps if centralized_texture-depth_packing_content_interpretation_type is equal to 3

For centralized_texture-depth_packing_content_interpretation_type equal to 0, 1, 2, or 3 and quarter_reduced_depth_line_number, texture and depth frames that are arranged by the specific order.

When centralized_texture-depth_packing_content_interpretation_type is not equal to 0, 1, 2, or 3, quarter_reduced_depth_line_number is reserved for future use by ITU-T | ISO/IEC. When centralized_texture-depth_packing_content_interpretation_type is not equal to 0, 1, 2, or 3, decoders shall ignore the value of quarter_reduced_depth_line_number.

depth_spatial_flipping_flag indicates that the reduced depth region are spatially flipped relative to its intended orientation for display or other such purposes.

When depth_spatial_flipping_flag is equal to 1, the type of depth spatial flipping that is indicated is as follows:

–	If centralized_texture-depth_packing_content_interpretation_type is equal to 0, the indicated depth spatial flipping is vertical flipping. The depth spatial vertical flipping process for each color component with Y, Cb, and Cr is given as:

for( x = 0; x < W; x++ )

for( y = 0; y < HHD; y++ )

{

tRDT[x][y] = RDT[x][HHD −y−1]

tRDB[x][y] = RDB[x][HHD −y−1]

}

where W and HHD are the width and the height of RDT and RDB regions of the decoded picture in samples in Figure D-X1, respectively. RDT[x][y], RDB[x][y] and tRDT[x][y], tRDB[x][y] are the sample values at the location (x, y) before and after the vertical flipping of the decoded picture, respectively. It is noted that the tRDT and tRDB are the temporary memories to store the sample values after the flipping. Finally, the temporary memories are restored as RDT = tRDT, RDB = tRDB.

–	If centralized_texture-depth_packing_content_interpretation_type is equal to 1, the indicated depth spatial flipping is horizontal flipping. The depth spatial horizontal flipping process for each component with Y, Cb, and Cr is given as:

for( x = 0; x < WHD; x++ )

for( y = 0; y < H; y++ )

{

tRDL[x][y] =RDL[WHD −x−1][y]

tRDR[x][y] = RDR[WHD −x−1][y]

}

where WHD and H are the width and the height of RDL and RDR regions of the decoded picture in samples in Figure D-X2, respectively. RDL[x][y], RDR[x][y] and tRDL[x][y], tRDR[x][y] denote the sample values at the location (x, y) before and after the horizontal flipping, respectively. It is noted that the tRDL and tRDR are the temporary memories to store the sample values after the flipping. Finally, the temporary memories are restored as RDL = tRDL and RDR = tRDR.

–	If centralized_texture-depth_packing_content_interpretation_type is equal to 2, the indicated depth spatial flipping is vertical flipping. The depth spatial vertical flipping process for each component with Y, Cb, and Cr is given as:

for( x = 0; x < W/2; x++ )

for( y = 0; y < HHD; y++ )

{

tRDLT[x][y] = RDLT[x][HHD −y−1]

tRDRT[x][y] = RDRT[x][HHD −y−1]

tRDLB[x][y] = RDLB[x][HHD −y−1]

tRDRB[x][y] = RDRB[x][HHD −y−1]

}

where HHD and W/2 are the height and the width of RDLT, RDRT, RDLB and RDRB regions of the decoded picture in samples in Figure D-X3, respectively. RDLT[x][y], RDRT[x][y], RDLB[x][y], RDRB[x][y] and tRDLT[x][y], tRDRT[x][y], tRDLB[x][y], tRDRB[x][y] are the sample values at the location (x, y) before and after the vertical flipping, respectively. It is noted that the tRDLT, tRDRT, tRDLB, and tRDRB are the temporary memories to store the sample values after the flipping. Finally, the temporary memories are restored as RDLT = tRDLT, RDRT = tRDRT, RDLB = tRDLB and RDRB = tRDRB.

–	If centralized_texture-depth_packing_content_interpretation_type is equal to 3, the indicated depth spatial flipping is horizontal flipping. The depth spatial horizontal flipping process for each component with Y, Cb, and Cr is given as:

for( x = 0; x < WHD; x++ )

for( y = 0; y < H/2; y++ )

{

tRDTL[x][y] =RDTL[WHD −x−1][y]

tRDTR[x][y] =RDTR[WHD −x−1][y]

tRDBL[x][y] = RDBL[WHD −x−1][y]

tRDBR[x][y] = RDBR[WHD −x−1][y]

}

where WHD and H/2 are the width and the height of RDTL, RDTR, RDBL and RDBR regions of the decoded picture in samples in Figure D-X4, respectively. RDLT[x][y], RDRT[x][y], RDLB[x][y], RDRB[x][y] and tRDLT[x][y], tRDRT[x][y], tRDLB[x][y], tRDRB[x][y] are the sample values at the location (x, y) before and after the vertical flipping, respectively. Finally, the temporary memories are restored as RDTL = tRDTL, RDTR = tRDTR, RDBL = tRDBL and RDBR = tRDBR.

–	Otherwise (depth_spatial_flipping_flag is equal to 0), the indicated spatial flipping is not necessary.

After spatial flipping (depth_spatial_flipping_flag is equal to 0) or no spatial flipping (depth_spatial_flipping_flag is equal to 0), two corresponding reduced depth regions should be merged togther to become one color reduced depth region.

–	If centralized_texture-depth_packing_content_interpretation_type is equal to 0, the top and bottom reduced depth regions for each color component with Y, Cb, and Cr are vertically merged as:

for( x = 0; x < W; x++ )

for( y = 0; y < HHD; y++ )

{

CRD [x][y] = RDT[x][ y]

CRD [x][ HHD +y] = RDB[x][ y]

}

where W and HHD are the width and the height of RDT and RDB regions.

–	If centralized_texture-depth_packing_content_interpretation_type is equal to 1, the left and right reduced regions for each component with Y, Cb, and Cr are horizontally merged as:

for( x = 0; x < WHD; x++ )

for( y = 0; y < H; y++ )

{

CRD [x][y] =RDL[x][y]

CRD[x+WHD ][y] = RDR[x][y]

}

where WHD and H are the width and the height of RDL and RDR regions.

–	If centralized_texture-depth_packing_content_interpretation_type is equal to 2, the top and bottom reduced depth regions of left and right views for each component with Y, Cb, and Cr are vertically merged as:

for( x = 0; x < W/2; x++ )

for( y = 0; y < HHD; y++ )

{

CRDL[x][y] = RDLT[x][ y]

CRDL[x][y+ HHD] = RDLB[x][y]

CRDR[x][y] = RDRT[x][y]

CRDR[x][y+ HHD] = RDRB[x][y]

}

where HHD and W/2 are the height and the width of RDLT, RDRT, RDLB and RDRB regions

–	If centralized_texture-depth_packing_content_interpretation_type is equal to 3, the left and right reduced depth regions of top and bottom views for each component with Y, Cb, and Cr are horizontally merged as::

for( x = 0; x < WHD; x++ )

for( y = 0; y < H/2; y++ )

{

CRDT[x][y] =RDTL[x][y]

CRDT[x+ WHD][y] =RDTR[x][y]

CRDB[x][y] = RDBL[x][y]

CRDB[x+ WHD][y] = RDBR[x][y]

}

where WHD and H/2 are the width and the height of RDTL, RDTR, RDBL and RDBR regions.

When centralized_texture-depth_packing_content_interpretation_type is not equal to 0, 1, 2, or 3, it is a requirement of bitstream conformance that depth_spatial_flipping_flag shall be equal to 0. When centralized_texture-depth_packing_content_interpretation_type is not equal to 0, 1, 2, or 3, the value 0 or 1 for depth_sampling_factor is reserved for future use by ITU-T | ISO/IEC. When centralized_texture-depth_packing_content_interpretation_type is not equal to 0, 1, 2, or 3, decoders shall ignore the value 1 for depth_spatial_flipping_flag.

depth_chroma_sampling_type indicates that the depth chroma sampling method from 444 to 422 or 420 format is used by the CTDP packer as specified in Table D-X4. According to depth_chroma_sampling_type, YCbCr colour 422 or 420 format is converted to YCbCr colour 444 format.

Table D-X4– Definition of depth_chroma_sampling_type

| Value | Interpretation |
| --- | --- |
| 0 | CTDP depacker is informed that the direct sample is used to convert the chroma sample from 444 to 422 or 420 format according to marked chroma sample location.. |
| 1 | CTDP depacker is informed that the mean of two (four) samples is used to convert the chroma sample from 444 to 422 (420) format. |

Values of depth_chroma_sampling_type that do not appear in Table D-X4 are reserved for future specification by ITU-T |ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore centralized texture-depth packing SEI messages that contain reserved values of depth_chroma_sampling_type.

The YCbCr colour plane of the CRD, CRDL, CRDR, CRDT, or CRDB is converted to the RGB colour plane of the the CRD, CRDL, CRDR, CRDT, or CRDB by using inverse colour format conversion by

RGB=1.1644−0.00011.59601.1644−0.3917−0.8131.16442.0173−0.0001YCbCr−16128128.                        	(D-X5)

depth_subpixel_arrangement_type indicates that the arrangement order of R, G, and B channels of a color reduced depth region are rearranged into three depth pixels of resized depth region.

–	When depth_subpixel_arrangement_type is equal to 0 indicates that R, G, and B channels of the reduced depth region are rearranged into three depth pixels of the resized depth region by using the sequence order specified in Figure D-X5.

if centralized_texture-depth_packing_content_interpretation_type is equal to 0 or 2, one horizontal RGB-channel line of color reduced depth (CRD) frame is expanded to three horizontal single-channel lines of resized depth (RD) frame such that the height, H’ (2HHD) will be increased to 3 times, 3H’ (6HHD), where the color reduced depth regions could be CRD and CRDL and CRDR regions and the reduced depth regions could be RD and RDL and RDR regions specified in Figure D-X1 and Figure D-X3, respectively.

if centralized_texture-depth_packing_content_interpretation_type is equal to 1 or 3, one vertical line of color reduced depth (CRD) region is expanded to three vertical lines of resized depth (RD) region such that the width WHD will be increased to 3 times, 6WHD, where the reduced depth region could be CRD and CRDT and CRDB regions and the reduced depth regions could be RD and RDT and RDB regions specified in Figure D-X2 and Figure D-X4, respectively.

Figure D-X5. RGB-channels of a W’× H’ color reduced depth region are arranged to three single-channels of resized depth in sequential order if depth_subpixel_arrangement_type equal to 0

–	When depth_subpixel_arrangement_type is equal to 1 indicates that R, G, and B channels of the color reduced depth (CRD) region are rearranged into three depth pixels of the resized depth (RD) region by using the rotation order specified in Figure D-X6.

if centralized_texture-depth_packing_content_interpretation_type is equal to 0 or 2, one horizontal RGB-channel line of color reduced depth (CRD) frame is expanded to three horizontal single-channel lines of resized depth (RD) frame such that the height, H’ (2HHD) will be increased to 3 times, 3H’ (6HHD), where the color reduced depth region could be CRD and CRDL and CRDR regions and the reduced depth region could be RD and RDL and RDR regions specified in Figure D-X1 and Figure D-X3, respectively.

if centralized_texture-depth_packing_content_interpretation_type is equal to 1 or 3, one vertical line of the color reduced depth (CRD) region is expanded to three vertical lines of the resized depth (RD) region such that the width W’ (2WHD) will be increased to 3 times, 3W’ (6WHD), where the color reduced depth region could be CRD and CRDT and CRDB regions and the reduced depth regions could be RD and RDT and RDB regions specified in Figure D-X2 and Figure D-X4, respectively.

Figure D-X6 RGB-channels of a W’× H’ reduced depth region are arranged to three single-channels of resized depth in rotation order if depth_subpixel_arrangement_type equal to 1

baseline_dist_flag equal to 0 specifies that the syntax elements specifying the baseline distance are not present in the syntax structure. baseline_dist_flag equal to 1 specifies that the syntax elements specifying the baseline distance value are present in the syntax structure.

focal_length_flag equal to 0 specifies that the syntax elements specifying the focal length are not present in the syntax structure. focal_length_pixel_flag equal to 1 specifies that the syntax elements specifying the focal length value are present in the syntax structure.

z_near_flag equal to 0 specifies that the syntax elements specifying the nearest depth value are not present in the syntax structure. z_near_flag equal to 1 specifies that the syntax elements specifying the nearest depth value are present in the syntax structure.

z_far_flag equal to 0 specifies that the syntax elements specifying the farthest depth value are not present in the syntax structure. z_far_flag equal to 1 specifies that the syntax elements specifying the farthest depth value are present in the syntax structure.

d_min_flag equal to 0 specifies that the syntax elements specifying the minimum disparity value are not present in the syntax structure. d_min_flag equal to 1 specifies that the syntax elements specifying the minimum disparity value are present in the syntax structure.

d_max_flag equal to 0 specifies that the syntax elements specifying the maximum disparity value are not present in the syntax structure. d_max_flag equal to 1 specifies that the syntax elements specifying the maximum disparity value are present in the syntax structure.

depth_representation_type specifies the representation definition of decoded luma samples of auxiliary pictures as specified in Table D-X5. In Table D-X5, disparity specifies the horizontal displacement between two texture views and Z value specifies the distance from a camera.

The variable maxVal is set equal to ( 1 << ( 8 + bit_depth_luma_minus8 ) ) − 1, where bit_depth_luma_minus8 is the value included in or inferred for the active SPS of the layer with nuh_layer_id equal to targetLayerId.

Table D-X5– Definition of depth_representation_type

| depth_representation_type | Interpretation |
| --- | --- |
| 0 | Each decoded luma sample value of an auxiliary picture represents an inverse of Z value that is uniformly quantized into the range of 0 to maxVal, inclusive. When z_far_flag is equal to 1, the luma sample value equal to 0 represents the inverse of ZFar (specified below). When z_near_flag is equal to 1, the luma sample value equal to maxVal represents the inverse of ZNear (specified below). |
| 1 | Each decoded luma sample value of an auxiliary picture represents disparity that is uniformly quantized into the range of 0 to maxVal, inclusive. When d_min_flag is equal to 1, the luma sample value equal to 0 represents DMin (specified below). When d_max_flag is equal to 1, the luma sample value equal to maxVal represents DMax (specified below). |
| 2 | Each decoded luma sample value of an auxiliary picture represents a Z value uniformly quantized into the range of 0 to maxVal, inclusive. When z_far_flag is equal to 1, the luma sample value equal to 0 corresponds to ZFar (specified below). When z_near_flag is equal to 1, the luma sample value equal to maxVal represents ZNear (specified below). |
| 3 | Each decoded luma sample value of an auxiliary picture represents a nonlinearly mapped disparity, normalized in range from 0 to maxVal, as specified by depth_nonlinear_representation_num_minus1 and depth_nonlinear_representation_model[i]. When d_min_flag is equal to 1, the luma sample value equal to 0 represents DMin (specified below). When d_max_flag is equal to 1, the luma sample value equal to maxVal represents DMax (specified below). |
| Other values | Reserved for future use |

The variables in the x column of Table D-X6 are derived from the respective variables in the s, e, n and v columns of Table D-X6 as follows:

–     If the value of e is in the range of 0 to 127, exclusive, x is set equal to ( −1 )s* 2e - 31 * ( 1 + n ÷ 2v ).

–     Otherwise (e is equal to 0), x is set equal to ( −1 )s * 2−( 30 + v ) * n.

NOTE 1 – The above specification is similar to that found in IEC 60559:1989, Binary floating-point arithmetic for microprocessor systems.

Table D-X6– Association between depth parameter variables and syntax elements

| x | s | e | n | v |
| --- | --- | --- | --- | --- |
| Bdistance | BdistanceSign | BdistanceExp | BdistanceMantissa | BdistanceManLen |
| Flength | FlengthSign | FlengthExp | FlengthMantissa | FlengthManLen |
| ZNear | ZNearSign | ZNearExp | ZNearMantissa | ZNearManLen |
| ZFar | ZFarSign | ZFarExp | ZFarMantissa | ZFarManLen |
| DMax | DMaxSign | DMaxExp | DMaxMantissa | DMaxManLen |
| DMin | DMinSign | DMinExp | DMinMantissa | DMinManLen |

The DMin and DMax values, when present, are specified in units of a luma sample width of the coded picture.

The units for the ZNear and ZFar, when present, are identical but unspecified.

When depth_representation_type is equal to 0 or 2, the disparity D can further be obtained from the value of Z, Bdistance, and Flength.

D = Bdistance × Flength ÷ Z

depth_nonlinear_representation_num_minus1 plus 2 specifies the number of piecewise linear segments for mapping of depth values to a scale that is uniformly quantized in terms of disparity.

depth_nonlinear_representation_model[ i ] for i ranging from 0 to depth_nonlinear_representation_num_minus1 + 2, inclusive, specify the piecewise linear segments for mapping of decoded luma sample values of an auxiliary picture to a scale that is uniformly quantized in terms of disparity. The values of depth_nonlinear_representation_model[0] and depth_nonlinear_representation_model[ depth_nonlinear_representation_num_minus1 + 2 ] are both inferred to be equal to 0.

NOTE 2 – When depth_representation_type is equal to 3, an auxiliary picture contains nonlinearly transformed depth samples. The variable DepthLUT[ i ], as specified below, is used to transform decoded depth sample values from the nonlinear representation to the linear representation, i.e., uniformly quantized disparity values. The shape of this transform is defined by means of line-segment approximation in two-dimensional linear-disparity-to-nonlinear-disparity space. The first(0, 0) and the last ( maxVal, maxVal ) nodes of the curve are predefined. Positions of additional nodes are transmitted in form of deviations (depth_nonlinear_representation_model[ i ]) from the straight-line curve. These deviations are uniformly distributed along the whole range of 0 to maxVal, inclusive, with spacing depending on the value of nonlinear_depth_representation_num_minus1.

The variable DepthLUT[ i ] for i in the range of 0 to maxVal, inclusive, is specified as follows:

for( k = 0; k <= depth_nonlinear_representation_num_minus1 + 1; k++ ) {

pos1 = ( maxVal * k ) / (depth_nonlinear_representation_num_minus1 + 2 )

dev1 = depth_nonlinear_representation_model[ k ]

pos2 = ( maxVal * ( k + 1 ) ) / (depth_nonlinear_representation_num_minus1 + 2 )

dev2 = depth_nonlinear_representation_model[ k + 1 ] (G-10)

x1 = pos1 − dev1

y1 = pos1 + dev1

x2 = pos2 − dev2

y2 = pos2 + dev2

for( x = Max( x1, 0 ); x <= Min( x2, maxVal ); x++ )

DepthLUT[ x ] = Clip3( 0, maxVal, Round( ( ( x − x1 ) * ( y2 − y1 ) ) ÷ ( x2 − x1 ) + y1 ) )

}

When depth_representation_type is equal to 3, DepthLUT[ dS ] for all decoded luma sample values dS of an auxiliary picture in the range of 0 to maxVal, inclusive, represents disparity that is uniformly quantized into the range of 0 to maxVal, inclusive.

centralized_texture_depth_packing_persistence_flag specifies the persistence of the centralized texture-depth packing arrangement SEI message.

centralized_texture-depth_packing_persistence_flag equal to 0 specifies that the centralized texture-depth packing arrangement SEI message applies to the current decoded frame only.

centralized_texture-depth_packing_persistence_flag equal to 1 specifies that the centralized texture-depth packing arrangement SEI message persists in output order until any of the following conditions are true:

–	A new CVS begins.

–	The bitstream ends.

–	A frame in an access unit containing a centralized texture-depth packing SEI message with the same

- D.3.X	View synthesis representation information element syntax

| view_syn_rep_info_element( OutSign, OutExp, OutMantissa, OutManLen ) { | Descriptor |
| --- | --- |
| da_sign_flag | u(1) |
| da_exponent | u(7) |
| da_mantissa_len_minus1 | u(5) |
| da_mantissa | u(v) |
| } |  |

The syntax structure specifies the value of an element in the view synthesis representation information SEI message.

The syntax structure sets the values of the OutSign, OutExp, OutMantissa and OutManLen variables that represent a floating-point value. When the syntax structure is included in another syntax structure, the variable names OutSign, OutExp, OutMantissa and OutManLen are to be interpreted as being replaced by the variable names used when the syntax structure is included.

da_sign_flag equal to 0 indicates that the sign of the floating-point value is positive. da_sign_flag equal to 1 indicates that the sign is negative. The variable OutSign is set equal to da_sign_flag.

da_exponent specifies the exponent of the floating-point value. The value of da_exponent shall be in the range of 0 to 27 − 2, inclusive. The value 27 − 1 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 27 − 1 as indicating an unspecified value. The variable OutExp is set equal to da_exponent.

da_mantissa_len_minus1 plus 1 specifies the number of bits in the da_mantissa syntax element. The value of da_mantissa_len_minus1 shall be in the range of 0 to 31, inclusive. The variable OutManLen is set equal to da_mantissa_len_minus1 + 1.

da_mantissa specifies the mantissa of the floating-point value. The variable OutMantissa is set equal to da_mantissa.

- D.4.X Reconstruction of texture and depth information from the CTDP frame

The following steps describe the reconstruction of texture and depth information from CTDP frame by applying the centralized texture-depth packing SEI message syntax and semantics:

If the current decoded picture is not 4:4:4 colour YCbCr format, it should be converted to 4:4 4 colour YCbCr format. Let W and H be the width and height as pic_width_in_luma_samples and pic_height_in_luma_samples, respectively, of the decoded frame output from the decoder in units of luma and chroma samples.

When centralized_texture-depth_packing_content_interpretation_type equal to 0.

- According to quarter_reduced_depth_line_number, the RDT, RT, and RDB regions from top to bottom in Figure D-X1 are divided from the decoded CTDP TB frame by horizontal splitting.

- The texture (T) frame is vertically upsampled from the reduced texture (RT) region by a factor of H:HRT, where HRT stated in (D-X3) is determined by quarter_reduced_depth_line_number.

- The color reduced depth (CRD) region is reconstructed by the combination of two, RDT and RDB, reduced depth top and bottom regions if depth_spatial_flipping_flag equal to 0 or the combination of vertical spatial flips of RDT and RDB regions if depth_spatial_flipping_flag equal to 1. The CRD region is merged in order of RDT, and RDB from top to bottom.

- According to depth_chroma_sampling_type, YCbCr colour 422 or 420 format is converted to YCbCr colour 444 format in the color reduced depth (CRD) region.

- The YCbCr colour plane of the color reduced depth (CRD) frame is converted to the RGB colour plane by using inverse colour format conversion defined in (D-X7).

- According to depth_subpixel_arrangement_type, the three depth pixels in the vertical direction of resized depth (RD) region are rearranged from one color depth pixel of the CRD region with R, G, B subpixel channels.

- The depth (D) frame is upsampled in the vertical direction from the reduced depth (RD) region with a factor of H:6HHD, where HHD stated in (D-X1) is determined by quarter_reduced_depth_line_number.

When centralized_texture-depth_packing_content_interpretation_type equal to 1.

- According to quarter_reduced_depth_line_number, the RDL, RT, and RDR regions from left to right in Figure D-X2 are divided from the decoded CTDP LR frame by vertical splitting.

- The texture (T) frame is horizontally upsampled from reduced texture (RT) region by a factor of W:WRT, where WRT stated in (D-X6) is determined by quarter_reduced_depth_line_number.

- The color reduced depth (CRD) region is reconstructed by the combination of two, RDL and RDR reduced depth left and right regions if depth_spatial_flipping_flag equal to 0 or the combination of horizontal spatial flips of RDL and RDR regions if depth_spatial_flipping_flag equal to 1. The CRD region is merged in order of RDL, and RDR from left to right.

- According to depth_chroma_sampling_type, YCbCr colour 422 or 420 format is converted to YCbCr colour 444 format in the color reduced depth (CRD) region.

- The YCbCr colour plane of the color reduced depth (CRD) region is converted to the RGB colour plane by using inverse colour format conversion stated in (D-X7).

- According to depth_subpixel_arrangement_type, the three depth pixels in the horizontal direction of the resized depth (RD) region are rearranged from one color depth pixel of the color reduced depth (CRD) frame with R, G, B subpixel channels.

- The depth (D) frame is upsampled in the horizontal direction from the resized depth (RD) region with a factor of W:6WHD, where WHD stated in (D-X2) is determined by quarter_reduced_depth_line_number.

When centralized_texture-depth_packing_content_interpretation_type equal to 2.

- According to quarter_reduced_depth_line_number, the RDLT, RDRT, RTL, RTR, RDLB, and RDRB regions in Figure D-X3 are extracted from the decoded picture by vertical and horizontal splitting.

- The leftview and rightview textures, TL and TR are respectively upsampled from reduced texture leftview and rightview regions, RTL and RTR with a factor of H:HRT in the vertical direction and a factor of 2:1 in the horizontal direction, where HRT stated in (D-X5) is determined by quarter_reduced_depth_line_number.

- The color depth leftview and rightview regions CRDL and CRDR are respectively reconstructed by combinations of RDLT and RDLB regions and RDRT and RDRB regions if depth_spatial_flipping_flag equal to 0 or the combinations of vertical spatial flips of RDLT and RDLB regions and RDRT and RDRB regions if depth_spatial_flipping_flag equal to 1. The color depth leftview and rightview regions, CRDL and CRDR are reconstructed in order of RDLT and RDLB regions and RDRT and RDRB regions from top to bottom, respectively.

- According to depth_chroma_sampling_type, YCbCr colour 422 or 420 format is converted to YCbCr colour 444 format in the color depth leftview and rightview regions, CRDL and CRDR.

- The YCbCr colour planes of the color depth leftview and rightview regions, CRDL and CRDR are converted into RGB colour planes by using inverse color format conversion stated in (D-X7).

- According to depth_subpixel_arrangement_type, the three depth pixels in the vertical direction of the reduced depth leftview and rightview regions, RDL and RDR are rearranged from one color depth pixel of the CRDL and CRDR regions with R, G, B subpixel channels.

- The depth leftview and rightview frames, DL and DR are respectively upsampled from the reduced depth leftview and rightview regions, RDL and RDR by a factor of H:6HHD in vertical direction and a factor of 2:1 in horizontal direction, where HHD stated in (D-X1) is determined by quarter_reduced_depth_line_number.

When centralized_texture-depth_packing_content_interpretation_type equal to 3.

- According to quarter_reduced_depth_line_number the RDTL, RDBL, RTT, RTB, RDTR, and RDBR regions in Figure D-X4 are obtained from the decoded CTDP 2LR frame by horizontal and vertical splitting.

- The texture topview and bottomview frames, TT and TB are respectively upsampled from reduced texture topview and bottomview regions, RTT and RTB and by a factor of W:WRT in horizontal direction and a factor of 2:1 in vertical direction, where WRT stated in (D-X6) is determined by quarter_reduced_depth_line_number.

- The color reduced depth topview and bottomview regions, CRDT and CRDB are respectively reconstructed by combinations of RDTL and RDTR regions and RDBL and RDBR regions if depth_spatial_flipping_flag equal to 0 or the combinations of horizontal spatial flips of RDTL and RDTR regions and RDBL and RDBR regions if depth_spatial_flipping_flag equal to 1. The color reduced depth topview and bottomview regions, CRDT and CRDB regions are reconstructed in order of RDTL and RDTR regions and RDBL and RDBR regions from left to right, respectively.

- According to depth_chroma_sampling_type, YCbCr colour 422 or 420 format is converted to YCbCr colour 444 format in the color reduced depth topview and bottomview regions, CRDT and CRDB.

- The YCbCr colour planes of the color reduced depth topview and bottomview regions, CRDT and CRDB are converted into RGB colour planes by using inverse color format conversion stated in (D-X7).

- According to depth_subpixel_arrangement_type, the three depth pixels in the horizontal direction of the reduced depth topview and bottomview regions, RDT and RDB are rearranged from one color-depth pixel, of the CRDT and CRDB regions with depth R, G, B subpixel channels, respectively.

- The depth topview and bottomview frames, DT and DB are respectively upsampled from the reduced depth topview and bottomview regions, RDT and RDB by a factor of W:6WHD in the horizontal direction and a factor of 2:1 in the vertical direction, where WHD stated in (D-X2) is determined by quarter_reduced_depth_line_number.

# Technology Adoptions

In order to promote the CTDP formats, we have signed two Memorandum of Understandings (MOUs) with Chunghwa Telecom (CHT) Company and Hsin Yeong An (HYA) Cable TV Company, while the former with about 1.4 million customers is a national-wise company in Taiwan and the later with about 145 thousand customers is a local cable company near the National Cheng Kung University.

For CHT Company, the test and adoption plan with time lines are designed as the following:

1. 04/01/2016: Propose and confirm the test and adoption detailed plan

2. 07/01/2016: Test of downloaded 3D CTDP services with computer clients

(Internal test in CHT and TOUCH Center only)

3. 02/01/2017: Services of 3D MOD services for computer clients

(Charge and open to the public and final CTDP format selection)

4. 02/01/2018: Test of 3D MOD services through Set-top-Boxes with separated CTDP depacking module (Open for selected users)

5. 02/01/2019：Test of 3D MOD services through Set-top-Boxes with Integrated CTDP depacking module (charged 3D services)

For HYA Cable TV Company, the signed agreement in English is shown in Fig. 3.1(b). The test and adoption plan with time lines are designed as the following:

1. 04/01/2016: Propose and confirm the test and adoption detailed plan

2. 07/01/2016: Test of downloaded 3D CTDP services with computer clients

(Internal test in HYA and TOUCH Center only)

3. 02/01/2017: Services of 3D CATV services for computer clients

(Charge and open to the public and final CTDP format selection)

4. 02/01/2018: Test of 3D CATV services through Set-top-Boxes with separated CTDP depacking module (Open for selected users)

5. 02/01/2019：Test of 3D CATV services through Set-top-Boxes with Integrated CTDP depacking module (charged 3D services)

# Conclusions

This contribution proposes modified CTDP SEI Message which is an update of the prior contribution JCT3V-M0021 by revising depth related syntax. Before the available of 3D broadcasting systems, we believe that the proposed CTDP formats with SEI Message could help to deliver 3D videos in the current 2D broadcasting systems simply and efficiently.

# References

[1] J.-F. Yang, H.-M. Wang, and Chiu-Yu Chen, Centralized Texture-Depth Packing SEI Message for H.264/AVC, Document, no. JCT3V-M0021, Oct. 2015.

[2] J.-F. Yang, H.-M. Wang, and Chiu-Yu Chen, Revised Centralized Texture-Depth Packing SEI Message and Its Performance Improvement by Depth Enhancement, Document, no. JCT3V-L0022, June 2015.

[3] J.-F. Yang, K.-Y. Liao. H.-M. Wang, and Y.-H. Hu, Centralized Texture-Depth Packing (CTDP) SEI Message, Document no. JCT3V-K0027, Feb. 2015.

[4]	J.-F. Yang, K.-Y. Liao. H.-M. Wang, and Y.-H. Hu, Centralized Texture-Depth Packing (CTDP) SEI Message Syntax, Document no. JCT3V-J0108, Jan. 2013.

[5]	J.-F. Yang, K.-Y. Liao. H.-M. Wang, and Y.-H. Hu, Additional Definitions of FPA SEI Message for Inclusion of Centralized Color-Depth Packing (CCDP) Formats, Document no. JCTVC-S0031, Jan. 2013.

[6] Recommendation ITU-T, H.265, High Efficiency Video Coding, April 2015

# Patent rights declaration(s)

National Cheng Kung University has US and EU pending patents, “METHOD, DEVICE AND SYSTEM FOR RESIZING ORIGINAL DEPTH FRAME INTO RESIZED DEPTH FRAME”, “METHOD, DEVICE AND SYSTEM FOR RESTORING RESIZED DEPTH FRAME INTO ORIGINAL DEPTH FRAME”, “METHOD, DEVICE AND SYSTEM FOR PACKING COLOR FRAME AND ORIGINAL DEPTH FRAME” and “UNPACKING METHOD, DEVICE AND SYSTEM OF PACKED FRAME” relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)..