| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: San Diego, USA, 22–26 Feb. 2016 | Document: JCT3V-N0025 |
| --- | --- |

| Title: | [AHG 5] Further confirmation on MV-HEVC conformance streams |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kei Kawamura Sei Naito 2-1-15 Ohara Fujimino-shi, Saitama Japan | Tel: Email: | +81 49 278 7411 ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI R&D Laboratories, Inc.) |  |  |

_____________________________

# Abstract

This contribution reports a cross-check results of test bitstreams for MV-HEVC conformance by an alternative decoder. The decoder was implemented independently with the reference software. All conformance stream can be decoded correctly, which hash value by the decoder is matched with that in the conformance stream.

# Introduction

MV-HEVC conformance draft text and streams are being developed [1][2]. Test bitstreams are specified to confirm functions proper to MV-HEVC prediction structure like 2-views and 3-views. Revised bitstreams based on the HTM15.1 are collected during the meeting cycle.

We had implemented an alternative decoder, which is independent with the reference software, and revised it to conform the latest reference software, HTM16.0. As the similar way of the previous report, an additional restriction is further implemented to the decoder based on the test specifications [3]. When the stream is not compliant with the specified prediction structure, decoder will stop.

The decoder was run on 64-bit Windows 7 environments. Through MVHEVCS_A to I, the prediction structure checks are passed and hash values are also matched.

It is noted that the decoding process of MVHEVCS_H is changed from the previous one due to a bug. The ticket is #106 (Inter-view bi-prediction mismatch with identical motion) and is already fixed in HTM15.1. An identical motion check is one of the decoder optimization, which was proposed in JCTVC- G415/G438. Although this check is implemented in the TComPrediction.cpp file, this is not specified in the specification text. This bug-fix may affect coding performance of a super multi-view case with IBP structure.

# Conclusion

We had verified the MV-HEVC conformance streams, which were collected in the meeting cycle. Provided streams were conformed to the specified prediction structure. All hash value by the alternative decoder are also matched with that in each bitstream.

# References

- Y. Chen, T. Ikai, K. Kawamura, S. Shimizu, and T. Suzuki, “MV-HEVC and 3D-HEVC Conformance Draft 3,” Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-K1008, Feb. 2015.

- Y. Chen, T. Ikai, K. Kawamura, S. Shimizu, and T. Suzuki, “HEVC Conformance testing development (AHG5),” Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-M0005, Oct. 2015.

- K. Kawamura and S. Naito, “[AHG 7] Cross-check result of test bitstreams for MV-HEVC conformance,” Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-L0024, June 2015.