| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 27–31 May 2016 | Document: JCT3V-O0022r2 |
| --- | --- |

| Title: | Centralized Texture-Depth Packing SEI Message for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V – Revision of JCT3V-N0022 |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jar-Ferr Yang Chiu-Yu Chen Wei-Jong Yang Hung-Ming Wang Department of Electrical Engineering, National Cheng Kung University 1 University Road, Tainan, Taiwan 701 | Tel: Email: | +886-6-2757575#62362 jefyang@mail.ncku.edu.tw |
| Source: | National Cheng Kung University (NCKU), 1 University Road, Tainan, Taiwan 701 |  |  |

_____________________________

# Abstract

This contribution modifies the prior contribution, JCT3V-N0022 [1], entitled “Centralized Texture-Depth Packing SEI Message for HEVC” by proposing an emulated YCbCr color representation to pack the color depth pixels. Thus, depth_chroma_sampling_type and depth_subpixel_arrangement_type are not needed and removed. To further improve the CTDP-2TB and CTDP-2LR performances, two_views_texture_packing_type is added to allow two texture views in CTDP-2TB format to be packed by either side-by-side (SbS) or checkerboard side-by-side (C-SbS) [2] method and those in CTDP-2LR format could be packed by either top-and-bottom (TaB) or checkerboard top-and-bottom (C-TaB) [2]. To evaluate quality of depacked texture, depth and synthesized view, the performances of CTDP formats with the emulated YCbCr 420 color representation as well as the two-view texture packing with SbS and C-SbS in uncoded and HEVC-coded cases are exhibited, where the uncoded and HEVC-coded results are compared to the ground truth results and the results achieved by 3D-HEVC, respectively.

# Centralized_texture-depth_packing SEI Message Syntax

- Replace the following specifications at clause 7.4.4:

general_non_packed_constraint_flag equal to 1 specifies that there are neither frame packing arrangement SEI messages nor segmented rectangular frame packing arrangement SEI messages nor centralized texture-depth packing SEI messages present in the CVS. general_non_packed_constraint_flag equal to 0 indicates that there may or may not be one or more frame packing arrangement SEI messages, segmented rectangular frame packing arrangement SEI messages or centralized texture-depth packing SEI messages present in the CVS.

NOTE 1– Decoders may ignore the value of general_non_packed_constraint_flag, as there are no decoding process requirements associated with the presence or interpretation of frame packing arrangement SEI messages, segmented rectangular frame packing arrangement SEI messages or centralized texture-depth packing SEI messages.

In Subclause D.2.1 - General SEI message syntax, adds “centralized_texture-depth_packing (payloadSize)” :

| else if( payloadType == X ) | Descriptor |
| --- | --- |
| centralized_texture-depth_packing ( payloadSize ) /* specified in Annex D */ |  |

where “X” is an appropriate payloadType number in the sequence of defined payloadType numbers.

Add a new Subclause D.2.X “centralized_texture-depth_packing SEI message syntax”, where. X will be replaced with an appropriate subclause number in the sequence of subclauses present in the text.

D.2.X	Centralized texture-depth packing (CTDP) SEI message syntax

| centralized_texture-depth_packing( payloadSize ) { | Descriptor |
| --- | --- |
| centralized_texture-depth_packing_cancel_flag | u(1) |
| if( ! centralized_texture-depth_packing_cancel_flag ) { |  |
| centralized_texture-depth_packing_content_interpretation_type | u(3) |
| quarter_resized_depth_line_number | u(12) |
| depth_spatial_flipping_flag | u(1) |
| two_views_texture_packing_type | u(2) |
| baseline_dist_flag | u(1) |
| focal_length_flag | u(1) |
| z_near_flag | u(1) |
| z_far_flag | u(1) |
| d_min_flag | u(1) |
| d_max_flag | u(1) |
| depth_representation_type | ue(v) |
| if( baseline_dist_flag ) |  |
| view_syn_rep_info_element( BdistanceSign, BdistanceExp, BdistanceMantissa, BdistanceManLen ) |  |
| if( focal_length_pixel_flag ) |  |
| view_syn_rep_info_element( FlengthSign, FlengthExp, FlengthMantissa, FlengthManLen ) |  |
| if( z_near_flag ) |  |
| view_syn_rep_info_element( ZNearSign, ZNearExp, ZNearMantissa, ZNearManLen ) |  |
| if( z_far_flag ) |  |
| view_syn_rep_info_element ( ZFarSign, ZFarExp, ZFarMantissa, ZFarManLen ) |  |
| if( d_min_flag ) |  |
| view_syn_rep_info_element ( DMinSign, DMinExp, DMinMantissa, DMinManLen ) |  |
| if( d_max_flag ) |  |
| view_syn_rep_info_element ( DMaxSign, DMaxExp, DMaxMantissa, DMaxManLen ) |  |
| if( depth_representation_type = = 3 ) { |  |
| depth_nonlinear_representation_num_minus1 | ue(v) |
| for( i = 1; i <= depth_nonlinear_representation_num_minus1 + 1; i++ ) |  |
| depth_nonlinear_representation_model[ i ] } |  |
| centralized_texture-depth_packing_persistence_flag | u(1) |
| } |  |
| } |  |

In Table F.4, add a new persistence scope for centralized_texture-depth_packing SEI message as follows:

| - SEI message | - Persistence scope |
| --- | --- |
| centralized_texture-depth_packing | - Specified by the syntax of the SEI message |

Add a new Subclause D.3.X “Centralized texture-depth packing SEI message semantics” where X will be replaced with an appropriate subclause number in the sequence of subclauses present in the text. In the following Subclause D.3.X, the numbers of tables, figures, and equations started from Table D-X1, Figure D-X1, D-X1 will be also replaced with an appropriate numbers in the sequence of tables, figures, and equations present in the text, respectively.

- D.3.X Centralized texture-depth packing SEI message semantics

This SEI message informs the decoder that the output cropped decoded picture contains samples of multiple distinct spatially packed constituent frames that are packed into one frame using a centralized texture-depth packing arrangement. This information can be used by the decoder to appropriately rearrange the samples and process the samples of the constituent frames appropriately for display or other purposes (which are outside the scope of this Specification).

When general_non_packed_constraint_flag is equal to 1 in the active SPS for the current layer, there shall be no centralized texture-depth packing SEI message applicable for any picture of the CLVS of the current layer.

When a frame packing arrangement SEI message or a segmented rectangular frame packing arrangement SEI message is applicable for any picture of the CLVS of the current layer, there shall be no centralized texture-depth packing SEI messages applicable for any picture of the CLVS of the current layer.

centralized_texture-depth_packing_cancel_flag equal to 1 indicates that the centralized texture-depth packing (CTDP) arrangement SEI message cancels the persistence of any previous the centralized texture-depth packing in output order. centralized_texture-depth_packing_cancel_flag equal to 0 indicates that indicates that centralized texture-depth packing follows.

The frame format in YCbCr color representation after the CTDP packing process is sent to the HEVC encoder for compression. Accordingly, the frame format in YCbCr color representation of the CTDP depacker will be received from the HEVC decoder.

centralized_texture-depth_packing_content_interpretation_type indicates the intended interpretation of the constituent frames as specified in Table D-X1.

Table D-X1 – Definition of centralized_texture-depth_packing_content_interpretation_type

| Value | Interpretation |
| --- | --- |
| 0 | Indicates the CTDP-1TB type that the CTDP packed frame, which provides one texture-plus-depth view, is arranged in the order of color_resized_depth_top (CRDT), resized_texture (RT), and color_resized_depth_bottom (CRDB) regions from top to bottom as illustrated in Figure D-X1. |
| 1 | Indicates the CTDP-1LR type that the CTDP packed frame is arranged in the order of color_resized_depth_left (CRDL), resized_texture (RT), and color_resized_depth_right (RDR) regions from left to right as illustrated in Figure D-X2. |
| 2 | Indicates the CTDP-2TB type that the CTDP packed frame, which provides two texture-plus-depth views, is arranged in the order of color_resized_depth_leftview_top (CRDLT), resized_texture_leftview (RTL), and color_resized_depth_leftview_bottom (CRDLB) regions in the left-half frame and color_resized_depth_rightview_top (CRDRT), resized_texture_rightview (RTR), and color_resized_depth_rightview_bottom (CRDRB) regions in the right-half frame from top to bottom as illustrated in Figure D-X3. |
| 3 | Indicates the CTDP-2LR type that the CTDP packed frame, which provides two texture-plus-depth views, is arranged in the order of color_resized_depth_topview_left (CRDTL), resized_texture_topview (RTT), and color_resized_depth_topview_right (CRDTR) regions in the top-half frame and color_resized_depth_bottomview_left (CRDBL), resized_texture_bottomview (RTB), and color_resized_depth_bottomview_right (CRDBR) regions in the bottom-frame from left to right as illustrated in Figure D-X4. |

Values of centralized_texture-depth_packing_content_interpretation_type that do not appear in Table D-X1 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore centralized texture-depth packing SEI messages that contain reserved values of centralized_texture-depth_packing_content_interpretation_type.

quarter_resized_depth_line_number indicates the quarter number of horizontal lines of the resized depth if centralized_texture-depth_packing_content_interpretation_type is equal to 0 or 2. The quarter_resized_depth_line_number defines the quarter number of vertical lines of the resized depth if centralized_texture-depth_packing_content_interpretation_type is equal to 1 or 3. The detailed descriptions of the sizes of color resized depth, resized depth, and resized texture regions according to centralized_texture-depth_packing_content_interpretation_type are stated as follows.

Let W and H be the width and height as pic_width_in_luma_samples and pic_height_in_luma_samples, respectively, of the decoded frame output from the decoder in units of luma samples.

If centralized_texture-depth_packing_content_interpretation_type is equal to 0 and 2, which respectively indicate CTDP-TB and CTDP-2TB packing types, the height of top and bottom color resized depth regions, CRDT and RDB shown in Figure D-X1 and the height of top and bottom color resized depth regions of left and right views, CRDLT, CRDRT, CRDLB, and CRDRB shown in Figure D-X3 is given as

HHD = 2*quarter_resized_depth_line_number.                                        (D-X1)

The height of RT region shown in Figure D-X1 and RTL and RTR regions shown in Figure D-X3 is specified in units of frame luma samples as:

HRT = H  HHD *2 = H 4*quarter_resized_depth_line_number. 	(D-X2)

The general guidelines for selecting the above quarter_resized_depth_line_number are: 1) to be a divisible factor of H such that it will result in simple downsample and upsampling VLSI implementations; 2) to be less than H/16 such that the downsampling factor of texture frame is less than that of depth map to achieve good packing quality. For UHD (3840x2160) and HD (1920x1080) video formats for examples, the possible selections of horizontal quarter_resized_depth_line_numbers and the corresponding HHD and HRT, which result in texture and depth vertical downsampling factors, are enlisted in Table D-X2.

Table D-X2 – Selections of horizontal quarter_resized_depth_line_number for UHD and HD video formats

| Video Formats | quarter_resized_ depth_ line_number | HHD | HRT | Texture Vertical Downsampling factor | Depth Vertical Downsampling factor |
| --- | --- | --- | --- | --- | --- |
| UHD (H=2160) | 90 | 180 | 1800 | 5:6 | 1:2 |
|  | 60 | 120 | 1920 | 8:9 | 1:3 |
|  | 45 | 90 | 1980 | 11:12 | 1:4 |
| HD (H=1080) | 45 | 90 | 900 | 5:6 | 1:2 |
|  | 30 | 60 | 960 | 8:9 | 1:3 |

If centralized_texture-depth_packing_content_interpretation_type is equal to 1 and 3, which respectively indicate CTDP-LR and CTDP-2LR packing types, the width of the left and right color resized depth regions, CRDL and CRDR shown in Figure D-X2 and the width of the left and right color resized depth regions of top and bottom views, CRDTL, CRDTR, CRDBL and CRDBR shown in Figure D-X4 is given as,

WHD = 2*quarter_resized_depth_line_number.                                        (D-X3)

The width of resized texture, RT region shown in Figure D-X2 and the width of resized textures of top and bottom views, RTT and RTB shown in Figure D-X4 in units of frame luma samples is specified as:

WRT = W WHD *2 = W 4*quarter_resized_depth_line_number.	(D-X4)

The general guidelines for determining the above quarter_resized_depth_line_number are: 1) to be a divisible factor of W such that it will result in simple downsample and upsampling VLSI implementations; 2) to be less than W/16 such that the downsampling factor of texture frame is less than that of depth map to achieve good packing quality. For UHD (3840x2160) and HD (1920x1080) video formats as examples, the possible selections of vertical quarter_resized_depth_line_numbers and the corresponding HHD and HRT, which result in texture and depth horizontal downsampling factors, are enlisted in Table D-X3.

Table D-X3 – Selections of vertical quarter_resized_depth_line_number for UHD and HD video formats

| Video Formats | quarter_resized_ depth_ line_number | WHD | WRT | Texture Horizontal Downsampling factor | Depth Horizontal Downsampling factor |
| --- | --- | --- | --- | --- | --- |
| UHD (W=3840) | 160 | 320 | 3200 | 5:6 | 1:2 |
|  | 120 | 240 | 3360 | 8:9 | 1:3 |
|  | 80 | 160 | 3520 | 11:12 | 1:4 |
| HD (W=1920) | 80 | 160 | 1600 | 5:6 | 1:2 |
|  | 60 | 120 | 1680 | 8:9 | 1:3 |
|  | 40 | 80 | 1760 | 11:12 | 1:4 |

- When centralized_texture-depth_packing_content_interpretation_type is equal to 0, the centralized texture-depth packing frame is composed of three resized rectangular regions identified as CRDT, RT and CRDB in the order from top to bottom as illustrated in Figure D-X1. The height of CRDT and CRDB regions is specified as HHD and the height of the resized texture, RT region is specified as HRT in units of frame luma samples. The width of CRDT, RT and CRDB regions is W in units of frame luma samples.

Followed the 6 reconstruction steps of texture and depth information stated in Subclause D.5.X, as illustrated in Figure D-X1, the CTDP frame is depacked to the texture (T) and depth (D), whose width and height are finally recovered back to W and H in units of frame luma samples, respectively.

Figure D-X1 Arrangement of the CTDP-1TB frame and its depacking steps if centralized_texture-depth_packing_content_interpretation_type is equal to 0

- When centralized_texture-depth_packing_content_interpretation_type is equal to 1, the centralized texture-depth packing frame is composed of three-resized rectangular regions identified as CRDL, RT, and CRDR in the order from left to right as illustrated in Figure D-X2. The width of CRDL and CRDR regions is specified as WHD and the width of RT region is specified as WRT in units of frame luma samples.. The height of CRDL, RT and CRDR regions is H in units of frame luma samples.

Followed the 6 reconstruction steps of texture and depth information stated in Subclause D.5.X, as illustrated in Figure D-X2, the CTDP frame is depacked to the texture (T) and depth (D), whose width and height are finally recovered back to W and H in units of frame luma samples, respectively.

Figure D-X2 Arrangement of the CTDP-1LR frame and its depacking steps if centralized_texture-depth_packing_content_interpretation_type is equal to 1

Figure D-X3 Arrangement of the CTDP-2TB frame and its depacking steps if centralized_texture-depth_packing_content_interpretation_type is equal to 2

- When centralized_texture-depth_packing_content_interpretation_type is equal to 2, the centralized texture-depth packing frame is composed of six resized rectangular regions identified as CRDLT, CRDRT, RTL, RTR, CRDLB, and CRDRB in the order from top to bottom as illustrated in Figure D-X3. The height of CRDLT, CRDRT, CRDLB, and CRDRB regions is specified as HHD and the height of RTL and RTR regions is specified as HRT in units of frame luma sample. The width of RDLT, RTL, RDLB, RDRT, RTR and RDRB regions is W/2 in units of frame luma samples.

Followed the 6 reconstruction steps of texture and depth information stated in Subclause D.5.X, as illustrated in Figure D-X3, the CTDP frame is depacked to the left texture (TL), the right texture (TR), the left depth (DL) and the right depth (DR), whose width and height are finally recovered back to W and H in units of frame luma samples, respectively.

- When centralized_texture-depth_packing_content_interpretation_type is equal to 3, the centralized texture-depth packing frame is composed of six resized rectangular regions identified as CRDTL, CRDBL, RTT, RTB, CRDTR and CRDBR in the order from left to right as illustrated in Figure D-X4. The width of CRDTL, CRDBL, CRDTR and CRDBR regions is specified as WHD and the width of RTT and RTB regions is specified as WRT in units of frame luma samples. The height of CRDTL, CRDBL, RTT, RTB, CRDTR and CRDBR regions is H/2 in units of frame luma samples.

Followed the 6 reconstruction steps of texture and depth information stated in Subclause D.5.X, as illustrated in Figure D-X4, the CTDP frame is depacked to the top texture (TT), bottom texture (TB), top depth (DT) and bottom depth (DB), whose width and height are finally recovery back to W and H in units of frame luma samples, respectively.

Figure D-X4 Arrangement of the CTDP-2LR frame and its depacking steps if centralized_texture-depth_packing_content_interpretation_type is equal to 3

For centralized_texture-depth_packing_content_interpretation_type equal to 0, 1, 2, or 3 and the quarter_resized_depth_line_number specifies the regions of the texture and depth information arranged in the specific order.

When centralized_texture-depth_packing_content_interpretation_type is not equal to 0, 1, 2, or 3, quarter_resized_depth_line_number is reserved for future use by ITU-T | ISO/IEC. When centralized_texture-depth_packing_content_interpretation_type is not equal to 0, 1, 2, or 3, decoders shall ignore the value of quarter_resized_depth_line_number.

depth_spatial_flipping_flag indicates that the resized depth region are spatially flipped relative to its intended orientation for display or other such purposes.

When depth_spatial_flipping_flag is equal to 1, the type of depth spatial flipping that is indicated is as follows:

–	If centralized_texture-depth_packing_content_interpretation_type is equal to 0, the indicated depth spatial flipping is vertical flipping. The depth spatial vertical flipping process for all depth color components with Y, Cb, and Cr is given as:

for( x = 0; x < W; x++ )

for( y = 0; y < HHD; y++ )

{

tRDT[x][y] = CRDT[x][HHD −y−1]

tRDB[x][y] = CRDB[x][HHD −y−1]

}

where W and HHD are the width and the height of CRDT and CRDB regions of the decoded picture in samples in Figure D-X1, respectively. CRDT[x][y], CRDB[x][y] and tRDT[x][y], tRDB[x][y] are the sample values at the location (x, y) before and after the vertical flipping of the decoded picture, respectively. It is noted that the tRDT and tRDB are the temporary memories to store the sample values after the flipping. Finally, the temporary memories are restored as CRDT = tRDT, CRDB = tRDB.

–	If centralized_texture-depth_packing_content_interpretation_type is equal to 1, the indicated depth spatial flipping is horizontal flipping. The depth spatial horizontal flipping process for all depth color components with Y, Cb, and Cr is given as:

for( x = 0; x < WHD; x++ )

for( y = 0; y < H; y++ )

{

tRDL[x][y] =CRDL[WHD −x−1][y]

tRDR[x][y] = CRDR[WHD −x−1][y]

}

where WHD and H are the width and the height of CRDL and CRDR regions of the decoded picture in samples in Figure D-X2, respectively. CRDL[x][y], CRDR[x][y] and tRDL[x][y], tRDR[x][y] denote the sample values at the location (x, y) before and after the horizontal flipping, respectively. It is noted that the tRDL and tRDR are the temporary memories to store the sample values after the flipping. Finally, the temporary memories are restored as CRDL = tRDL and CRDR = tRDR.

–	If centralized_texture-depth_packing_content_interpretation_type is equal to 2, the indicated depth spatial flipping is vertical flipping. The depth spatial vertical flipping process for all depth color components with Y, Cb, and Cr is given as:

for( x = 0; x < W/2; x++ )

for( y = 0; y < HHD; y++ )

{

tRDLT[x][y] = CRDLT[x][HHD −y−1]

tRDRT[x][y] = CRDRT[x][HHD −y−1]

tRDLB[x][y] = CRDLB[x][HHD −y−1]

tRDRB[x][y] = CRDRB[x][HHD −y−1]

}

where HHD and W/2 are the height and the width of CRDLT, CRDRT, CRDLB and CRDRB regions of the decoded picture in samples in Figure D-X3, respectively. CRDLT[x][y], CRDRT[x][y], CRDLB[x][y], CRDRB[x][y] and tRDLT[x][y], tRDRT[x][y], tRDLB[x][y], tRDRB[x][y] are the sample values at the location (x, y) before and after the vertical flipping, respectively. It is noted that the tRDLT, tRDRT, tRDLB, and tRDRB are the temporary memories to store the sample values after the flipping. Finally, the temporary memories are restored as CRDLT = tRDLT, CRDRT = tRDRT, CRDLB = tRDLB and CRDRB = tRDRB.

–	If centralized_texture-depth_packing_content_interpretation_type is equal to 3, the indicated depth spatial flipping is horizontal flipping. The color resized depth spatially horizontal flipping process for all depth color components with Y, Cb, and Cr is given as:

for( x = 0; x < WHD; x++ )

for( y = 0; y < H/2; y++ )

{

tRDTL[x][y] = CRDTL[WHD −x−1][y]

tRDTR[x][y] = CRDTR[WHD −x−1][y]

tRDBL[x][y] = CRDBL[WHD −x−1][y]

tRDBR[x][y] = CRDBR[WHD −x−1][y]

}

where WHD and H/2 are the width and the height of CRDTL, CRDTR, CRDBL and CRDBR regions of the decoded picture in samples in Figure D-X4, respectively. CRDLT[x][y], CRDRT[x][y], CRDLB[x][y], CRDRB[x][y] and tRDLT[x][y], tRDRT[x][y], tRDLB[x][y], tRDRB[x][y] are the sample values at the location (x, y) before and after the vertical flipping, respectively. Finally, the temporary memories are restored as CRDTL = tRDTL, CRDTR = tRDTR, CRDBL = tRDBL and CRDBR = tRDBR.

–	Otherwise (depth_spatial_flipping_flag is equal to 0), the indicated spatial flipping is not necessary.

When centralized_texture-depth_packing_content_interpretation_type is not equal to 0, 1, 2, or 3, it is a requirement of bitstream conformance that depth_spatial_flipping_flag shall be equal to 0. When centralized_texture-depth_packing_content_interpretation_type is not equal to 0, 1, 2, or 3, decoders shall ignore the value 1 for depth_spatial_flipping_flag.

After spatial flipping (depth_spatial_flipping_flag equal to 1) or no spatial flipping (depth_spatial_flipping_flag equal to 0), two corresponding color resized depth regions should be merged together to become color resized depth regions.

–	If centralized_texture-depth_packing_content_interpretation_type is equal to 0, the top and bottom color resized depth regions for each color component with Y, Cb, and Cr are vertically merged as:

for( x = 0; x < W; x++ )

for( y = 0; y < HHD; y++ )

{

CRD [x][y] = CRDT[x][ y]

CRD [x][ HHD +y] = CRDB[x][ y]

}

where W and HHD are the width and the height of CRDT and CRDB regions. After the above merging process, the color reduced depth, CRD region in color YCbCr color representation can be obtained. The width and the height of CRD region now become W and 2HHD, respectively.

–	If centralized_texture-depth_packing_content_interpretation_type is equal to 1, the left and right resized regions for each component with Y, Cb, and Cr are horizontally merged as:

for( x = 0; x < WHD; x++ )

for( y = 0; y < H; y++ )

{

CRD [x][y] =CRDL[x][y]

CRD[x+WHD ][y] = CRDR[x][y]

}

where WHD and H are the width and the height of CRDL and CRDR regions. After the above merging process, the color reduced depth, CRD region in color YCbCr color representation can be obtained. The width and the height of CRD region now becomes 2WHD and H, respectively.

–	If centralized_texture-depth_packing_content_interpretation_type is equal to 2, the top and bottom color resized depth regions of left and right views for each component with Y, Cb, and Cr are vertically merged as:

for( x = 0; x < W/2; x++ )

for( y = 0; y < HHD; y++ )

{

CRDL[x][y] = CRDLT[x][ y]

CRDL[x][y+ HHD] = CRDLB[x][y]

CRDR[x][y] = CRDRT[x][y]

CRDR[x][y+ HHD] = CRDRB[x][y]

}

where HHD and W/2 are the height and the width of CRDLT, CRDRT, CRDLB and CRDRB regions. After the above merging process, the color reduced depth leftview and rightview, CRDL and CRDR regions in color YCbCr color representation can be obtained. The width and the height of CRDL and CRDR regions now become W/2 and 2HHD, respectively.

–	If centralized_texture-depth_packing_content_interpretation_type is equal to 3, the left and right color resized depth regions of top and bottom views for each component with Y, Cb, and Cr are horizontally merged as::

for( x = 0; x < WHD; x++ )

for( y = 0; y < H/2; y++ )

{

CRDT[x][y] =CRDTL[x][y]

CRDT[x+ WHD][y] = CRDTR[x][y]

CRDB[x][y] = CRDBL[x][y]

CRDB[x+ WHD][y] = CRDBR[x][y]

}

where WHD and H/2 are the width and the height of CRDTL, CRDTR, CRDBL and CRDBR regions. After the above merging process, the color reduced depth topview and bottom view, CRDT and CRDB regions in color YCbCr color representation can be obtained. The width and the height of CRDT and CRDB regions now become 2WHD and H/2, respectively.

After the merging step, the color space translation and color-to-depth conversion steps should be performed to recover the reduced depth regions. The color resized depth, CRD, CRDL, CRDR, CRDT, CRDB regions in YCbCr color representation, should convert to the color depth in RGB color representation. The color depth CRD, CRDL, CRDR, CRDT, CRDB regions in RGB color representation are then respectively depacked to reduced depth, RD, RDL, RDR, RDT, RDB regions by expanding one color pixel to three depth pixels.

The regular YCbCr to RGB translation is computed in single color pixel bases for texture frames. Unlike the regular YCbCr to RGB translation, the emulated YCbCr translation for the color depth regions is translated in a set of 4 color depth pixels, which are spatially indexed by subscripts 1, 2, 3, and 4. Thus, there are 12 color subpixels, which could be {Ri, Gi, Bi, for i = 1 ,2, 3, 4} or {Yi, Cbi, Cri, for i = 1 ,2, 3, 4} as shown in Figure D-X6.

Figure D-X6 Emulated YCbCr 444 to RGB translation: (a) vertical color packing; (b) horizontal color packing

According to the HEVC decoder, if it adopts YCbCr 444 chroma format as shown in Figure D-X6.The whole YCbCr 444 to RGB translation is performed with three color conversion stages.

- In the YCbCr 420 color conversion stage, Y1, Y2, Y3, Y4 luma components are first converted to 4 selected subpixels, R1, G2, G3, and B4 by the range mappers and Cb1 and Cr1 chroma components are successively transformed to 2 selected subpixels, B1 and R4 by the range mappers of residuals plus the compensation of DPCM predictions. The detailed formulations are addressed as follows:

R1 = 1.1644  (Y1 – 16); 	(D-X5)

G2 = 1.1644  (Y2 – 16); 	(D-X6)

G3 = 1.1644  (Y3 – 16); 	(D-X7)

B4 = 1.1644  (Y4 – 16). 	(D-X8)

- For vertical color depacking (CTDP-1TB and CTDP-2TB types), B1 and R4 are given as:

B1 = 2.2768  (Cb1–128) + 0.5  (R1 +G3); 	(D-X9)

R4 = 2.2768  (Cr1–128) + 0.5  (G2 +B4), 	(D-X10)

where 0.5(R1+G3) and 0.5(G2+B4) are the DPCM predictors of B1 and R4, respectively.

- For horizontal color depacking (CTDP-1LR and CTDP-2LR types), B1 and R4 are computed as:

B1 = 2.2768  (Cb1–128) + 0.5  (R1 +G2); 	(D-X11)

R4 = 2.2768  (Cr1–128) + 0.5  (G3 +B4); 	(D-X12)

where 0.5(R1+G2) and 0.5(G3+B4) are the DPCM predictors of B1 and R4, respectively.

In this stage, six selected RGB subpixels located in interlaced positions are converted.

- In the YCbCr 422 color conversion stage, Cb2 and Cr2 chroma components are successively transformed to 2 selected RGB subpixels by range mappers of residuals plus the compensation of DPCM predictions. The detailed formulations are addressed as follows:

- For vertical color depacking procedures (CTDP-1TB and CTDP-2TB), R2 and B3 can be computed as:

R2 = 2.2768  (Cb2–128) + 0.5  (R1 +G2); 	(D-X13)

B3 = 2.2768  (Cr2–128) + 0.5  (G3 +B4); 	(D-X14)

where 0.5(R1+G2) and 0.5(G3+B4) are the DPCM predictors of R2 and B3, respectively.

- For horizontal color depacking (CTDP-1LR and CTDP-2LR), R3 and B2 are given as:

R3 = 2.2768  (Cb2–128) + 0.5  (R1 +G3); 	(D-X15)

B2 = 2.2768  (Cr2–128) + 0.5  (G2 +B4); 	(D-X16)

where 0.5(R1+G3) and 0.5(G2+B4) are the DPCM predictors of R3 and B2, respectively.

In this stage, two selected RGB subpixels located in interlaced positions are converted.

- In the YCbCr 444 color conversion stage, Cb3, Cr3, Cb4 and Cr4 chroma components are successively transformed to the final 4 selected RGB subpixels by range mappers of residuals plus the compensation of DPCM predictions. The detailed formulations are addressed as follows:

- Two RGB subpixels, G1 and G4 can be computed from Cb3 and Cr3 as:

G1 = 2.2768  (Cb3–128) + 0.5  (R1 +B1); 	(D-X17)

G4 = 2.2768  (Cr3–128) + 0.5  (R4 +B4) ; 	(D-X18)

where 0.5(R1+B1) and 0.5(R4+B4) are the DPCM predictors of G1 and G4, respectively.

- For vertical color depacking (CTDP-1TB and CTDP-2TB), the final two RGB subpixels, R3 and B2 can be recovered from U4 and V4 as:

R3 = 2.2768  (Cb4–128) + 0.5  (B1 +G3); 	(D-X19)

B2 = 2.2768  (Cr4–128) + 0.5  (G2 +R4) ; 	(D-X20)

where 0.5(B1+G3) and 0.5(G2+R4) are the DPCM predictors of R3 and B2, respectively.

- For horizontal color depacking (CTDP-1LR and CTDP-2LR), the final two RGB subpixels, R2 and B4 can be computed from U4 and V4 as:

R2 = 2.2768  (Cb4–128) + 0.5  (B1 +G2); 	(D-X21)

B3 = 2.2768  (Cr4–128) + 0.5  (G3 +R4) ; 	(D-X22)

where 0.5(B1+G2) and 0.5(G3+R4) are the DPCM predictors of R2 and B3, respectively.

In this stage, the final four RGB subpixels located in interlaced positions are converted. If the HEVC coder adopts YCbCr 444 color format, the emulated YCbCr 444 to RGB conversion will translate all 12 YCbCr components to obtain 12 RGB components, which could well represent all 12 depth values in either vertical or horizontal fashion as shown in Figure D-X6.

Figure D-X7 Emulated YCbCr 422 to RGB translation with 4 missing subpixels (black color):

(a) vertical color packing; (b) horizontal color packing

According to the HEVC coder, if it adopts YCbCr 422 chroma format as shown in D-X7 for vertical and horizontal color depth depacking, the color translation only have four luma components, Y1, Y2, Y3, and Y4 and four chroma components, Cb1, Cr1, Cb2 and Cr2, the procedure of the emulated YCbCr 422 to RGB translation will involve the first and second color translation stages of YCbCr 444 to RGB color translation to recover eight RGB subpixels. There are four missing RGB subpixels (in black color) as shown in Figure D-X7 should be recovered by a proper interpolation method from the neighboring known RGB subpixels The neighboring known RGB subpixels might be extended to nearby sets of four YCbCr components for better interpolation performance..

Figure D-X8 Emulated YCbCr 420 to RGB translation with 6 missing subpixels (black color):

(a) vertical color packing; (b) horizontal color packing

According to the HEVC decoder, if it adopts YCbCr 420 chroma format as shown in Figure D-X8 for vertical and horizontal color depacking, the color translation only have four luma components, Y1, Y2, Y3, and Y4 and two chroma components, Cb1 and Cr1. the procedure of the emulated YCbCr 420 to RGB translation will only involve the first color translation stage of YCbCr 444 to RGB color translation to recover six RGB subpixels. There are six missing RGB subpixels (in black color) as shown in Figure D-X8 should be recovered by a proper interpolation method from the neighboring known RGB subpixels. The neighboring known RGB subpixels might be extended to nearby sets of four YCbCr components for better interpolation performance.

After the emulated YCbCr 444, YCbCr 422, or YCbCr 420 to RGB color translation, the YCbCr colour reduced depth, CRD, CRDL, CRDR, CRDT, and CRDB regions are respectively converted to the full RGB color reduced depth, CRD, CRDL, CRDR, CRDT, and CRDB regions.

By depacking one color pixel (3 subpixels) to 3 depth pixels, the RGB color resized depth, CRD, CRDL, CRDR, CRDT, or CRDB regions will be depacked into the resized depth, RD, RDL, RDR, RDT, RDB regions, respectively. By depacking color pixels, the one pixel with R, G, and B channels (subpixels) of the color resized depth region are rearranged into three depth pixels of the resized depth region by using the sequence order specified in Figure D-X9.

    - if centralized_texture-depth_packing_content_interpretation_type is equal to 0 or 2, one horizontal RGB-channel line of color resized depth (CRD) frame is expanded to three horizontal single-channel lines of resized depth (RD) frame such that the height, 2HHD will be increased to 3 times, 6HHD, where the color resized depth regions could be CRD and CRDL and CRDR regions and the resized depth regions could be RD and RDL and RDR regions as specified in Figure D-X1 and Figure D-X3, respectively.

    - if centralized_texture-depth_packing_content_interpretation_type is equal to 1 or 3, one vertical line of color resized depth (CRD) region is expanded to three vertical lines of resized depth (RD) region such that the width WHD will be increased to 3 times, 6WHD, where the resized depth region could be CRD and CRDT and CRDB regions and the resized depth regions could be RD and RDT and RDB regions specified in Figure D-X2 and Figure D-X4, respectively.

Figure D-X9 Color RGB-channels of a W’× H’ color resized depth region is depacked to W’× 3H’ resized depth map for vertical color depacking or 3W’× H’ resized depth map for horizontal color depacking

two_views_texture_packing_type indicates that the method for packing two texture frames of two views into one texture frame is performed by either normal 2:1 downsampling or line-offset 2:1 downsampling.

For horizontal two-view texture packing, if two_views_texture_packing_type equals to 0, the CTDP packer performs the side-by-side (SbS) packing, which includes vertical downsizing and normal 2:1 horizontal downsampling of leftview and rightview texture frames and combination of them in left and right arrangement. If two_views_texture_packing_type equals to 1, the CTDP packer performs the checkerboard side-by-side (C-SbS) packing, which includes vertical downsizing and the line-offset 2:1 horizontal downsampling of leftview and rightview texture frames and combination of them in left and right arrangement.

For vertical two-view texture packing, if two_views_texture_packing_type equals to 0, the CTDP packer performs the top-and-bottom (TaB) packing, which includes horizontal downsizing, the normal 2:1 vertical downsampling of topview and bottomview texture frames and combination of them in top and down arrangement. If two_views_texture_packing_type equals to 1, the CTDP packer performs checkerboard top-and-bottom (C-TaB) packing, which includes horizontal downsizing, the line-offset 2:1 vertical downsampling of topview and bottomview texture frames and combination of them in top and down arrangement.

Table D-X5– Definition of two_views_texture_packing_type

| Value | Interpretation |
| --- | --- |
| 0 | CTDP depacker is informed that the normal 2:1 downsampling of two views is used for packing two texture views together. |
| 1 | CTDP depacker is informed that the line-offset 2:1 downsample of two views is used for packing two texture views together. |

Values of two_views_texture_packing_type that do not appear in Table D-X5 are reserved for future specification by ITU-T |ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore centralized texture-depth packing SEI messages that contain reserved values of texture two_view_packing_type.

- If centralized_texture-depth_packing_content_interpretation_type is equal to 0 or 1, only one view is existed. Thus, two_views_texture_packing_type will be ignored.

- If centralized_texture-depth_packing_content_interpretation_type is equal to 2, the resized_texture_leftview (RTL), resized_texture_rightview (RTR), resized_depth_leftview (RDL), and resized_depth_rightview (RDR) as shown in Figure D-X3 should be 2:1 horizontally upsampled.

If two_views_texture_packing_type equals to 0, the normal 2:1 horizontal upsampling procedure as show in Figure D-X9 is performed and followed by a proper interpolation filter to interpolate the missing pixels (in gray color).

If two_views_texture_packing_type equals to 1, the line-offset 2:1 horizontal upsampling procedure as shown in Figure D-X9 is performed and followed by a proper interpolation filter to interpolate the missing pixels (in gray color).

Figure D-X9 Normal and line-offset 2:1 horizontal upsampling procedures (gray=missing pixel)

- If centralized_texture-depth_packing_content_interpretation_type is equal to 3, the resized_texture_topview (RTT), resized_texture_bottomview (RTB), resized_depth_topview (RDT), and resized_depth_bottomview (RDB) as shown in Figure D-X4 should be 2:1 horizontally upsampled.

If two_views_texture_packing_type equals to 0, the normal 2:1 vertical upsampling procedure as show in Figure D-X10 is performed and followed by a proper interpolation filter to interpolate the missing pixels (in gray color).

If two_views_texture_packing_type equals to 1, the line-offset 2:1 vertical upsampling procedure as shown in Figure D-X10 is performed and followed by a proper interpolation filter to interpolate the missing pixels (in gray color).

Figure D-X10 Normal and line-offset 2:1 vertical upsampling procedures (gray =missing pixel)

baseline_dist_flag equal to 0 specifies that the syntax elements specifying the baseline distance are not present in the syntax structure. baseline_dist_flag equal to 1 specifies that the syntax elements specifying the baseline distance value are present in the syntax structure.

focal_length_flag equal to 0 specifies that the syntax elements specifying the focal length are not present in the syntax structure. focal_length_pixel_flag equal to 1 specifies that the syntax elements specifying the focal length value are present in the syntax structure.

z_near_flag equal to 0 specifies that the syntax elements specifying the nearest depth value are not present in the syntax structure. z_near_flag equal to 1 specifies that the syntax elements specifying the nearest depth value are present in the syntax structure.

z_far_flag equal to 0 specifies that the syntax elements specifying the farthest depth value are not present in the syntax structure. z_far_flag equal to 1 specifies that the syntax elements specifying the farthest depth value are present in the syntax structure.

d_min_flag equal to 0 specifies that the syntax elements specifying the minimum disparity value are not present in the syntax structure. d_min_flag equal to 1 specifies that the syntax elements specifying the minimum disparity value are present in the syntax structure.

d_max_flag equal to 0 specifies that the syntax elements specifying the maximum disparity value are not present in the syntax structure. d_max_flag equal to 1 specifies that the syntax elements specifying the maximum disparity value are present in the syntax structure.

depth_representation_type specifies the representation definition of decoded luma samples of auxiliary pictures as specified in Table D-X5. In Table D-X6, disparity specifies the horizontal displacement between two texture views and Z value specifies the distance from a camera.

The variable maxVal is set equal to ( 1 << ( 8 + bit_depth_luma_minus8 ) ) − 1, where bit_depth_luma_minus8 is the value included in or inferred for the active SPS of the layer with nuh_layer_id equal to targetLayerId.

Table D-X6– Definition of depth_representation_type

| depth_representation_type | Interpretation |
| --- | --- |
| 0 | Each decoded luma sample value of an auxiliary picture represents an inverse of Z value that is uniformly quantized into the range of 0 to maxVal, inclusive. When z_far_flag is equal to 1, the luma sample value equal to 0 represents the inverse of ZFar (specified below). When z_near_flag is equal to 1, the luma sample value equal to maxVal represents the inverse of ZNear (specified below). |
| 1 | Each decoded luma sample value of an auxiliary picture represents disparity that is uniformly quantized into the range of 0 to maxVal, inclusive. When d_min_flag is equal to 1, the luma sample value equal to 0 represents DMin (specified below). When d_max_flag is equal to 1, the luma sample value equal to maxVal represents DMax (specified below). |
| 2 | Each decoded luma sample value of an auxiliary picture represents a Z value uniformly quantized into the range of 0 to maxVal, inclusive. When z_far_flag is equal to 1, the luma sample value equal to 0 corresponds to ZFar (specified below). When z_near_flag is equal to 1, the luma sample value equal to maxVal represents ZNear (specified below). |
| 3 | Each decoded luma sample value of an auxiliary picture represents a nonlinearly mapped disparity, normalized in range from 0 to maxVal, as specified by depth_nonlinear_representation_num_minus1 and depth_nonlinear_representation_model[i]. When d_min_flag is equal to 1, the luma sample value equal to 0 represents DMin (specified below). When d_max_flag is equal to 1, the luma sample value equal to maxVal represents DMax (specified below). |
| Other values | Reserved for future use |

The variables in the x column of Table D-X7 are derived from the respective variables in the s, e, n and v columns of Table D-X7 as follows:

–     If the value of e is in the range of 0 to 127, exclusive, x is set equal to ( −1 )s* 2e - 31 * ( 1 + n ÷ 2v ).

–     Otherwise (e is equal to 0), x is set equal to ( −1 )s * 2−( 30 + v ) * n.

NOTE 1 – The above specification is similar to that found in IEC 60559:1989, Binary floating-point arithmetic for microprocessor systems.

Table D-X7– Association between depth parameter variables and syntax elements

| x | s | e | n | v |
| --- | --- | --- | --- | --- |
| Bdistance | BdistanceSign | BdistanceExp | BdistanceMantissa | BdistanceManLen |
| Flength | FlengthSign | FlengthExp | FlengthMantissa | FlengthManLen |
| ZNear | ZNearSign | ZNearExp | ZNearMantissa | ZNearManLen |
| ZFar | ZFarSign | ZFarExp | ZFarMantissa | ZFarManLen |
| DMax | DMaxSign | DMaxExp | DMaxMantissa | DMaxManLen |
| DMin | DMinSign | DMinExp | DMinMantissa | DMinManLen |

The DMin and DMax values, when present, are specified in units of a luma sample width of the coded picture.

The units for the ZNear and ZFar, when present, are identical but unspecified.

When depth_representation_type is equal to 0 or 2, the disparity D can further be obtained from the value of Z, Bdistance, and Flength.

D = Bdistance × Flength ÷ Z

depth_nonlinear_representation_num_minus1 plus 2 specifies the number of piecewise linear segments for mapping of depth values to a scale that is uniformly quantized in terms of disparity.

depth_nonlinear_representation_model[ i ] for i ranging from 0 to depth_nonlinear_representation_num_minus1 + 2, inclusive, specify the piecewise linear segments for mapping of decoded luma sample values of an auxiliary picture to a scale that is uniformly quantized in terms of disparity. The values of depth_nonlinear_representation_model[0] and depth_nonlinear_representation_model[ depth_nonlinear_representation_num_minus1 + 2 ] are both inferred to be equal to 0.

NOTE 2 – When depth_representation_type is equal to 3, an auxiliary picture contains nonlinearly transformed depth samples. The variable DepthLUT[ i ], as specified below, is used to transform decoded depth sample values from the nonlinear representation to the linear representation, i.e., uniformly quantized disparity values. The shape of this transform is defined by means of line-segment approximation in two-dimensional linear-disparity-to-nonlinear-disparity space. The first(0, 0) and the last ( maxVal, maxVal ) nodes of the curve are predefined. Positions of additional nodes are transmitted in form of deviations (depth_nonlinear_representation_model[ i ]) from the straight-line curve. These deviations are uniformly distributed along the whole range of 0 to maxVal, inclusive, with spacing depending on the value of nonlinear_depth_representation_num_minus1.

The variable DepthLUT[ i ] for i in the range of 0 to maxVal, inclusive, is specified as follows:

for( k = 0; k <= depth_nonlinear_representation_num_minus1 + 1; k++ ) {

pos1 = ( maxVal * k ) / (depth_nonlinear_representation_num_minus1 + 2 )

dev1 = depth_nonlinear_representation_model[ k ]

pos2 = ( maxVal * ( k + 1 ) ) / (depth_nonlinear_representation_num_minus1 + 2 )

dev2 = depth_nonlinear_representation_model[ k + 1 ] (G-10)

x1 = pos1 − dev1

y1 = pos1 + dev1

x2 = pos2 − dev2

y2 = pos2 + dev2

for( x = Max( x1, 0 ); x <= Min( x2, maxVal ); x++ )

DepthLUT[ x ] = Clip3( 0, maxVal, Round( ( ( x − x1 ) * ( y2 − y1 ) ) ÷ ( x2 − x1 ) + y1 ) )

}

When depth_representation_type is equal to 3, DepthLUT[ dS ] for all decoded luma sample values dS of an auxiliary picture in the range of 0 to maxVal, inclusive, represents disparity that is uniformly quantized into the range of 0 to maxVal, inclusive.

centralized_texture_depth_packing_persistence_flag specifies the persistence of the centralized texture-depth packing arrangement SEI message.

centralized_texture-depth_packing_persistence_flag equal to 0 specifies that the centralized texture-depth packing arrangement SEI message applies to the current decoded frame only.

centralized_texture-depth_packing_persistence_flag equal to 1 specifies that the centralized texture-depth packing arrangement SEI message persists in output order until any of the following conditions are true:

–	A new CVS begins.

–	The bitstream ends.

–	A frame in an access unit containing a centralized texture-depth packing SEI message with the same

- D.4.X	View synthesis representation information element syntax

| view_syn_rep_info_element( OutSign, OutExp, OutMantissa, OutManLen ) { | Descriptor |
| --- | --- |
| da_sign_flag | u(1) |
| da_exponent | u(7) |
| da_mantissa_len_minus1 | u(5) |
| da_mantissa | u(v) |
| } |  |

The syntax structure specifies the value of an element in the view synthesis representation information SEI message.

The syntax structure sets the values of the OutSign, OutExp, OutMantissa and OutManLen variables that represent a floating-point value. When the syntax structure is included in another syntax structure, the variable names OutSign, OutExp, OutMantissa and OutManLen are to be interpreted as being replaced by the variable names used when the syntax structure is included.

da_sign_flag equal to 0 indicates that the sign of the floating-point value is positive. da_sign_flag equal to 1 indicates that the sign is negative. The variable OutSign is set equal to da_sign_flag.

da_exponent specifies the exponent of the floating-point value. The value of da_exponent shall be in the range of 0 to 27 − 2, inclusive. The value 27 − 1 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 27 − 1 as indicating an unspecified value. The variable OutExp is set equal to da_exponent.

da_mantissa_len_minus1 plus 1 specifies the number of bits in the da_mantissa syntax element. The value of da_mantissa_len_minus1 shall be in the range of 0 to 31, inclusive. The variable OutManLen is set equal to da_mantissa_len_minus1 + 1.

da_mantissa specifies the mantissa of the floating-point value. The variable OutMantissa is set equal to da_mantissa.

- D.5.X Reconstruction of texture and depth information from the CTDP frame

The following steps describe the reconstruction of texture and depth information from CTDP frame by applying the centralized texture-depth packing SEI message syntax and semantics:

The input of the CTDP depacker, which is YCbCr 444, 422, or 420 format, will be the decoded frame of the video decoder. Let W and H be the width and height as pic_width_in_luma_samples and pic_height_in_luma_samples, respectively, of the decoded frame output from the decoder in units of luma and chroma samples.

When centralized_texture-depth_packing_content_interpretation_type is equal to 0.

- According to quarter_resized_depth_line_number, CRDT, RT, and CRDB regions from top to bottom are obtained by horizontally splitting the decoded CTDP-1TB frame as shown in Figure D-X1.

- The texture (T) frame with height H and width W is vertically upsampled from the resized texture (RT) region by a factor of H:HRT, where HRT stated in (D-X2) is determined by quarter_resized_depth_line_number.

- The color resized depth (CRD) region is reconstructed by the combination of two, CRDT and CRDB, i.e., color resized depth top and bottom regions if depth_spatial_flipping_flag equals to 0 or the combination of vertical spatial flips of CRDT and CRDB regions if depth_spatial_flipping_flag equal to 1. The CRD region is merged in order of CRDT, and RCDB from top to bottom.

- The YCbCr colour components of the color resized depth (CRD) region are converted to the RGB color components of the color resized depth (CRD) region by using the emulated color conversion formulas to obtain six selected RGB subpixels as defined from (D-X5) to (D-X10) in the first stage, to retrieve two selected RGB subpixels as defined by (D-X13) and (D-X14) in the second stage, and to find the final four RGB subpixels as stated by (D-X17), (D-X18), (D-X19), and (D-X20) in the third stage. If the video decoder adopts YCbCr 422 format, the third stage will be skipped. If the video decoder adopts YCbCr 420 format, the second and third stages will be skipped. Thus, for YCbCr 422 and YCbCr 420 formats, the proper interpolations of missing RGB subpixelsts should be performed.

- One RGB colour component of the color resized depth (CRD) region is vertically depacked to three gray components of resized depth (RD) region, which the height becomes 6HHD and the width is W, where HHD stated in (D-X1) is determined by quarter_resized_depth_line_number.

- The depth (D) frame with height H and width W is upsampled in the vertical direction from the resized depth (RD) region with a factor of H:6HHD, where HHD stated in (D-X1) is determined by quarter_resized_depth_line_number..

When centralized_texture-depth_packing_content_interpretation_type is equal to 1.

- According to quarter_resized_depth_line_number, CRDL, RT, and CRDR regions from left to right are obtained by vertically splitting the decoded CTDP-1LR frame shown in Figure D-X2.

- The texture (T) frame with height H and width W is horizontally upsampled from the resized texture (RT) region by a factor of W:WRT, where WRT stated in (D-X4) is determined by quarter_resized_depth_line_number.

- The color resized depth (CRD) region is reconstructed by the combination of two, CRDL and CRDR, i.e., color resized depth left and right regions if depth_spatial_flipping_flag equals to 0 or the combination of horizontal spatial flips of RDL and RDR regions if depth_spatial_flipping_flag equals to 1. The CRD region is merged in order of CRDL, and CRDR from left to right.

- The YCbCr colour components of the color resized depth (CRD) region are converted to the RGB color components of the color resized depth (CRD) region by using the emulated color conversion formulas to obtain six selected RGB subpixels as defined from (D-X5) to (D-X8), (D-X11), and D(X-12) in the first stage, to retrieve two selected RGB subpixels as defined by (D-X15) and (D-X16) in the second stage, and to find the final four RGB subpixels as stated in (D-X17), (D-X18), (D-X21), and (D-X22). If the video decoder adopts YCbCr 422 format, the third stage will be skipped. If the video decoder adopts YCbCr 420 format, the second and third stages will be skipped. Thus, for YCbCr 422 and YCbCr 420 formats, the proper interpolations of missing RGB components should be performed.

- One RGB colour component of the color resized depth (CRD) region is horizontally depacked to three gray components of resized depth (RD) region, which the height is H and the width becomes 6WHD, where WHD stated in (D-X3) is determined by quarter_resized_depth_line_number.

- The depth (D) frame with height H and width W is upsampled in the horizontal direction from the resized depth (RD) region with a factor of W:6WHD, where WHD stated in (D-X3) is determined by quarter_resized_depth_line_number.

When centralized_texture-depth_packing_content_interpretation_type is equal to 2.

- According to quarter_resized_depth_line_number, the CRDLT, CRDRT, RTL, RTR, CRDLB, and CRDRB regions in Figure D-X3 are extracted from the decoded picture by vertical and horizontal splitting.

- The texture leftview and rightview, TL and TR regions with height H and width W are respectively upsampled from resized texture leftview and rightview, RTL and RTR regions. The RTL and RTR regions are first performed by a horizontal 2:1 upsampling method defined by two_views_texture_packing_type and followed by vertical H:HRT upsampling.

If two_views_texture_packing_type equals to 0, the normal horizontal 2:1 upsampling is performed as shown in Figure D-X9. If two_views_texture_packing_type equals to 1, the line-offset 2:1 upsampling is performed as shown in Figure D-X9.

Then, the texture regions are vertically upsampled with a factor of H:HRT, where HRT stated in (D-X2) is determined by quarter_resized_depth_line_number.

- The color depth leftview and rightview regions CRDL and CRDR are respectively reconstructed by combinations of CRDLT and CRDLB regions and CRDRT and CRDRB regions if depth_spatial_flipping_flag equal to 0 or the combinations of vertical spatial flips of CRDLT and CRDLB regions and CRDRT and CRDRB regions if depth_spatial_flipping_flag equal to 1. The color depth leftview and rightview regions, CRDL and CRDR are reconstructed in order of CRDLT and CRDLB regions and CRDRT and CRDRB regions from top to bottom, respectively.

- The YCbCr colour components of the color resized depth leftview and rightview, CRDL and CRDR regions are respectively converted to the RGB components of the color resized depth leftview and rightview, CRDL and CRDR regions by using the emulated color conversion formulas to obtain six selected RGB subpixels as defined from (D-X5) to (D-X10) in the first stage, to retrieve two selected RGB subpixels as defined by (D-X13) and (D-X14) in the second stage, and to find the final four selected RGB subpixels as stated in (D-X17), (D-X18), (D-X19), and (D-X20) in the third stage. If the video decoder adopts YCbCr 422 format, the third stage will be skipped. If the video decoder adopts YCbCr 420 format, the second and third stages will be skipped. Thus, for YCbCr 422 and YCbCr 420 formats, the proper interpolations of missing RGB subpixels should be performed.

- One RGB colour component of the CRDL and CRDR regions is vertically depacked to three gray depth components of the resized depth leftview and resized depth rightview, RDL and RDR regions, which the height becomes 6HHD and the width is W/2, where HHD stated in (D-X1) is determined by quarter_resized_depth_line_number.

- The depth leftview and rightview frames, DL and DR regions with height H and width W are respectively upsampled from the resized depth leftview and rightview, RDL and RDR regions. The RDL and RDR regions are first performed by horizontal 2:1 upsampling and followed by vertical H:HRT upsampling, where HHD stated in (D-X1) is determined by quarter_resized_depth_line_number.

When centralized_texture-depth_packing_content_interpretation_type is equal to 3.

- According to quarter_resized_depth_line_number the CRDTL, CRDBL, RTT, RTB, CRDTR, and CRDBR regions in Figure D-X4 are obtained from the decoded CTDP-2LR frame by horizontal and vertical splitting.

- The texture topview and bottomview, TT and TB regions with height H and width W are respectively upsampled from the resized texture topview and bottomview, RTT and RTB regions. The RTT and RTB regions are first performed by a vertical 2:1 upsampling method defined by two_views_texture_packing_type  and followed by horizontal W:WRT upsampling.

If two_views_texture_packing_type equals to 0, the normal vertical 2:1 upsampling is performed as shown in Figure D-X10. If two_views_texture_packing_type equals to 1, the line-offset 2:1 vertical upsampling is performed as shown in Figure D-X10.

Then, the texture regions are horizontally upsampled with a factor of W:WRT, where WRT stated in (D-X4) is determined by quarter_resized_depth_line_number.

- The color resized depth topview and bottomview regions, CRDT and CRDB are respectively reconstructed by combinations of CRDTL and CRDTR regions and CRDBL and CRDBR regions if depth_spatial_flipping_flag equal to 0 or the combinations of horizontal spatial flips of CRDTL and CRDTR regions and CRDBL and CRDBR regions if depth_spatial_flipping_flag equal to 1. The color resized depth topview and bottomview regions, CRDT and CRDB regions are reconstructed in order of CRDTL and CRDTR regions and CRDBL and CRDBR regions from left to right, respectively.

- The YCbCr colour components of the color resized depth topview and bottomview, CRDT and CRDB regions are respectively converted to the RGB components of the color resized depth topview and bottomview, CRDT and CRDB regions by using the emulated color conversion formulus to obtain six selected RGB subpixels as defined from (D-X5) to (D-X8), (D-X11), and (D-X12) in the first stage, to retrieve two selected RGB subpixels defined by (D-X15) and (D-X16) in the second stage, and to find the final four RGB subpixels as stated by (D-X17), (D-X18), (D-X21), and (D-X22). If the video decoder adopts YCbCr 422 format, the third stage will be skipped. If the video decoder adopts YCbCr 420 format, the second and third stages will be skipped. Thus, for YCbCr 422 and YCbCr 420 formats, proper interpolations of missing RGB subpixels should be performed.

- One RGB colour components of the CRDT and CRDB regions is horizontally depacked to three gray depth components of the resized depth topview and resized depth bottomtview, RDT and RDB regions, which the height is H/2 and the width becomes 6WHD, where WHD stated in (D-X3) is determined by quarter_resized_depth_line_number.

- The depth topview and bottomview frames, DT and DB with height H and width W are respectively upsampled from the resized depth topview and bottomview, RDT and RDB regions. The RDT and RDB regions are first performed by vertical 2:1 upsampling and followed by horizontal W:6WHD upsampling, where WHD stated in (D-X3) is determined by quarter_resized_depth_line_number.

# Performance Evaluations of Emulated YCbCr Conversion

The previous contributions [1]-[6] have reported the performances of CTDP formats, CTDP-1TB, CTDP-1LR CTDP-2TB and CTDP-2LR types, for both single-view and stereo-view 3D videos, where the synthesized views are generated by VSRS software by using natural and CG videos. Suggested in JCT-3V Meeting held on Feb. 2016, the additional color packing in YCbCr chroma domain should be considered instead of the traditional YCbCr. If the coding system adopts YCbCr 420 format, the performance evaluations of the CTDP formats with the regular and emulated RGB/YCbCr conversions in various conditions are shown and discussed in the following subsections.

2.1 Performance Evaluations of CTDP Formats in YCbCr420 Chroma Representation

Without going through any video coding systems, the performances of the CTDP formats are totally affected by: 1) the resized texture; 2) the resized depth; 3) the drop of three Cb and Cr components if YCbCr 420 chroma representation is used; and 4) the 2:1 downsizing top-and-bottom or side-by-side views and depths for two-view and two-depth packing. In simulations, we adopt Shark and UndoDancer sequences for performance evaluations of uncoded CTDP videos with the regular YCbCr and the emulated YCbCr representations. In various CTDP formats, Table 2.1, 2.2, 2.3, and 2.4 show the PSNR and SSIM performances of depacked texture, depth, and their synthesized virtual views generated by VSRS rendering system. The simulation results show that the depacked depth with the emulated YCbCr conversion shows much better PSNR and better SSIM performances than that with the regular YCbCr conversion. Besides, the synthesized virtual view with the emulated YCbCr translation also exhibits better PSNR and SSIM performances than that with the regular YCbCr conversion.

Table 2.1. Objective performances for uncoded CTDP-1TB texture-5/6 packing videos

| Performances | Depacked Texture and Depth |  |  | Synthesized Virtual View |  |
| --- | --- | --- | --- | --- | --- |
|  | Texture | Depth |  | Virtual View Generated by VSRS |  |
|  | 5/6_1TB | Regular YCbCr | Emulated YCbCr | Regular YCbCr | Emulated YCbCr |
| PSNR | 45.2634 | 41.4285 | 45.1297 | 37.8784 | 38.6801 |
| SSIM | 0.9922 | 0.9832 | 0.9902 | 0.9820 | 0.9843 |

Table 2.2. Objective performances for uncoded CTDP-1TB texture-8/9 packing videos

| Performances | Depacked Texture and Depth |  |  | Synthesized Virtual View |  |
| --- | --- | --- | --- | --- | --- |
|  | Texture | Depth |  | Virtual View Generated by VSRS |  |
|  | 8/9_1TB | Regular YCbCr | Emulated YCbCr | Regular YCbCr | Emulated YCbCr |
| PSNR | 46.1485 | 39.9886 | 43.9897 | 37.4524 | 38.7200 |
| SSIM | 0.9938 | 0.9802 | 0.9881 | 0.9824 | 0.9853 |

Table 2.3. Objective performances for uncoded CTDP-2TB texture-5/6 packing videos

| Performances | Depacked Texture and Depth |  |  | Synthesized Virtual View |  |
| --- | --- | --- | --- | --- | --- |
|  | Texture | Depth |  | Virtual View Generated by VSRS |  |
|  | 5/6_2TB | Regular YCbCr | Emulated YCbCr | Regular YCbCr | Emulated YCbCr |
| PSNR | 35.0175 | 39.1883 | 40.6999 | 33.6607 | 34.1535 |
| SSIM | 0.9382 | 0.9779 | 0.9850 | 0.9365 | 0.9375 |

Table 2.4. Objective performances for uncoded CTDP-2TB texture-8/9 packing videos

| Performances | Depacked Texture and Depth |  |  | Synthesized Virtual View |  |
| --- | --- | --- | --- | --- | --- |
|  | Texture | Depth |  | Virtual View Generated by VSRS |  |
|  | 8/9_2TB | Regular YCbCr | Emulated YCbCr | Regular YCbCr | Emulated YCbCr |
| PSNR | 35.1908 | 38.3579 | 40.2682 | 33.6330 | 34.3702 |
| SSIM | 0.9413 | 0.9740 | 0.9832 | 0.9379 | 0.9409 |

The C-SbS and C-TaB packing methods show better stereo packing performances than the SbS and TaB, respectively [2]. In order to further improve the packing performances for CTDP two view cases, the uncompressed texture with normal 2:1 downsample and line-offset downsample are presented. Tables 2.5 and 2.6 shows the PSNR and SSIM performances of depacked texture, depacked depth, and synthesized virtural view for 5/6-texture and 8/9-texture CTDB-2TB formats, respectively. In these tables, the depth is packed by emulated YUV and the two-view texture is packed by normal 2:1 subsampling (SbS format) and line-offset 2:1 subsampling (C-SbS). Thus, the simulation results show that the CTDP-2TB with checkboard side-by-side (C-SbS) method shows better performance than that with regular side-by-side (SbS) method for packing the two texture views together.

Table 2.5. Objective performances for uncoded CTDP-2TB texture-5/6 by using emulated YUV transform for depth packing while two texture views are packed by side-by-side (SbS) and checkerboard side-by-side (C-SbS).

| Performances | Depacked Texture and Depth |  |  |  | Synthesized Virtual View |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Texture |  | Depth |  | Virtual View by VSRS |  |
|  | Emulated YUV +SbS | Emulated YUV +C-SbS | Emulated YUV +SbS | Emulated YUV +C-SbS | Emulated YUV+SbS | Emulated YUV +C-SbS |
| PSNR | 35.0175 | 38.0633 | 40.6999 | 41.3703 | 34.1535 | 35.9455 |
| SSIM | 0.9382 | 0.9633 | 0.9850 | 0.9852 | 0.9375 | 0.9596 |

Table 2.6. Objective performances for uncoded CTDP-2TB texture-8/9 by using emulated YUV transform for depth packing while two texture views are packed by side-by-side (SbS) and checkerboard side-by-side (C-SbS).

| Performances | Depacked Texture and Depth |  |  |  | Synthesized Virtual View |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Texture |  | Depth |  | Virtual View by VSRS |  |
|  | Emulated YUV +SbS | Emulated YUV +C-SbS | Emulated YUV +SbS | Emulated YUV +C-SbS | Emulated YUV+SbS | Emulated YUV +C-SbS |
| PSNR | 35.1908 | 38.5203 | 40.2682 | 40.9213 | 34.3702 | 36.1371 |
| SSIM | 0.9413 | 0.9666 | 0.9832 | 0.9834 | 0.9409 | 0.9622 |

2.2 Performance Evaluations of HEVC-Coded CTDP with in Comparisons of 3D-HEVC

To test if the emulated YCbCr conversion fit the latest HEVC coding system, the CTDP formats (CTDP-1TB, CTDP-1LR, CTDP-2TB, and CTDP-2LR) with the resized texture ratios of 5/6 and 8/9 are encoded by HEVC HM13.0 and compared to the same 3D video encoded by 3D-HEVC HTM8.0. The QP setting for both HM13.0 and HTM8.0 are 27, 32, 37, and 42. Two CG 3D video test sequences, Shark and UndoDancer (1920x1088) sequences were removed top and bottom 4 horizontal lines to become the size of 1920x1080. In simulation, the coding performances of depacked texture, the depacked depth, and the synthesized virtual view by VSRS with the regular YCbCr conversion, the regular YCbCr conversion with the enhanced processes proposed in [1], and the emulated YCbCr conversion are exhibited. Comparing to the regular YCbCr color depth, Tables 2.7 – 2.14 shows that the proposed emulated YCbCr color depth can significantly improve the BDBR and BDPSNR performances of synthesized virtual views for Texture-5/6 CTDP-1TB, Texture-8/9 CTDP-1TB, Texture-5/6 CTDP-2TB, and Texture-8/9 CTDP-2TB.

Table 2.7. Average BDBR (%) results for Texture-5/6 CTDP-1TB format with HEVC compared to 3D-HEVC

| Coding Mode | Depacked Texture |  | Depacked Depth |  |  | Virtual View by VSRS |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Regular YUV | Emulated YUV | Regular YUV | Regular YUV + Enhance | Emulated YUV | Regular YUV + Enhance. | Emulated YUV |
| ra | 19.1613 | 21.0704 | -40.6867 | -48.4200 | -43.7626 | 45.0708 | 25.5579 |
| ldp | 13.6528 | 16.5524 | -57.0321 | -61.5932 | -56.0816 | 29.8237 | 15.7390 |
| ai | 11.6244 | 12.1774 | -15.1046 | -28.0788 | -32.1844 | 54.2097 | 27.9285 |
| ave | 14.8128 | 16.6000 | -37.6078 | -46.0307 | -44.0095 | 43.0347 | 23.0751 |

Table 2.8. Average BDPSNR (dB) results for Texture 5/6 CTDP-1TB format with HEVC compared to 3D-HEVC

| Coding Mode | Depacked Texture |  | Depacked Depth |  |  | Virtual View by VSRS |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Regular YUV | Emulated YUV | Regular YUV | Regular YUV + Enhance | Emulated YUV | Regular YUV + Enhance. | Emulated YUV |
| ra | -0.5741 | -0.6299 | 1.8158 | 2.4889 | 2.2445 | -1.0416 | -0.6718 |
| ldp | -0.4308 | -0.5127 | 3.6340 | 4.3047 | 3.9586 | -0.7667 | -0.4522 |
| ai | -0.4129 | -0.4329 | 0.4578 | 1.2294 | 1.6828 | -1.3641 | -0.8275 |
| ave | -0.4726 | -0.5251 | 1.9692 | 2.6743 | 2.6286 | -1.0575 | -0.6505 |

Table 2.9. Average BDBR (%) results for Texture 8/9 CTDP-1TB format with HEVC compared to 3D-HEVC

| Coding Mode | Depacked Texture |  | Depacked Depth |  |  | Virtual View by VSRS |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Regular YUV | Emulated YUV | Regular YUV | Regular YUV + Enhance | Emulated YUV | Regular YUV + Enhance. | Emulated YUV |
| ra | 15.6515 | 15.7538 | -20.4062 | -30.4340 | -23.2366 | 53.2643 | 30.0211 |
| ldp | 10.3883 | 10.6721 | -46.7615 | -52.0132 | -47.0105 | 36.3050 | 18.0217 |
| ai | 8.3173 | 9.0084 | 7.7358 | -8.0741 | -7.0863 | 62.7299 | 33.8656 |
| ave | 11.4523 | 11.8114 | -19.8106 | -30.1738 | -25.7778 | 50.7664 | 27.3028 |

Table 2.10. Average BDPSNR (dB) results for Texture 8/9 CTDP-1TB format with HEVC compared to 3D-HEVC

| Coding Mode | Depacked Texture |  | Depacked Depth |  |  | Virtual View by VSRS |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Regular YUV | Emulated YUV | Regular YUV | Regular YUV + Enhance | Emulated YUV | Regular YUV + Enhance. | Emulated YUV |
| ra | -0.4186 | -0.4791 | 0.6767 | 1.2684 | 0.9576 | -1.1861 | -0.7665 |
| ldp | -0.2996 | -0.3410 | 2.6069 | 3.2358 | 2.9329 | -0.8902 | -0.5036 |
| ai | -0.1949 | -0.3244 | -0.7220 | 0.1314 | 0.1938 | -1.5130 | -0.9675 |
| ave | -0.3043 | -0.3815 | 0.8539 | 1.5452 | 1.3614 | -1.1964 | -0.7458 |

Table 2.11. Average BDBR (%) results for Texture 5/6 CTDP-2TB format with HEVC compared to 3D-HEVC

| Coding Mode | Depacked Texture |  | Depacked Depth |  |  | Virtual View by VSRS |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Regular YUV | Emulated YUV | Regular YUV | Regular YUV + Enhance | Emulated YUV | Regular YUV + Enhance. | Emulated YUV |
| ra | 281.0796 | 261.1590 | -14.3619 | -23.1498 | -23.5458 | 226.3722 | 179.0697 |
| ldp | 212.6822 | 193.6546 | -46.3458 | -51.1193 | -50.5875 | 166.7185 | 129.8933 |
| ai | 120.6750 | 105.5605 | -40.1254 | -46.5875 | -47.8140 | 133.9196 | 94.3508 |
| ave | 204.8122 | 186.7913 | -33.6110 | -40.2855 | -40.6491 | 175.6701 | 134.4379 |

Table 2.12. Average BDPSNR (dB) results for Texture 5/6 CTDP-2TB format with HEVC compared to 3D-HEVC

| Coding Mode | Depacked Texture |  | Depacked Depth |  |  | Virtual View by VSRS |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Regular YUV | Emulated YUV | Regular YUV | Regular YUV + Enhance | Emulated YUV | Regular YUV + Enhance. | Emulated YUV |
| ra | -3.4523 | -3.2323 | 0.2035 | 0.6046 | 0.7165 | -3.1486 | -2.7377 |
| ldp | -3.1293 | -2.8768 | 2.0161 | 2.4799 | 2.6986 | -2.7743 | -2.3497 |
| ai | -2.1034 | -1.7525 | 1.2915 | 1.8502 | 2.2525 | -2.3466 | -1.7677 |
| ave | -2.8950 | -2.6205 | 1.1703 | 1.6449 | 1.8892 | -2.7562 | -2.2850 |

Table 2.13. Average BDBR (%) results for Texture 8/9 CTDP-2TB format with HEVC compared to 3D-HEVC

| Coding Mode | Depacked Texture |  | Depacked Depth |  |  | Virtual View by VSRS |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Regular YUV | Emulated YUV | Regular YUV | Regular YUV + Enhance | Emulated YUV | Regular YUV + Enhance. | Emulated YUV |
| ra | 275.0123 | 248.8300 | 7.6578 | -6.2878 | -1.0379 | 230.8110 | 178.7589 |
| ldp | 209.0862 | 186.9224 | -34.4792 | -40.7517 | -41.1088 | 171.1121 | 130.8688 |
| ai | 117.8838 | 99.6945 | -25.0388 | -33.3960 | -34.7983 | 137.6251 | 93.1213 |
| ave | 200.6608 | 178.4823 | -17.2867 | -26.8118 | -25.6483 | 179.8494 | 134.2490 |

Table 2.14. Average BDPSNR (dB) results for Texture 8/9 CTDP-2TB format with HEVC compared to 3D-HEVC

| Coding Mode | Depacked Texture |  | Depacked Depth |  |  | Virtual View by VSRS |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Regular YUV | Emulated YUV | Regular YUV | Regular YUV + Enhance | Emulated YUV | Regular YUV + Enhance. | Emulated YUV |
| ra | -3.4115 | -3.1865 | -0.5479 | -0.0245 | -0.2115 | -3.1663 | -2.7616 |
| ldp | -3.1054 | -2.8270 | 1.2126 | 1.6748 | 1.9263 | -2.8149 | -2.3627 |
| ai | -2.1073 | -1.7225 | 0.2496 | 0.8719 | 1.2903 | -2.4196 | -1.8039 |
| ave | -2.8747 | -2.5786 | 0.3047 | 0.8407 | 1.0017 | -2.8003 | -2.3094 |

In order to further evaluate the packing performances for packing two view cases, the HEVC coded CTDP-2TB and the two-view texture with normal 2:1 downsample and line-offset 2:1 downsample in comparisons of 3D-HEVC are presented. Tables 2.15 – 2-18 show the BDBR and BDPSNR performances in comparison of 3D-HEVC. The results also verify that the checkerboard SbS and checkerboard TaB for two-view packing can further improve the CTDP packing BDBR and BDPSNR performances.

Table 2.15. Average BDBR (%) results for Texture 5/6 CTDP-2TB format with HEVC compared to 3D-HEVC

| Performances | Depacked Texture and Depth |  |  |  | Synthesized Virtual View |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Texture |  | Depth |  | Virtual View by VSRS |  |
|  | Emulated YUV +SbS | Emulated YUV +C-SbS | Emulated YUV +SbS | Emulated YUV +C-SbS | Emulated YUV+SbS | Emulated YUV +C-SbS |
| ra | 261.1590 | 198.0049 | -23.5458 | -17.7369 | 179.0697 | 163.9154 |
| ldp | 193.6546 | 138.5016 | -50.5875 | -49.2544 | 129.8933 | 112.6531 |
| ai | 105.5605 | 45.7359 | -47.8140 | -41.9267 | 94.3508 | 59.5660 |
| ave | 186.7913 | 127.4141 | -40.6491 | -36.3060 | 134.4379 | 112.0448 |

Table 2.16. Average BDPSNR (dB) results for Texture 5/6 CTDP-2TB format with HEVC compared to 3D-HEVC

| Performances | Depacked Texture and Depth |  |  |  | Synthesized Virtual View |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Texture |  | Depth |  | Virtual View by VSRS |  |
|  | Emulated YUV +SbS | Emulated YUV +C-SbS | Emulated YUV +SbS | Emulated YUV +C-SbS | Emulated YUV+SbS | Emulated YUV +C-SbS |
| ra | -3.2323 | -2.9204 | 0.7165 | 0.5966 | -2.7377 | -2.5803 |
| ldp | -2.8768 | -2.4768 | 2.6986 | 2.4951 | -2.3497 | -2.1423 |
| ai | -1.7525 | -1.2001 | 2.2525 | 1.6918 | -1.7677 | -1.5160 |
| ave | -2.6205 | -2.1991 | 1.8892 | 1.5945 | -2.2850 | -2.0795 |

Table 2.17. Average BDBR (%) results for Texture 8/9 CTDP-2TB format with HEVC compared to 3D-HEVC

| Performances | Depacked Texture and Depth |  |  |  | Synthesized Virtual View |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Texture |  | Depth |  | Virtual View by VSRS |  |
|  | Emulated YUV +SbS | Emulated YUV +C-SbS | Emulated YUV +SbS | Emulated YUV +C-SbS | Emulated YUV+SbS | Emulated YUV +C-SbS |
| ra | 248.8300 | 150.8311 | -1.0379 | 0.7539 | 178.7589 | 164.9953 |
| ldp | 186.9224 | 129.0255 | -41.1088 | -38.6711 | 130.8688 | 109.7341 |
| ai | 99.6945 | 42.0673 | -34.7983 | -25.9210 | 93.1213 | 60.3334 |
| ave | 178.4823 | 107.3080 | -25.6483 | -21.2794 | 134.2490 | 111.6876 |

Table 2.18. Average BDPSNR (dB) results for Texture 8/9 CTDP-2TB format with HEVC compared to 3D-HEVC

| Performances | Depacked Texture and Depth |  |  |  | Synthesized Virtual View |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Texture |  | Depth |  | Virtual View by VSRS |  |
|  | Emulated YUV +SbS | Emulated YUV +C-SbS | Emulated YUV +SbS | Emulated YUV +C-SbS | Emulated YUV+SbS | Emulated YUV +C-SbS |
| ra | -3.1865 | -2.5949 | -0.2115 | -0.0822 | -2.7616 | -2.5835 |
| ldp | -2.8270 | -2.3486 | 1.9263 | 1.7120 | -2.3627 | -2.0930 |
| ai | -1.7225 | -1.1272 | 1.2903 | 0.6309 | -1.8039 | -1.5503 |
| ave | -2.5786 | -2.0236 | 1.0017 | 0.7536 | -2.3094 | -2.0756 |

# Technology Adoptions

In order to promote the CTDP formats, we have signed two Memorandum of Understandings (MOUs) with Chunghwa Telecom (CHT) Company and Hsin Yeong An (HYA) Cable TV Company, while the former with about 1.4 million customers is a national-wise company in Taiwan and the later with about 145 thousand customers is a local cable company near the National Cheng Kung University.

For CHT Company, the test and adoption plan with time lines are designed as the following:

1. 08/01/2016: Propose and confirm the test and adoption detailed plan

2. 11/01/2016: Test of downloaded 3D CTDP services with computer clients

(Internal test in CHT and TOUCH Center only)

3. 06/01/2017: Services of 3D MOD services for computer clients

(Charge and open to the public and final CTDP format selection)

4. 06/01/2018: Test of 3D MOD services through Set-top-Boxes with separated CTDP depacking module (Open for selected users)

5. 06/01/2019：Test of 3D MOD services through Set-top-Boxes with Integrated CTDP depacking module (charged 3D services)

For HYA Cable TV Company, the signed agreement in English is shown in Fig. 3.1(b). The test and adoption plan with time lines are designed as the following:

1. 08/01/2016: Propose and confirm the test and adoption detailed plan

2. 11/01/2016: Test of downloaded 3D CTDP services with computer clients

(Internal test in HYA and TOUCH Center only)

3. 06/01/2017: Services of 3D CATV services for computer clients

(Charge and open to the public and final CTDP format selection)

4. 06/01/2018: Test of 3D CATV services through Set-top-Boxes with separated CTDP depacking module (Open for selected users)

5. 06/01/2019：Test of 3D CATV services through Set-top-Boxes with Integrated CTDP depacking module (charged 3D services)

# Conclusions

This contribution proposes modified CTDP SEI Message which is an update of the prior contribution JCT3V-N0022 by revising depth related syntax. This contribution removes depth_chroma_sampling_type and depth_subpixel_arrangement_type and adds two_views_ texture_packing_type. By replacing regular YCbCr representation of color depth [1], this contribution proposes an emulated YCbCr representation based on 4-pixel arrangements, where four Y components are directly range-mapped to 4 selected RGB subpixels and eight Cb and Cr components are range-mapped to the residuals of selected RGB subpixels plus their corresponding DPCM predictions. The newly-added syntax, two_views_ texture_packing_type allows the regular two-view texture packing methods, side-by-side (SbS) and top-and-bottom (TaB) to be replaced by checkerboard side-by-side (C-SbS) and checkerboard top-and-bottom (C-TaB) [2] for two-view texture packing used in CTDP-2TB and CTDP-2LR types, respectively. After simulations of uncoded and HEVC-coded CTDP formats, the results show that both emulated YCbCr chroma representation for packing the color depth and checkerboard SbS (TaB) packing method for the two-view texture can greatly improve the CTDP packing performances.

# References

[1] J.-F. Yang, H.-M. Wang, and Chiu-Yu Chen, Centralized Texture-Depth Packing SEI Message for HEVC, Document, no. JCT3V-N0022, Feb. 2016.

[2] A.-T. Chiang, H.-M. Wang, J.-F. Yang, and J. F. Wang, “A New Stereo Packing Format Based on Checkerboard Sub-sampling for Efficient Stereo Video Coding,” Proc. of IEEE International Symposium on Circuits and Systems, Seoul, May 2012.

[3] J.-F. Yang, H.-M. Wang, and Chiu-Yu Chen, Centralized Texture-Depth Packing SEI Message for HEVC, Document, no. JCT3V-M0022, Oct. 2015.

[4] J.-F. Yang, H.-M. Wang, and Chiu-Yu Chen, Revised Centralized Texture-Depth Packing SEI Message and Its Performance Improvement by Depth Enhancement, Document, no. JCT3V-L0022, June 2015.

[5] J.-F. Yang, K.-Y. Liao. H.-M. Wang, and Y.-H. Hu, Centralized Texture-Depth Packing (CTDP) SEI Message, Document no. JCT3V-K0027, Feb. 2015.

[6]	J.-F. Yang, K.-Y. Liao. H.-M. Wang, and Y.-H. Hu, Centralized Texture-Depth Packing (CTDP) SEI Message Syntax, Document no. JCT3V-J0108, Jan. 2013.

[7]	J.-F. Yang, K.-Y. Liao. H.-M. Wang, and Y.-H. Hu, Additional Definitions of FPA SEI Message for Inclusion of Centralized Color-Depth Packing (CCDP) Formats, Document no. JCTVC-S0031, Jan. 2013.

[8] Recommendation ITU-T, H.265, High Efficiency Video Coding, April 2015

# Patent rights declaration(s)

National Cheng Kung University has US and EU pending patents, “METHOD, DEVICE AND SYSTEM FOR RESIZING ORIGINAL DEPTH FRAME INTO RESIZED DEPTH FRAME”, “METHOD, DEVICE AND SYSTEM FOR RESTORING RESIZED DEPTH FRAME INTO ORIGINAL DEPTH FRAME”, “METHOD, DEVICE AND SYSTEM FOR PACKING COLOR FRAME AND ORIGINAL DEPTH FRAME” and “UNPACKING METHOD, DEVICE AND SYSTEM OF PACKED FRAME” relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)..