| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 27–31 May 2016 | Document: JCT3V-O1004 |
| --- | --- |

| Title: | Conceptual study of potential centralized texture depth packing SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Output document of JCT-3V – Revision of JCT3V-O0022 |  |  |
| Purpose: | Output document |  |  |
| Author(s) or Contact(s): | Jar-Ferr Yang (NCKU, Taiwan) Jens-Rainer Ohm (RWTH Aachen Univ.) | Tel: Email: | +886-6-2757575#62362 jefyang@mail.ncku.edu.tw ohm@ient.rwth-aachen.de |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document was developed based on contribution JCT3V-O0022. Its purpose is to reflect possible improvements that had been discussed related to the proposed text during the 15th JCT-3V meeting, in the event that the parent bodies might later decide to include a “Centralized Texture Depth Packing SEI Message” into their standards. The purpose of such an SEI message would be to deliver one texture view plus one depth map, or two texture views plus two depth maps, packed into the samples of a monoscopic (texture only) stream in the proposed manner.

# Centralized texture depth packing SEI message syntax

- Replace specification of general_non_packed_constraint_flag semantics in clause 7.4.4 with the following:

general_non_packed_constraint_flag equal to 1 specifies that there are neither frame packing arrangement SEI messages nor segmented rectangular frame packing arrangement SEI messages nor centralized texture depth packing SEI messages present in the CVS. general_non_packed_constraint_flag equal to 0 indicates that there may or may not be one or more frame packing arrangement SEI messages, segmented rectangular frame packing arrangement SEI messages or centralized texture depth packing SEI messages present in the CVS.

NOTE 2 – Decoders may ignore the value of general_non_packed_constraint_flag, as there are no decoding process requirements associated with the presence or interpretation of frame packing arrangement SEI messages, segmented rectangular frame packing arrangement SEI messages or centralized texture depth packing SEI messages.

In subclause D.2.1 (General SEI message syntax), add the following rows at the position in the table corresponding to the payloadType number X:

| else if( payloadType == X ) | Descriptor |
| --- | --- |
| centralized_texture_depth_packing( payloadSize ) /* specified in Annex D */ |  |

where “X” is an appropriate payloadType number in the sequence of defined payloadType numbers of the respective specification.

Add a new subclause D.2.X “centralized texture depth packing SEI message syntax”, where. X will be replaced with an appropriate subclause number in the sequence of subclauses present in the text of the respective standard.

D.2.X	Centralized texture depth packing (CTDP) SEI message syntax

| centralized_texture_depth_packing( payloadSize ) { | Descriptor |
| --- | --- |
| centralized_texture_depth_packing_cancel_flag | u(1) |
| if( ! centralized_texture_depth_packing_cancel_flag ) { |  |
| centralized_texture_depth_packing_type | u(3) |
| quarter_depth_size | u(12) |
| depth_spatial_flipping_flag | u(1) |
| two_views_texture_packing_type | u(2) |
| baseline_dist_flag | u(1) |
| focal_length_flag | u(1) |
| z_near_flag | u(1) |
| z_far_flag | u(1) |
| d_min_flag | u(1) |
| d_max_flag | u(1) |
| depth_representation_type | ue(v) |
| if( baseline_dist_flag ) |  |
| view_syn_rep_info_element( BdistanceSign, BdistanceExp, BdistanceMantissa, BdistanceManLen ) |  |
| if( focal_length_sample_flag ) |  |
| view_syn_rep_info_element( FlengthSign, FlengthExp, FlengthMantissa, FlengthManLen ) |  |
| if( z_near_flag ) |  |
| view_syn_rep_info_element( ZNearSign, ZNearExp, ZNearMantissa, ZNearManLen ) |  |
| if( z_far_flag ) |  |
| view_syn_rep_info_element ( ZFarSign, ZFarExp, ZFarMantissa, ZFarManLen ) |  |
| if( d_min_flag ) |  |
| view_syn_rep_info_element ( DMinSign, DMinExp, DMinMantissa, DMinManLen ) |  |
| if( d_max_flag ) |  |
| view_syn_rep_info_element ( DMaxSign, DMaxExp, DMaxMantissa, DMaxManLen ) |  |
| if( depth_representation_type = = 3 ) { |  |
| depth_nonlinear_representation_num_minus1 | ue(v) |
| for( i = 1; i <= depth_nonlinear_representation_num_minus1 + 1; i++ ) |  |
| depth_nonlinear_representation_model[ i ] |  |
| centralized_texture_depth_packing_persistence_flag | u(1) |
| } |  |
| } |  |
| } |  |

In Table F.4, add the persistence scope for the centralized texture depth packing SEI message as follows:

| - SEI message | - Persistence scope |
| --- | --- |
| centralized texture depth packing | - Specified by the syntax of the SEI message |

Add a new subclause D.3.X “centralized texture depth packing SEI message semantics” where X will be replaced with an appropriate subclause number in the sequence of subclauses present in the text. In the following subclause D.3.X, the numbers of tables, figures, and equations started from Table D-X1, Figure D-X1, D-X1 will be also replaced with appropriate numbers in the sequence of tables, figures, and equations present in the text, respectively.

- D.3.X Centralized texture depth packing SEI message semantics

This SEI message informs the decoder that the output cropped decoded picture contains a specific packing arrangement of texture and depth samples, associated with one or two views. This information can be used by a process applied after decoding to appropriately interpret the samples and process them appropriately for display of 3D video or other purposes (which are outside the scope of this Specification).

When general_non_packed_constraint_flag is equal to 1 in the active SPS for the current layer, there shall be no centralized texture depth packing SEI message applicable for any picture of the CLVS of the current layer.

When a frame packing arrangement SEI message or a segmented rectangular frame packing arrangement SEI message is applicable for any picture of the CLVS of the current layer, there shall be no centralized texture depth packing SEI messages applicable for any picture of the CLVS of the current layer.

centralized_texture_depth_packing_cancel_flag equal to 1 indicates that the centralized texture_depth packing (CTDP) arrangement SEI message cancels the persistence of any previous centralized texture depth packing SEI message in output order. centralized_texture_depth_packing_cancel_flag equal to 0 indicates that centralized texture depth packing is applicable.

centralized_texture_depth_packing_type indicates the intended interpretation of the constituent frames as specified in Table D-X1.

Table D-X1 – Definition of centralized_texture_depth_packing _type

| Value | Interpretation |
| --- | --- |
| 0 | Indicates the CTDP-1TB type with an arrangement of subsample depth top (SDT), subsample texture (ST), and subsample depth bottom (SDB) regions from top to bottom, as illustrated in Figure D-X1. |
| 1 | Indicates the CTDP-1LR type with an arrangement of subsample depth left (SDL), subsample texture (ST), and_subsample depth right (SDR) regions from left to right, as illustrated in Figure D-X2. |
| 2 | Indicates the CTDP-2TB type with an arrangement of_subsample depth leftview top (SDLT), subsample texture leftview (STL), and_subsample depth leftview bottom (SDLB) regions in the left-half picture and subsample depth rightview top (SDRT), subsample texture rightview (STR), and subsample depth rightview bottom (SDRB) regions in the right-half picture from top to bottom, as illustrated in Figure D-X3. |
| 3 | Indicates the CTDP-2LR type that the CTDP packed frame is arranged in the order of subsample depth top view left (SDTL), subsample texture top view (STT), and_subsample depth top view right (SDTR) regions in the top-half picture and_subsample depth bottom view left (SDBL), subsample texture bottom view (STB), and subsample depth bottom view right (SDBR) regions in the bottom-half picture from left to right, as illustrated in Figure D-X4. |

Values of centralized_texture_depth_packing_type that do not appear in Table D-X1 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore centralized texture depth packing SEI messages that contain reserved values of centralized_texture_depth_packing type. Subsequently, centralized_texture_depth_packing type is called as packing type.

quarter_depth_size multiplied by four indicates the number of lines carrying the vertically subsampled depth map if centralized_texture_depth_packing_type is equal to 0 or 2. It indicates the number of columns carrying the horizontally subsampled depth map if centralized_texture_depth_packing_type is equal to 1 or 3. The quarter_depth_size specifies the arrangements of subsample texture and subsample depth information. When centralized_texture_depth_packing_type is not equal to 0, 1, 2, or 3, quarter_depth_size is ignored and reserved for future use by ITU-T | ISO/IEC. The detailed interpretation how the subsampled texture positions relate to the original full resolution positions of the texture frame is addressed and the detailed interpretation how the subsampled depth positions relate to the original full resolution positions of the depth map is given as follows.

Let W and H be the width and height expressing pic_width_in_luma_samples and pic_height_in_luma_samples, respectively, of the decoded picture output from the decoder. It is assumed that the original texture and depth pictures are both of size WxH.

In case of packing type equal 0 or 2, the packed frame from top to bottom is horizontally divided into top subsample depth, subsample texture, and bottom subsample depth regions. The height of top and bottom subsample depth regions in units of frame luma samples is given as

HHD = 2 * quarter_depth_size,								(D-X1)

while the height of subsample texture regions in units of frame luma samples becomes:

HST = H − 4 * quarter_depth_size		(D-X2)

When packing type is equal to 0 as illustrated in Figure D-X1, the packed frame with HHD and HST is divided into SDT, ST and SDB regions. The width of SDT, ST and SDB regions is W in units of frame luma samples. When packing type is equal to 2, as illustrated in Figure D-X3, they are further vertically divided into two left and right half regions to obtain top subsample depth leftview and top subsample depth rightview, subsample texture leftview and subsample texture rightview, bottom subsample depth leftview and bottom subsample depth rightview respectively identified as SDLT and SDRT, STL and STR, and SDLB, and SDRB, whose width becomes W/2 in units of frame luma samples.

In case of packing type equal 1 or 3, the packed frame from left to right is vertically divided into left subsample depth, subsample texture, and right subsample depth regions. The width of top and bottom subsample depth regions in units of frame luma samples is given as

WHD = 2 * quarter_depth_size,								(D-X3)

while the width of subsample texture regions in units of frame luma samples becomes:

WST = W − 4 * quarter_depth_size.		(D-X4)

When packing type is equal to 1, as illustrated in Figure D-X2, the packed frame with WHD and WST is divided into SDL, ST, and SDR regions. The height of SDL, ST and SDR regions is H in units of frame luma samples. When packing type is equal to 3, as illustrated in Figure D-X4, these three regions are further horizontally divided into top and bottom half regions to obtain left subsample depth top view and left subsample depth bottom view, subsample texture top view and subsample texture bottom view, right subsample depth top view and right subsample depth bottom view respectively identified as SDTL, SDBL, STT, STB, SDTR and SDBR regions, whose height becomes H/2 in units of frame luma samples.

Figure D-X1 Arrangement of the CTDP-1TB type and its depacking steps (packing type = 0)

Figure D-X2 Arrangement of the CTDP-1LR type and its depacking steps (packing type = 1)

Figure D-X3 Arrangement of the CTDP-2TB type and its depacking steps (packing type = 2)

Figure D-X4 Arrangement of the CTDP-2LR type and its depacking steps (packing type = 3)

depth_spatial_flipping_flag indicates that the sample positions in the subsampled depth regions are spatially flipped relative to the original positions.

When depth_spatial_flipping_flag is equal to 1, all the subsample depth regions are spatially flipped as follows:

–	If packing type is equal to 0 or 2, the indicated depth spatial flipping is performed vertically. As shown in Figures D-X1 and D-X3, the subsample depth region, which is SDT, SDB, SDLT, SDRT, SDLB, or SDRB in YCbCr representation is vertically flipped. If the subsample depth region is denoted as SDXs, the subsample depth spatial vertical flipping process is given as:

for( x = 0; x < Wd; x++ )

for( y = 0; y < quarter_depth_size; y++ ) {

k = SDXs[ x ][ y ]

SDXs[ x][ y ] = SDXs[ x ] [HHD − y − 1 ]

SDXs[ x ][HHD − y − 1 ] = k

}

where quarter_depth_size is the half of the height of the subsample depth regions in units of frame luma samples. If SDXs is SDT or SDB region, with Wd= W in units of frame luma samples, the flipped result, SDXs is still named SDT or SDB region, respectively. If SDXs is SDLT, SDRT, SDLB, or SDRB region, with Wd= W/2 in units of frame luma samples, the flipped result is still named as SDLT, SDRT, SDLB, or SDRB region, respectively,

–	If packing type is equal to 1 or 3, the indicated depth spatial flipping is performed horizontally. As shown in Figures D-X2 and D-X4, the subsample depth region, which is SDL, SDR, SDTL, SDBL, SDTR, or SDBR in YCbCr representation is horizontally flipped. If the subsample depth region is denoted as SDXs, the subsample depth spatial horizontal flipping process is given as:

for( x = 0; x < quarter_depth_size; x++ )

for( y = 0; y < Hd; y++ ) {

temp= SDXs[x][y]

SDXs[x][y] =SDXs[WHD−x−1][y]

SDXs[WHD−x−1][y] = temp

}

where quarter_depth_size is the half of the width of the subsample depth regions in units of frame luma samples. If SDXs is SDL or SDR region, with Hd = H in units of frame luma samples, the flipped result is still named as SDL or SDR region, respectively. If SDXs is SDTL, SDBL, SDTR, or SDBR region, with Hd =H/2 in units of frame luma samples, the result is still named as SDTL, SDBL, SDTR, or SDBR region, respectively.

–	Otherwise (depth_spatial_flipping_flag is equal to 0), the indicated spatial flipping is not necessary.

When centralized_texture_depth_packing_type is not equal to 0, 1, 2, or 3, it is a requirement of bitstream conformance that depth_spatial_flipping_flag shall be equal to 0. When centralized_texture_depth_packing_type is not equal to 0, 1, 2, or 3, decoders shall ignore the value 1 for depth_spatial_flipping_flag.

After spatial flipping (depth_spatial_flipping_flag equal to 1) or no spatial flipping (depth_spatial_flipping_flag equal to 0), a pair of corresponding subsample depth regions should be merged together to become the subsample depth region .

–	If packing type is equal to 0 or 2, as shown in Figures D-X1 and D-X3, the pair of top and bottom subsample depth regions, which are SDT and SDB, SDLT and SDLB, or SDRT and SDRB in YCbCr representation are vertically merged. If the pair of top and bottom subsample depth regions are respectively denoted as SDXT and SDXB, the subsample depth after merging process is obtained as:

for( x = 0; x < Wd; x++ )

for( y = 0; y < HHD; y++ ) {

SDX[x][y] = SDXT[x][ y]

SDX[x][HHD +y] = SDXB[x][ y]

}

where Wd and HHD are the width and the height of top and bottom subsample depth regions in units of frame luma samples, respectively. If packing type is equal to 0, Wd = W and SDXT and SDXB respectively denote SDT and SDB regions, the merged result SDX becomes subsample depth, SD as shown in Figure D-X1. If packing type is equal to 2, Wd = W/2 and SDXT and SDXB respectively denote SDLT and SDLB or SDRT and SDRB regions, the merged result SDX becomes subsample depth leftview, SDL or subsample depth rightview, SDR, respectively as shown in Figure D-X3. After vertical merging process, the height of the subsample depth regions, SD, SDT, and SDB becomes 2HHD, while the width of the subsample depth is unchanged.

–	If packing type is equal to 1 or 3, as shown in Figures D-X2 and D-X4, the pair of left and right subsample depth regions, which are SDL and SDR, SDTL and SDTR, or SDBL and SDBR in YCbCr representation are horizontally merged. If the pair of left and right subsample depth regions are respectively denoted as SDXL and SDXR, the subsample depth after merging process is obtained as:

for( x = 0; x < WHD; x++ )

for( y = 0; y < Hd; y++ ) {

SDX[x][y] =SDXL[x][y]

SDX[ x+WHD ][ y ] = SDXR[ x ][ y ]

}

where WHD and Hd are the width and the height of left and right subsample depth regions in units of frame luma samples, respectively. If packing type is equal to 1, Hd = H and SDXL and SDXR respectivel denote SDL and SDR regions, the merged result SDX become the subsample depth SD as shown in Figure D-X2. if packing type is equal to 3, Hd = H/2 and SDXL and SDXR respectively denote SDTL and SDTR or SDBL and SDBR regions, the merged result SDX becomes subsample depth top view, SDT or subsample depth bottom view, SDB, respectively as shown in Figure D-X4., After horizontal merging process, the width of the subsample depth regions, SD, SDT, and SDB becomes 2WHD, while the height of the subsample depth is unchanged.

After the merging step, Y, Cb, Cr components in three separate frames are virtually merged into one, which is converted to subsample depth with single-sample values. As shown in Figure D-X5, the subsample depth in YCbCr 4:4:4 representation is treated as the subsample depth in spatially-positioned YCbCr 4:4:4 as follows:

- If packing type is equal to 0 or 2, subsample depth, SD, SDL, or SDR region in YCbCr 4:4:4 representation is stacked together as spatial vertically-positioned YCbCr 4:4:4. The height of subsample depth, SD, SDL, or SDR is expanded 3 times to become 6HHD, while the width of the subsample depth is unchanged..

- If packing type is equal to 1 or 3, subsample depth, SD, SDT, or SDB in YCbCr 4:4:4 representation is stacked together as spatial horizontally-positioned YCbCr 4:4:4. The width of the subsample depth, SD, SDT, or SDB is expanded 3 times to become 6 * WHD, while the height of the subsample depth is unchanged.

Figure D-X5 Virtual interpretation of spatial positions in YCbCr 4:4:4 representation of subsample depth

For subsample depth regions, Y, Cb. Cr components are grouped in every four Y, Cb, Cr components, which are marked by red rectangular in Figure D-X5. The grouped Y, Cb, Cr components are spatially indexed by subscripts 1, 2, 3, and 4. Referring to video coding systems, the YCbCr representations of subsample depth are different from the original YCbCr representation. The detailed depacking procedures for converting YCbCr 4:4:4, YCbCr 4:2:2, and YCbCr 4:2:0 components to depth values of subsample depth are described as follows:

For YCbCr 4:4:4 representation of subsample depth, each group contains four Y, Cb, Cr components,{Yi, Cbi, Cri, for i = 1, 2, 3, 4}. With the same spatial position as shown in Figure D-X6, the 12 depth values, {Di1, Di2, Di3, for i = 1, 2, 3, 4} of subsample depth are computed in order as:

Di1 = (11644 * (Yi − 16) + 5000) / 10000	(D-X5)

Di2 = (22768 * (Cbi − 128) + 5000) / 10000 + Di1	(D-X6)

Di3 = (22768 * (Cbi − 128) + 5000) / 10000 + Di2	(D-X7)

where Di1 is linear mapping of Yi while Di2 and Di3 are linear mapping of Cbi and Cri, which are the residuals of DPCM predictions, Di1 and Di2, respectively. According to the packing type, each group of the corresponding 12 depth values with same spatial positions as the spatial-positioned YCbCr representation defined in Figure D-X5 are obtained as shown in Figure D-X6.

Figure D-X6 Four samples of YCbCr 4:4:4 components of subsample depth depacked to 12 depth values in: (a) vertical positioning for packing type 0 or 1; (b) horizontal positioning for packing type = 1 or 3.

If the video coding system adopts YCbCr 4:2:2 format, each group of four YCbCr 4:2:2 components contains 4 luma components, {Yi, for i = 1, 2, 3, 4} and 4 chroma components, {Cbi, Cri, for i = 1, 2}. Four luma components, Y1, Y2, Y3, Y4 are first converted to 4 selected depth samples, D11, D22, D32, and D43 as

D11 = (11644 * (Y1 − 16) + 5000) / 10000	(D-X8)

D22 = (11644 * (Y2 − 16) + 5000) / 10000	(D-X9)

D32 = (11644 * (Y3 − 16) + 5000) / 10000	(D-X10)

D43 = (11644 * (Y4 − 16) + 5000) / 10000	(D-X11)

- For packing type equal to 0 or 2, D13, D31, D41 and D23 are computed in order as:

D13 = (22768 * (Cb1 − 128) + 5000 * (D11 +D32) + 5000) / 10000	(D-X12)

D31 = (22768 * (Cr1 − 128) + 5000 * (D13 +D32) + 5000) / 10000	(D-X13)

D41 = (22768 * (Cr2 − 128) + 5000 * (D22 +D43) + 5000) / 10000	(D-X14)

D23 = (22768 * (Cb2 − 128) +5000 * (D22 +D41) + 5000) / 10000	(D-X15)

- For packing type equal to 1 or 3, D13, D21, D41 and D33 are computed in order as:

D13 = (22768 * (Cb1 − 128) + 5000 * (D11 +D22) + 5000) / 10000	(D-X16)

D21 = (22768 * (Cb2 − 128) + 5000 * (D13 +D22) + 5000) / 10000	(D-X17)

D41 = (22768 * (Cr2 − 128) + 5000 * (D32 +D43) + 5000) / 10000	(D-X18)

D33 = (22768 * (Cr1 − 128) + 5000 * (D32 +D41) + 5000) / 10000	(D-X19)

The above YCbCr 4:2:2 representation to depth conversion only recovers 8 depth samples arranged in partial quincunx fashions as shown in Figure D-X6. There are 4 missing depth samples (in black colour) should be upsampled by a proper interpolation method from the neighboring known depacked depth samples. The neighboring known depth samples might be extended to nearby groups of known depth samples for better interpolations.

Figure D-X6 Emulated four YCbCr 4:2:2 colour components depacking to 8 depth samples: (a) vertical colour depacking (packing type 0 or 1); (b) horizontal colour depacking (packing type = 1 or 3)

When the video coding system adopts YCbCr 4:2:0 format, each group of four YCbCr 4:2:0 components only contains 4 luma components, {Yi, for i = 1, 2, 3, 4} and two chroma components Cb1 and Cr1. Four luma components, Y1, Y2, Y3, Y4 are first converted to 4 selected depth samples, D11, D22, D32, and D43 by (D-X8), (D-X9), (D-X10), and (D-X11), respectively. The chroma components, Cb1 and Cr1 are successively transformed to 2 selected depth samples, D13 and D41 by the range mappers of residuals plus the compensation of DPCM predictions. The detailed formulations are addressed

- For packing type equal to 0 or 2, D13 and D41 are computed by (D-X12) and (D-X14), respectively.

- For packing type equal to 1 or 3, D13 and D41 are computed by (D-X16) and (D-X18), respectively.

The above YCbCr 4:2:0 representation to depth conversion only recovers 6 depth samples arranged in quincunx fashions as shown in Figure D-X5. There are 6 missing depth samples (in black colour) should be upsampled by a proper interpolation method from the neighboring known depacked depth samples. The neighboring known depth samples might be extended to nearby sets of known depth samples for better interpolation.

Figure D-X7 Grouped YCbCr 4:2:0 samples to 6 depth samples conversion: (a) vertical depacking for packing type 0 or 1; (b) horizontal depacking for packing type = 1 or 3

After the above conversion of YCbCr 4:2:0, YCbCr 4:2:2, or YCbCr 4:4:4 components to single-sample values for subsample depth regions, the height, 2HHD of subsample depth SD, SDL, and SDR regions for packing type = 0 or 2 is vertically magnified to 3 times to become 6HHD or the width, 2WHD of subsample depth SD, SDT, and SDB regions for packing type = 1 or 3 is horizontally magnified to 3 times to become 6WHD. To completely depack the CTDP frame, the final upsampling processes of the subsample depth and subsample texture regions are upsampled as as follows.

- For packing type =0, the subsample depth, SD with the size of WxHHD is vertically upsampled by a factor of H : 6 * HHD to obtain depth (D) region and the subsample texture, ST with the size of WxHST is vertically upsampled by a factor of H:HST to obtain texture (T) region both with the size of WxH in units of frame luma samples.

- For packing type =1, the subsample depth, SD with the size of WHDxH is horizontally upsampled by a factor of W : 6 WHD to obtain depth (D) region and the subsample texture, ST with the size of WSTxH is horizontally upsampled by a factor of W:WST to obtain texture (T) region both with the size of WxH in units of frame luma samples.

- For packing type =2, the subsample depth leftview and subsample depth rightview, SDL and SDR with the size of W/2xHHD are performed horizontal 2:1 upsampling and vertical H:6HHD upsampling to obtain depth leftview and depth rightview, DL and DR, respectively. The subsample texture leftview and subsample texture rightview, STL and STR with the size of W/2xHST are first performed horizontal 2:1 upsampling and then vertical H:HST upsampling to obtain texture leftview and texture rightview, TL and TR, respectively all with the size of WxH in units of frame luma samples. The horizontal 2:1 upsampling method will be further specified by two_views_texture_packing_type.

- For packing type =3, the subsample depth top view and subsample depth bottom view, SDT and SDB with the size of WHDxH/2 are performed vertical 2:1 upsampling and horizontal W:6WHD upsampling to obtain depth top view and depth bottom view, DT and DB, respectively. The subsample texture top view and subsample texture bottom view, STT and STB with the size of WSTxH/2 are first performed vertical 2:1 upsampling and then horizontal W:WST upsampling to obtain texture top view and texture bottom view, TT and TB, respectively all with the size of WxH in units of frame luma samples. The vertical 2:1 upsampling method will be further specified by two_views_texture_packing_type.

two_views_texture_packing_type indicates that the method packs two subsample texture views. For two_views_texture_packing_type equals to 0, the two texture views perform regular 2:1 downsampling and for two_views_texture_packing_type equals to 1, the two texture views perform line-offset 2:1 downsampling before packing.

Table D-X5 – Definition of two_views_texture_packing_type

| Value | Interpretation |
| --- | --- |
| 0 | CTDP depacker is informed that the regular 2:1 downsampling of two views is used for packing two subsample texture views together. |
| 1 | CTDP depacker is informed that the line-offset 2:1 downsampled of two views is used for packing two subsample texture views together. |

Values of two_views_texture_packing_type that do not appear in Table D-X5 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore centralized texture depth packing SEI messages that contain reserved values of texture two_view_packing_type.

When centralized_texture_depth_packing_type is equal to 0 or 1, two_views_texture_packing_type is ignored. When centralized_texture_depth_packing_type is equal to 2 or 3, two subsample texture views for performing 2:1 upsampling are described as follows:

- If centralized_texture_depth_packing_type is equal to 2, the subsample texture leftview (STL), subsample texture right view (STR) are 2:1 horizontally upsampled as shown in Figure D-X8. If two_views_texture_packing_type is equal to 0, the regular 2:1 horizontal upsampling procedure is performed. If two_views_texture_packing_type is equal to 1, the line-offset 2:1 horizontal upsampling procedure is performed. After a proper interpolation filter interpolates the missing samples (in gray colour), the texture leftview and texture rightview are vertically H:HST upsampled to the size of WxH in units of frame luma samples.

- Otherwise (centralized_texture_depth_packing_type is equal to 3), the subsample texture top view (STT), subsample texture bottom view (STB) are vertically 2:1 upsampled as shown in Figure D-X9. If two_views_texture_packing_type is equal to 0, the regular 2:1 vertical upsampling procedure is performed. If two_views_texture_packing_type is equal to 1, the line-offset 2:1 vertical upsampling procedure is performed. After a proper interpolation filter interpolates the missing samples (in black colour) and the texture top view and texture bottom view are vertically W:WST upsampled to the size of WxH in units of frame luma samples

Figure D-X8 Regular and line-offset 2:1 horizontal upsampling procedures (black=missing sample)

Figure D-X9 Regular and line-offset 2:1 vertical upsampling procedures (black =missing sample)

baseline_dist_flag equal to 0 specifies that the syntax elements specifying the baseline distance are not present in the syntax structure. baseline_dist_flag equal to 1 specifies that the syntax elements specifying the baseline distance value are present in the syntax structure.

focal_length_flag equal to 0 specifies that the syntax elements specifying the focal length are not present in the syntax structure. focal_length_sample_flag equal to 1 specifies that the syntax elements specifying the focal length value are present in the syntax structure.

z_near_flag equal to 0 specifies that the syntax elements specifying the nearest depth value are not present in the syntax structure. z_near_flag equal to 1 specifies that the syntax elements specifying the nearest depth value are present in the syntax structure.

z_far_flag equal to 0 specifies that the syntax elements specifying the farthest depth value are not present in the syntax structure. z_far_flag equal to 1 specifies that the syntax elements specifying the farthest depth value are present in the syntax structure.

d_min_flag equal to 0 specifies that the syntax elements specifying the minimum disparity value are not present in the syntax structure. d_min_flag equal to 1 specifies that the syntax elements specifying the minimum disparity value are present in the syntax structure.

d_max_flag equal to 0 specifies that the syntax elements specifying the maximum disparity value are not present in the syntax structure. d_max_flag equal to 1 specifies that the syntax elements specifying the maximum disparity value are present in the syntax structure.

depth_representation_type specifies the representation definition of decoded luma samples of auxiliary pictures as specified in Table D-X5. In Table D-X6, disparity specifies the horizontal displacement between two texture views and Z value specifies the distance from a camera.

The variable maxVal is set equal to ( 1 << ( 8 + bit_depth_luma_minus8 ) ) − 1, where bit_depth_luma_minus8 is the value included in or inferred for the active SPS of the layer with nuh_layer_id equal to targetLayerId.

Table D-X6 – Definition of depth_representation_type

| depth_representation_type | Interpretation |
| --- | --- |
| 0 | Each decoded luma sample value of an auxiliary picture represents an inverse of Z value that is uniformly quantized into the range of 0 to maxVal, inclusive. When z_far_flag is equal to 1, the luma sample value equal to 0 represents the inverse of ZFar (specified below). When z_near_flag is equal to 1, the luma sample value equal to maxVal represents the inverse of ZNear (specified below). |
| 1 | Each decoded luma sample value of an auxiliary picture represents disparity that is uniformly quantized into the range of 0 to maxVal, inclusive. When d_min_flag is equal to 1, the luma sample value equal to 0 represents DMin (specified below). When d_max_flag is equal to 1, the luma sample value equal to maxVal represents DMax (specified below). |
| 2 | Each decoded luma sample value of an auxiliary picture represents a Z value uniformly quantized into the range of 0 to maxVal, inclusive. When z_far_flag is equal to 1, the luma sample value equal to 0 corresponds to ZFar (specified below). When z_near_flag is equal to 1, the luma sample value equal to maxVal represents ZNear (specified below). |
| 3 | Each decoded luma sample value of an auxiliary picture represents a nonlinearly mapped disparity, normalized in range from 0 to maxVal, as specified by depth_nonlinear_representation_num_minus1 and depth_nonlinear_representation_model[i]. When d_min_flag is equal to 1, the luma sample value equal to 0 represents DMin (specified below). When d_max_flag is equal to 1, the luma sample value equal to maxVal represents DMax (specified below). |
| Other values | Reserved for future use |

The variables in the x column of Table D-X7 are derived from the respective variables in the s, e, n and v columns of Table D-X7 as follows:

- If the value of e is in the range of 0 to 127, exclusive, x is set equal to ( −1 )s* 2e − 31 * ( 1 + n ÷ 2v ).

- Otherwise (e is equal to 0), x is set equal to ( −1 )s * 2−( 30 + v ) * n.

NOTE 1 – The above specification is similar to that found in IEC 60559:1989, Binary floating-point arithmetic for microprocessor systems.

Table D-X7 – Association between depth parameter variables and syntax elements

| x | s | e | n | v |
| --- | --- | --- | --- | --- |
| Bdistance | BdistanceSign | BdistanceExp | BdistanceMantissa | BdistanceManLen |
| Flength | FlengthSign | FlengthExp | FlengthMantissa | FlengthManLen |
| ZNear | ZNearSign | ZNearExp | ZNearMantissa | ZNearManLen |
| ZFar | ZFarSign | ZFarExp | ZFarMantissa | ZFarManLen |
| DMax | DMaxSign | DMaxExp | DMaxMantissa | DMaxManLen |
| DMin | DMinSign | DMinExp | DMinMantissa | DMinManLen |

The DMin and DMax values, when present, are specified in units of a luma sample width of the coded picture.

The units for the ZNear and ZFar, when present, are identical but unspecified.

When depth_representation_type is equal to 0 or 2, the disparity D can further be obtained from the value of Z, Bdistance, and Flength.

D = Bdistance * Flength ÷ Z								(D-X20)

depth_nonlinear_representation_num_minus1 plus 2 specifies the number of piecewise linear segments for mapping of depth values to a scale that is uniformly quantized in terms of disparity.

depth_nonlinear_representation_model[ i ] for i ranging from 0 to depth_nonlinear_representation_num_minus1 + 2, inclusive, specify the piecewise linear segments for mapping of decoded luma sample values of an auxiliary picture to a scale that is uniformly quantized in terms of disparity. The values of depth_nonlinear_representation_model[ 0 ] and depth_nonlinear_representation_model[ depth_nonlinear_‌representation_num_minus1 + 2 ] are both inferred to be equal to 0.

NOTE 2 – When depth_representation_type is equal to 3, an auxiliary picture contains nonlinearly transformed depth samples. The variable DepthLUT[ i ], as specified below, is used to transform decoded depth sample values from the nonlinear representation to the linear representation, i.e., uniformly quantized disparity values. The shape of this transform is defined by means of line-segment approximation in two-dimensional linear-disparity-to-nonlinear-disparity space. The first(0, 0) and the last ( maxVal, maxVal ) nodes of the curve are predefined. Positions of additional nodes are transmitted in form of deviations (depth_nonlinear_representation_model[ i ]) from the straight-line curve. These deviations are uniformly distributed along the whole range of 0 to maxVal, inclusive, with spacing depending on the value of nonlinear_depth_representation_num_minus1.

The variable DepthLUT[ i ] for i in the range of 0 to maxVal, inclusive, is specified as follows:

for( k = 0; k <= depth_nonlinear_representation_num_minus1 + 1; k++ ) {

pos1 = ( maxVal * k ) / (depth_nonlinear_representation_num_minus1 + 2 )

dev1 = depth_nonlinear_representation_model[ k ]

pos2 = ( maxVal * ( k + 1 ) ) / (depth_nonlinear_representation_num_minus1 + 2 )

dev2 = depth_nonlinear_representation_model[ k + 1 ] (G-10)

x1 = pos1 − dev1

y1 = pos1 + dev1

x2 = pos2 − dev2

y2 = pos2 + dev2

for( x = Max( x1, 0 ); x <= Min( x2, maxVal ); x++ )

DepthLUT[ x ] = Clip3( 0, maxVal, Round( ( ( x − x1 ) * ( y2 − y1 ) ) ÷ ( x2 − x1 ) + y1 ) )

}

When depth_representation_type is equal to 3, DepthLUT[ dS ] for all decoded luma sample values dS of an auxiliary picture in the range of 0 to maxVal, inclusive, represents disparity that is uniformly quantized into the range of 0 to maxVal, inclusive.

centralized_texture_depth_packing_persistence_flag specifies the persistence of the centralized texture depth packing arrangement SEI message.

centralized_texture_depth_packing_persistence_flag equal to 0 specifies that the centralized texture depth packing arrangement SEI message applies to the current decoded frame only.

centralized_texture_depth_packing_persistence_flag equal to 1 specifies that the centralized texture depth packing arrangement SEI message persists in output order until any of the following conditions are true:

–	A new CVS begins;

–	The bitstream ends;

–	A frame in an access unit containing a centralized texture depth packing SEI message with the same.

- D.4.X	View synthesis representation information element syntax

| view_syn_rep_info_element( OutSign, OutExp, OutMantissa, OutManLen ) { | Descriptor |
| --- | --- |
| da_sign_flag | u(1) |
| da_exponent | u(7) |
| da_mantissa_len_minus1 | u(5) |
| da_mantissa | u(v) |
| } |  |

The syntax structure specifies the value of an element in the view synthesis representation information SEI message.

The syntax structure sets the values of the OutSign, OutExp, OutMantissa and OutManLen variables that represent a floating-point value. When the syntax structure is included in another syntax structure, the variable names OutSign, OutExp, OutMantissa and OutManLen are to be interpreted as being replaced by the variable names used when the syntax structure is included.

da_sign_flag equal to 0 indicates that the sign of the floating-point value is positive. da_sign_flag equal to 1 indicates that the sign is negative. The variable OutSign is set equal to da_sign_flag.

da_exponent specifies the exponent of the floating-point value. The value of da_exponent shall be in the range of 0 to 27 − 2, inclusive. The value 27 − 1 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 27 − 1 as indicating an unspecified value. The variable OutExp is set equal to da_exponent.

da_mantissa_len_minus1 plus 1 specifies the number of bits in the da_mantissa syntax element. The value of da_mantissa_len_minus1 shall be in the range of 0 to 31, inclusive. The variable OutManLen is set equal to da_mantissa_len_minus1 + 1.

da_mantissa specifies the mantissa of the floating-point value. The variable OutMantissa is set equal to da_mantissa.

# References

[1] J.-F. Yang, Chiu-Yu Chen, Wei-Jong Yang, and H.-M. Wang, “Centralized Texture Depth Packing SEI Message for HEVC”, Document JCT3V-O0022, May 2016.